3GPP TSG RAN Meeting #109			RP- 252330

Beijing, China, September 15th – 18th, 2025

Source:	Sharp

Title:	NTN deployment scenarios

Agenda Item:	8.2.1.1

Document for:	Discussion

# NTN deployment scenarios

For NTN deployment scenarios for 6G, parameters in TR38.811 [1] can be mostly reused. In addition, parameters for Ku-band introduced in Rel-19 should also be included in 6G deployment scenarios. Table 1 shows deployment scenarios for NTN.

Table 1: Example deployment scenarios for NTN

| Carrier Frequency | Around 2 GHz (S-band) Around 11 GHz for DL and 14 GHz for UL (Ku-band) Around 20 GHz for DL and 30 GHz for UL (Ka-band) |
| --- | --- |
| Duplex | HD-FDD, FDD, [TDD] |
| Aggregated system Bandwidth | Up to [40] MHz for S-band Up to [800] MHz for Ku- and Ka-band |
| Payload type | Transparent Regenerative |
| Altitude (Satellite orbit) | 600 km (LEO) 8000 km (MEO) 35786 km (GEO) |
| UE Distribution | 100% Outdoor |
| UE speed | 3, 30, 60, 120, 500, 1000, 1500 km/h |

For frequencies higher than 10 GHz (e.g., Ku-band and Ka-band), 60 cm equivalent aperture diameter is specified in TR38.811 for VSAT terminals. On the other hand, mobile VSAT terminals for both NGSO and GSO systems in the Ku- and Ka-bands, with small form factors (up to 20 × 20 cm), are in demand from the automotive industry [2]. These compact terminals are intended for installation on, e.g., vehicle rooftops. At the last RAN plenary meeting in June, it was agreed in principle that small form factors of up to 20 × 20 cm would be included in the RAN4 package for Rel-20 5G Advanced [3]. Therefore, we propose adding the following parameters related to VSAT form factor in the deployment scenario for NTN.

Table 2: Terminal type information for NTN deployment scenarios

| NTN terminal type | Handheld, mobile/fixed VSATs |
| --- | --- |
| VSAT form factor | 20 x 20 cm, 60 cm equivalent aperture diameter |

# Conclusion

Proposal: Consider the parameters in Table 1 and Table 2 for 6G deployment scenarios for NTN.

Table 1: Example deployment scenarios for NTN

| Carrier Frequency | Around 2 GHz (S-band) Around 11 GHz for DL and 14 GHz for UL (Ku-band) Around 20 GHz for DL and 30 GHz for UL (Ka-band) |
| --- | --- |
| Duplex | HD-FDD, FDD, [TDD] |
| Aggregated system Bandwidth | Up to [40] MHz for S-band Up to [800] MHz for Ku- and Ka-band |
| Payload type | Transparent Regenerative |
| Altitude (Satellite orbit) | 600 km (LEO) 8000 km (MEO) 35786 km (GEO) |
| UE Distribution | 100% Outdoor |
| UE speed | 3, 30, 60, 120, 500, 1000, 1500 km/h |

Table 2: Terminal type information for NTN deployment scenarios

| NTN terminal type | Handheld, mobile/fixed VSATs |
| --- | --- |
| VSAT form factor | 20 x 20 cm, 60 cm equivalent aperture diameter |

# References

[1] 3GPP TR38.811V15.4.0, Study on New Radio (NR) to support non-terrestrial networks (Release 15)

[2] RP-250856, LS on automotive priorities in 5G-Advanced NR-NTN Rel-20, 5GAA

[3] RP-251879, Way forward on RAN4 R20 package