3GPP TSG RAN Meeting #109	RP-252358

Beijing, China, Sep 15th - Sep 18th, 2025

Agenda item:	9.3.2.1

Source:	Qualcomm Incorporated

Title:	Updates to the WID for Rel-20 AI/ML for mobility in NR

Document for:	Discussion and decision

# Introduction

In the last RAN plenary meeting #108 [1], a nearly complete version of the WID document [2] for Rel-20 AI/ML for mobility in NR was agreed, with a few aspects left to be addressed based on the outcome of the last RAN2 meeting (RAN2 #131 [3]) on the Rel-19 AI/ML for mobility SI.

In the last RAN2 meeting, RAN2 #131 [3], agreements were made regarding the scenarios and cases that are supported for a network-side model for RRM measurement prediction.

RAN2 #131 agreement:

For network sided models for inference:

- For cell-level temporal domain Case A, sub-case 2 the following enhancement is considered UE can report “cell-level RRM measurement results at multiple time instances in one measurement report.

- For other cases there is no specification impact.   Can be discussed in WI phase whether any additional enhancements are needed and justified (i.e. multi-instances reporting of beam)

3	Study item can conclude that all scenarios and sub-cases are feasible.

In summary, it was agreed that, for a network-side model:

- All scenarios and sub-cases discussed in the SI are feasible to be supported.

- Other than the case of temporal domain prediction Case A, RRM sub-case 2, for the other cases there is no specification impact, and it is left up to the WI phase to discuss and determine any additional specification impact.

Proposal 1: The text of the WID for Rel-20 AI/ML for mobility in NR is updated to align with the following RAN2 #131 agreements on network-side model for RRM measurement prediction.

RAN2 #131 agreement:

For network sided models for inference:

- For cell-level temporal domain Case A, sub-case 2 the following enhancement is considered UE can report “cell-level RRM measurement results at multiple time instances in one measurement report.

- For other cases there is no specification impact.   Can be discussed in WI phase whether any additional enhancements are needed and justified (i.e. multi-instances reporting of beam)

3	Study item can conclude that all scenarios and sub-cases are feasible.

For L3 beam level prediction, the following were the RAN2 #131 agreements.

RAN2 #131 Agreements on L3 beam-level prediction

For RRM measurement prediction, L3 beam-level prediction is feasible for network sided model.  CB after spec impact/overhead etc on the different cases.

For RRM measurement prediction, L3 beam-level prediction is feasible for UE sided model.  However, there are concerns on complexity and other WGs.

An important use of L3 beam level prediction is that if it can be performed for a target cell the prediction results can be used by a target gNB to configure CFRA resources on a beam for a UE to access the target cell upon handover. This can reduce the handover latency considerably.

Though the RAN2 #131 agreement mentions complexity issues for a UE-side model, we think it should not be much more complex than L1 beam prediction, which has been extensively studied and specified in Rel-18/Rel-19 AI/ML for beam management. The input for L3 beam prediction for a cell can be L1 beam measurements or L3 beam measurements, which can be obtained from L1 beam measurements by processing them using configuration provided by RRC.

Observation 1: L3 beam level prediction results of a target cell can be used by a target gNB to configure CFRA resources on a beam for the UE to access the target cell upon handover, which can reduce handover latency considerably.

Observation 2: For a UE-side model, L3 beam prediction for a cell should not be much more complex than L1 beam prediction, which has been specified in Rel-19 AI/ML for beam management.

Proposal 2: The text of the WID for Rel-20 AI/ML for mobility in NR is updated to support L3 beam prediction for a UE-side model.

For measurement event prediction, it was agreed by RAN2 to aim for a single framework for direct and indirect event prediction.

- RAN2 #131 Agreements

Aim for a single framework for event prediction and performance monitoring.  From RAN2 point of view indirect event prediction, RSRP differences can be used as the performance metric for monitoring.  Capture in the TR that there is no consensus on the feasibility of performance monitoring of the direct event prediction and this would need to be resolved in WI phase before proceeding with specifying direct event prediction.

Proposal 3: The text of the WID for Rel-20 AI/ML for mobility in NR is updated to indicate that RAN2 aims for a single framework for direct and indirect event prediction, and for performance monitoring.

For both RRM measurement prediction and measurement even	t prediction, companies studied in their simulations the cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) and found that the cluster approach can improve performance. Both these approaches are valid approaches, and it is up to UE implementation to determine which one to use for a particular use case. We therefore propose to add to the WID text that both these approaches can be used for a UE-side model.

Proposal 4: The text of the WID for Rel-20 AI/ML for mobility in NR is updated to add the fact that the cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) can be used for a UE-side model.

In the Annex, we provide a draft of the updates to the text of the current WID document based on the above proposals.

# 3 Conclusion

Based on the above discussion, we recommend that RAN plenary discuss the following observations and proposals.

Proposal 1: The text of the WID for Rel-20 AI/ML for mobility in NR is updated to align with the following RAN2 #131 agreements on network-side model for RRM measurement prediction.

RAN2 #131 agreement:

For network sided models for inference:

- For cell-level temporal domain Case A, sub-case 2 the following enhancement is considered UE can report “cell-level RRM measurement results at multiple time instances in one measurement report.

- For other cases there is no specification impact.   Can be discussed in WI phase whether any additional enhancements are needed and justified (i.e. multi-instances reporting of beam)

3	Study item can conclude that all scenarios and sub-cases are feasible.

Observation 1: L3 beam level prediction results of a target cell can be used by a target gNB to configure CFRA resources on a beam for the UE to access the target cell upon handover, which can reduce handover latency considerably.

Observation 2: For a UE-side model, L3 beam prediction for a cell should not be much more complex than L1 beam prediction, which has been specified in Rel-19 AI/ML for beam management.

Proposal 2: The text of the WID for Rel-20 AI/ML for mobility in NR is updated to support L3 beam prediction for a UE-side model.

Proposal 3: The text of the WID for Rel-20 AI/ML for mobility in NR is updated to indicate that RAN2 aims for a single framework for direct and indirect event prediction, and for performance monitoring.

Proposal 4: The text of the WID for Rel-20 AI/ML for mobility in NR is updated to add the fact that the cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) can be used for a UE-side model.

In the Annex, we provide a draft of the updates to the text of the current WID document based on the above proposals.

# References

[1] Chair notes, RAN plenary #108 meeting.

[2] RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, CATT.

[3] Chair notes, RAN2 #131 meeting.

# Annex: Updates to the text of the WID document on Rel-20 AI/ML for mobility

### 4.1	Objective of SI or Core part WI or Testing part WI

The objectives of this work item focuses on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, RRM measurement configuration and report framework in RRC layer are taken as baseline.

The objectives are as the following:

- Specify support for RRM measurement prediction for both UE and network sided models [RAN2]:

- Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction

- Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction

- At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level prediction is supported. For a UE-side model, L3 beam level prediction is supported.

Note 1: For UE sided model, spatial domain prediction is not supported. For a network-side model, spatial domain prediction is supported without any specification impact foreseen.

- Specify support for measurement event prediction for UE sided models [RAN2]:

- Specify signaling and protocol aspect to enable reporting inference outcome of UE sided model for measurement event prediction

    - Measurement event A1~A6 are in the scope in this release

    - Aim for a single framework for direct and indirect event prediction, and for performance monitoring.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY)

- Inference configuration

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline

- Both cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) can be used for a UE-side model.

Note 3: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation.

- For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]:

- Network sided data collection, at least

    - UE is configured to log RRM measurement results.

    - UE supports periodical logging and L3 event triggered logging.

    - UE can indicate logged data availability.

Note 4: The above objectives to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

- For both RRM measurement prediction and measurement event prediction [RAN4]:

- Specify core requirements for the use cases and scenarios supported by the previous objectives

- Specify LCM-related core requirements to support the LCM functionality management for both UE and network sided functionalities, if any

Note 6: Can check in RAN#109 whether RAN4-related objectives need further updates, based on RAN4’s further discussions.