3GPP TSG-RAN4 Meeting #116 	R4-2509879

# Bengaluru, India, 25th August 2025 - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2926 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) CR to add Lcrb in Figure 5.3.1-1 - TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A change is needed: - Since 4G LTE and Figure 5.6-1 (Definition of channel bandwidth and transmission bandwidth configuration for one E UTRA carrier), LCRB is not shown in that figure even though mentioned 193 times in Rel-19 TS 36.101 (also in Table 6.5.2.3.1-1, it is written “NOTE 6: LCRB is the Transmission Bandwidth (see Figure 5.6-1)” but LCRB is not shown there). - Whereas it may be considered late and not essential to fix that issue for 4G LTE in figure 5.6-1, it is not for 5G NR. - In 5G NR, Figure 5.3.1-1 (Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel), LCRB is also not shown in that figure even though mentioned more than 300 times (including in many table headers) in Rel-19 TS 38.101-1 (also in Table 6.4.2.3-1, it is written “NOTE 6: LCRB is the Transmission Bandwidth (see Figure 5.3.3-1)” but LCRB is not shown there again). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Figure 5.3.1-1: - Change “Transmission Bandwidth [RB]” to “Transmission Bandwidth LCRB [RB]”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It complicates the understanding of a simple parameter (not shown in a figure supposed to show it clearly) and that may even perpetuate to 6G. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

## 5.3	UE channel bandwidth

### 5.3.1	General

The UE channel bandwidth supports a single NR RF carrier in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple carriers to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

The relationship between the channel bandwidth, the guardband and the maximum transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel

<<Unchanged parts of the section skipped>>

<<End of change>>