3GPP TSG-RAN4 Meeting #116	R4-2509853

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5806 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) Update on SUL test scenario (Rel19) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current SUL definition on Rel-15 spec points out to a Secondary Cell (SCell) that will become SUL. However, Rel-15 specification does not include any Carrier Aggregation test scenario which were added in Rel-16 and onwards. Hence, our understanding is this SUL test scenario is a PCell + SUL carrier without Carrier Aggregation configuration for a Secondary Cell (SCell) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed all references to Secondary Cell (SCell) and replacing to SUL instead. Removed all test configurations defining Carrier Aggregation configurations Updated Table A.6.5.4.1.1-3 to accommodate test configuration for PCell and SUL carriers Removed whole Table A.6.5.4.1.1-4 as SUL configurations have been added in Table A.6.5.4.1.1-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A.6.5.4.1 will remain wrongly defined for Rel-15 specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Cat A CR aligned with R4-2509849 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### A.6.5.4	UE UL carrier RRC reconfiguration Delay

#### A.6.5.4.1	UE UL carrier RRC reconfiguration Delay

Table A.6.5.4.1-1 - Table A.6.5.4.1-4 : Void

##### A.6.5.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that when the UE receives a RRC message implying NR UL or Supplementary UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within the time limits specified in clause 8.4.2 and 8.4.3 for configuring and deconfiguring, respectively.

There is one cell with two UL carriers: FR1 PCell and FR1 SUL. Both NR uplink and supplementary uplink are broadcast by ServingCellConfigCommonSIB. The test parameters for PCell and SUL are given in Table A. 6.5.4.1.1-1, Table A.6.5.4.1.1-2, Table A.6.5.4.1.1-3 and Table A.6.5.4.1.1-4 below.  The test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, only NR uplink of PCell is configured to UE. At the start of T2, a supplementary uplink is configured to UE through RRCReconfiguration, then UE shall start transmission on the supplementary uplink. At the start of T3, the supplementary uplink is released through RRCReconfiguration.

Table A.6.5.4.1.1-1: Supported test configurations

| Configuration | PCell | SUL |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | SUL: 15 kHz SCS, 10 MHz bandwidth, SUL duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | SUL: 30kHz SCS, 40 MHz bandwidth, SUL duplex mode |
| 3 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | SUL: 15 kHz SCS, 10 MHz bandwidth, SUL duplex mode |
| 4 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | SUL: 30kHz SCS, 40 MHz bandwidth, SUL duplex mode |
| 5 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | SUL: 15 kHz SCS, 10 MHz bandwidth, SUL duplex mode |
| 6 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | SUL: 30kHz SCS, 40 MHz bandwidth, SUL duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Void |  |  |

Table A.6.5.4.1.1-2: General test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on Pcell

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1,2,3, 4, 5, 6 | 1, 2 | Two UL radio channels are used for this test. |
| Active cell |  | Config 1,2,3, 4, 5, 6 | FR1 PCell FR1 SUL | PCell on RF channel number 1 FR1 SUL on RF channel number 2 |
| CP length |  | Config 1,2,3, 4, 5, 6 | Normal |  |
| DRX |  | Config 1,2,3, 4, 5, 6 | OFF |  |
| Measurement gap pattern Id |  | Config 1,2,3, 4, 5, 6 | OFF |  |
| Filter coefficient |  | Config 1,2,3, 4, 5, 6 | 0 | L3 filtering is not used |
| T1 | s | Config 1,2,3, 4, 5, 6 | 5 |  |
| T2 | s | Config 1,2,3, 4, 5, 6 | 5 |  |
| T3 | s | Config 1,2,3, 4, 5, 6 | 5 |  |

Table A.6.5.4.1.1-3: NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on PCell

| Parameter | Unit | Test Configuration | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| Channel number for NR UL carrier |  | Conf 1, 2, 3, 4, 5, 6 | 1 |  |  |
| Channel number for supplementary UL |  | Conf 1, 2, 3, 4, 5, 6 | 2 |  |  |
| TDD configuration |  | Conf 1, 2 | N/A |  |  |
|  |  | Conf 3, 4 | TDD Conf.1.1 |  |  |
|  |  | Conf 5, 6 | TDD Conf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 2, 3, 4 | 10: NRB,c = 52 |  |  |
|  |  | Conf 5, 6 | 40: NRB,c = 10 |  |  |
| BWchannel for supplementary UL | MHz | Conf 1, 3, 5 | 10: NRB,c = 52 |  |  |
|  |  | Conf 2, 4, 6 | 40: NRB,c = 106 |  |  |

| PUSCH parameters for NR UL carrier |  | Conf 1, 2, 3, 4 | G-FR1-A3-9 in [13] | G-FR1-A3-9 in [13] | G-FR1-A3-9 in [13] |
| --- | --- | --- | --- | --- | --- |
|  |  | Conf 5, 6 | G-FR1-A3-13 in [13] | G-FR1-A3-13 in [13] | G-FR1-A3-13 in [13] |
| PUCCH parameters For NR UL carrier |  | Conf 1, 2, 3, 4 | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |
|  |  | Conf 5, 6 | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] |
| PUSCH parameters for supplementary UL |  | Conf 1, 3, 5 | N/A | G-FR1-A3-9 in [13] | N/A |
|  |  | Conf 2, 4, 6 | N/A | G-FR1-A3-13 in [13] | N/A |
| PUCCH parameters for supplementary UL |  | Conf 1, 3, 5 | N/A | N/A | N/A |
|  |  | Conf 2, 4, 6 | N/A | N/A | N/A |

| PDSCH reference measurement channel as defined in clause A.3.1.1 |  | Conf 1, 2 | SR.1.1 FDD |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Conf 3, 4 | SR.1.1 TDD |  |  |
|  |  | Conf 5, 6 | SR 2.1 TDD |  |  |
| RMSI CORESET reference measurement channel as defined in clause A.3.1.2 |  | Conf 1, 2 | CR.1.1 FDD |  |  |
|  |  | Conf 3, 4 | CR.1.1 TDD |  |  |
|  |  | Conf 5, 6 | CR.2.1 TDD |  |  |
| RMC CORESET reference measurement channel as defined in clause A.3.1.3 |  | Conf 1, 2 | CCR.1.1 FDD |  |  |
|  |  | Conf 3, 4 | CCR.1.1 TDD |  |  |
|  |  | Conf 5, 6 | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| SSB configuration |  | Conf 1, 2, 3, 4 | SSB.1 FR1 |  |  |
|  |  | Conf 5, 6 | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1, 2 | TRS.1.1 FDD |  |  |
|  |  | Conf 3, 4 | TRS.1.1 TDD |  |  |
|  |  | Conf 5, 6 | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4, 5, 6 | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4 | -102 |  |  |
|  | dBm/ SCS | Conf 1,2,3,4 | -102 |  |  |
|  |  | Conf 5, 6 | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1,2,3,4 | -86 | -86 | -86 |
|  |  | Conf 5, 6 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1,2,3,4 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16 MHz | Conf 5, 6 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6 | 1 x 2 |  |  |
| SUL RSRP threshold (rsrp-ThresholdSSB-SUL) | dBm | Conf 1,2,3,4 | -75 |  |  |
|  |  | Conf 5, 6 | -72 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void. NOTE 5: Void. NOTE 6: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[18] with configured BWchannel. |  |  |  |  |  |

##### A.6.5.4.1.2	Test Requirements

The UE shall be ready to start transmission on the supplementary uplink carrier on SUL within 20ms from the start of T2.

The UE shall stop the transmission on the supplementary uplink carrier on SUL within 20ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed UE UL carrier configuration delay and UE UL carrier release delay to be counted as correct. The rate of correct observed UE UL carrier configuration delay and UE UL carrier release delay during repeated tests shall be at least 90%.

## <<< END OF CHANGES >>>