3GPP TSG-RAN WG4 Meeting # 116	R4-2510069

# Bengaluru, India, August 25 – 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0813 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_CA_R16_intra-Core) CR to TR 38.101-2 on symbols for NR carrier aggregation_R19_CAT_A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause 5.3A.1 mentioned in the symbols “NRB,low”, “NRB,high”, “SCSlow” and “SCShigh” does not exist. It should be clause 5.3A.2 instead. Furthermore, the symbols “BWChannel,block”, “Foffset,high” and “Foffset,low” are missing in clause 3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the clause 5.3A.1 to 5.3A.2 for symbols “NRB,low”, “NRB,high”, “SCSlow” and “SCShigh”. - Add the symbol definition for “BWChannel,block”, “Foffset,high” and “Foffset,low”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definitions of the mentioned symbols will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

3.2	Symbols

For the purposes of the present document, the following symbols apply:

∆EIRPBC	The beam correspondence tolerance, where ∆EIRPBC = EIRP2 – EIRP1

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔRB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB	Allowed reference sensitivity relaxation due to support for inter-band CA operation

ΔRIBC	Allowed reference sensitivity relaxation due to support for intra-band contiguous CA operation

ΔRIBNC	Allowed reference sensitivity relaxation due to support for intra-band non-contiguous CA operation

ΔRIB,P,n	Allowed relaxation to reference sensitivity due to support for inter-band CA operation, per supported band in a combination.

ΔRIB,S,n	Allowed relaxation to EIS spherical coverage due to support for inter-band CA operation, per supported band in a combination.

ΔTIB	Allowed relaxation to EIRP requirements due to support for inter-band CA operation

ΔTIB,P,n	Allowed relaxation to peak EIRP requirements due to support for inter-band CA operation, per supported band in a combination.

ΔTIB,S,n	Allowed relaxation to EIRP spherical coverage due to support for inter-band CA operation, per supported band in a combination.

ΔTSTxMP	Allowed relaxation to EIRP requirements due to support for simultaneous transmission to multiple directions.

ΔMBP,n	Allowed relaxation to each, minimum peak EIRP and reference sensitivity due to support for multi-band operation, per supported band in a combination.

ΔMBS,n	Allowed relaxation to each, EIRP spherical coverage and EIS spherical coverage due to support for multi-band operation, per supported band in a combination.

∆MPR	Allowed increase in Maximum Power Reduction for 256QAM.

ΔMPRSTxMP	Allowed relaxation to MPR requirement due to support for simultaneous transmission to multiple directions, per each of indicated TCI states.

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWGB	max(GBChannel,low, GBChannel,high)

BWinterferer	Bandwidth of the interferer

BWintraCA	Aggregated channel bandwidth for intraband CA with or without contiguous RBs allocation, expressed in MHz. BWintraCA = BWChannel_CA in case of contiguous RBs allocation.  BWintraCA is the frequency separation between the lower edge of the lowest allocated RB and the higher edge of the highest allocated RB in case of non-contiguous RBs allocation.

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EIRP1	The measured total EIRP based on the beam the UE chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping

EIRP2	The measured total EIRP based on the beam yielding highest EIRP in a given direction, which is based on beam correspondence with relying on UL beam sweeping

EIRPmax	The applicable maximum EIRP as specified in sub-clause 6.2.1

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

F_center	The center frequency of an allocated block of PRBs

FC	Center frequency of a carrier for a numerology defined by the RF reference frequency on the channel raster mapped to the carrier according to subclause 5.4.2.2

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block.

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block.

FC, low	The Fc of the lowest carrier, expressed in MHz.

FC, high	The Fc of the highest carrier, expressed in MHz.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge, low	The lower edge of Aggregated Channel Bandwidth, expressed in MHz. Fedge, low = FC, low - Foffset, low.

Fedge, high	The upper edge of Aggregated Channel Bandwidth, expressed in MHz. Fedge, high = FC, high + Foffset, high.

FInterferer	Frequency of the interferer

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FUL_Meas	The sub-carrier frequency for which the equalizer coefficient is evaluated

GBChannel	Minimum guard band defined in sub-clause 5.3.3, expressed in kHz

GBChannel(i)	Minimum guard band defined in clause 5.3.3 of carrier i

GBChannel,low	Minimum guard band defined in clause 5.3.3 for the lowest assigned component carrier in clause 5.3A.2

GBChannel,high	Minimum guard band defined in clause 5.3.3 for the highest assigned component carrier in clause 5.3A.2

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

LCRB,Max	Maximum number of RB for a given Channel bandwidth and sub-carrier spacing

Max()	The largest of given numbers

Min()	The smallest of given numbers

MPRf,c	Maximum output power reduction for carrier f of serving cell c

MPRnarrow	Maximum output power reduction due to narrow PRB allocation

MPRWT	Maximum power reduction due to modulation orders, transmit bandwidth configurations, waveform types

nPRB	Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB,low	Transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.2

NRB,high	Transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.2

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c

Pint	The intermediate power point as defined in table 6.3.4.2-2

PInterferer	Modulated mean power of the interferer

Pmax	The maximum UE output power as specified in sub-clause 6.2.1

Pmin	The minimum UE output power as specified in sub-clause 6.3.1

P-MPRf,c	The Power Management UE Maximum Power Reduction for carrier f of serving cell c

PPowerClass	Nominal UE power class (i.e., no tolerance) as specified in sub-clause 6.2.1

PRB	The transmitted power per allocated RB, measured in dBm

PTMAX,f,c	The measured total radiated power for carrier f of serving cell c

PUMAX	The measured configured maximum UE output power

Pw	Power of a wanted DL signal

RBstart	Indicates the lowest RB index of transmitted resource blocks

SCSlow	SCS for the lowest assigned component carrier in clause 5.3A.2, expressed in kHz

SCShigh	SCS for the highest assigned component carrier in clause 5.3A.2, expressed in kHz

SSREF	SS block reference frequency position

T(∆P)	The tolerance T(∆P) for applicable values of ∆P (values in dB)

TRPmax	The maximum TRP for the UE power class as specified in sub-clause 6.2.1

==============================================================

<< End of changes >>