3GPP TSG RAN WG4 Meeting #116                  	                                          R4-2512561

# Bengaluru, IN, 25th - 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0719 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.104: BS demodulation performance requirements for MMSE-IRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It has been agreed that RAN4 will introduce BS demodulation performance requirements with MMSE-IRC receiver. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of BS demodulation performance requirements with MMSE-IRC receiver. Editorial updates for 11.2.1.13 - 11.2.1.15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.X (New) 11.2.1.13 11.2.1.14 11.2.1.15 11.2.1.X (New) B.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.141-1 38.141-2 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

Table 8.2.15.2-4: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 70% | G-FR1-A4-11 | pos1 | 10.1 |
| 2 | 2 | Normal | TDLC300-100 Low | 70% | G-FR1-A4-25 | pos1 | 18.5 |

### 8.2.X	Enhanced performance requirements for PUSCH with synchronous interference

#### 8.2.X.1	General

The performance requirement of PUSCH with enhanced performance requirements is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions.

Table: 8.2.X.1-1 Test parameters with enhanced performance requirements

| Parameter |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 (Note 1) |  | Interferer 2 (Note 1) |
| Transform precoding |  | Disabled |  |  |  |
| Default TDD UL-DL pattern (Note 2) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |  |  |  |
| INR (dB, Note 3) | Set 1 | N/A | 5.35 for 2Rx tests 7.28 for 4/8 RX tests | N/A for 2Rx tests -2.52 for 4/8 RX tests |  |
|  | Set 2 | N/A | 9.82 for for 2Rx tests 12.38 for 4/8 RX tests | N/A for 2Rx tests -0.97 for 4/8 RX tests |  |
| Interference Model |  | N/A | As specified in B.X.2 |  |  |
| HARQ | Maximum number of HARQ transmissions | 4 | N/A |  |  |
|  | RV sequence | 0, 2, 3, 1 | N/A |  |  |
| DM-RS | DM-RS configuration type | 1 |  |  |  |
|  | DM-RS duration | single-symbol DM-RS |  |  |  |
|  | Additional DM-RS position | pos1 |  |  |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |  |  |
|  | DM-RS port | {0} |  |  |  |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=1, nSCID =0 |  | NID0=2, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |  |  |  |
|  | Start symbol | 0 |  |  |  |
|  | Allocation length | 14 |  |  |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |  |  |
|  | Frequency hopping | Disabled |  |  |  |
| Code block group based PUSCH transmission |  | Disabled |  |  |  |
| NOTE 1: One explicit interferer, i.e., interferer 1, is modelled for tests with 2 RX antennas. Two explicit interferers are modelled for tests with 4 or 8 RX antennas. NOTE 2: The same requirements are applicable to TDD with different UL-DL pattern NOTE 3: INR is defined in Annex B.X.1 NOTE 4: All cells are time-synchronous. |  |  |  |  |  |

#### 8.2.X.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in tables 8.2.X.2-1 to 8.2.X.2-8 at the given SNR. FRCs are defined in annex A.

Table 8.2.X.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-8 | pos1 | 14.2 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.7 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.1 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.1 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.1 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.X.2-2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-10 | pos1 | 14.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.7 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 11.1 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.6 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.8 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.X.2-3: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-11 | pos1 | 14.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.8 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.4 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.5 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.X.2-4: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-14 | pos1 | 14.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.7 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.8 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.8 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.5 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.X.2-5: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-8 | pos1 | 14.0 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.6 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.2 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.1 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.X.2-6: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-10 | pos1 | 14.2 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.8 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.9 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.2 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.X.2-7: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-11 | pos1 | 14.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.7 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.8 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.5 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.7 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.X.2-8: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-14 | pos1 | 14.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.4 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.8 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.8 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.6 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

<Unchanged part Skipped>

11.2.1.13	Requirements for PUSCH with DM-RS bundling

Apply the requirements defined in clause 8.2.13 for 2Rx.

#### 11.2.1.14	Requirements for PUSCH for ATG

Apply the requirements defined in clause 8.2.14 for 2Rx.

#### 11.2.1.15	Requirements for PUSCH with enhanced DM-RS

Apply the requirements defined in clause 8.2.15 for 2Rx.

#### 11.2.1.X	Enhanced performance requirements for PUSCH with synchronous interference

Apply the requirements defined in clause 8.2.X for 2Rx.

<Unchanged part Skipped>

B.7	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval.

For FDD the averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step.

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

-	Thus   is calculated using in the expressions above and  is calculated using  in the  calculation.

-	Thus we get:

EVM=maxEVMframe,l,EVMframe,h

For TDD, let NdlTDD be the number of slots with downlink symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

B.X	Interference model for enhanced performance requirements for PUSCH with synchronous interference

B.X.1	Interference to Noise ratio (INR)

Each inter-cell interferer involved in enhanced performance requirements is characterized by its associated interferer to noise ratio (INR) value:

INRi=IiN

where Ii is the is the total signal energy in the slot from the i-th interfering PUSCH involved in the requirement scenario and N is the noise energy with definition provided in section 8.1.

B.X.2	Interference model for synchronous scenario

This clause provides interference modelling for each explicitly modelled inter-cell interferer in the requirement scenario where the inter-cell interferer(s) are time-synchronous with the tested signal.

In each subframe, each interferer shall transmit 16QAM randomly modulated data over the entire PUSCH region and the same resource blocks as the tested signal. The demodulation reference signal (DM-RS), configured according to Table 8.2.X.1-1 for enhanced performance requirements is transmitted associated with the transmission of PUSCH.

<Changed part End>