3GPP TSG-RAN4 Meeting #116	R4-2509149

# Bengaluru, IN, 25th - 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0191 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_perf_enh-Perf) Correction on release independent rules for PMI and CA test requirements (R19 A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During the discussion in the Rel-16 NR_perf_enh-Perf WI, RAN4 have agreed the release independent rules for PMI and CA requirements, which has been correctly reflected in the latest TS38.307 v16.16.0. The v17.x 38.307 should also capture the above release independent rules with the R17 agreed CR R4-2100789. However, some parts in R4-2100789 are never implemented from the v17.x spec which will cause mis-alignment between v16 and v17/18/19 specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Missing parts in the agreed R17 CR R4-2100789 are re-added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will remain mis-alignment between v16 and v17/18/19 TS38.307 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3 5.2.1 5.2.2 5.4 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Cat F CR R4-2509147 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BW	Bandwidth

CA	Carrier Aggregation

CC	Component carrier

DL	Downlink

EN-DC	Dual connectivity between E-UTRA and NR

(e)RedCap	Redcap or eRedCap

eRedCap	enhanced Reduced Capability

FDD	Frequency Division Duplex

FR1	Frequency range 1

FR2	Frequency range 2

NR	New radio

PMI	Precoding Matrix Indicator

RedCap	Reduced Capability

REL	Release

SDL	Supplementary downlink

SUL	Supplementary uplink

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

<Unchanged part skipped>

5.2	Additional NR CA configurations for NR frequency range 1

5.2.1	Intra-band CA

Requirements for additional NR intraband CA configurations within FR1 of  TS 38.101-1 in Rel-P [2] are introduced via this clause.

Requirements for additional NR intraband CA configurations with UL MIMO within FR1 of TS 38.101-1 in Rel-P [2] are introduced via this clause.

Table 5.2.1-1: NR intra-band contiguous CA within FR1

| Feature | DL/UL | CA BW Class | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- |
| Intra-band contiguous CA configurations within FR1 | DL | B, C, D, E, M, N, O | FDD, TDD | Rel-15 | Table B.3.1-1, Table B.4.2-1 |
|  | UL | A,B,C | FDD, TDD | Rel-15 |  |
| Intra-band contiguous CA configurations with UL MIMO within FR1 | UL | C | TDD | Rel-15 | Table B.4.2-2 |

Table 5.2.1-2: NR intra-band non-contiguous CA within FR1

| Feature | DL/UL | number of sub-blocks | maximum number of CCs within a sub-block | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| Intra-band non-contiguous CA configurations within FR1 | DL | 2 | 1 | FDD, TDD | Rel-15 | Table B.3.1-1, Table B.4.2-1 |
|  |  | 3 | 1 | FDD, TDD | Rel-15 |  |
|  |  | 4 | 1 | TDD | Rel-15 |  |
|  | UL | 1 | 1 | FDD, TDD | Rel-15 |  |
|  |  | 2 | 1 | FDD, TDD | Rel-15 |  |

5.2.2	Inter-band CA

Requirements for additional NR inter-band CA configurations within FR1 of TS 38.101-1 in Rel-P [2] are introduced via this clause.

Requirements for additional NR inter-band CA configurations with UL MIMO within FR1 of TS 38.101-1 in Rel-P [2] are introduced via this clause.

Requirements for additional NR inter-band CA configurations with Tx Diversity within FR1 of TS 38.101-1 in Rel-P [2] are introduced via this clause.

Table 5.2.2-0: NR inter-band CA UE power class within FR1

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Inter-band CA Power Class 2 | TDD, FDD and TDD | Rel-15 | Table B.4.2-1 |
| Inter-band CA Power Class 3 | FDD, TDD, SDL and TDD, FDD and TDD | Rel-15 |  |
| Inter-band CA Power Class 1.5 | TDD, FDD and TDD | Rel-17 | Table B.4.2-3, Table B.4.2-4 |

Table 5.2.2-1: NR inter-band CA within FR1

| Feature | DL/UL | Maximum number of bands | Maximum number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations within NR FR1 | DL | 6 | 9 | A, B, C, D, E | TDD, FDD, SDL and FDD, SDL and TDD, FDD and TDD, FDD and SDL and TDD | Rel-15 | Table B.3.1-1, Table B.4.2-1 |
|  | UL | 2 | 2 | A | TDD, FDD, FDD and TDD | Rel-15 |  |
|  |  | 1 | 2 | B, C, 2A |  |  |  |
|  |  | 2 | 3 | A, B, C |  |  |  |
| Inter-band CA configurations with UL MIMO within FR1 | DL | 2 | 2 | A | TDD, FDD and TDD | Rel-171 | Table B.4.2-3 |
|  | UL | 2 | 2 | A | TDD, FDD and TDD | Rel-171 |  |
| Inter-band CA configurations with Tx Diversity within FR1 | DL | 2 | 2 | A | FDD and TDD | Rel-171 | Table B.4.2-4 |
|  | UL | 2 | 2 | A | FDD and TDD | Rel-171 |  |
| Inter-band CA configurations with 1Tx-2Tx switching within NR FR1 | UL | 2 | 2 | A | TDD, FDD, FDD and TDD | Rel-16 | Table B.4.14-1 |
| Inter-band CA configurations with 1Tx-2Tx switching within NR FR1 | UL | 2 | 3 | A, B, C | TDD, FDD, FDD and TDD | Rel-17 | Table B.4.14-1 |
| Inter-band CA configurations with 2Tx-2Tx switching within NR FR1 | UL | 2 | 2 | A | TDD, FDD, FDD and TDD | Rel-17 | Table B.4.14-1 |
|  |  | 2 | 3 | A, B, C |  |  |  |
| NOTE 1: This is applied to FWA UE only. |  |  |  |  |  |  |  |

<Unchanged part skipped>

5.4	Other release independent features for NR frequency range 1

This clause covers requirements for a Rel-P UE coming from all other release independent features that are not covered under clause 5.1, 5.2 and 5.3, e.g. generic baseband requirements or requirements that are not band/CA/SUL configuration specific.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| RRM requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.1-1 | Rel-16 WI NR_HST introduced band independent RRM requirements: see Table C.1-1 |
| UE demodulation requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.2-1 | Rel-16 WI NR_HST introduced band independent UE demodulation requirements: see Table C.2-1 |
| Precoding matrix indicator (PMI) reporting requirements for TypeI-SinglePanel and TypeII Codebooks with more than 8TX and up to 32TX | Rel-15 | Table D.1-1, Table D.2-1 | Rel-16 NR_perf_enh-Perf WI introduced band independent PMI reporting requirements for 16TX and 32TX TypeI-SinglePanel Codebook, and 16TX TypeII Codebook: see Table D.1-1 and Table D.2-1 respectively. |
| PDSCH demoulation requirements with LTE CRS rate matching for TDD band | Rel-15 | Table E.1-1 | Rel-16 NR_perf_enh-Perf WI introduced band independent PDSCH demoulation requirements with LTE CRS rate mataching for TDD band: see Table E.1-1. |
| RF requirements for 4Rx UEs | Rel-15 | Table B.4.10-1, Table B.4.10-2 |  |
| Transparent Tx diversity | Rel-15 | Table B.4.11-1 | Rel-17 WI NR_RF_TxD introduced transparent Tx diversity requirements: see Table B.4.11-1 |
| UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference | Rel-15 | Table B.3.3-1 | Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration. |
| RRM enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 2) | Table C.3-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent RRM enhancement: see Table C.3-1 |
| HST-SFN CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 3) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-SFN CA demodulation enhancement: see Table C.4-1 |
| HST-DPS CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-15 (NOTE 4) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-DPS CA demodulation enhancement: see Table C.4-1 |
| PDSCH absolute physical layer throughput requirements with link adaptation | Rel-17 | Table B.3.4-1 | Rel-18 WI NR_demod_enh3-Perf: see Table B.3.4-1. These requirements are optional for Rel-17 and can be executed based on UE declaration. |
| RF requirements for 8Rx UE | Rel-17 | Table B.4.x-1, Table B.4.x-2 |  |
| Demodulation requirements for 8Rx UE | Rel-17 | Table B.3.5-1 |  |
| NOTE 1: Rel-15 UEs supporting the high speed train are assumed to read the Rel-16 high speed train scenario information, which is broadcast to all UEs. NOTE 2: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 3: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 4: Rel-15 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. |  |  |  |

<Unchanged part skipped>

6.2	Additional NR CA configurations for NR frequency range 2

6.2.1	Intra-band CA

Requirements for additional NR intra-band CA configurations within FR2 of TS 38.101-2 in Rel-P [3] are introduced via this clause.

Table 6.2.1-1: NR intra-band contiguous CA within FR2

| Feature | DL/UL | CA BW Class | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- |
| Intra-band contiguous CA configurations within FR2 | DL | B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-15 | Table B.3.1-1, Table B.4.2-1 |
|  |  | R2, R3, R4, R5, R6, R7, R8, R9, R10, R11, R12 | TDD | Rel-17 |  |
|  | UL | A, B, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-15 |  |
|  |  | R2, R3, R4, R5 | TDD | Rel-17 |  |

Table 6.2.1-2: NR non-contiguous intra-band CA within FR2

| Feature | DL/UL | number of sub-blocks | maximum number of CCs within a sub-block | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| Intra-band non- contiguous CA configurations within | DL | 2 | 7 | TDD | Rel-15 | Table B.3.1-1, Table B.4.2-1 |
| FR2 |  | 3 | 4 | TDD | Rel-15 |  |
|  |  | 4 | 4 | TDD | Rel-15 |  |
|  |  | 5 | 4 | TDD | Rel-15 |  |
|  |  | 6 | 4 | TDD | Rel-15 |  |
|  |  | 7 | 3 | TDD | Rel-15 |  |
|  |  | 8 | 3 | TDD | Rel-15 |  |
|  |  | 9 | 2 | TDD | Rel-15 |  |
|  |  | 10 | 2 | TDD | Rel-15 |  |
|  | UL | 1 | 7 | TDD | Rel-15 |  |
|  |  | 2 | 2 | TDD | Rel-15 |  |
|  |  | 3 | 1 | TDD | Rel-15 |  |

<End of change>