3GPP TSG-RAN WG4 Meeting #116	R4-2509526

# Bangalore, India,  25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.741 | CR | 0008 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_LSband-Core) Corrections and clarifications to the NR NTN band n254 NS flags |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Globalstar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_LSband-Core |  |  |  |  |  | Date: |  |  | 2025-07-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In addition to the existing band n254 NS flags NS_04N and NS_05N it was agreed to introduce additional flags, NS_11N and NS_12N, to enforce when needed more stringent in-band PSD restrictions. While the corresponding changes were already implemented in TS 38.101-5, the TR does not contain the corresponding explanations for these flags. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In sub-clause 6.2.1.3, additional information on NS flags NS_11N and NS_12N is added. In sub-clause 6.2.1.3a, NS_11N and NS_12N is associated with the same A-MPR values as NS_04N and NS_05N, respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be incomplete information on the NTN band n254 NS flags. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3, 6.2.1.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.1.3	Emission requirements and NS values

There are additional NS values defined for this band as summarised in Table 6.2.1.3-1 below:

a)	NS_03N is for the FCC regulatory domain. Since FCC does not introduce any special in-band emission requirements for sub-ranges, same requirements will be applicable for the whole range and for all the channel sizes.

b)	ETSI regulations have different in-band emission masks depending on the operating sub-range, so two additional NS flags are needed to differentiate between them.

c)	NS_04N applicable range is limited to 1610-1618.25MHz according to the ETSI requirements. And since the total spectrum block size is 8.25MHz, it can accommodate only 5MHz standard NR channel, i.e. the applicable NR channel size is 5MHz.

d)	NS_05N is for the ETSI range 1618.25-1626.5MHz, which can also accommodate only 5MHz standard NR channel. However, ETSI document also clarifies that the same in-band emission requirements will apply if the channel is partially configured in that range. It means that any 10 and 15MHz channel configured in 1610-1626.5MHz range will inevitably fall into the 1618.25-1626.5MHz range and thus they will follow NS_04N requirements.

e)	In addition to ETSI flags NS_04N and NS_05N associated with a more relaxed PSD limit of 27dBm/4kHz, two NS flags are neeed to enforce when required a more stringent PSD limit of 15dBm/4kHz (refer to sub-clause 5.0 for more information on the regulatory requirements and corresponding provisions).

Table 6.2.1.3-1: NTN L-/S-band NS values

| Regulatory domain | NS value | Applicable range (MHz) | Applicable channel (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| FCC | NS_03N | 1610-1626.5 | 5, 10, 15 |  |
| ETSI | NS_04N | 1610-1618.25 | 5 | 27dBm/4kHz (mean) |
|  | NS_05N | 1618.25-1626.5 | 5 |  |
|  |  | 1610-1626.5 | 10, 15 |  |
|  | NS_11N | 1610-1618.25 | 5 | 15dBm/4kHz (peak limit) |
|  | NS_12N | 1618.25-1626.5 | 5 |  |
|  |  | 1610-1626.5 | 10, 15 |  |

Mapping between values of additionalSpectrumEmission and NS values is given in the Table 6.2.3.1-2 below.

Table 6.2.1.3-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N | NS_11N | NS_12N |  |  |

#### 6.2.1.3a	NS values and associated A-MPR

For the 5MHz channel, Figure 6.2.1.3a-1 below presents a graphical summary of the required power back-off values for different 5MHz channels:

-	The 5MHz channel right at the lower edge of the band requires power back-off even for small RB allocations irrespective of the fact whether it is FCC or ETSI regulations. If the channel is shifted by 1.4MHz, then at least small RB allocations will not require power back-off exceeding baseline MPR requirements. Shifting the 5MHz channel beyond the 1615.7MHz centre frequency results in no A-MPR at all.

-	As for the upper edge of the band, the 5MHz channel under the FCC regulations will not require any additional A-MPR requirements even if it is right at the edge because there are no out-of-band emission requirements for the upper frequencies. However, with the ETSI regulations we will need to provide offset of 1.6MHz, upon which we will not be exceeding baseline MPR requirements.

-	Because of the same regulations for the lower frequencies, A-MPR values can be the same for NS_03N (FCC) and NS_04N (ETSI lower range) at centre frequencies 1612.5 <= fc < 1615.7MHz. Furthermore, it is possible to split A-MPR values into two regions – 1612.5 <= fc < 1613.9MHz and 1613.9 <= fc < 1615.7MHz – whereupon the second region will not require A-MPR for small RB allocations.

-	There is no need for A-MPR for NS_03N (FCC) at centre frequencies above 1615.7MHz, but NS_05N (ETSI upper range) requires A-MPR for centre frequencies at 1622.4 < fc <= 1624MHz.

Figure 6.2.1.3a-1: Summary of required power back-off regions for the 5MHz channel for FCC (top) and ETSI (bottom) regulations.

For the 10MHz channel:

-	For the channel right at the lower edge of the band, A-MPR might be needed even for small RB allocations at the channel edges because of the strict out-of-band emission requirements. Shifting the channel farther away from the lower edge basically improves A-MPR with the switching point at the centre frequency of around 1620.1MHz (i.e. 1.4MHz offset from the upper edge), after which at least for the ETSI requirements emission requirements from the upper frequencies start influencing allocations at the channel’s upper edge. Based on that A-MPR values can be considered the same for NS_03N (FCC) and NS_05N (ETSI) for the centre frequencies at 1615 <= fc < 1620.1MHz.

-	For the channel right at the upper edge of the band, A-MPR is generally not needed for small allocations under the FCC regulations, which is reflected in the corresponding A-MPR for NS_03 for the channel at 1621.5MHz. However, with NS_05N (ETSI) A-MPR is still required because of the out-of-band emission requirements for upper frequencies.

-	As can be seen from Figure 6.2.1.3a-2 below, the only major difference between the FCC and ETSI A-MPR regions is that the former can have optimised 10MHz channel placed right at the upper edge of the band.

Figure 6.2.1.3a-2: Summary of required power back-off regions for the 10MHz channel for FCC (top) and ETSI (bottom) regulations.

For the 15MHz channel, more or less same A-MPR is needed for NS_03N (FCC) and NS_05N (ETSI) irrespective of the fact where exactly the channel is configured and whether it is under the FCC or ETSI regulations.

Figure 6.2.1.3a-3: A-MPR regions for NS_03N (5, 10 and 15MHz channels).

Table 6.2.1.3a-1: A-MPR values for NS_03N (FCC)

| Channel BW | Carrier Centre Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

Figure 6.2.1.3a-4: A-MPR regions for NS_04N and NS_11N (5MHz channels).

Table 6.2.1.3a-2: A-MPR values for NS_04N and NS_11N (ETSI lower sub-range)

| Channel BW | Carrier Centre Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |

|  | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 |
|  | QPSK | 2.5 | 4.0 | 2.5 |
|  | 16QAM | 3.0 | 4.5 | 3.0 |
|  | 64QAM | 3.5 | 5 | 3.5 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 |

Figure 6.2.1.3a-5: A-MPR regions for NS_05N and NS_12N (5, 10 and 15MHz channels).

Table 6.2.1.3a-3: A-MPR values for NS_05N and NS_12N (ETSI upper sub-range)

| Channel BW | Carrier Centre Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1622.4 < fc <= 1624 | >= 3.6 | > 0 | A3 |
|  |  |  | >= 2.88 | A1 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc <= 1621.5 |  | <= 7.2 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  |  | > 7.2 | > 0 | A6 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 5.0 | 1.5 | 6.5 | 6.5 | 2.0 |
|  | QPSK | 1.5 | 5.0 | 1.5 | 6.5 | 7.0 | 2.5 |
|  | 16QAM | 2.0 | 5.5 | 2.0 | 7.0 | 7.5 | 3.0 |
|  | 64QAM | 2.5 | 6 | 2.5 | 7.5 | 8 | 3.5 |
|  | 256QAM | 3.5 | 7 | 3.5 | 8.5 | 9 | 4.5 |
| CP-OFDM | QPSK | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 16QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 64QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 256QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |