3GPP TSG-RAN4 Meeting #116	R4-2512831

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0666 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.141-2: 7MHz channel bandwidth introduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_7MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of 7MHz channel bandwidth for NR. Technical content of this CR was Endorsed during RAN4#115 in R4-2507926. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of 7MHz channel bandwidth for NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreement on 7 MHz channel would not be properly reflected in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.2, 6.4.3.5.1, 6.6.3.5.1, 6.7.2.4.2, 6.7.3.5.1, 7.2.5.2, 7.3.5.2, 7.4.5.2, 7.5.1.5.2, 7.5.2.5.2, 7.8.5.1, 7.9.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 37.105, 37.113, 38.104, 38.106, 38.113 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 37.141, 37.145-1, 37.145-2, 38.115-1, 38.141-1, 38.176-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

#### 4.1.2.2	Measurement of transmitter

The maximum OTA Test System uncertainty for OTA transmitter tests minimum requirements are given in tables 4.1.2.2-1 and 4.1.2.2-2. Details for derivation of OTA Test System uncertainty are given in corresponding clauses in TR 37.941 [29].

Table 4.1.2.2-1: Maximum OTA Test System uncertainty for FR1 OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 6.2 Radiated transmit power | Normal condition: ±1.1 dB, f ≤ 3 GHz ±1.3 dB, 3 GHz < f ≤ 7.125 GHz ±1.8 dB for band n96 |
|  | Extreme condition: ±2.5 dB, f ≤ 3 GHz ±2.6 dB, 3 GHz < f ≤ 7.125 GHz |
| 6.3 OTA base station output power | ±1.4 dB, f ≤ 3.0 GHz ±1.5 dB, 3.0 GHz < f ≤ 4.2 GHz ±1.5 dB, 4.2 GHz < f ≤ 7.125 GHz |
| 6.4.2 OTA RE power control dynamic range | N/A |
| 6.4.3 OTA total power dynamic range | ±0.4 dB |
| 6.5.1 OTA transmitter OFF power | ±3.4 dB, f ≤ 3.0 GHz ±3.6 dB, 3.0 GHz < f ≤ 7.125 GHz (NOTE 1) |
| 6.5.2 OTA transmitter transient period | N/A |
| 6.6.2 OTA frequency error | ±12 Hz |
| 6.6.3 OTA modulation quality | ±1 % |
| 6.6.4 OTA time alignment error | ±25 ns |
| 6.7.2 OTA occupied bandwidth | ±100 kHz, BWChannel 5 MHz, 7 MHz, 10 MHz ±300 kHz, BWChannel 15 MHz, 20 MHz, 25 MHz, 30 MHz, 35 MHz, 40 MHz, 45 MHz, 50 MHz ±600 kHz, BWChannel 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz |
| 6.7.3 OTA ACLR/CACLR | f ≤ 3.0 GHz ±1 dB, BW ≤ 20MHz ±1 dB, BW > 20MHz 3.0 GHz < f ≤ 7.125 GHz ±1.2 dB, BW ≤ 20MHz ±1.2 dB, BW > 20MHz Absolute power ±2.2 dB, f ≤ 3.0 GHz Absolute power ±2.7 dB, 3.0 GHz < f ≤ 4.2 GHz Absolute power ±2.7 dB, 4.2 GHz < f ≤ 7.125 GHz |
| 6.7.4 OTA operating band unwanted emissions | Absolute power ±1.8 dB, f ≤ 3.0 GHz Absolute power ±2 dB, 3.0 GHz < f ≤ 4.2 GHz Absolute power ±2 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 6.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz < f ≤ 6 GHz ±4.2 dB, 6 GHz < f ≤ 26 GHz |
| 6.7.5.3 OTA transmitter spurious emissions, protection of BS receiver | ±3.1 dB, f ≤ 3 GHz ±3.3 dB, 3 GHz < f ≤ 4.2 GHz ±3.4, 4.2 GHz < f ≤ 7.125 GHz (NOTE 1) |
| 6.7.5.4 OTA transmitter spurious emissions, additional spurious emissions requirements | ±2.6 dB, f ≤ 3 GHz ±3.0, 3 GHz < f ≤ 4.2 GHz ±3.5, 4.2 GHz < f ≤ 6 GHz |
| 6.7.5.5 OTA transmitter spurious emissions, co-location | ±3.1 dB, f ≤ 3 GHz ±3.3 dB, 3 GHz < f ≤ 4.2 GHz ±3.4, 4.2 GHz < f ≤ 7.125 GHz (NOTE 1) |
| 6.8 OTA transmitter intermodulation | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests in6.7.3 (ACLR), 6.7.4 (OBUE) and 6.7.5 (spurious emissions) which have to be carried out in the presence of the interferer. ±3.2 dB, f ≤ 3.0 GHz ±3.4 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.5 dB, 4.2 GHz < f ≤ 6 GHz (NOTE 1) |
| 6.9 OTA spatial emission | 6.425 GHz < f ≤ 7.075 GHz |
| NOTE 1: Fulfilling the criteria for CLTA selection and placement in clause 4.12 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate measurement uncertainty budget in TR 37.941 [29] shall be used for evaluating the test system uncertainty. NOTE 2: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

------------------------------ Next modified section -------------------------

##### 6.4.3.5.1	BS type 1-O

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.4.3.5.1-1.

Table 6.4.3.5.1-1: Total power dynamic range

| BS channel bandwidth (MHz) | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.5 | 10 | N/A |
| 7 | 15.0 | N/A | N/A |
| 10 | 16.7 | 13.4 | 10 |
| 15 | 18.5 | 15.3 | 12.1 |
| 20 | 19.8 | 16.6 | 13.4 |
| 25 | 20.8 | 17.7 | 14.5 |
| 30 | 21.6 | 18.5 | 15.3 |
| 35 | 22.7 | 19.6 | 16.4 |
| 40 | 22.9 | 19.8 | 16.6 |
| 45 | 23.8 | 20.7 | 17.6 |
| 50 | 23.9 | 20.8 | 17.7 |
| 60 | N/A | 21.6 | 18.5 |
| 70 | N/A | 22.3 | 19.2 |
| 80 | N/A | 22.9 | 19.8 |
| 90 | N/A | 23.4 | 20.4 |
| 100 | N/A | 23.9 | 20.9 |

------------------------------ Next modified section -------------------------

##### 6.6.3.5.1	BS type 1-O

For BS type 1-O, the EVM of each NR carrier for different modulation schemes on PDSCH shall be less than the limits in table 6.6.3.5.1-1.

Table 6.6.3.5.1-1: EVM requirements for BS type 1-O

| Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 18.5 |
| 16QAM | 13.5 |
| 64QAM | 9 |
| 256QAM | 4.5 |
| 1024QAM | 3.5 %1 3.8 %2 |
| NOTE 1: This requirement is applicable for frequencies equal to or below 4.2 GHz. NOTE 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

EVM shall be evaluated for each NR carrier over all allocated resource blocks and downlink slots. Different modulation schemes listed in table 6.6.3.5.1-1 shall be considered for rank 1.

For NR, for all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink slots within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

Table 6.6.3.5.1-2, 6.6.3.5.1-3, 6.6.3.5.1-4 below specify the EVM window length (W) for normal CP for BS type 1-O.

Table 6.6.3.5.1-2: EVM window length for normal CP, FR1, 15 kHz SCS

| Channel bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| 7 | 512 | 36 | 14 | 40 |
| 10 | 1024 | 72 | 28 | 40 |
| 15 | 1536 | 108 | 44 | 40 |
| 20 | 2048 | 144 | 58 | 40 |
| 25 | 2048 | 144 | 72 | 50 |
| 30 | 3072 | 216 | 108 | 50 |
| 35 | 3072 | 216 | 108 | 50 |
| 40 | 4096 | 288 | 144 | 50 |
| 45 | 4096 | 288 | 144 | 50 |
| 50 | 4096 | 288 | 144 | 50 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

------------------------------ Next modified section -------------------------

##### 6.7.2.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	Set the BS to transmit signal.

6)	Measure the spectrum emission of the transmitted signal using at least the number of measurement points, and across a span, as listed in table 6.7.2.4.2-1, table 6.7.2.4.2-2, and table 6.7.2.4.2-3. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less.

NOTE:	The detection mode of the spectrum analyzer will not have any effect on the result if the statistical properties of the out-of-OBW power are the same as those of the inside-OBW power. Both are expected to have the Rayleigh distribution of the amplitude of Gaussian noise. In any case where the statistics are not the same, though, the detection mode is power responding. There are at least two ways to be power responding. The spectrum analyser can be set to "sample" detection, with its video bandwidth setting at least three times its RBW setting. Or the analyser may be set to respond to the average of the power (root-mean-square of the voltage) across the measurement cell.

Table 6.7.2.4.2-1: Span and number of measurement points for OBW measurements for FR1

| Bandwidth | BS channel bandwidth BWChannel (MHz) |  |  |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 7 | 10 | 15 | 20 | > 20 | > 20 |
| Span (MHz) | 10 | 14 | 20 | 30 | 40 | 2×BWChannel |  |
| Minimum number of measurement points | 400 | 400 | 400 | 400 | 400 | 2×BWChannel100kHz |  |

------------------------------ Next modified section -------------------------

##### 6.7.3.5.1	BS type 1-O

For the OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.1-1/2a or the OTA ACLR absolute limits in table 6.7.3.5.1-2 shall apply, whichever is less stringent. The OTA CACLR limits in table 6.7.3.5.1-3 or the OTA CACLR absolute limits in table 6.7.3.5.1-3a shall apply, whichever is less stringent.

The CACLR in a sub-block gap and Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.1-3 and the filters on the assigned channels are defined in table 6.7.3.5.1-4.

For operation in paired and unpaired spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1-1.

Table 6.7.3.5.1-1: BS type 1-O ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0 – 3 GHz) | OTA ACLR limit (3 – 6 GHz) | OTA ACLR limit (6.0 – 7.125 GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) | NA |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) | NA |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.1-2.

Table 6.7.3.5.1-2: BS type 1-O ACLR absolute limit

| BS category / BS class | OTA ACLR absolute limit |
| --- | --- |
| Category A Wide Area BS | -4 dBm/MHz |
| Category B Wide Area BS | -6 dBm/MHz |
| Medium Range BS | -16 dBm/MHz |
| Local Area BS | -23 dBm/MHz |
| NOTE 1: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 2: Void |  |

For operation in non-contiguous spectrum or multiple bands, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1-2a.

Table 6.7.3.5.1-2a: BS type 1-O ACLR limit in non-contiguous spectrum or multiple bands

| BS channel bandwidth of NR carrier transmitted BWChannel adjacent to sub-block gap or inter RF Bandwidth gap (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0-3GHz) | OTA ACLR limit (3-6GHz) | OTA ACLR limit (6.0 – 7.125 GHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20 | Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8 dB |
|  | Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8 dB |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8 dB |
|  | Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 7, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |  |  |

The OTA CACLR measurement result shall not less than the OTA CACLR limit specified in table 6.7.3.5.1-3.

Table 6.7.3.5.1-3: BS type 1-O CACLR limit

| BS channel bandwidth of NR carrier transmitted BWChannel adjacent to sub-block gap or inter RF Bandwidth gap (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA CACLR limit (0-3 GHz) | OTA CACLR limit (3-6 GHz) | OTA CACLR limit (6.0 – 7.125 GHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20 | 5 ≤ Wgap < 15 (Note 3) 5 ≤ Wgap < 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8dB |
|  | 10 < Wgap < 20 (Note 3) 10 ≤ Wgap < 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8dB |
| 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 | 20 ≤ Wgap < 60 (Note 4) 20 ≤ Wgap < 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8dB |
|  | 40 < Wgap < 80 (Note 4) 40 ≤ Wgap < 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB | 36.8dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 7, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |  |  |

------------------------------ Next modified section -------------------------

#### 7.2.5.2	Test requirements for BS type 1-H and BS type 1-O

For each measured carrier, the throughput measured in step 9 of clause 7.2.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.5.2-1.

Table 7.2.5.2-1: EIS levels

| BS channel | Sub-carrier | Reference | OTA sensitivity level, EIS (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| bandwidth (MHz) | spacing (kHz) | measurement channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | 6.0 GHz < f ≤ 7.125 GHz |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 |  |  |  |  |
| 10, 15 | 30 | G-FR1-A1-2 |  |  |  |  |
| 10, 15 | 60 | G-FR1-A1-3 | Declared | Declared | Declared | Declared |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | minimum EIS | minimum EIS | minimum EIS | minimum EIS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | + 1.3 | + 1.4 | + 1.6 | + 1.9 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 |  |  |  |  |
| NOTE: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

------------------------------ Next modified section -------------------------

Table 7.3.5.2-1: Wide Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | OTA reference sensitivity level, EISREFSENS (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | 6.0 GHz < f ≤ 7.125 GHz |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -100.4 – ΔOTAREFSENS | -100.3 – ΔOTAREFSENS | -100.1 – ΔOTAREFSENS | NA |
| 10, 15 | 30 | G-FR1-A1-2 | -100.5 – ΔOTAREFSENS | -100.4 – ΔOTAREFSENS | -100.2 – ΔOTAREFSENS | NA |
| 10, 15 | 60 | G-FR1-A1-3 | -97.6 – ΔOTAREFSENS | -97.5 – ΔOTAREFSENS | -97.3 – ΔOTAREFSENS | NA |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -94 – ΔOTAREFSENS | -93.9 – ΔOTAREFSENS | -93.7 – ΔOTAREFSENS | -92.4 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -94.3 – ΔOTAREFSENS | -94.2 – ΔOTAREFSENS | -94 – ΔOTAREFSENS | -92.7 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -94.4 – ΔOTAREFSENS | -94.3 – ΔOTAREFSENS | -94.1 – ΔOTAREFSENS | -92.8 – ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

Table 7.3.5.2-2: Medium Range BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | EISREFSENS (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | 6.0 GHz < f ≤ 7.125 GHz |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -95.4 – ΔOTAREFSENS | -95.3 – ΔOTAREFSENS | -95.1 – ΔOTAREFSENS | NA |
| 10, 15 | 30 | G-FR1-A1-2 | -95.5 – ΔOTAREFSENS | -95.4 – ΔOTAREFSENS | -95.2 – ΔOTAREFSENS | NA |
| 10, 15 | 60 | G-FR1-A1-3 | -92.6 – ΔOTAREFSENS | -92.5 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS | NA |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -89 – ΔOTAREFSENS | -88.9 – ΔOTAREFSENS | -88.7 – ΔOTAREFSENS | -87.4 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -89.3 – ΔOTAREFSENS | -89.2 – ΔOTAREFSENS | -89 – ΔOTAREFSENS | -87.7 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -89.4 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS | -89.1 – ΔOTAREFSENS | -87.8 – ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

Table 7.3.5.2-3: Local Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | OTA reference sensitivity level, EISREFSENS (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | 6.0 GHz < f ≤ 7.125 GHz |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -92.4 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS | -92.1 – ΔOTAREFSENS | NA |
| 10, 15 | 30 | G-FR1-A1-2 | -92.5 – ΔOTAREFSENS | -92.4 – ΔOTAREFSENS | -92.2 – ΔOTAREFSENS | NA |
| 10, 15 | 60 | G-FR1-A1-3 | -89.6 – ΔOTAREFSENS | -89.5 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS | NA |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -86 – ΔOTAREFSENS | -85.9 – ΔOTAREFSENS | -85.7 – ΔOTAREFSENS | -84.4 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -86.3 – ΔOTAREFSENS | -86.2 – ΔOTAREFSENS | -86 – ΔOTAREFSENS | -84.7 - ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86.4 – ΔOTAREFSENS | -86.3 – ΔOTAREFSENS | -86.1 – ΔOTAREFSENS | -84.8 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

------------------------------ Next modified section -------------------------

Table 7.4.5.2-1: Wide Area BS dynamic range for f ≤ 6.0 GHz

| BS | Subcarrier | Reference | Wanted signal mean power (dBm) |  |  | Interfering | Type of |
| --- | --- | --- | --- | --- | --- | --- | --- |
| channel bandwidth (MHz) | spacing (kHz) | measurement channel (annex A.2) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | signal mean power (dBm) / BWConfig | interfering signal |
| 5 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -82.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -81.0 – ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -79.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -76.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -75.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -74.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -73.7– ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -73.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -72.6– ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -72.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -71.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.2-1a: Wide Area BS dynamic range for 6.0 < f ≤ 7.125 GHz

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.2) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20 | 15 | G-FR1-A2-4 | -63.2 – ΔOTAREFSENS | -75.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -63.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -63.5– ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -63.2 – ΔOTAREFSENS | -73.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -63.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -63.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -63.2 – ΔOTAREFSENS | -72.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -63.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -63.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -63.2 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -63.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -63.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -63.2 – ΔOTAREFSENS | -70.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -63.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -63.2 – ΔOTAREFSENS | -69.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -63.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -63.2 – ΔOTAREFSENS | -69.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -63.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -63.2 – ΔOTAREFSENS | -68.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -63.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -63.2 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -63.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.4.5.2-2: Medium Range BS dynamic range for f ≤ 6.0 GHz

| BS | Subcarrier | Reference | Wanted signal mean power (dBm) |  |  | Interfering | Type of |
| --- | --- | --- | --- | --- | --- | --- | --- |
| channel bandwidth (MHz) | spacing (kHz) | measurement channel (annex A.2) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | signal mean power (dBm) / BWConfig | interfering signal |
| 5 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -76.0 – ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -74.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -72.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -71.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -70.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -69.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -68.7– ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -67.6– ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -67.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -66.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.2-2a: Medium Range BS dynamic range for 6.0 < f ≤ 7.125 GHz

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.2) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20 | 15 | G-FR1-A2-4 | -58.2 – ΔOTAREFSENS | -70.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -58.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -58.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -58.2 – ΔOTAREFSENS | -68.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -58.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -58.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -58.2 – ΔOTAREFSENS | -67.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -58.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -58.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -58.2 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -58.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -58.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -58.2 – ΔOTAREFSENS | -65.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -58.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -58.2 – ΔOTAREFSENS | -64.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -58.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -58.2 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -58.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -58.2 – ΔOTAREFSENS | -63.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -58.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -58.2 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -58.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.4.5.2-3: Local Area BS dynamic range for f ≤ 6.0 GHz

| BS | Subcarrier | Reference | Wanted signal mean power (dBm) |  |  | Interfering | Type of |
| --- | --- | --- | --- | --- | --- | --- | --- |
| channel bandwidth (MHz) | spacing (kHz) | measurement channel (annex A.2) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | signal mean power (dBm) / BWConfig | interfering signal |
| 5 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -74.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -73.0 – ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -71.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -69.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -68.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -66.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -65.7 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -64.6 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -63.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

------------------------------ Next modified section -------------------------

Table 7.5.1.5.2-1: OTA ACS requirement for BS type 1-O for f ≤ 6.0 GHz

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) |  |  | Interfering signal mean power (dBm) |
| --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |
| 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 (Note 1) | EISminSENS + 6 dB |  |  | Wide Area BS: -52 – ΔminSENS Medium Range BS: -47– ΔminSENS Local Area BS: -44– ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.2.1. |  |  |  |  |

Table 7.5.1.5.2-1a: OTA ACS requirement for BS type 1-O for band 6.0 GHz < f ≤ 7.125 GHz

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 20, 30, 40, 50, 60, 70, 80, 90, 100 (Note 1) | EISminSENS + 6 dB | Wide Area BS: -55 – ΔminSENS Medium Range BS: -50– ΔminSENS Local Area BS: -47– ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.2.1. |  |  |

Table 7.5.1.5.2-2: OTA ACS interferer frequency offset for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 7 | ±2.5075 |  |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 30 | ±9.4725 |  |
| 35 | ±9.4625 |  |
| 40 | ±9.4675 |  |
| 45 | ±9.4725 |  |
| 50 | ±9.4625 |  |
| 60 | ±9.4725 |  |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

------------------------------ Next modified section -------------------------

Table 7.5.2.5.2-1: General OTA blocking requirement for BS type 1-O

| BS channel bandwidth of the | Wanted signal mean power (dBm) |  |  | Interfering signal mean | Interfering signal centre frequency minimum offset | Type of interfering |
| --- | --- | --- | --- | --- | --- | --- |
| lowest/highest carrier received (MHz) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 7.125 GHz | power (dBm) | from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | signal |
| 5, 7, 10, 15, 20 | EISREFSENS + x dB (NOTE 2, NOTE 4) |  |  | Wide Area BS: -43 - ΔOTAREFSENS Medium Range BS: -38 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS (NOTE 2) | ±7.5 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
|  | EISminSENS + x dB (NOTE 3, NOTE 4) |  |  | Wide Area BS: -43 - ΔminSENS Medium Range BS: -38 - ΔminSENS Local Area BS: -35 - ΔminSENS (NOTE 3) |  |  |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + x dB (NOTE 2, NOTE 4) |  |  | Wide Area BS: -43 - ΔOTAREFSENS Medium Range BS: -38 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS (NOTE 2) | ±30 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
|  | EISminSENS + x dB (NOTE 3, NOTE 4) |  |  | Wide Area BS: -43 - ΔminSENS Medium Range BS: -38 - ΔminSENS Local Area BS: -35 - ΔminSENS (NOTE 3) |  |  |
| NOTE 1: EISREFSENS and EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.3.2 and 10.2.1. NOTE 2: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 3: This test requirement is only applied in the OTA minSENS receiver target reference direction. NOTE 4: For a BS capable of single band operation only, "x" is equal to 6 dB. For a BS capable of multi-band operation, "x" is equal to 6 dB in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in the in-band blocking frequency range of an adjacent or overlapping operating band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, "x" is equal to 1.4 dB. |  |  |  |  |  |  |

Table 7.5.2.5.2-2: OTA narrowband blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA Wanted signal mean power (dBm) |  |  | OTA Interfering signal mean power (dBm) |
| --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |
| 5, 7, 10, 15, 20 | EISREFSENS + 6 dB (NOTE 3) |  |  | Wide Area BS: -49 - ΔOTAREFSENS Medium Range BS: -44 - ΔOTAREFSENS Local Area BS: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB (NOTE 4) |  |  | Wide Area BS: -49 - ΔminSENS Medium Range BS: -44 - ΔminSENS Local Area BS: -41 - ΔminSENS |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB (NOTE 3) |  |  | Wide Area BS: -49 - ΔOTAREFSENS Medium Range BS: -44 - ΔOTAREFSENS Local Area BS: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB (NOTE 4) |  |  | Wide Area BS: -49 - ΔminSENS Medium Range BS: -44 - ΔminSENS Local Area BS: -41 - ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: EISREFSENS and EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.3.2 and 10.2.1. NOTE 3: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 4: This test requirement is only applied in the OTA minSENS receiver target reference direction. NOTE 5: 7.5 kHz shift is not applied to the wanted signal. |  |  |  |  |

Table 7.5.2.5.2-3: OTA narrowband blocking interferer frequency offsets for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±(350 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 7, 10 | ±(355 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±(360 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±(350 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 30 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 35 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 40 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 45 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 50 | ±(560 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 60 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 70 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 80 | ±(560 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 90 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 100 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| NOTE 1: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

------------------------------ Next modified section -------------------------

Table 7.8.5.1-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper base station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 7 | ±7.45 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 10 | ±7.465 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 15 | ±7.43 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 20 | ±7.395 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 25 | ±7.465 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 30 | ±7.43 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 35 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal, (Note 2) |
| 40 | ±7.45 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 45 | ±7.37 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal, (Note 2) |
| 50 | ±7.35 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 60 | ±7.49 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 70 | ±7.42 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 80 | ±7.44 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 90 | ±7.46 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 100 | ±7.48 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| NOTE 1: For the 15 kHz subcarrier spacing, the number of RB is 25. For the 30 kHz subcarrier spacing, the number of RB is 10. NOTE 2: For the 15 kHz subcarrier spacing, the number of RB is 100. For the 30 kHz subcarrier spacing, the number of RB is 50. For the 60 kHz subcarrier spacing, the number of RB is 24. NOTE 3: The RBs shall be placed adjacent to the transmission bandwidth configuration edge which is closer to the Base Station RF Bandwidth edge. |  |  |

Table 7.8.5.1-3: Narrowband intermodulation performance requirement in FR1

| BS class | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB (Note 1) | -52 - ΔOTAREFSENS | See table 7.8.5.1-4 |
|  | EISminSENS + 6 dB (Note 1) | -52 - ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6 dB (Note 1) | -47 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB (Note 1) | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -44 - ΔminSENS |  |
| NOTE: EISREFSENS and EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.3.2 and 10.2.1. |  |  |  |

Table 7.8.5.1-4: Interfering signals for narrowband intermodulation requirement in FR1

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±360 | CW |
|  | ±1420 | 5MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 7 | ±400 | CW |
|  | ±1240 | 5MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 10 | ±370 | CW |
|  | ±1960 | 5MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 15 (NOTE 2) | ±380 | CW |
|  | ±1960 | 5MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 20 (NOTE 2) | ±390 | CW |
|  | ±2320 | 5MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 25 (NOTE 2) | ±325 | CW |
|  | ±2350 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 30 (NOTE 2) | ±335 | CW |
|  | ±2350 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 35 (NOTE 2) | ±345 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 40 (NOTE 2) | ±355 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 45 (NOTE 2) | ±365 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 50 (NOTE 2) | ±375 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 60 (NOTE 2) | ±395 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 70 (NOTE 2) | ±415 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 80 (NOTE 2) | ±435 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 90 (NOTE 2) | ±365 | CW |
|  | ±2530 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 100 (NOTE 2) | ±385 | CW |
|  | ±2530 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 3: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

------------------------------ Next modified section -------------------------

Table 7.9.5.1-1: Wide Area BS in-channel selectivity for f ≤ 6 GHz

| BS channel bandwidth | Subcarrier spacing | Reference measurement | Wanted signal mean power (dBm) |  |  | Interfering signal mean | Type of interfering |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (MHz) | (kHz) | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | power (dBm) | signal |
| 5 | 15 | G-FR1-A1-7 | -98.9-ΔminSENS | -98.5-ΔminSENS | -98.2-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 | 15 | G-FR1-A1-7 | -98.9-ΔminSENS | -98.5-ΔminSENS | -98.2-ΔminSENS | -79.6 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -97-ΔminSENS | -96.6-ΔminSENS | -96.3-ΔminSENS | -77.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -90.6-ΔminSENS | -90.2-ΔminSENS | -89.9-ΔminSENS | -71.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -99.6-ΔminSENS | -99.2-ΔminSENS | -98.9-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -97.1-ΔminSENS | -96.7-ΔminSENS | -96.4-ΔminSENS | -78.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -90.9-ΔminSENS | -90.5-ΔminSENS | -90.2-ΔminSENS | -71.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -96.5-ΔminSENS | -96.1-ΔminSENS | -95.8-ΔminSENS | -78.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -91-ΔminSENS | -90.6-ΔminSENS | -90.3-ΔminSENS | -71.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.9.5.1-1a: Wide Area BS in-channel selectivity for 6 < f ≤ 7.125 GHz

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.1) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20, 30 | 15 | G-FR1-A1-1 | -94.9 -ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 50 | 15 | G-FR1-A1-4 | -88.5 -ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 20, 30 | 30 | G-FR1-A1-2 | -95.0 -ΔminSENS | -77.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -88.8 -ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 20, 30 | 60 | G-FR1-A1-9 | -94.4 -ΔminSENS | -77.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -88.9 -ΔminSENS | -70.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |

Table 7.9.5.1-2: Medium Range BS in-channel selectivity for f ≤ 6.0 GHz

| BS channel bandwidth | Subcarrier spacing | Reference measurement | Wanted signal mean power (dBm) |  |  | Interfering signal mean | Type of interfering |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (MHz) | (kHz) | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | power (dBm) | signal |
| 5 | 15 | G-FR1-A1-7 | -93.9-ΔminSENS | -93.5-ΔminSENS | -93.2-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 | 15 | G-FR1-A1-7 | -93.9-ΔminSENS | -93.5-ΔminSENS | -93.2-ΔminSENS | -74.6 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -92-ΔminSENS | -91.6-ΔminSENS | -91.3-ΔminSENS | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -85.6-ΔminSENS | -85.2-ΔminSENS | -84.9-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -94.6-ΔminSENS | -94.2-ΔminSENS | -93.9-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -92.1-ΔminSENS | -91.7-ΔminSENS | -91.4-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -85.9-ΔminSENS | -85.5-ΔminSENS | -85.2-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -91.5-ΔminSENS | -91.1-ΔminSENS | -90.8-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86-ΔminSENS | -85.6-ΔminSENS | -85.3-ΔminSENS | -66.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.9.5.1-2a: Medium Range BS in-channel selectivity for 6.0 < f ≤ 7.125 GHz

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.1) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20, 30 | 15 | G-FR1-A1-1 | -89.9 -ΔminSENS | -71.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 50 | 15 | G-FR1-A1-4 | -83.5 -ΔminSENS | -65.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 20, 30 | 30 | G-FR1-A1-2 | -90.0 -ΔminSENS | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -83.8 -ΔminSENS | -65.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 20, 30 | 60 | G-FR1-A1-9 | -89.4 -ΔminSENS | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -83.9 -ΔminSENS | -65.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |

Table 7.9.5.1-3: Local area BS in-channel selectivity for f ≤ 6.0 GHz

| BS channel bandwidth | Subcarrier spacing | Reference measurement | Wanted signal mean power (dBm) |  |  | Interfering signal mean | Type of interfering |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (MHz) | (kHz) | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | power (dBm) | signal |
| 5 | 15 | G-FR1-A1-7 | -90.9-ΔminSENS | -90.5-ΔminSENS | -90.2-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 | 15 | G-FR1-A1-7 | -90.9-ΔminSENS | -90.5-ΔminSENS | -90.2-ΔminSENS | -71.6 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -89-ΔminSENS | -88.6-ΔminSENS | -88.3-ΔminSENS | -69.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -82.6-ΔminSENS | -82.2-ΔminSENS | -81.9-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -91.6-ΔminSENS | -91.2-ΔminSENS | -90.9-ΔminSENS | -73.4 - ΔminSENS | DFT-s- NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -89.1-ΔminSENS | -88.7-ΔminSENS | -88.4-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -82.9-ΔminSENS | -82.5-ΔminSENS | -82.2-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -88.5-ΔminSENS | -88.1-ΔminSENS | -87.8-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -83-ΔminSENS | -82.6-ΔminSENS | -82.3-ΔminSENS | -63.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

------------------------------ End of modified section -------------------------