3GPP TSG-RAN Meeting #109	RP-252491

Beijing, China, September 15th- 18th 2025

Agenda Item:	9.6.4.1

Source:	Qualcomm Incorporated

Title:	Release independence for 6L

Document for:	Discussion and Decision

1	Introduction

Introduction of 6Rx with 4L/6L has been a part of Rel-19 package under the UE RF enhancements WI [1]. Introduction of some of the requirements in a release independent way has been discussed in RAN4 but the discussion was not concluded. Also, some questions on the feasibility of introducing requirements in a release independent way was raised. In this paper we discuss the release independence handling of the requirements for 6L and we briefly touch on the general approach to defining requirements in a release independent way.

2	Discussion

2.1 Release Independent Introduction of Requirements and Features

3GPP has introduced the release independence concept such that certain features or functionalities introduced later can be implemented from a previous release. This mechanism enables faster time to market and simpler implementation since it is not necessary for a UE to support all other features/functionalities defined up to the current release. The mechanism has been extensively used for new frequency bands (usually made available from the 1st release of a certain RAT), band combinations, higher order MIMO, etc.

The release independence mechanism has in general two components: the signaling defined in RAN2 which is introduced such that a feature can be advertised and configured from a previous release and the RAN4 requirements referenced in TS 38.307 indicating the requirements which should be met by a UE implementing the feature from an earlier release.

For the RAN2 signaling part, the RAN2 CRs with “magic” sentence “implementable from an earlier release” are captured in Annex C of TS 38.331 to indicate that the feature can be implemented from an earlier release. For example, CRs with signalling changes for BCS4/5, NS value extension and FGB2 BW classes for FR2 are listed there.

Observation 1: Release independent handling of features is possible through RAN2 signaling and RAN4 requirement referencing in 38.307.

Observation 2: The release independence mechanism is important to enable faster time to market and simpler inter-dependence of features.

2.2 Release independence handling of 6Rx/6L

RAN4 has discussed introducing the requirements for 6Rx/6L and/or 6Rx/4L for FWA/handheld devices in a release independent manner from Rel-18. A tentative agreement was reached to introduce 6Rx/6L requirements for FWA in a release independent manner from Rel-18, however, the RAN4 chair has expressed some concerns about the possible misuse of release independence in this case as new signaling for this feature might be needed. An LS asking RAN2 whether supporting this feature from Rel-18 is feasible from a RAN2 specification point of view was agreed, however the LS does not contain any other actions for RAN2. According to the RAN4 discussion, plenary guidance will be sought on the use of the release independence mechanism.

Based on the observations in Section 2.1, introduction of the 6Rx/6L requirements is clearly feasible. Also, only a new capability signaling is needed for this feature whose definition in the RAN2 specification is trivial. New signaling in order to configure the UE with this feature is not needed as it is already supported by current RRC signaling.

Furthermore, considering the RAN4 discussions quoted below and the strong request from operators [3], RAN plenary should already agree to introduce the 6Rx/6L requirements for FWA in a release independent manner from Rel-18.

RAN4 chiar notes from RAN4#116:

Chair’s opinion:

- RAN4 requests RAN2 to provide feedback that if the signaling associated with new 6Rx requirements in R19 will be specified. If YES, can this signaling be introduced in R18.

- Depending on RAN2’s response, RAN4 can further discuss if release independence is feasible for this feature with new R19 signaling .

- If the answers in both 1) and 2) are YES, the following agreement will be followed.

  - Support 6Rx requirements with up to 4 MIMO layers for handheld UE from Rel-19

  - Support 6Rx requirements for FWA UE from Rel-18 including 4 MIMO and 6 MIMO layer requirements

- Since the new 6Rx requirements are performance part, the corresponding release independence discussion should be performance part too. That means the discussion beyond Sept. plenary will not delay the completion of the core part of this WI.

Proposal 1: Introduce 6Rx/6L requirements from Rel-18.

2.3 General Handling of Release Independence

Based on the RAN4 discussion, plenary should discuss the general use of release independence and for which features/requirements it is appropriate. As shown in Section 2.1 this mechanism has existed for a long time to support the needs of the 3GPP ecosystem. 3GPP should continue to treat the introduction of release independence on a case by case basis considering technical aspects and market needs.

Proposal 2: Continue to treat release independence on a case by case basis.

2.4 RAN2 CRs on 6-layer MIMO UE capability

For the introduction of 6-layer MIMO UE capability parameter, RAN2 has endorsed two sets of CRs, for RAN plenary to approve one of the two, as follows.

Set 1: Without “FWA” text (Xiaomi, Intel Corporation, Oppo, Ericsson, Nokia, Qualcomm Incorporated, ZTE, Sanechips, CATT, T-mobile USA, CHTTL)

- Rel-19 CR to 38.331 in R2-2506259

- Rel-19 CR to 38.306 in R2-2506260

Set2: With “FWA” text (Huawei, HiSilicon, Samsung, MediaTek Inc., Apple)

- Rel-19 CR to 38.331 in R2-2506466

- Rel-19 CR to 38.306 in R2-2506467

RAN2 has been defining UE capability parameters as per RAN4 request. The following features indeed are only applicable to FWA according to RAN4 specification, but RAN2 never mentioned “FWA” in the corresponding UE capability parameters in TS38.306.

- 8Rx (implicitly supporting 8L)

- PC1, PC5 for FR2

- Different MPR/A-MPR values for PC1.5

As RAN4 makes enhances on features, correctly reflecting the latest status of RAN4 in TS38.306 has never been an easy task in RAN2. In some cases, discrepancies between RAN4 and RAN2 were left as they are without being corrected for a long time. Recently RAN2 has been more careful about this issue and trying to avoid capturing something in TS38.306 when the same is clearly defined in RAN4 specification. This allows RAN4 to make further changes and enhancements without causing such inconsistency between RAN4 and RAN2. We believe RAN2 should continue this practice.

Proposal 3: RAN plenary to approve the set 1 CRs not mentioning “FWA”.

2.5 RAN4 CR to TS 38.307

RAN4 can enable 6Rx/6L feature release-independent from Rel-18 by submitting CR to Rel-19 version of TS 38.307. An example of CR to Rel-19 TS 38.307 is drafted as below.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| -------- Skip the unchanged context -------- |  |  |  |
| RF requirements for 6Rx UE | Rel-18 | Table B.4.x-1 | Support 6Rx requirements for FWA UE from Rel-18 including 4 MIMO and 6 MIMO layer requirements |
| Demodulation requirements for 6Rx UE | Rel-18 | Table B.3.y-1 | Support 6Rx requirements for FWA UE from Rel-18 including 4 MIMO and 6 MIMO layer requirements |
| NOTE 1: Rel-15 UEs supporting the high speed train are assumed to read the Rel-16 high speed train scenario information, which is broadcast to all UEs. NOTE 2: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 3: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 4: Rel-15 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. |  |  |  |

-------- Skip the unchanged context --------

B.3.y	Common PDSCH demodulation and CQI requirements with 6Rx

The requirements and test cases listed in Table B.3.y-1 are specified in Rel-P version of TS 38.101-4 [5].

Table B.3.y-1: UE PDSCH demodulation requirements with 6Rx

| Section / Clause | Description |
| --- | --- |
| 5.2.x | PDSCH demodulation requirements for 6Rx TDD |
| 6.2.x | CQI requirements with 6Rx |

-------- Skip the unchanged context --------

B.4.x-1	Common UE RF requirements for 6Rx

The requirements and test cases listed in Table B.4.x-1 are specified in REL-P version of TS 38.101-1 [2].

Table B.4.x-1: Common UE RF requirements for 6Rx for single band in FR1

| Clause | Description |
| --- | --- |
| 6.2.4 | Configured output power |
| 7.3 | Reference sensitivity |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | Spurious emissions |

Observation 3: RAN4 can enable 6Rx/6L feature release independent from Rel-18 by submitting an CR to Rel-19 TS 38.307. The above CR draft can be used as a reference. The specific CR content could be further discussed at RAN4#116bis meeting.

3	Conclusion

In this paper we briefly discussed the mechanism of introducing new features or requirements in a release independent manner, how this applies to the 6Rx/6L requirements and the general handling of release independence for different features. We made the following observations and proposals:

Observation 1: Release independent handling of features is possible through RAN2 signaling and RAN4 requirement referencing in 38.307.

Observation 2: The release independence mechanism is important to enable faster time to market and simpler inter-dependence of features.

Proposal 1: Introduce 6Rx/6L requirements from Rel-18.

Proposal 2: Continue to treat release independence of features and/or RAN4 requirements on a case by case basis.

Observation 3: RAN4 can enable 6Rx/6L feature release independent from Rel-18 by submitting an CR to Rel-19 TS 38.307. The above CR draft can be used as a reference. The specific CR content could be further discussed at RAN4#116bis meeting.

References

[1]	RP-251816, “Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, Hisilicon, TSG RAN#108.

[2] 	R4-2511898, “LS on Release Independence of 6Rx”, T-Mobile USA, TSG-RAN4#116

[3]	R4-2509968, “On Release Independence of 6Rx”, T-Mobile USA, BT, SK Telecom. TELUS, Verizon, Deutsche Telekom, Bell Mobility, Boost Mobile, Southern Linc, Telecom Italia, Telstra, TSG-RAN4#116