3GPP TSG RAN Meeting #109						                RP-252559

Beijing, China, 15 - 18 September, 2025

Agenda Item:	9.6.3.1

Source:			China Unicom, CMCC

Title:			WI Summary for  Data collection for SON 					                      (Self-Organising Networks)/MDT (Minimization of Drive                             				Tests) in NR standalone and MR-DC (Multi-Radio Dual 					Connectivity) Phase 4

WI code(s):		NR_ENDC_SON_MDT_Ph4-Core

leading WG:		RAN WG3

Release:		Rel-19

1	Introduction

This WI specified further NR SON/MDT enhancement, supporting Mobility Robustness Optimisation (MRO) enhancement for intra-CU Lower layer triggered mobility (LTM), MRO enhancement for CHO with candidate SCGs, MRO enhancement for subsequent CPAC, SON/MDT enhancements for intra-NTN mobility, SON/MDT enhancements for Network Slicing, RACH optimization for SDT, MHI enhancement for SCG Deactivation/Activation, MRO for MR-DC SCG failure.

2	Description

The key functionalities of this WI are described as below.

MRO enhancement for intra-CU LTM

MRO for intra-CU LTM failure is to detect intra-CU LTM cell switch failures that occur due to Too Late LTM cell switch, or Too Early LTM cell switch, or LTM cell switch to Wrong Cell. In case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the gNB-CU forwards the RLF report, which is associated to an LTM mobility event from the UE or via the Failure indication message over Xn, to the last serving gNB-DU in case of too late LTM cell switch, or to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell.

Additionally, there are 3 MRO LTM use cases supported over F1 interface, including Beam Failure Recovery (BFR) after successful LTM Cell Switch to wrong beam(case 1), RACH-less LTM Cell Switch failure due to wrong beam (case 2), and Cell Switch failure due to outdated Timing Advance(TA) (case 3). For case1, the gNB-DU can do optimisation of target beam selection based on the information of SSB Index of the recovery beam used at successful Beam Failure Recovery. For case2, the gNB-DU can do optimisation of target beam selection or TA related parameters based on the information of SSB Index of the re-established or recovery beam after LTM Cell Switch Failure. For case3, the gNB-DU can optimise TA related parameters or target beam selection based on the information of TA value used at successful Random Access during LTM recovery or re-establishment after a Cell Switch failure in same cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) happened in UE side after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt). The target gNB-DU performs initial analysis and sends the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. The target gNB-DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU.

The gNB-CU is responsible for detecting LTM ping-pong issue, and gNB-CU sends relevant information to the gNB-DU whenever gNB-CU decides this is necessary with last consecutive LTM cell switch entries to the gNB-DU. Then the gNB-DU can be able to resolve LTM ping-pong based on the LTM UE History Information(UHI) information.

MRO enhancement for CHO with candidate SCGs

MRO for CHO with candidate SCGs is to detect connection failures that occur due to Too Late CHO with candidate SCG(s), Too Early CHO with candidate SCG and CHO Execution to Wrong Cell.

In case of Too Late Handover when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may send a Handover Report over Xn to the candidate NG-RAN node(s) in order to control the candidate PSCell(s) whose mobility configuration caused the failure.

In case of Too Early Handover or Handover to Wrong Cell when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may inform the NG-RAN node to control the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure has been caused by a different M-NG-RAN node, the serving M-NG-RAN node sends SCG Failure Indication message to the M-NG-RAN node that caused the failure. If it is an SN that caused the failure, the M-NG-RAN node sends SCG FAILURE INFORMATION REPORT message to the SN.

MRO enhancement for subsequent CPAC

MRO for subsequent CPAC(S-CPAC) failure is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell.

For MN initiated S-CPAC, the MN performs the root cause analysis. For initial S-CPAC failure during a MN initiated S-CPAC, the forwarding mechanism is the same as that for MN initiated CPC. For a following S-CPC failure during an MN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, the MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the initiating SN, and the initiating SN performs root cause analysis. For initial S-CPAC failure during a SN initiated S-CPAC, the forwarding mechanism is the same as that for SN initiated CPC. The source SN is also the initiating SN at S-CPAC preparation.For a following S-CPC failure during a SN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the initiating SN at CPAC preparation, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation.

In case of S-CPAC, when an SN is released, the MN may provide the latest SCG UHI to the SN using the M-NG-RAN node initiated S-NG-RAN node Release or the S-NG-RAN node initiated S-NG-RAN node Release procedures.

SON/MDT enhancements for intra-NTN mobility

To enable the NTN feature supported in MDT, the network configures a list of up to 8 geographical areas in MDT configuration to specify the area scope of the specific MDT task, while UE will only log measurements when camping in any cell that belongs to the configured geographical areas.  Both logged and immediate MDT are supported within the geographical area scope.

SON/MDT enhancements for Network Slicing

To enable the network slicing feature supported in MDT, the network configures Network Slice Area Scope in MDT configuration for MDT measurement collection, while UE will only support log measurements when using the particular slice(s) that belongs to the configured Network Slice Area Scope.

Support of the leftovers in Rel-18 SON/MDT

There are three leftovers functionalities in Rel-18, including RACH optimization for SDT, MHI Enhancement for SCG Deactivation/Activation, and MRO for MR-DC SCG failure.

The functionality of RACH optimization for SDT supports to detect and optimize the non-optimal the configurations of the conditions and parameters of SDT which may result in SDT data transmission fails. The UE can log and report SDT related configuration parameters in the RA report for SDT.

For MHI enhancement for SCG Deactivation/Activation, the information of SCG activation and/or deactivation could assist the network to obtain the actual utilization status of Dual Connectivity and further optimize the configuration of Dual Connectivity. To enable the network to control the coverage of network and adjust the mobility related parameters, both UHI and MHI are enhanced to include the information about time spent in the PSCell in activated and deactivated state.

MRO for MR-DC SCG failure includes both EN-DC scenario and NGEN-DC scenario. To enhance EN-DC case, the new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface is introduced to transfer SCG failure related information between MN and SN for root cause analysis.

3	References

[1]	RP-234038, New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2]	RP-252558, SR for NR_ENDC_SON_MDT_phase 4