3GPP TSG-RAN Meeting #109	RP-252573

Beijing, China, September 15th – 8th 2025

Agenda Item:	9.1.4.5

Source:	Ericsson

Title:	RAN4 led 5G-A Rel-20 demodulation objectives

Document for:	Discussion

# 1	Introduction

In this contribution, we present our view on 5G-A Rel-20 RAN4 led demodulation objectives.

# 2	Discussion

The WF discussed at RAN4#108 contained the following conclusions on demodulation objectives:

The high-level scope for WI on Demod enhancement (the exact scope and WID draft will be discussed and decided in RAN#109):

- Normative work on SCM based requirements for SU-MIMO in FR1

  - The exact scope also depends on the conclusion of R19 study

- Study on SCM for MU-MIMO

- Performance requirements for 6Rx UE with interference

  - Up to 4 MIMO layers for the target cell is considered

- No RAN1/2 impact is expected

- Target 0.5 TU allocation from Q1’26 and 1 TU from Q2’26.

Development of a spatial channel model is important in order that advanced MIMO features, that are based on AAS basestation systems that aim to take advantage of spatial characteristics of the radio channel function properly and bring a return on the investment on the network and UE sides. During Rel-15, there was an attempt to introduce CDL channel models for demodulation requirements, but this was not successful due to lack of convergence in the available time. Meanwhile, all MIMO feature development and evaluation in RAN1 is based on CDL.

During Release 19, a Study has taken place in RAN4 to develop a stable, reproducible spatial channel model and to verify that the CDL model is indeed the most relevant model to consider. The study is now converging, at least for the case of FR1 and SU-MIMO.

During Release 20, it is useful to create an additional UE requirement based on the spatial channel model, in order to enhance 5G and to be ready for 6G. The requirement should be straightforward and should be in addition to, not a replacement of, existing requirements. We propose that the following is considered for a release 20 requirement:

- PDSCH requirement for SU-MIMO (8 layers)

- Type II PMI reporting requirement for SU-MIMO

Create optional demodulation requirements based on SCM:

  - PDSCH requirement for SU-MIMO (8 layers)

  - Type II PMI reporting requirement for SU-MIMO

In addition, it is proposed to continue to study the spatial channel model. Areas that have not been studied in Rel-19 include MU-MIMO, FR2 and SCM for AI. For the MU-MIMO case, the study is essentially about an interference model for a multi-user system. Regarding AI, a key issue to study is how to extend the model to incorporate more diversity into the model parameters and in particular the spatial directions of arrival, in order to adequately test that the AI model generalizes and is not overtrained for specific spatial modes. The SCM should also be studied for 6G. For 5G, we propose to continue with a study of MU-MIMO.

Study further MU-MIMO SCM. Within the context of 6G, further study SCM for AI/ML.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Create optional demodulation requirements based on SCM:

o	PDSCH requirement for SU-MIMO (8 layers)

o	Type II PMI reporting requirement for SU-MIMO

Proposal 2	Study further MU-MIMO SCM. Within the context of 6G, further study SCM for AI/ML.