3GPP TSG-RAN Meeting #109	RP-252574

Beijing, China, September 14th – 18th September 2025

Agenda Item:	9.1.4.7

Source:	Ericsson

Title:	On cross-area related aspects of the RAN4 led Rel-20 5G-A package

Document for:	Discussion

# 1	Introduction

This document presents considerations on cross-area aspects of the 5G-A RAN4 led package.

# 2	Discussion

The WF discussed at RAN4#108 on cross-area topics was as follows:

The following two WI are agreed to be introduced in R20.

- Extension of LBCA (the exact scope and WID draft will be discussed and decided in RAN#109):

  - Further discussion on the possible scoping in RAN#109 based on the following two objectives

    - Study and if feasible, specify non-RRC based switching solution, e.g. dynamic switching, by taking the RAN1 impacts into consideration

    - Dual DL solution

    - RAN1/2 impact is expected and will be further discussed in RAN#109

  - Target 0.5 TU (0.25 RF and 0.25 RRM) in Q4’25 and 0.75 TU allocation (0.25 RF and 0.5 RRM) from Q1’26

- UAV (the exact scope and WID draft will be discussed and decided in RAN#109):

  - UAV UE with antenna array with up to 33dBm maximum output power on TDD spectrum

  - RAN1/2 scope is not expected

  - Target 0.5 TU allocation (0.25 RF and 0.25 RRM)

The following two proposals will be discussed at RAN#109, where it will be decided whether or not to include either of the proposals

  - Fragmented DL:

    - Introducing RF/RRM requirements related to new UE behavior for reducing the number of UE Rx RF chains

    - RAN1/2 scope is TBC

  - Intra-band non-collocated EN-DC/NR-CA deployment

    - Type 3 UE requirements assuming MRTD<CP and power imbalance < [xx] dB for handheld

    - RAN1/2 scope is TBC

Two cross-topic proposals have been agreed for inclusion into the release-20 packaging; LB-LB switching and high-power UAV.

The LB-LB switching follows on from the Rel-19 WI, which considered switching between FDD (UL/DL) and SDL carriers. The Release 20 extension would incorporate the “dual DL” architecture, which would enable switching between FDD (UL/DL) and (SDL + FDD DL). This would mean that the FDD DL carrier would be available all of the time.

It is not desirable to consider a fast switching between the carriers that would require tight co-ordination; e.g. cross-carrier based scheduling. However, with dual downlink simpler possibilities for enabling a greater flexibility of switching patterns exist. Greater flexibility would be very beneficial; in particular for ensuring that during UL bursts, the UE could rapidly switch to continuous FDD in order not to lose the coverage advantage of the low band. A potential example of more flexible switching would be switching to FDD when uplink is scheduled. The mechanism would be similar to those used for UL switching and would greatly improve the benefits of the LB-LB switching feature.

For LB-LB CA, consider non-RRC based switching that is similar to UL switching and does not have large RAN1 impact, e.g. uplink activity based.

The high power UAV proposal is related to the envisaged vertical economy. To enable high power UAVs, co-existence study is needed. However, co-existence between ground networks and UAVs is different to co-existence between different ground systems. An aggressor UE flying directly above a victim BS would be at an angle with respect to the BS antenna with low-gain. However, a UAV further away from a victim BS would be closer to the horizon, for which there would be a greater amount of BS antenna gain. UAVs in the air have a line of sight propagation to basestations, which means that interference power from UAVs that are several km or 10s of km away from the BS may be received. A victim ground basestation could be impacted by accumulated power from a potentially large number of UEs that are in the order of up to 10s of km away and at the horizon.

For UAV, co-existence study is required. The co-existence study needs to take into account the special circumstances of UAV with accumulated interference over long distances.

It is also important to consider the impact of adjacent channel networks on UAVs, even though this is not directly related to HPUE operation. A UAV may have line of sight to a large number of TN basestations. For basestations directly below, the antenna gain of the basestations may be low, but more distant basestations for which the UAV is on the horizon may contribute more interference power. It may be important to establish that the accumulated power from the ground basestations does not cause degradation to the UAVs. We note that, during the ATG WI, it was found that for below 3000m, there may be losses due to selectivity towards adjacent channel basestations.

Study of co-existence degradations to UAV receivers is also necessary.

In case the UAV UE performs beamforming, there may also be RRM impacts.

RRM impacts of UAV based UE beamforming should be considered.

Two further cross-topic proposals remain for further consideration; fragmented carriers and intra-band non-colocated type 3 UE.

It should be noted that the workload for the other parts of the RAN4 package is already very high, and in our view there is no room to accommodate further work.

There is no room to accommodate additional work on intra-band non-co-located or fragmented carriers on top of the other parts of the package discussed at RAN4#108.

Regarding fragmented carriers, the Study Item established that there may be significant sensitivity degradation with single RX chain for FDD band combinations. It is essential that PCell is not degraded, and so any single RX operation should be restricted to SCell. The degradation will not exist for TDD. As discussed during the Study Item, from a network point of view, degrading ACS is highly undesirable, and any WI should not consider ACS degradation.

Due to the extremely high workload of RAN4, any WI on fragmented carriers should be extremely simple, and there should be a very clear need to consider the WI for 5G.

Any fragmented carriers WI needs to be very simple in scope and have a very commercial motivation why it is needed in the 5G-A Rel-20 timescale.

Any fragmented carriers WI should as a basis not degrade PCell and not relax requirements.

Regarding the inter-band non-co-located, the general issue of enabling CA for non-co-located deployments has been solved with a variety of UE architectures over two Releases. During these discussions, it has also become clear that the type 3 receiver is unlikely to be implementable on either the UE or the network side. Since good solutions already exist to the issue and the optimization of type 3 would be difficult to agree, and considering the overall RAN4 workload situation it is preferable not to address this scenario further.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For LB-LB CA, consider non-RRC based switching that is similar to UL switching and does not have large RAN1 impact, e.g. uplink activity based.

Proposal 2	For UAV, co-existence study is required. The co-existence study needs to take into account the special circumstances of UAV with accumulated interference over long distances.

Proposal 3	Study of co-existence degradations to UAV receivers is also necessary.

Proposal 4	RRM impacts of UAV based UE beamforming should be considered.

Proposal 5	There is no room to accommodate additional work on intra-band non-co-located or fragmented carriers on top of the other parts of the package discussed at RAN4#108.