3GPP TSG-RAN Meeting #109	                RP-252591

Beijing, China, 15th – 18th September 2025

Agenda Item:	9.1.4.3

Source:	Eutelsat Group

Title:	Motivation for New SID OTA Testability for VSAT

Document for:	Information

# 1	Background

In May 2024 at RAN4 #111, the issue of VSAT testability was first formally presented in [1]. In parallel at RAN5 #103, a draft WID to develop VSAT conformance test requirements was presented in [2]. Following offline discussions at RAN4 #111, it was decided that RAN5 would not present [2] to RAN #104 but defer to the outcome of a draft SID on VSAT testability which was presented to RAN #104 in [3]. The precedent for RAN4 first studying new test methods ahead of RAN45 developing conformnce test cases was established in the revised RAN4 terms of reference in [4].

Since RAN #104, several attempts to start work on NTN testability have not been agreed. However, at RAN #108 in [5] it was agreed that future work on NTN testability would be covered under the Rel-20 OTA track and be separated from the NTN track.

# 2	Discussion

When RAN approved the WI NR_NTN_enh for VSATs operating above 10 GHz there was an implicit and obvious consequence on test methods since the size of VSATs is much larger than the size of the FR2 handsets used to develop the original FR2 NR radiated test systems whose quiet zone was only 15 cm (now 40 cm). Also, the FR2-NTN frequency range has a lower limit of 17.3 GHz, well below the 24.25 GHz of existing approved FR2 radiated test methods. When RAN approved the WI Introduction of Ku bands for NR NTN in June 2024, the lower frequency dropped further to 10.7 GHz.

In addition to the frequency range and VSAT device size issues, the introduction of Ka band added the issue of polarization. Current radiated test systems are all based on cross polarized antennas, but Ka band operates with circular polarization. Unfortunately, there is no obvious way to convert existing test systems to use circular polarization without reverting to using a vector network analyzer approach which would mean the loss of the L3 signalling connectivity normally provided by a base station emulator, which lacks the phase control to generate stable circular polarization.

The need to address the large and growing gap in VSAT testability is further evidenced by a Feb 2025 industry press release [6] which announced the first successful connection between a Ku band VSAT using an NR air interface and a LEO constellation. This indicates the demand for 3GPP-based NTN standards since current commercial developments are ahead of 3GPP rather than being a couple of years behind the standards There has already been a 16-month delay since VSAT  testability issues were first raised at RAN4 #111 and this is impeding the ability of RAN5 to work on conformance test for Ka band and shortly Ku band.

The key current and future VSAT testability issues are:

- VSAT device sizes exceed by far what is currently testable in existing 3GPP FR2 test systems

- The introduction of Ka band with its 17.3 GHz lower limit falls well below the range of existing FR2 test systems of 24.25 GHz.

- The introduction of Ku band (10.7 and 14.5 GHz) further challenges the operating range of existing test methods.

- The need to support circular polarization adds a significant challenge to existing test systems which may require regressing to using non-signalling methods.

The new SID in [7] is the proposal for how to progress these issues.

# 3	 Proposal

Proposal: It is proposed to approve at RAN #109 the new SID in [7], which has broad industry support, to commence the work on developing VSAT test methods following the completion in June 2024 of the core requirements specification for Ka band VSAT, and the completion of the priority 1 bands in the Rel-19 WI NR_NTN_Ku_bands WI to be approved at RAN #109.

# 4	References

[1] R4-2407232 VSAT performance requirements and testability (Eutelsat Group, RAN4 #111 Fukuoka May 2024)

[2] R5-243355 New WID on UE Conformance - NR NTN (Non-Terrestrial Networks) enhancements plus CT aspects (Qualcomm Inc, China Unicom, MediaTek, RAN5 #103 Fukuoka May May 2024)

[3] RP-241171 New SID on VSAT test methods (Eutelsat Group, RAN #104 Shanghai June 2024)

[8] RP-210786 Terms of Reference (ToR) for 3GPP TSG RAN WG4 (RAN4), RAN#91-e, Mar 2021

[5] R4-251875 Way forward on RAN4 R20 package (RAN4 Chair Apple)

[6] https://www.eutelsat.com/en/news/press.html#/pressreleases/eutelsat-mediatek-and-airbus-announce-worlds-first-5g-non-terrestrial-network-connection-leveraging-oneweb-leo-satellites-3371519

[7] RP-252590 New SID OTA Testability for VSAT (Eutelsat Group, RAN #109 Incheon September 2025)