3GPP TSG RAN WG1 #122	        		   			R1-2506662

# Bangalore, India, August 25th – 29th, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0723 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Non-Terrestrial Networks (NTN) for NR Phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-09-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Duplicated text with TS 38.214 v19.0.0 in clause 8.4. - Missing description for Rel-19 intra-slot PDCCH repetitions in clause 10.1. - Missing description for the counting of PDCCH candidates for inter-slot PDCCH repetition and for intra-slot PDCCH repetition in clause 10.1. - Incorrect timeline conditions for HD-UE in NTN in clause 17.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove duplicated text with TS 38.214 v19.0.0 in clause 8.4. - Add description for Rel-19 intra-slot PDCCH repetitions in clause 10.1. - Capture the counting of PDCCH candidates for inter-slot PDCCH repetition and for intra-slot PDCCH repetition in clause 10.1. - Correct the timeline conditions for HD-UE in NTN in clause 17.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete/incorrect specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4, 10.1, 17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged parts are omitted ***

## 8.4	PDSCH with UE contention resolution identity

In response to a PUSCH transmission scheduled by a RAR UL grant or corresponding PUSCH retransmission scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI provided in the corresponding RAR message when a UE has not been provided a C-RNTI, the UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding TC-RNTI scheduling a PDSCH that includes a UE contention resolution identity [11, TS 38.321]. If SIB1 provides msg4-NumberofRepetitions, the UE may indicate FG-XYZ in the PUSCH transmission.

In response to the PDSCH reception with the UE contention resolution identity, the UE transmits HARQ-ACK information in a PUCCH. The PUCCH transmission is within a same active UL BWP as the PUSCH transmission. A minimum time between the last symbol of the PDSCH reception and the first symbol of the corresponding PUCCH transmission with the HARQ-ACK information is equal to NT,1+0.5 msec. NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

When detecting a DCI format in response to a PUSCH transmission scheduled by a RAR UL grant, as described in [11, TS 38.321], or corresponding PUSCH retransmission scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI provided in the corresponding RAR message [11, TS 38.321], the UE may assume the PDCCH carrying the DCI format has the same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block the UE used for PRACH association, as described in clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format.

*** Unchanged parts are omitted ***

## 10.1	UE procedure for determining physical downlink control channel assignment

*** Unchanged parts are omitted ***

For search space sets si and sj that include searchSpaceLinkingId or searchSpaceLinkingId-r19 with same value, a UE monitors, in monitoring occasions with same index according to each of search space sets si and sj in a slot, PDCCH candidates msi,nCI(L) and msj,nCI(L), with msi,nCI(L)=msj,nCI(L), for detection of a DCI format with same information. The UE expects ksi=ksj, osi=osj, Tsi=Tsj, Msi(L)=Msj(L), and a same number of non-overlapping PDCCH monitoring occasions per slot based on corresponding monitoringSymbolsWithinSlot, for search space sets si and sj.

For CORESET pi associated with the search space set si and for CORESET pj associated with the search space set sj, the UE is provided tci-PresentInDCI or tci-PresentDCI-1-2 for either none or both of CORESETs pi and pj. For CORESET pi associated with the search space set si and for CORESET pj associated with the search space set sj, the UE is either not provided coresetPoolIndex value of 1 for any of the two CORESETs, or is provided coresetPoolIndex value of 1 for both CORESETs.

A UE can indicate by numBD-twoPDCCH a capability for counting PDCCH candidates msi,nCI(L) and msj,nCI(L) associated with searchSpaceLinkingId either as 2 PDCCH candidates or as 3 PDCCH candidates.

A UE can indicate by numBD-twoPDCCH-r19 a capability for counting PDCCH candidates msi,nCI(L) and msj,nCI(L) associated with searchSpaceLinkingId-r19 either as 1 PDCCH candidate or as 2 PDCCH candidates.

For search space sets si and sj that include searchSpaceLinkingId with same value, and for search space sets sk and sl that include searchSpaceLinkingId with same value, a UE expects to simultaneously monitor PDCCH candidates msi,nCI,1(8)=msj,nCI,1(8), and msk,nCI,2(16)= msl,nCI,2(16) only if a first CCE of msi,nCI,1(8) or msj,nCI,1(8) has different index than a first CCE of msk,nCI,2(16) or msl,nCI,2(16) in a CORESET configured with cce-REG-MappingType = 'nonInterleaved' and with duration of one symbol.

If a UE

-	is provided monitoringCapabilityConfig = r16monitoringcapability for a downlink cell,

-	is provided, by searchSpaceLinkingId a same value for search space sets si and sj on the downlink cell, and

-	indicates numBD-twoPDCCH with value of 3

the UE counts each PDCCH candidate for the one of the search space sets si and sj that the UE monitors PDCCH in the later span, as two PDCCH candidates. The UE does not expect a first PDCCH candidate from search space set si or sj and a second PDCCH candidate from a search space set sk that does not include searchSpaceLinkingId to use a same set of CCEs and same scrambling in a same CORESET, and provide respective first and second DCI formats with same size, in any span other than the first span in a slot.

If the PBCH payload bit aA+7 has value 1 and subject to UE capability, a UE counts each PDCCH candidate for the Type0-PDCCH CSS set that the UE monitors in slot n0+1, as described in clause 13, as two PDCCH candidates.

A UE does not expect to be provided freqMonitorLocations for a search space set s in a serving cell if intraCellGuardBandsDL-List indicates that no intra-cell guard-bands are configured for the serving cell.

*** Unchanged parts are omitted ***

## 17.2	Half-Duplex UE in paired spectrum

A half-duplex UE (HD-UE) in paired spectrum is not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum. This clause is applicable for communication of a HD-UE on a serving cell with paired spectrum. Procedures for a HD-UE are same as described for a UE in all other clauses of this document unless stated otherwise.

A HD-UE that operates on a non-NTN serving cell, or a HD-UE that operates on an NTN serving cell and does not indicate CollisionHandlingOfHDFDDOperation, does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols. A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates CollisionHandlingOfHDFDDOperation, is scheduled to receive a PDSCH or CSI-RS in a set of symbols based on an indication by a first DCI format, and is scheduled to transmit a PUSCH, PUCCH, or PRACH that overlap with any symbol from the set of symbols based on an indication by a second DCI format, determines

-	to either receive the PDSCH or the CSI-RS or transmit the PRACH based on the HD-UE implementation;

-	to either receive the PDSCH or transmit the PUSCH or PUCCH based on the HD-UE implementation, if the first DCI format is provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set;

-	to receive the PDSCH or the CSI-RS if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is not provided ntnRedCapCollisionCase4UlPriority, and the transmission of the PUCCH or the PUSCH would not start before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format;

-	to transmit the PUSCH or PUCCH, if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and

-	the HD-UE is provided ntnRedCapCollisionCase4UlPriority, or

-	the HD-UE is not provided ntnRedCapCollisionCase4UlPriority and the transmission of the PUCCH or the PUSCH would start before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format.

A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates CollisionHandlingOfHDFDDOperation, is scheduled to receive a PDSCH or CSI-RS based on an indication by a first DCI format, and is scheduled to transmit an SRS based on an indication by a second DCI format and the transmission of the SRS in a set of symbols overlaps with the PDSCH reception or CSI-RS reception, determines

-	to either receive the PDSCH or transmit the SRS based on the HD-UE implementation, if the first DCI format is provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set;

-	to receive the PDSCH or the CSI-RS if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is not provided ntnRedCapCollisionCase4UlPriority, and the first symbol from the set of symbols is not before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format;

-	to transmit the SRS if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and

-	the HD-UE is provided ntnRedCapCollisionCase4UlPriority, or

-	the HD-UE is not provided ntnRedCapCollisionCase4UlPriority and any symbol from the set of symbols is before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format

When a PDCCH reception by a HD-UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1, the end of the PDCCH reception is the end of the PDCCH candidate that ends later.

*** Unchanged parts are omitted ***