3GPP TSG-RAN Meeting 109	RP-252652

Beijing, China, September 15-18, 2025

Agenda Item:	9.3.2.1

Source:	Huawei, HiSilicon

Title:	Discussion on AI/ML based mobility WID scope

Document for:	Discussion

1	Introduction

In the last TSG-RAN meeting, many companies have proposed their expectations with respect to the scope of AI/ML for mobility WI in Rel-20. In this contribution, we discuss the FFS points from RAN#108 meeting, considering the latest progress in RAN2 meetings.

2	Discussion

2.1 NW-side model support

In the previous RAN plenary meeting, the following note was captured in the WID regarding spatial domain prediction [1].

The objectives are as the following: - Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI. Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI.

The WID currently states that for UE sided model, spatial domain is not supported while whether to support it for the NW-sided model is to be checked in RAN#109 based on the final outcome of the SI.

RAN2 discussed this topic during our previous meeting and the following conclusion was made:

For network sided models for inference: - For cell-level temporal domain Case A, sub-case 2 the following enhancement is considered UE can report “cell-level RRM measurement results at multiple time instances in one measurement report. - For other cases there is no specification impact. Can be discussed in WI phase whether any additional enhancements are needed and justified (i.e. multi-instances reporting of beam) 3 Study item can conclude that all scenarios and sub-cases are feasible.

As can be seen above, RAN2 concluded that there are no impacts related to inference for spatial domain prediction. Hence the only impacts will be related to NW-side data collection which has been already analyzed in previous RAN2 meetings and is already included in the WID scope.

Observation 1: There is no additional specifications impact to support spatial domain prediction for NW-sided model other than NW-side data collection which is largely common to all use cases and sub-cases.

Proposal 1: Capture in the WID that spatial domain prediction is to be supported for NW-sided model.

Another aspect that is worth clarifying based on the RAN2 conclusions as mentioned above is that all sub-use cases are to be supported for NW-sided model. This may have an impact on the type of data to be supported for NW-sided data collection, as per the agreement made by RAN2:

For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed)

Since all the sub-cases were deemed feasible, we think it is worth clarifying this in the WID, e.g. by adding a corresponding note.

Proposal 2: Clarify in the WID, e.g. with a NOTE, that for NW-sided model, all studied sub-use cases for RRM measurement prediction are supported.

2.2 Beam-level prediction and direct event prediction

Another leftover issue from the last RAN plenary meeting is whether L3 beam level prediction is also supported in the WI phase.

In the last RAN2 meeting, the following agreements are achieved [2].

Agreements on L3 beam-level prediction For RRM measurement prediction, L3 beam-level prediction is feasible for network sided model. CB after spec impact/overhead etc on the different cases. For RRM measurement prediction, L3 beam-level prediction is feasible for UE sided model. However, there are concerns on complexity and other WGs.

We can see that L3 beam-level prediction is feasible for network sided model. Thus, RAN should include L3 beam-level prediction for network sided model in Rel-20 WI scope.

Proposal 3: Capture in the WID that L3 beam-level prediction for network sided model is supported.

Regarding UE-sided beam-level prediction, the agreement is nuanced: "For RRM measurement prediction, L3 beam-level prediction is feasible for UE sided model. However, there are concerns on complexity and other WGs." While technically feasible, there were some concerns raised that support of L3-beam level prediction has additional workload in RAN4.

Observation 2: Based on RAN2 analysis, beam-level prediction is beneficial and technically feasible, but to support it additional workload would have to imposed on RAN4.

When it comes to measurement event prediction with direct method, RAN2 concluded the following:

Aim for a single framework for event prediction and performance monitoring. From RAN2 point of view indirect event prediction, RSRP differences can be used as the performance metric for monitoring. Capture in the TR that there is no consensus on the feasibility of performance monitoring of the direct event prediction and this would need to be resolved in WI phase before proceeding with specifying direct event prediction.

Even though RAN2 aimed to have a common framework for both indirect and direct measurement event framework, it was not possible to achieve that, at least in terms of performance monitoring metrics. Performance monitoring is a crucial feature which ensures that the mobility performance is not deteriorated when AIML based mobility is applied.

Observation 3: Performance monitoring is a crucial element of AIML based mobility procedures. Without it, applying AIML based mobility may have a negative impact on mobility performance which is against the goals of this feature.

Observation 4: There is no consensus on the feasibility of performance monitoring of the direct event prediction.

At the same time, it should be noted that RAN4 also did not have time to fully discuss direct event prediction. There was some discussion on it, but the TR captures RAN4 conclusions only for indirect method.

#### 6.2.2.2 Potential RRM requirements For indirect event prediction, the requirements for the predicted event triggered reporting including, but not limited to, delay and accuracy requirements and the performance metrics are considered. If the report includes the predicted RSRPs corresponding to the predicted event occurrence time, the absolute and/or relative accuracy requirement for the predicted RSRP will be defined.

Observation 5: RAN4 did not have time to fully study direct event prediction and did not make any conclusions, e.g. on potential RRM requirements for direct method.

Based on observation 3-5, we believe that, in case there are concerns on the workload in RAN4, direct event prediction should be first to down-scope. In case this does not entirely address workload issues, then it is acceptable to us to drop beam level prediction for UE-sided model from the WID scope as well.

Proposal 4: In case AIML WID needs to be down-scoped due to concerns with RAN4 workload:

- Firstly, direct event prediction should be precluded from the WID as its feasibility was not confirmed.

- In case RAN4 workload issue is not resolved with bullet 1), RAN should further consider precluding beam-level prediction for UE-sided model.

3. Conclusions

In this paper, we provide our further considerations for AI/ML based mobility study in Rel-20 and give the following observation and proposals:

NW-side model support

Observation 1: There is no additional specifications impact to support spatial domain prediction for NW-sided model other than NW-side data collection which is largely common to all use cases and sub-cases.

Proposal 1: Capture in the WID that spatial domain prediction is to be supported for NW-sided model.

Proposal 2: Clarify in the WID, e.g. with a NOTE, that for NW-sided model, all studied sub-use cases for RRM measurement prediction are supported.

Beam-level prediction and direct event prediction

Observation 2: Based on RAN2 analysis, beam-level prediction is beneficial and technically feasible, but to support it additional workload would have to imposed on RAN4.

Observation 4: There is no consensus on the feasibility of performance monitoring of the direct event prediction.

Observation 5: RAN4 did not have time to fully study direct event prediction and did not make any conclusions, e.g. on potential RRM requirements for direct method.

Proposal 3: Capture in the WID that L3 beam-level prediction for network sided model is supported.

Observation 3: Performance monitoring is a crucial element of AIML based mobility procedures. Without it, applying AIML based mobility may have a negative impact on mobility performance which is against the goals of this feature.

Proposal 4: In case AIML WID needs to be down-scoped due to concerns with RAN4 workload:

- Firstly, direct event prediction should be precluded from the WID as its feasibility was not confirmed.

- In case RAN4 workload issue is not resolved with bullet 1), RAN should further cosidder precluding beam-level prediction for UE-sided model.

References

RP-251864 New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

R2_131_Chairnotes_25-08-29_13-00