3GPP TSG RAN meeting #109		                                   RP-252657

Beijing, China, September 15-18, 2025

Title: 	Discussions on SCS and Minimum Bandwidth considerations for 6G SI

Source: 	Tejas Networks Ltd. , Reliance Jio, CEWiT

Agenda item:	8.3

Document for:	Discussion and decision

# Introduction

In the 3GPP RP#108 meeting, the objectives for 6G study items were outlined in the “RP-251881_6G_WG_SID” document, focusing on Physical Layer aspects for 6G Radio (6GR). These objectives include:

Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and are also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random-access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1

As part of the SID further recommended to avoid multiple numerologies for the same band / sub-range. Also, there were aspects to define the minimum and maximum channel bandwidth for the bands/sub-bands.

At the RAN WG1 #122 meeting, agreements under AI 11.3.2 (Frame Structure) focused on subcarrier spacing (SCS) and ensuring compatibility with 5G NR to support efficient Multi-RAT Spectrum Sharing (MRSS). Additionally, under AI 11.1 (6GR Air Interface Overview), discussions addressed consulting the RAN Plenary regarding supported RF and UE bandwidths without spectrum aggregation for low-tier device types.

(AI 11.3.2) Frame Structure Including numerology and frame structure (for all duplex types), as well as compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). …. Omitted …. Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz …. Omitted …. (AI 11.1) Overview of 6GR air interface …. Omitted …. Agreement - Study and identify the lessons learned from NR spectrum utilization and aggregation framework - DC is subject to RANP decision in June 2026 - Note: MRSS aspects are separate discussion Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different …. Omitted …

This document presents Tejas Networks perspectives on the 7.5 kHz SCS for sub-GHz bands and carrier bandwidth requirements for low-tier device types in the 6GR framework.

# Views on 7.5KHz SCS for sub-GHz Bands

Background

The transition to 6G networks highlights an urgent need for improved coverage, especially in rural, remote, and complex propagation areas. Existing 5G NR numerologies, which use a minimum subcarrier spacing of 15 kHz for data channels, reveal shortcomings in environments where power is limited and coverage is critical. Although NR allows for narrower subcarrier spacings (such as 1.25 or 5 kHz) for PRACH long formats, similar coverage-oriented options are not currently available for control and data channels.

Sub-GHz bands (600-900 MHz) inherently provide better propagation yet often face challenges in reaching sufficient link budgets at the cell edge—most notably for control channel reception and low-rate data applications. The implementation of a dedicated coverage numerology has the potential to extend network reach, all while staying compatible with existing infrastructure.

To address these issues, this proposal recommends evaluating a 7.5 kHz subcarrier spacing (μ = -1) as a coverage-oriented enhancement for 6G deployment. This approach is particularly suited for sub-GHz scenarios, where favorable propagation and advanced RF transceiver phase noise performance enable practical and effective adoption.

## Justification

The deployment of a 7.5 kHz subcarrier spacing (SCS) presents notable advantages in enhancing network coverage, by allowing for greater power spectral density concentration. This configuration permits the concentration of transmitted power into narrower bandwidths, yielding substantial improvements in signal-to-noise ratio (SNR). Specifically, when the physical resource block (PRB) bandwidth is reduced from 180 kHz (15 kHz SCS) to 90 kHz (7.5 kHz SCS), theoretical SNR gains of approximately 3 dB per PRB can be realized. In the context of rural macrocell environments, where the typical path loss exponent is approximately 3.5, such SNR improvements may translate into a coverage radius extension of approximately 22%, equating to a 49% increase in the area covered. Empirical data from sub-GHz deployments further indicate that coverage limitations are most often imposed by the reception quality of control channels—such as the Physical Downlink Control Channel (PDCCH) and system information broadcasts (SSB/PBCH)—rather than by data throughput requirements. Addressing these constraints, the 7.5 kHz numerology specifically enhances the performance of these coverage-critical channels, all while preserving acceptable latency for control plane operations.

Observation 1: 7.5 kHz subcarrier spacing enables Coverage Enhancement through Power Spectral Density Concentration

The adoption of 7.5 kHz subcarrier spacing (SCS) in sub-GHz frequency bands is facilitated by the superior phase noise characteristics of contemporary RF transceivers. High-performance devices, such as the ADI ADRV9009 series, exhibit close-in phase noise figures of -125 to -130 dBc/Hz at 10 kHz offset for operation within the 600-900 MHz range. This allows for normalized carrier frequency offset (CFO) factors to remain exceptionally low—typically around 0.047 at 700 MHz with a 0.5 ppm oscillator tolerance. Such values are well within the robust tracking capabilities of modern receiver architectures.

Further, integrated phase noise assessments yield RMS phase errors of approximately 0.4-0.6° for sub-GHz settings, which correspond to a signal-to-noise ratio (SNR) degradation of less than 0.5 dB. When compared to the 3 dB SNR improvement afforded by power concentration in 7.5 kHz SCS, this results in a net system gain exceeding 2.5 dB in coverage-limited conditions. The favorable phase noise and oscillator performance of current sub-GHz transceivers ensure that the deployment of 7.5 kHz SCS is both practical and effective. The minimal impairment introduced by phase noise is substantially outweighed by the system-level coverage gains, thereby establishing 7.5 kHz SCS as a technically sound solution for enhancing coverage in challenging environments.

Observation 2: Sub-GHz Phase Noise Characteristics of RF Transceivers enable Viable Implementation of 7.5 kHz SCS.

The increased symbol duration afforded by the 7.5 kHz subcarrier spacing substantially enhances multipath resilience for large cell deployments, particularly within rural environments. Specifically, the symbol period expands from 66.67 μs (15 kHz) to 133.33 μs (7.5 kHz), with appropriately lengthened cyclic prefix options that can accommodate delay spreads of up to 10-15 μs, thus mitigating the risk of inter-symbol interference. This extended delay spread tolerance is highly advantageous in rural macro cell scenarios and in challenging propagation environments characterized by foliage, terrain-induced reflections, or significant building penetration. Empirical sub-GHz measurements indicate that RMS delay spreads often exceed 5 μs in such rural settings, a threshold at which the performance of 15 kHz numerology approaches its practical limits. The proposed 7.5 kHz configuration, by contrast, provides considerable additional margin for reliable demodulation.

Observation 3: Delay Spread Tolerance Matches Rural Propagation Requirements

The integration of 7.5 kHz subcarrier spacing (SCS) is facilitated by its seamless compatibility with established sampling rate families and FFT architectures. This numerology aligns with standard LTE/NR base rates—such as 7.68, 15.36, and 30.72 Msps—allowing for efficient implementation within existing digital signal processing frameworks. The required FFT sizes scale predictably; for example, supporting a 20 MHz bandwidth would utilize a 4096-point FFT as compared to a 2048-point FFT for 15 kHz SCS, representing an incremental computational adjustment rather than a complete architectural overhaul.

Furthermore, both base station and user equipment vendors can enable μ = -1 support through software or firmware updates to current platforms, which minimizes the need for hardware redevelopment and accelerates deployment schedules. This strategy of backward compatibility effectively lowers implementation barriers, thus fostering swift market adoption.

Observation 4: Hardware Implementation Leverages Existing Infrastructure

7.5 kHz subcarrier spacing (μ = -1) should be evaluated as a dedicated coverage numerology for 6G networks, with focus on sub-GHz frequency bands.

Define the Numerology parameter definition and scope for 7.5 kHz SCS.

We propose clarifying that the requirement in point (d), “avoiding multiple numerologies for the same band/sub-range”, should not be interpreted as prohibiting the use of a dedicated sub-band with 7.5 kHz SCS in sub-GHz bands. A strict “single SCS per band/sub-range” rule could be overly restrictive and limit further study of enhancing solutions (coverage, throughput ..etc) .

# Views on Minimum Carrier bandwidth

In Release 18, the introduction of 3 MHz channel bandwidth marked a major step toward enabling narrowband spectrum operation, especially for mission-critical and IoT use cases. The work extended NR operation below the legacy 5 MHz minimum by standardizing a 600 kHz synchronization raster, a punctured 12-RB SSB design, and band-specific raster configurations that allowed reliable initial access in constrained spectrum. These solutions ensured that UEs could perform synchronization, cell ID detection, and initial access procedures even in channels as narrow as 3 MHz, while maintaining architectural consistency with NR.

Adoption of the 3MHz Channel bandwidths in Release-18 is already a precedent for this bandwidth in 6G-R, along with a new 600KHz synchronization raster for corresponding bands. In NR, initial access was an afterthought and supported using punctured SSB’s.

The following scenarios necessitate the support of a 3 MHz bandwidth in various frequency bands for telecommunications applications:

In India, the telecommunications sector is organized into 22 distinct geographical areas known as telecom circles or Licensed Service Areas (LSAs) for the purpose of spectrum allocation, licensing, and service provision.

- 850 MHz Band in India: Within the 850 MHz spectrum band, approximately six telecom circles in India have spectrum allocations ranging between 3 MHz and 5 MHz. If 5 MHz is chosen as the minimum bandwidth, then these allotted bandwidths are not suitable for 6G radio (6GR) deployment due to bandwidth constraints.

- 900 MHz Band in India: Out of India’s 22 telecom circles, six circles have unsold spectrum blocks in the 900 MHz band, each with a bandwidth of less than 5 MHz, limiting their utilization for advanced mobile communication technologies.

- Public Protection and Disaster Relief (PPDR) in Europe: In Band 28, a paired 2 x 3 MHz Frequency Division Duplex (FDD) configuration has been designated to support PPDR operations, ensuring robust communication for emergency services.

- Band 26 and Band 8 in the United States: These bands utilize channels with a 3 MHz bandwidth to facilitate mobile telecommunications, accommodating specific operational requirements in the U.S. market.

- Rail Communication in Europe: In the 900 MHz band, a 3 MHz bandwidth is allocated to support dedicated rail communication systems, enabling reliable connectivity for railway operations across Europe.

Overall, there is a need to have a 3MHz channel BW configuration in 6G-R is sub-GHz.

There is a need to have a 3MHz channel BW configuration in 6G at least for sub-GHz frequency range.

Low-power devices, including mission-critical equipment for Public Protection and Disaster Relief (PPDR), Internet of Things (IoT) devices, and reduced-complexity devices such as those used in smart grids, smart power meters, trackside sensors, and rail communication systems, can operate effectively within a 3 MHz bandwidth. Hence, support for a 3 MHz bandwidth is essential for the low-power device category to ensure compatibility and efficient performance across these applications.

Support the 3MHz for narrowband allocations and low power device category.

For 6G, in RAN#122 various contributions recognized the clear need to continue supporting narrow bandwidths in the 3–5 MHz range. The main points of convergence were that 5 MHz serves as the practical baseline because it aligns with existing NR structures, offers around 10–15% complexity reduction compared to 20 MHz, and avoids excessive redesign of control channels. At the same time, 3 MHz was acknowledged as essential in scenarios where spectrum allocations are fragmented or where low-tier, LPWA, or IoT devices dominate.

There are technical concerns about the impact of sub-5 MHz operation on SSB and CORESET0 design, as well as PDCCH performance, emphasizing the need for specialized solutions instead of punctured SSBs. Overall, the combined conclusion is that 6G should support both 3 MHz and 5 MHz minimum bandwidths.5 MHz as the default minimum for broad device categories and performance balance, and 3 MHz as a conditional option to ensure coverage of narrowband allocations and ultra-low-cost devices. This dual approach ensures continuity with 5G NR, accommodates diverse spectrum situations, and provides a unified path forward for both mainstream and constrained device types.

While supporting 3 MHz channel bandwidth, these objectives need to be considered:

- To design unified initial access procedure compatible with 3 MHz spectrum.

- Allow to have a compact CORESET#0 and narrowband SSB structure.

- To ensure coverage and detection reliability even with restricted bandwidth.

# Conclusion

Observation 1: 7.5 kHz subcarrier spacing enables Coverage Enhancement through Power Spectral Density Concentration

Observation 2: Sub-GHz Phase Noise Characteristics of RF Transceivers enable Viable Implementation of 7.5 kHz SCS.

Observation 3: Delay Spread Tolerance Matches Rural Propagation Requirements

Observation 4: Hardware Implementation Leverages Existing Infrastructure

- 7.5 kHz subcarrier spacing (μ = -1) should be evaluated as a dedicated coverage numerology for 6G networks, with focus on sub-GHz frequency bands.

Define the Numerology parameter definition and scope for 7.5 kHz SCS.

We propose clarifying that the requirement in point (d), “avoiding multiple numerologies for the same band/sub-range”, should not be interpreted as prohibiting the use of a dedicated sub-band with 7.5 kHz SCS in sub-GHz bands. A strict “single SCS per band/sub-range” rule could be overly restrictive and limit further study of enhancing solutions (for coverage, throughput. Etc).

- Adoption of the 3MHz Channel bandwidths in Release-18 is already a precedent for this bandwidth in 6G-R, along with a new 600KHz synchronization raster for corresponding bands. In NR, initial access was an afterthought and supported using punctured SSB’s.

- There is a need to have a 3MHz channel BW configuration in 6G at least for sub-GHz frequency range.

Support the 3MHz for narrowband allocations and low power device category.

While supporting 3 MHz channel bandwidth, these objectives need to be considered:

- To design unified initial access procedure compatible with 3 MHz spectrum.

- Allow to have a compact CORESET#0 and narrowband SSB structure.

- To ensure coverage and detection reliability even with restricted bandwidth.

# References

RP-251881, “New SID: Study on 6G Radio”, RAN#108, June. 2025.

R1-250XXXX “Draft Report of 3GPP TSG RAN WG1 #122 v0.1.0” Aug 2025

R1-250XXXX “Chair Notes RAN1#122 eom” Aug  2025