3GPP TSG-RAN Meeting #109	RP-252662

Beijing, China, Sept 15th – 18th, 2025

Source:	Nokia

Title:	Considerations on RAN4-led Rel-20 5G-Adv RRM topics

Agenda item:	9.1.4.4

Document for:	Discussion

Introduction

In the last RAN plenary meeting #108 in Prague, the WF for RAN4 led REL-20 topics in [1] was endorsed. During the meeting a number of agreements were made while some aspects are still open for further discussions. In this paper we discuss these remaining open aspects regarding the RRM aspects of the RAN4 led Rel-20 package.

Discussion

## 2.1	RRM enhancements

In the last plenary meeting, the discussion related to the RRM part of the Rel-20 package discussion concluded as follows [1]:

Agreements: The high-level scope for WI on RRM enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - (e)RedCap UE enhancement - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 LT5 WI - This also include example bands, UE RF scope including potential complexity reduction, UE demod - Fast SCell activation - Further discussion on the possible down scoping in RAN#109 based on the following two objectives - SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026. - A-TRS/TRS based unknown SCell activation including both inter-band CA and intra-band non-contiguous CA - Consider simplification of solution to maintain reasonable work scope in consideration of the TU allocation - No RAN1/2 scope is expected - Target 0.5 TU (0.25RD and 0.25RF) in Q4’25 and 1 TU allocation (0.75RD,0.25RF) from Q2’26

The high-level scope of the RRM part includes two main topics:

- (e)RedCap UE enhancement

- Fast SCell activation

(e)RedCap UE enhancement:

These two RRM enhancements are endorsed to be included in the Rel-20 package. The Objectives for (e)RedCap enhancement scope are agreed on high level while the detailed description remains to be further discussed and decided in RAN#109 plenary.

Regarding (e)RedCap UE enhancement two objectives have been endorsed, namely:

- RLM/BFD relaxation, and

- Less than 5MHz for (e)RedCap.

Both objectives shall be developed according to the already developed UE requirements for non-(e)RedCap devices in earlier releases.

Hence, for the RLM/BFD relaxation the WI should apply the same requirements for 2Rx capable (e)RedCap devices as defined for non-(e)RedCap devices. If defined for 1Rx (e)RedCap devices RAN4 would need to study and agree on necessary relaxation due to the 1Rx assumption. We assume that for this objective, requirements will be defined for both FR1 and FR2.

Concerning Less than 5MHz for (e)RedCap objective the same approach must be applied. Hence, RAN4 will apply the UE requirements as developed in earlier releases directly to the 2Rx capable (e)RedCap device. For 1Rx (e)RedCap devices RAN4 would need to study and agree on necessary relaxation due to the 1Rx assumption. We assume that for this objective, requirements will be defined only for FR1.

Both objectives for (e)RedCap would need to develop requirements for (e)RedCap devices supporting the following:

- Number of receive branches: 1Rx and 2Rx.

- Duplex operation: HD-FDD operation.

We propose agreeing on the following for the two objectives for (e)RedCap for the RAN4 Rel-20 package:

For RLM/BFD relaxation:
- include both 1Rx and 2Rx receive branches; 
- include HD-FDD operation;
- define requirements for both FR1 and FR2.

For Less than 5MHz for (e)RedCap:
- include both 1Rx and 2Rx receive branches;
- include HD-FDD operation;
- define requirements for FR1 only.

Fast SCell activation:

Related to Fast SCell enhancements, the current scope is based on a down selection and compromise among the many proposals from multiple companies, in earlier plenary discussions. The overall scope is still open and very broad and there is a need for more discussion and down-scoping. When considering the objectives we need to consider overall RRM workload and available TUs together with predictability of the estimated workload. Additionally, we see it important to limit impact on other WGs. Hence, our preference is to down-scope from the listed objective, proposals having impact on other WGs.

Exclude proposals which have impact on other WGs.

Accounting the allocated TUs for Rel-20 5G work, and considering that (e)RedCap work is already agreed to progress, we think that only one Objective should be agreed under Fast SCell activation.

Agree only one objective for Fast SCell activation.

Our understanding is that proposals related to A-TRS/TRS-assisted SCell activation address the scenarios for the Fast SCell activation feature which were left out during the initial development of the Fast SCell activation requirements.

Currently, for Fast SCell activation, RAN4 has not defined requirements when:

- The SCell to be activated is an unknown FR2 SCell in a band in which there is not already an active cell.

- The SCell to be activated is an unknown FR1 SCell which is not contiguous to an active cell in the same band.

- The SCell to be activated is an unknown FR1 SCell which is not in the same band as an active cell.

Regarding 1, as discussed earlier, this scenario is very difficult from network point of view simply because network would not have a good understanding where to transmit any A-TRS. Hence, we do not think this should be included into the Rel-20 work.

Due to lack of gain, do not include into the Fast SCell activation objective: The SCell to be activated is an unknown FR2 SCell in a band in which there is not already an active cell.

Regarding 2 and 3, these could likely be defined within RAN4 without involving other WGs. When looking at these two aspects there are possible synergies between them and the proposed Objective ‘SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting’. However, as it was agreed in RAN4#116 not to expand the serving cell measurement relaxation to include FR1, we believe the reasoning and gain from working on 2 and 3 in R20 is rather low. For case 3, we additionally have concerns on the network complexity in supporting such scenario in real deployments.

Due to lack of gain, do not include into the Fast SCell activation objective: SCell to be activated is an unknown FR1 SCell which is not contiguous to an active cell in the same band.

Due to lack of gain, do not include into the Fast SCell activation objective: The SCell to be activated is an unknown FR1 SCell which is not in the same band as an active cell.

Hence, we suggest not including any objective related to A-TRS/TRS aspects under the Fast SCell activation objective.

RAN4 does not work on any objective related to A-TRS/TRS aspects under the Fast SCell activation objective.

Proposals covering other scenarios not defined within the Fast SCell activation feature scope, would likely need additional effort for defining necessary procedure and/or additional signalling support. Both such activities would in our view require work in other WGs.

Considering the above proposal regarding fast SCell activation and the SCell activation delay reduction objective:

- SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting

From the contributions and the discussions among companies it seems that if any work related to fast SCell activation is to be included in the Rel-20 package, it is most useful to work on an objective which gives gains, does not change existing procedures very much and does not impact other WGs.

Hence, one aspect which could be discussed, and which has been discussed also earlier in RAN4, is to discuss and if needed specify how the UE could be scheduled on an SCell before having sent a valid CSI report to the network. Currently, the total SCell activation delay is currently defined as (38.133, 8.3.2):

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

- THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

- Tactivation_time is the SCell activation delay in milliseconds.

- TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

The time between THARQ and UE sending the valid CSI report (TCSI_Reporting) depends on numerous configuration factors and the UE conditions, but may be relatively long. The understanding is that it may be possible to schedule the UE even before UE sends a valid CSI report to the network. If, for example, the UE has acquired DL synchronization and is therefore able to receive in DL, but has not yet been able to send a valid CSI report in UL, e.g. due to UL resources not being available, this adds an artificial delay in the SCell activation, where the UE could possibly be scheduled.

Enabling earlier SCell scheduling during SCell activation will increase use of network resources. We see that this could be done without changing the existing activation procedure start and end points (current minimum requirements listed above) and without impact on other WGs.

Hence, for the fast SCell activation delay objective we suggest working on enabling early SCell scheduling during SCell activation without changing the existing activation procedure start and end points.

Fast SCell activation objective: enable early SCell scheduling during SCell activation prior to sending the valid CSI report.

We do not see it feasible to change the existing overall SCell activation and hence we propose:

Fast SCell activation work shall not change the existing SCell activation delay start and end points.

Based on the objective outcome of RAN#108 meeting we are fine working on this objective assuming intra-band collocated scenario and including SSB-less cases.

Conclusion

Overall, the TU allocation for 5G-Adv topics in Rel-20 needs to take into account the time required for the 6G SI. Great care therefore needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items.

The following proposals and observations were made:

(e)RedCap UE enhancement:

- For RLM/BFD relaxation:
- include both 1Rx and 2Rx receive branches; 
- include HD-FDD operation;
- define requirements for both FR1 and FR2.

For Less than 5MHz for (e)RedCap:
- include both 1Rx and 2Rx receive branches;
- include HD-FDD operation;
- define requirements for FR1 only.

Fast SCell activation:

Exclude proposals which have impact on other WGs.

Agree only one objective for Fast SCell activation.

- Due to lack of gain, do not include into the Fast SCell activation objective: The SCell to be activated is an unknown FR2 SCell in a band in which there is not already an active cell.

- Due to lack of gain, do not include into the Fast SCell activation objective: SCell to be activated is an unknown FR1 SCell which is not contiguous to an active cell in the same band.

- Due to lack of gain, do not include into the Fast SCell activation objective: The SCell to be activated is an unknown FR1 SCell which is not in the same band as an active cell.

RAN4 does not work on any objective related to A-TRS/TRS aspects under the Fast SCell activation objective.

Fast SCell activation objective: enable early SCell scheduling during SCell activation prior to sending valid CSI report.

Fast SCell activation work shall not change the existing SCell activation delay start and end points.

- References

- RP-251879; WF for RAN4 led REL-20 topics; RAN4 Chair

- R4-2509420, Considerations on RAN4-led REL-20 5G-Adv topics, Nokia