3GPP TSG-RAN WG3 Meeting #129	R3-255773

Bengaluru, India, 25th ~29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR | 0134 | Rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Muting of always-on signals in 5G broadcast [5GB_CASMuting] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Qualcomm Incorporated, EBU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to support Muting of always-on signals in 5G broadcast, it is needed for the MCE to inform the CAS Muting Parameters (K_CAS and N_CAS) to the gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the CAS Muting Parameters IE (includes sub-IEs K_CAS and N_CAS) in the MBSFN Area Configuration Item Ies IE in the M2AP: MBMS SCHEDULING INFORMATION message. Impact Analysis: This CR has isolated impact with the previous version of the specification (same release) because it only includes additional optional IE to support Muting of always-on signals in 5G broadcast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support Muting of always-on signals in 5G broadcast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2, 9.1.7, 9.2.1.xx(new), 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: fix typo. |  |  |  |  |  |  |  |  |

---------------Start of the First Change---------------

## 8.4	MBMS Scheduling Information

### 8.4.1	General

The purpose of the MBMS Scheduling Information Procedure is to provide MCCH related information, and optional session suspension decision to the eNB.

The procedure uses non MBMS-Service-associated signalling.

### 8.4.2	Successful Operation

Figure 8.4.2-1. MBMS Scheduling Information procedure. Successful operation.

The MCE initiates the procedure by sending the MBMS SCHEDULING INFORMATION message to the eNB. The eNB shall store the MBSFN Area Configuration Item IE, apply the MCCH update from the modification period defined in the MCCH Update Time IE, and transmit the MCCH according to the MCCH configuration for the given MBSFN area indicated by the MCE. If an empty PMCH Configuration List IE is contained in this message for an MBSFN area, the eNB shall update the content of the corresponding MCCH so as not to include PMCH related information over the corresponding MCCH. If the Modulation and Coding Scheme 2 IE is included, the eNB shall ignore the Modulation and Coding Scheme IE and use the Modulation and Coding Scheme 2 IE instead. If the received PMCH Configuration IE contains the MCH Scheduling Period Extended IE, the eNB shall take its value into account instead of the value signalled in the MCH Scheduling Period IE. If the received PMCH Configuration IE contains the MCH Scheduling Period Extended 2 IE, the eNB shall take its value into account instead of the value signalled in the MCH Scheduling Period IE. The eNB shall schedule the MBMS services in the MCCH according to the order defined in the MBMS Session List per PMCH IE. If the MBMS Suspension Notification List IE is included in this message for an MBSFN area, the eNB shall broadcast the suspension decision over the air interface from the radio frame defined by the SFN IE, until the end of the Modification Period just before the "MCCH Update Time".

If the MBMS SCHEDULING INFORMATION message contains the Subframe Allocation Extended IE included in the MBSFN Subframe Configuration IE, the eNB shall, if supported, store it and take it into account.

If the MBMS SCHEDULING INFORMATION message contains the CAS Muting Parameters IE in the MBSFN Area Configuration Item Ies IE, the eNB shall, if supported, take it into account to transmit the non-MBSFN subframes containing the cell acquisition signals in the MBMS-dedicated cells, as specified in TS 36.211 [12].

### 8.4.3	Abnormal Conditions

Void.

---------------Start of the Next Change---------------

### 9.1.7	MBMS SCHEDULING INFORMATION

This message is sent by the MCE to provide MCCH related information to the eNB.

Direction: MCE  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MCCH Update Time | M |  | 9.2.1.19 |  | YES | reject |
| MBSFN Area Configuration List |  | 1 |  |  | YES | reject |
| >MBSFN Area Configuration Item Ies |  | 1 to <maxnoofMBSFNareas> |  |  |  |  |
| >>PMCH Configuration List |  | 1 |  |  | YES | reject |
| >>>PMCH Configuration Item Ies |  | 0 to <maxnoofPMCHsperMBSFNarea> |  |  | EACH | reject |
| >>>>PMCH Configuration | M |  | 9.2.1.8 |  | - |  |
| >>>>MBMS Session List per PMCH | M |  | 9.2.1.9 |  | - |  |
| >>Subframes Configuration List |  | 1 |  |  | YES | reject |
| >>>Subframes Configuration Item Ies |  | 1 to <maxnoofMBSFN-Allocations> |  |  | EACH | reject |
| >>>>MBSFN Subframe Configuration | M |  | 9.2.1.17 |  | - |  |
| >>Common Subframe Allocation Period | M |  | 9.2.1.18 |  | YES | reject |
| >>MBSFN Area ID | M |  | 9.2.1.14 |  | YES | reject |
| >>MBMS Suspension Notification List |  | 0..1 |  |  | YES | ignore |
| >>>MBMS Suspension Notification Item Ies |  | 1 to <maxnoofPMCHsperMBSFNarea> |  |  | EACH | ignore |
| >>>>SFN | M |  | INTEGER (0..1023) | SFN of the first radio frame containing the information that the MBMS session(s) are to be suspended. | - | - |
| >>>>MBMS Sessions To Be Suspended List per PMCH |  | 1 |  |  | - | - |
| >>>>>MBMS Sessions To Be Suspended List per PMCH Item |  | 1 to <maxnoofSessionsPerPMCH> |  |  | - | - |
| >>>>>>MBMS Service Identity | M |  | TMGI 9.2.3.3 |  | - | - |
| >>CAS Muting Parameters | O |  | 9.2.1.xx |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSFNareas | Maximum no. of MBSFN areas served by a single eNB. The value for maxnoofMBSFNareas is 256. |
| maxnoofPMCHsperMBSFNarea | Maximum no. of PMCHs possible per MBSFN .The value for maxnoofMCHsperMBSFNarea is 15. |
| maxnoofMBSFN-Allocations | Maximum no. of MBSFN frame allocations with different offset. The value for maxnoofMBSFN-Allocations is 8. |
| maxnoofSessionsPerPMCH | Maximum no. of Sessions per PMCH. The value for maxnoofSessionsPerPMCH is 29. |

---------------Start of the Next Change---------------

#### 9.2.1.xx	CAS Muting Parameters

This IE indicates the parameters of where the non-MBSFN subframes containing the cell acquisition signals in an MBMS-dedicated cell shall only be transmitted, as defined in TS 36.211 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| K-CAS | M |  | INTEGER (4..63, ...) | The same encoding as the k-CAS-r19 in the cas-MutingConfig-r19 contained in the SystemInformationBlockType1-MBMS-v19xy IE as specified in TS 36.331 [11]. |
| N-CAS | M |  | ENUMERATED (n2, n4, n8, n16, ...) | The same encoding as the n-CAS-r19 in the cas-MutingConfig-r19 contained in the SystemInformationBlockType1-MBMS-v19xy IE as specified in TS 36.331 [11]. |

---------------Start of the Next Change---------------

### 9.3.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for M2AP.

--

-- **************************************************************

M2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) m2ap (4) version1 (1) m2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

ENB-MBMS-Configuration-data-Item,

ENB-MBMS-Configuration-data-ConfigUpdate-Item,

ENB-MBMS-M2AP-ID,

ENBname,

GlobalENB-ID,

GlobalMCE-ID,

MBSFN-Area-ID,

MBMS-Service-Area,

MBMS-Session-ID,

MBMSsessionListPerPMCH-Item,

MBMS-Service-associatedLogicalM2-ConnectionItem,

MBSFN-Subframe-Configuration,

MCCH-Update-Time,

MCCHrelatedBCCH-ConfigPerMBSFNArea-Item,

MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item,

MCE-MBMS-M2AP-ID,

MCEname,

PMCH-Configuration,

Common-Subframe-Allocation-Period,

TimeToWait,

TMGI,

TNL-Information,

SFN,

MBMSsessionsToBeSuspendedListPerPMCH-Item,

SC-PTM-Information,

CASMutingParameters

FROM M2AP-Ies

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

M2AP-PRIVATE-IES,

M2AP-PROTOCOL-EXTENSION,

M2AP-PROTOCOL-IES,

M2AP-PROTOCOL-IES-PAIR

FROM M2AP-Containers

id-MCE-MBMS-M2AP-ID,

id-ENB-MBMS-M2AP-ID,

id-TMGI,

id-MBMS-Session-ID,

id-MBMS-Service-Area,

id-TNL-Information,

id-Alternative-TNL-Information,

id-CriticalityDiagnostics,

id-Cause,

id-MBSFN-Area-Configuration-List,

id-MBSFN-Subframe-Configuration-Item,

id-MBSFN-Subframe-Configuration-List,

id-MCCH-Update-Time,

id-PMCH-Configuration-List,

id-PMCH-Configuration-Item,

id-Common-Subframe-Allocation-Period,

id-GlobalENB-ID,

id-ENBname,

id-ENB-MBMS-Configuration-data-List,

id-ENB-MBMS-Configuration-data-Item,

id-GlobalMCE-ID,

id-MCEname,

id-MCCHrelatedBCCH-ConfigPerMBSFNArea,

id-MCCHrelatedBCCH-ConfigPerMBSFNArea-Item,

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea,

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item,

id-TimeToWait,

id-ENB-MBMS-Configuration-data-List-ConfigUpdate,

id-ENB-MBMS-Configuration-data-ConfigUpdate-Item,

id-MBSFN-Area-ID,

id-ResetType,

id-MBMS-Service-associatedLogicalM2-ConnectionItem,

id-MBMS-Service-associatedLogicalM2-ConnectionListResAck,

id-MBMS-Counting-Request-Session,

id-MBMS-Counting-Request-Session-Item,

id-MBMS-Counting-Result-List,

id-MBMS-Counting-Result-Item,

id-MBMS-Suspension-Notification-List,

id-MBMS-Suspension-Notification-Item,

id-PMCH-Overload-Status,

id-Overload-Status-Per-PMCH-List,

id-Active-MBMS-Session-List,

id-SC-PTM-Information,

id-CASMutingParameters,

maxnoofMBSFN-Allocations,

maxnoofMBSFNareas,

maxnoofPMCHsperMBSFNarea,

maxnoofCells,

maxnoofMBMSServiceAreasPerCell,

maxnoofSessionsPerPMCH,

maxnooferrors,

maxNrOfIndividualM2ConnectionsToReset,

maxnoofCountingService

FROM M2AP-Constants;

//skip unchanged part

-- **************************************************************

--

#### -- MBMS SCHEDULING INFORMATION

--

-- **************************************************************

MbmsSchedulingInformation ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{ MbmsSchedulingInformation-Ies}},

...

}

MbmsSchedulingInformation-Ies M2AP-PROTOCOL-IES ::= {

{ ID id-MCCH-Update-Time					CRITICALITY reject	TYPE MCCH-Update-Time						PRESENCE mandatory }|

{ ID id-MBSFN-Area-Configuration-List		CRITICALITY reject	TYPE MBSFN-Area-Configuration-List			PRESENCE mandatory },

...

}

MBSFN-Area-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofMBSFNareas)) OF ProtocolIE-Container { { MBSFN-Area-Configuration-Item } }

MBSFN-Area-Configuration-Item M2AP-PROTOCOL-IES ::= {

{ ID id-PMCH-Configuration-List				CRITICALITY reject	TYPE PMCH-Configuration-List				PRESENCE mandatory }|

{ ID id-MBSFN-Subframe-Configuration-List	CRITICALITY reject	TYPE MBSFN-Subframe-ConfigurationList		PRESENCE mandatory }|

{ ID id-Common-Subframe-Allocation-Period	CRITICALITY reject	TYPE Common-Subframe-Allocation-Period		PRESENCE mandatory }|

{ ID id-MBSFN-Area-ID						CRITICALITY reject	TYPE MBSFN-Area-ID							PRESENCE mandatory }|

{ ID id-MBMS-Suspension-Notification-List	CRITICALITY ignore	TYPE MBMS-Suspension-Notification-List		PRESENCE optional }|

{ ID id-CASMutingParameters					CRITICALITY reject	TYPE CASMutingParameters						PRESENCE optional },

...

}

PMCH-Configuration-List ::= SEQUENCE (SIZE(0.. maxnoofPMCHsperMBSFNarea)) OF ProtocolIE-Single-Container { { PMCH-Configuration-ItemIEs } }

PMCH-Configuration-ItemIEs M2AP-PROTOCOL-IES ::= {

{ ID id-PMCH-Configuration-Item	 CRITICALITY reject	TYPE PMCH-Configuration-Item	PRESENCE mandatory },

...

}

---------------Start of the Next Change---------------

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

M2AP-Ies {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) m2ap (4) version1 (1) m2ap-Ies (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

//skip unchanged part

AllocationAndRetentionPriority-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- B

BitRate ::= INTEGER (0..10000000000)

#### -- C

CASMutingParameters ::= SEQUENCE {

k-CAS						INTEGER (4..63,...),

n-CAS						ENUMERATED {n2, n4, n8, n16, ...},

iE-Extensions				ProtocolExtensionContainer { { CASMutingParameters-ExtIEs} } OPTIONAL,

...

}

CASMutingParameters-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

nAS					CauseNAS,

protocol			CauseProtocol,

misc				CauseMisc,

...

}

---------------Start of the Next Change---------------

### 9.3.7	Constant definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

M2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) m2ap (4) version1 (1) m2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

//skip unchanged part

id-MCH-Scheduling-PeriodExtended2											ProtocolIE-ID ::= 48

id-Subcarrier-SpacingMBMS													ProtocolIE-ID ::= 49

id-SubframeAllocationExtended												ProtocolIE-ID ::= 50

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item 								ProtocolIE-ID ::= 51

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea									ProtocolIE-ID ::= 52

id-SubframeAllocationFurtherExtension										ProtocolIE-ID ::= 53

id-AdditionalConfigParameters												ProtocolIE-ID ::= 54

id-CASMutingParameters														ProtocolIE-ID ::= xx

END

---------------End of the Changes---------------