3GPP TSG-RAN WG3 Meeting #129	R3-255751

Bengaluru, India, 25th ~29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1970 | Rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RRC establishment Cause for CIoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, ZTE, Nokia, Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-13, in the RRCConnectionRequest-NB message and the RRCConnectionResumeRequest-NB message, the EstablishmentCause-NB-r13 IE is provided from the UE: RRCConnectionRequest-NB-r13-IEs ::= SEQUENCE { ue-Identity-r13 InitialUE-Identity, establishmentCause-r13 EstablishmentCause-NB-r13, multiToneSupport-r13 ENUMERATED {true} OPTIONAL, multiCarrierSupport-r13 ENUMERATED {true} OPTIONAL, earlyContentionResolution-r14 BOOLEAN, cqi-NPDCCH-r14 CQI-NPDCCH-NB-r14, spare BIT STRING (SIZE (17)) } RRCConnectionResumeRequest-NB-r13-IEs ::= SEQUENCE { resumeID-r13 ResumeIdentity-r13, shortResumeMAC-I-r13 ShortMAC-I, resumeCause-r13 EstablishmentCause-NB-r13, earlyContentionResolution-r14 BOOLEAN, cqi-NPDCCH-r14 CQI-NPDCCH-NB-r14, anr-InfoAvailable-r16 BOOLEAN, spare BIT STRING (SIZE (3)) } In Rel-15, EDT was introduced during the discussion of CIoT: - new RRCEarlyDataRequest message was introduced, with a new establishmentCause-r15 field contained in the RRCEarlyDataRequest-r15-IEs IE. And in Rel-16, MT-EDT was further introduced, and a new establishmentCause-v1610 field contained in the RRCEarlyDataRequest-5GC-r16-IEs IE was introduced. RRCEarlyDataRequest-r15-IEs ::= SEQUENCE { s-TMSI-r15 S-TMSI, establishmentCause-r15 ENUMERATED {mo-Data, delayTolerantAccess}, dedicatedInfoNAS-r15 DedicatedInfoNAS, nonCriticalExtension RRCEarlyDataRequest-v1590-IEs OPTIONAL } RRCEarlyDataRequest-v1610-IEs ::= SEQUENCE { establishmentCause-v1610 ENUMERATED {mt-Access, spare3, spare2, spare1}, nonCriticalExtension SEQUENCE {} OPTIONAL } - New RRCEarlyDataRequest-NB message was introduced, with a new establishmentCause-r15 field contained in RRCEarlyDataRequest-NB-r15-IEs IE RRCEarlyDataRequest-NB-r15-IEs ::= SEQUENCE { s-TMSI-r15 S-TMSI, establishmentCause-r15 ENUMERATED {mo-Data, mo-ExceptionData, delayTolerantAccess, mt-Access-v1610}, cqi-NPDCCH-r15 CQI-NPDCCH-NB-r14 OPTIONAL, dedicatedInfoNAS-r15 DedicatedInfoNAS, nonCriticalExtension RRCEarlyDataRequest-NB-v1590-IEs OPTIONAL } But currently the cause value related to the fields/IEs introduced above are missing/incorrect in the S1AP: RRC Establishment Cause IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the S1AP: RRC Establishment Cause to refer to the correct fields in TS 36.331 in case of EDT. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts the RRC Establishment Cause in case of EDT. This CR is functional NBC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong cause will be provided from the eNB to the MME in case of EDT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Separating NB-IoT related descriptions. Adding co-source company. |  |  |  |  |  |  |  |  |

-------------------------Start of the Change-------------------------

#### 9.2.1.3a	RRC Establishment Cause

The purpose of the RRC Establishment Cause IE is to indicate to the MME the reason for RRC Connection Establishment or RRC Connection Resume as received from the UE in the EstablishmentCause IE, or ResumeCause IE as defined in TS 36.331 [16], or the reason for RRC Early Data Request as received from the UE in the establishmentCause-r15 contained in the RRCEarlyDataRequest-r15-IEs IE or the establishmentCause-v1610 contained in the RRCEarlyDataRequest-v1610-IEs IE as defined in TS 36.331 [16].

In case of NB-IoT, this IE indicates the reason for RRC Connection Establishment or for RRC Connection Resume as received from the UE in the EstablishmentCause-NB-r13 IE, or the reason for RRC Early Data Request as received from the UE in the establishmentCause-r15 contained in the RRCEarlyDataRequest-NB-r15-IEs IE as defined in TS 36.331 [16].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC Establishment Cause | M |  | ENUMERATED(emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, …,delayTolerantAccess, mo-VoiceCall, mo-ExceptionData) |  |

-------------------------End of the Change-------------------------