3GPP TSG-RAN Meeting #109							RP-252700

Beijing, China, 15 - 18 September, 2025

Agenda Item:		8.3	Other 6G documents

Source:	Robert Bosch GmbH

Title:		Views and Considerations on Automotive UE Evolution for 6G

Document for:	Discussion

# Introduction

This contribution proposes that the 6G RAN Study Item’s technical report (TR 38.914 [1]) explicitly capture an Automotive/vehicular UE type (V‑UE) and associated common RAN considerations, positioning vehicular communications within the mainstream 6G service continuum (including potential requirements for extended mobile broadband with AI/compute‑offload traffic and immersive communications), rather than as a separate “vertical” requiring dedicated RAN solutions.

Compared to the initial 5G studies (circa 2015), vehicular connectivity deployments and requirements are now substantially better understood, spanning commercial/public network operation as today’s status quo. Current V2N deployments enable wide range of connectivity functions, e.g., software‑defined vehicle (SDV) operations, cloud connectivity (for back-end distributed automotive functions), tele-operation/control, and early AI-assisted services (e.g., cloud-connected inference of conversional models)

The intent of this contribution is to scope, within the RAN studies, potential automotive requirements including deployment scenarios, diverse UE types, new and existing services, and KPI/TPR considerations, to reflect connected vehicles as one of 6G main users. This contribution complements the companion TDocs [4]-[6], which are submitted for discussion and decision at the same TSG RAN meeting.

# Objectives of Covering Automotive in TR 38.914

From automotive sector, the key points are:

- Automotive connectivity is a sustained consumer and commercial domain. Treating it as a separate “vertical” with a fully isolated slice(s) may reflect a possibly non-scalable approach adopted during early 5G introduction.

- Public/commercial networks are the primary substrate for connected vehicles. Currently, all vehicles are connected alongside with smartphone without dedicated fragmentation. Even though safety‑related applications remain important; however, modern vehicle platforms can and do adapt functionality to network conditions. This enables use of common RAN capabilities while still supporting stringent applications where feasible.

- Vehicular traffic patterns are diverse, e.g., AI/compute offload, cloud services, software update for SDV, telemetry/fleet management, and immersive/interactive services for occupants. These patterns map naturally to extended eMBB/Immersive communication profiles (with low‑latency and high reliability), rather than to the IoT low‑throughput region.

- Field feedback from 5G‑Advanced indicates that advanced connected services (e.g., AI/ML services, cloud‑assisted features, OTA updates) are already being introduced in the automotive domain. For 6G, RAN should evolve the support for automotive radio requirements through common mechanisms (e.g., with smartphones), without reliance mainly on vertical-specific or special-case designs.

Considering the above observations and motivations, it is proposed that TR 38.914 [1] includes a dedicated subsection on an Automotive/Vehicular UE type (V‑UE) capturing common RAN considerations (i.e., with other 6G devices, e.g., smartphones) and the relevant evaluation assumptions, with the following objectives:

# Vehicular UE type definition, commonalities/deltas, and services:

Vehicular UE (V-UE) shares many commonalities with smartphone or extended mobile broadband devices. However, there are still specific aspects dedicated for the automotive use cases, mobility requirements, and the possible characteristics associated with the vehicular formfactors. In RAN1#122 meeting, the following agreements were considered [7]:

Agreement

Study a scalable 6GR design for diverse device types, considering aspects:

- What should be commonly applicable to all 6G device types

- FFS: add-on features dedicated to specific device types, if any

Agreement

Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

Therefore, RAN (followed by RAN1) is requested to define V‑UE additional characteristics (i.e., deltas) relative to smartphone UE, including power classes, number and antenna characteristics, and other aspects associated with vehicular form factors and potential installation (in vehicular mounting, roof-top mounted, etc.). Additionally, for V-UE, consider relevant characteristics associated with the mobility profiles (urban, suburban, highway) and on‑board compute/storage.

Regarding automotive services, currently, the vehicles are equipped with infotainment/cockpit functions with passenger connectivity and cloud-assisted AI applications. Vehicles can also connect to remote services for monitoring, extended automotive functionalities, and allow for on-device AI applications connected to cloud services (e.g., natural language assistants’ models in the cloud).

For 6G, extended AI capabilities, more cloud-based services, and further functional offloading (e.g., advanced SDV functions) are expected to be deployed. This positions the vehicular UE (at least for passenger and commercial vehicles) within the extended mobile broadband/immersive communication devices. Smaller form-factor mobility devices (e.g., two-wheelers, e-bike) may require reduced capabilities relative to their compute, bandwidth, and battery limitation. See Figure 1 for exemplary V-UEs with advanced eMBB/Immersive communication capabilities and possible smaller form-factor mobility devices.

Figure 1: Illustration of 6G diverse device types with scalable 6GR design, where automotive UE devices are positioned in the extended MBB/Immersive communication

# RAN Services and requirements:

This objective defines representative data traffic profile for a V-UE operating on public/commercial networks and aggregating data flows from the vehicle’s different services and streams (where streams aggregation is out of RAN scope). For each profile, RAN should consider, at least, the following:

(a) AI service: for vehicular AI service, AI compute offload, and on-device AI; RAN should consider studying RAN-awareness, vehicular-AI awareness (e.g., on-device AI lifecycle management (LCM) exposure), and additional vehicular specific AI requirements including, e.g., UL/DL rate requirements associated with model execution, potential sensitivity to latency, and reliability associated to AI services.

Note: Potential sensitivity to latency and burst data rates for certain functions (AI services) need to be considered by RAN1-3 and mobility aspects in RAN4 performance part. Therefore, AI services may require studying possible introduction of joint latency/reliability ranges and data burstiness in case of future potential quantitative TPRs would be considered.

(b) Immersive communication: interactive/immersive media for occupants and advanced interactive cockpit; may require both DL and UL enhancements, RAN/vehicle awareness, etc. For these scenarios, the immersive communication profiles and KPIs that would be captured in TR 38.914 should be applied, with additional consideration of vehicular mobility requirements.

(c) eMBB: telemetry, fleet monitoring and management; traffic assumptions follow 5G baselines, including periodic transmissions (e.g., UL configured grants, DL SPS) and aperiodic scheduling as baseline for 3GPP 6G RAN studies.

(d) HRLLC: for safety‑related services, vehicle control (e.g., tele-operated driving), and distributed/offloaded automotive functions with stringent requirements. RAN should study HRLLC requirements for vehicular use cases, including enhanced joint reliability, reduced latency, optimized data rates, and higher mobility support.

(e) Positioning, timing and synchronization: RAN should capture requirements and study possible RAN-based positioning, timing, and synchronization relevant to vehicular use cases leveraging both TN and NTN. 5G positioning and timing/synchronization should be considered as baseline.

Figure 2: 6G RAN and connected automotive functionalities

# Deployment scenarios:

Identify typical deployments scenarios for vehicular UEs operating on public/commercial networks. Scenarios should include urban/suburban and highway/freeway environments as proposed in [4]. For each scenario, capture representative mobility ranges (including high-speed vehicular mobility), coverage characteristics, and service/data traffic assumptions. Also consider heterogeneous coverage of terrestrial non-terrestrial network (NTN), especially in transition and extended-coverage conditions. See our companion TDoc [4] for additional parameters and details.

# NTN applicability:

RAN should study the support of multiple vehicular NTN terminal antenna types for, e.g., passenger vehicles, trucks, and off-road vehicles. The associated antenna requirements should be considered for the first 6G release (Day-1 support). RAN should also consider mobility enhancements across TN/NTN, including reduction of connection interruption time budgets. Additionally, RAN should study the feasibility of dual access/dual traffic steering for V-UEs operating, at least, different services simultaneously on, e.g., concurrent TN and NTN connectivity legs, or on two NTNs (e.g., orbit 1 and 2) at the same time. Furthermore, RAN should consider GNSS-independent operation in 6G as the baseline for NTN operation.

Figure 3: 6G Proposed multi-service support with TN, NTN, multi-orbits, and dual-channel operation

# Automotive TPRs and the Joint Requirements (if agreed):

For vehicular/automotive use cases, define evaluation assumptions and KPI/TPR ranges in line with TR 38.914 values (e.g., for user experienced data rate, reliability, latency, positioning accuracy, …) at least for Immersive, HRLLC, and AI services. If Joint Requirements is aggregable, mobility should also be considered for the test environments of Immersive and HRLLC, i.e., assuming vehicular use cases.

Note: further AI service requirements to be identified during RAN WGs study item.

# Relationship with SA1/SA2

  - Align V‑UE service/traffic profiles and deployment scenarios with SA1 studies on 6G use cases and requirements (e.g., for automotive related AI-services, E2E AI, computational offloading) [2].

  - Coordinate with SA2 on any architectural capability that may impact RAN evaluation assumptions for automotive use cases including [3]:

    - WT#3 (E2E AI and network for AI),

    - WT#5 (data handling, collection, exposure and privacy),

    - WT#6 (computational offloading)

    - WT#8 (TN-NTN service continuity)

# Conclusion

For discussions, this proposal is considered:

Proposal 1: RAN is invited to:

- Agree to include an Automotive/Vehicular UE type (V‑UE) subsection in TR 38.914 as proposed above in [5], also to be among the diverse 6G UE device types.

- Incorporate the text proposal for automotive deployment scenario in [4] and automotive service requirements in [6].

- Study the objectives in Sections 2.1 – 2.5

- Align with SA1 and SA2 as described in section 3.

# References

[1] TR 38.914, “Study on 6G Scenarios and requirements,” TSG RAN 6G Study Technical Report.

[2] TR 22.870, 6G use cases and requirements, SA1 Technical Report version 0.3.1.

[3] S2-2505647, “New Study on Architecture for 6G System,” SA2 Meeting #169, Fukuoka, Japan, May 2025.

[4] RP-252177, “Text proposal for TR 38.914 for deployment scenarios for Vehicular Use Cases,” Bosch et al., TSG RAN Meeting#109, Beijing, China, September 2025.

[5] RP-251940, “Text proposal for TR 38.914 for Vehicular Device Type, BMW et al.,” TSG RAN Meeting#109, Beijing, China, September 2025.

[6] RP-252177, “Text proposal for Requirements of New and Existing Services for Vehicular Use Cases,” Bosch et al., TSG RAN Meeting#109, Beijing, China, September 2025.

[7] Chair notes, Draft Agenda of 3GPP TSG RAN WG1 #122, MCC Support, Bengaluru, India, August 2025