3GPP TSG RAN Meeting #109 											RP-252707

Beijing, China, 15th -18th September, 2025

Agenda Item:	9.2.3

Source:	CATT

Title:	Views on Positioning objective of Ambient IoT works in Rel-20

Document for:	Discussion and decision

# Introduction

In RAN#108, Rel-20 Ambient IoT SID was agreed in [1].  The scope of the Rel-20 Ambient IoT study for active devices 2b/C in outdoor deployment scenario D4T1 was agreed in the following with the objective (2)  of positioning/proximity determination to be determined in RAN#109,

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]:

- Deployment scenario 4 with topology 1 with no external carrier wave.

- Traffic types DO-DTT, DT, DO-A.

- Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command).

- Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs.

- FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.

- Assumption of the same air interface for Device 2b and Device C.

  - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769

- Positioning for outdoor scenarios is under objective 2.

This objective begins after RAN#108 (June 2025)

RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work.

This objective includes:

RAN1-led

- Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios

  - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios

- Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C.

- While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming

  - Maximum distance between Reader and Device of 50-500 m

  - Maximum transmit power between -3 dBm and +5 dBm for device C

    - RAN1 to recommend feasible values within this range

  - Maximum transmit power of -20 dBm and -10 dBm for device 2b

Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources.

- Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations.

- Define link budget calculation for coverage.

RAN4-led

- Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios.

- Use band n8 as an example band.

NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

- RAN#109 to decide whether to include an objective on positioning / proximity determination in the scope for Rel-20 study item

This paper discusses the use cases and the importance of the positioning/proximity determination for devices 2b/C in the outdoor deployment scenarios.

# Discussion of the Positioning/Proximity Determination in Rel-20 A-IoT study

During RAN#108 offline discussion of the Ambient IoT scope, the objective 2 of the A-IoT positioning in indoor and outdoor scenarios for active devices 2b/C were captured in [2] with the study focusing in RAN1 as follows,

- Study the support of A-IoT positioning in indoor and outdoor scenarios for active device(s), focusing on UL, i.e. in D2R, and network based positioning, and considering the findings from the Rel-19 study of proximity determination solution 2 [RAN1]

- D1T1 for indoor and D4T1 for outdoor

- Representative use cases rUC3 (indoor positioning) and rUC7 (outdoor positioning).

- Evaluate the achievable positioning accuracy [RAN1]

- Note: positioning solutions studied under this objective are expected to be equally applicable for outdoor and indoor scenarios for Device 2b/Device C

- RAN#109 to decide whether to include this objective in the scope for Rel-20, and attempt to further narrow-down the targeted candidate network-based positioning technique(s).

- RAN#111 (March 2026) will make a decision on whether to include positioning in Rel-20 normative work.

The positioning use cases rUC3 (Indoor positioning) and rUC7 (Outdoor positioning) are the most useful applications for the device tracking.   In particular in the outdoor deployment scenarios, the tracking of the A-IoT device location is critical for most of the A-IoT applications when A-IoT device could be moving to different location.   Although the Rel-19 proximity determination solution could be used to the device tracking and location identification, the accuracy of the A-IoT device positioning with UE mobility might not be sufficient when the coverage area targets 50-500 meter in the outdoor deployment scenarios.

The study of A-IoT positioning will focus on the D2R link to avoid increasing the complexity of the A-IoT device.   Since the Rel-20 A-IoT study targets the outdoor scenarios to have the coverage of 50-500 meters and the Tx power of devices 2b and devices C at the range of [-20, -10] dBm and [-5, 3] dBm respectively , the UL D2R signals used for UTDOA triangulation of positioning techniques might not be adequate since the D2R signals might not reach the neighboring readers.  Thus, the candidates of the positioning techniques would be limited to use the D2R signals from a single reader.

Proposal 1:  Due to the limited Tx power of A-IoT devices 2b/C, the candidate of the positioning techniques should focus on using the D2R signals at a single reader.

It was agreed in RAN1#122 to study the potential enhancement of the D2R signals to achieve the functionalities for A-IoT devices 2b/C in outdoor deployment scenarios as follows,

Agreement

For D2R signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing D2R signal(s) or to introduce new D2R signal(s) for the following functionalities (if needed):

- CFO estimation

- SFO estimation

- timing acquisition

- channel estimation

- measurement (e.g., signal strength, interference estimation)

- Device differentiation e.g. for collision resolution, for interference randomization

The potential enhancement of D2R signals for the functionalities of the CFO estimation, SFO estimation, timing acquisition, channel estimation, and signal strength could be potentially applied to the positioning techniques.  The framework of the D2R signal enhancement study for the single cell Positioning techniques had been established based on the potential functionalities enhancements of generic D2R signal detection and demodulation.  The addition time and effort of including the positioning/proximity determination would be minimal.

Proposal 2:  RAN1#122 agreements on the potential functionalities enhancements of D2R signals for the D2R signal demodulation and decoding could be used for the single cell Positioning technique.

Since the D2R signals enhancement for the functionalities to achieve the D2R signal demodulation and decoding for devices 2b/C in the outdoor deployments had been identified to be studied.  The addition function of the A-IoT device positioning could be included as one of the functionalities.   Most of the functionalities for demodulation and decoding could be reused for the A-IoT device Positioning techniques.  The Objective 2 of A-IoT device Positioning/proximity determination should be included in the scope of Rel-20 A-IoT study item.

Proposal 3: The Objective 2 of A-IoT device Positioning/proximity determination should be included in the scope of Rel-20 A-IoT study item.

# Conclusion

In this paper, we discuss the importance of the Positioning use cases of the devices 2b/C in the outdoor deployment scenarios and the standardization efforts.  We have the following proposals,

- Proposal 1:  Due to the limited Tx power of A-IoT devices 2b/C, the candidate of the positioning techniques should focus on using the D2R signals at a single reader.

- Proposal 2:  RAN1#122 agreements on the potential functionalities enhancements of D2R signals for the D2R signal demodulation and decoding could be used for the single cell Positioning technique.

- Proposal 3: The Objective 2 of A-IoT device Positioning/proximity determination should be included in the scope of Rel-20 A-IoT study item.

The proposed objectives of A-IoT device positioning/proximity determination are as follows,

- Study the support of A-IoT positioning in indoor and outdoor scenarios for active device(s), focusing on UL , i.e. in D2R signal(s) to a single reader, and network based positioning, and considering the findings from the Rel-19 study of proximity determination solution 2 [RAN1]

- D1T1 for indoor and D4T1 for outdoor

- Representative use cases rUC3 (indoor positioning) and rUC7 (outdoor positioning).

- Evaluate the achievable positioning accuracy [RAN1]

- Note: positioning solutions studied under this objective are expected to be equally applicable for outdoor and indoor scenarios for Device 2b/Device C

# References

- RP-251884	Study on enhancements for solutions for Ambient IoT 	(Internet of Things) in NR outdoor for active devices, LG

- RP-251788	Moderator's summary for Enhancements for solutions for Ambient IoT (Internet of Things) in NR,  Moderator (RAN1 vice-chair)