3GPP TSG-RAN Meeting #109	RP-252749

Beijing, China, September 15-18, 2025

Agenda Item:	9.1.4.1

Source:	Intel Corporation

Title:	Views on RAN4 Rel-20 5G-A package – UE RF topics

Document for:	Discussion and decision

# Introduction

This contribution summarizes Intel’s views on the scope of RAN4 5G-A Release 20 work scope for UE RF centric topics and provides further views on top of the WF endorsed in RAN#108 [1].

# Views on UE RF topics

The following agreements are provided in RAN #108 WF [1] for UE RF centric RAN4 topics:

The high-level scope of the WI on UE RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - FR1 HPUE enhancement - Further consider/discuss until RAN#109 on the detailed scope - This work is to focus on the TN application - 6MHz channel bandwidth - It can be prioritized to FDD band <1GHz - Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM , 256QAM - BS demod scope is included. - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact - The details for study is to be discussed until RAN#109 - This work will be started in Q1’26. If the similar scope (The examples include the enhancements enabled by BS advanced receiver and UE Tx signal modifications. It is noted that referring to these examples in this WF should not be deemed as taking these examples as the baseline or starting point in 6G study) is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, it is possible that 6G related conclusion can be applied to 5G. - No RAN1/2 scope is expected - Target 0.5 RF TU in Q4’25 and 1.25 TU (0.75 RF and [0.5] Demod for EVM relaxation and 6MHz) from Q1’26.

The agreed target TU allocation for UE RF packages is relatively small and further prioritization and down-scoping is recommended.

Proposal #1: Consider further prioritization and down-scoping of UE RF 5G-A Rel-20 objectives

## FR1 HPUE enhancements for TN

Views on objective scope/prioritization

In the recent releases a lot of efforts were put to enable new HPUE architectures for single carrier and CA scenarios. For Rel-20 we recommend that HPUE enhancements for TN should strive to provide meaningful improvements for FWA and handheld type of UEs for single carrier scenarios and the following single carrier HPUE sub-objectives are proposed to be prioritized:

- PC1 with 4Tx for FWA UEs for TDD bands

- PC1.5 with 2Tx for handheld UEs for FDD bands

Candidate objectives

- High power UE (HPUE) for single carrier in terrestrial network (TN)

  - Specify the generic requirements of high power UE (HPUE) for NR UL single carrier in NR FR1 bands

    - Power class 1 (PC1) UE for NR TDD for FWA type of devices

    - Power class 1.5 (PC1.5) UE for NR FDD for handheld type of devices

Proposal #2:	UE RF objective “FR1 HPUE enhancements for TN”

- The work objectives shall include definition of HPUE RF requirements for single carrier cases

  - PC1 with 4Tx for FWA UEs for TDD bands

  - PC1.5 with 2Tx for handheld UEs for FDD bands

## 6MHz channel bandwidth

Views on objective scope/prioritization

The scope of the work is relatively straightforward and the Rel-19 7MHz WI objectives can be taken as baseline.

- Generic RF requirements (CBW, spectrum utilization, UE RF, BS RF)

- Band specific RF requirements

- Necessary signalling

In addition, the following assumption are recommended:

- Limit 6 MHz CBW to 15kHz SCS

- To facilitate easier commercialization, it is recommended to introduce 6MHz as an optional feature from Rel-15

Candidate objectives

- Specify the generic core requirements to support 6 MHz NR FR1 channel bandwidth [RAN4]

  - Introduce the channel bandwidth 7MHz

    - Only include 15 kHz SCS

  - Specify the spectrum utilization

  - Specify UE RF requirements for UL and DL

  - Specify BS RF requirements for UL and DL

- Specify the band specific core requirements to introduce 6 MHz for NR band TBD [RAN4]

  - BS and UE Channel bandwidth per operating band

  - UE reference sensitivity

  - When needed:

    - MPR (relative bandwidth criteria)

    - Additional Maximum Power Reduction (A-MPR)

- Enable, if feasible, optional UE support of 6 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4]

- Introduce the necessary signalling for this channel bandwidth. [RAN2, RAN3]

- Specify necessary performance requirements [RAN4]

TU allocation and WI structure

- Expect limited workload and may consider shorter work duration (e.g., 12 months).

- Given that item includes both RF/RRM objectives, we recommend handling it as a separate work item

Proposal #3:	UE RF objective “6MHz channel bandwidth”

- The work objectives shall include definition of

  - Generic RF requirements (CBW, spectrum utilization, UE RF, BS RF)

  - Band specific RF requirements

  - Necessary signalling

- 6MHz CBW is limited to 15kHz SCS

- Introduce 6MHz as an optional feature from Rel-15

- Handle objective as a separate work item with 12 months duration

## Reduced UE MPR values with relaxed Tx EVM

Views on objective scope/prioritization

We anticipate a low chance for a wide roll-out of 5G-A commercial deployments supporting the “relaxed Tx EVM” feature since the efficient operation would require: 1) sufficient population of UEs support the feature in order to extract the benefits, 2) advanced BS receiver, which may potentially require hardware updates. Therefore, it is preferred to handle the enhancements as a part of 6G studies and include AI-ML enhancements from the very beginning. This would allow more time for thorough studies and give a better chance for the successful commercialization of the feature. So, we recommend to task RAN4 to include corresponding 6G AI-ML use case in October RAN4 #116bis meeting. To follow the spirit of agreements in RAN#108 we suggest that the 6G studies should also include studies of non-linear compensation algorithm based on existing 5G design (i.e. no RAN1 impact) and depending on the outcome of the work a normative stage of 5G can be triggered at a later stage in Rel-20 or Rel-21.

The scope of studies should include

- Evaluation of feasible Tx EVM relaxation for different UEs and assessment of associated MPR improvement.

- Evaluation of achievable performance improvement incl. link-level and system-level performance and various scenarios incl. MU-MIMO.

- Evaluation of BS complexity and implementation feasibility

The studies should include solutions for 1) 6G design with application of AI-ML receivers and 2) 5G design with application of non AI-ML non-linear receivers.

Proposal #4:	UE RF objective “Reduced UE MPR values with relaxed Tx EVM”

- Do not include “Reduced UE MPR values with relaxed Tx EVM” objective in the overall 5G-A Rel-20 package. RAN4 to include the EVM relaxation use case in the scope 6G RAN4 studies and study both 5G non-linear receiver and 6G AI-ML receiver altogether.

- The study objectives shall include

  - Evaluation of feasible Tx EVM relaxation for different UEs and assessment of associated MPR improvement.

  - Evaluation of achievable performance improvement incl. link-level and system-level performance and various scenarios incl. MU-MIMO.

  - Evaluation of BS complexity and implementation feasibility

# Conclusions

In summary, we make the following proposals on the scope of RAN4 Rel-20 5G-A UE RF topics:

Proposal #1: Consider further prioritization and down-scoping of UE RF 5G-A Rel-20 objectives

Proposal #2:	UE RF objective “FR1 HPUE enhancements for TN”

- The work objectives shall include definition of HPUE RF requirements for single carrier cases

  - PC1 with 4Tx for FWA UEs for TDD bands

  - PC1.5 with 2Tx for handheld UEs for FDD bands

Proposal #3:	UE RF objective “6MHz channel bandwidth”

- The work objectives shall include definition of

  - Generic RF requirements (CBW, spectrum utilization, UE RF, BS RF)

  - Band specific RF requirements

  - Necessary signalling

- 6MHz CBW is limited to 15kHz SCS

- Introduce 6MHz as an optional feature from Rel-15

- Handle objective as a separate work item with 12 months duration

Proposal #4:	UE RF objective “Reduced UE MPR values with relaxed Tx EVM”

- Do not include “Reduced UE MPR values with relaxed Tx EVM” objective in the overall 5G-A Rel-20 package. RAN4 to include the EVM relaxation use case in the scope 6G RAN4 studies and study both 5G non-linear receiver and 6G AI-ML receiver altogether.

- The study objectives shall include

  - Evaluation of feasible Tx EVM relaxation for different UEs and assessment of associated MPR improvement.

  - Evaluation of achievable performance improvement incl. link-level and system-level performance and various scenarios incl. MU-MIMO.

  - Evaluation of BS complexity and implementation feasibility

# References

RP-251789 “WF for RAN4 led REL-20 topics”, RAN4 Chair (Apple), RAN #108, June 2025