3GPP TSG-RAN Meeting #109	RP-252750

Beijing, China, September 15-18, 2025

Agenda Item:	9.1.4.4

Source:	Intel Corporation

Title:	Views on RAN4 Rel-20 5G-A package – RRM topics

Document for:	Discussion and decision

# Introduction

This contribution summarizes Intel’s views on the scope of RAN4 5G-A Release 20 work scope for RRM topics and provides further views on top of the WF endorsed in RAN#108 [1].

# Views on RRM topics

The following agreements are provided in RAN #108 WF [1] for RRM centric RAN4 topics:

The high-level scope for WI on RRM enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - (e)RedCap UE enhancement - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 LT5 WI - This also include example bands, UE RF scope including potential complexity reduction, UE demod - Fast SCell activation - Further discussion on the possible down scoping in RAN#109 based on the following two objectives - SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026. - A-TRS/TRS based unknown SCell activation including both inter-band CA and intra-band non-contiguous CA - Consider simplification of solution to maintain reasonable work scope in consideration of the TU allocation - No RAN1/2 scope is expected - Target 0.5 TU (0.25RD and 0.25RF) in Q4’25 and 1 TU allocation (0.75RD,0.25RF) from Q2’26

## (e)RedCap UE enhancement

Views on objective scope/prioritization

- Less than 5MHz for (e)RedCap

  - The work should introduce necessary requirements to enable 3 MHz operation for RedCap type of devices with 15 PRB transmission bandwidth configuration and 12 PRB SSB (i.e. 12 PRB transmission bandwidth for band n100 is not in scope)

  - The necessary RF/RRM/Demodulation requirement should be considered

  - The work should cover 1Rx/2Rx devices and HD-FDD

- RLM/BFD relaxation

  - The work should strive to reuse the conclusions of R17 NR UE power saving WI (NR_UE_pow_sav_enh).

  - The work should cover 2Rx and 1Rx types of (e)RedCap devices

  - The work on introducing relaxations for RedCap devices with ≥ 5MHz CBW should be prioritized (i.e. no overlap between objectives).

Candidate objectives

- Specify requirements for (e)RedCap to support 3 MHz channel bandwidth with 15 PRB transmission bandwidth configuration and 12 PRB SSB

  - Specify UE RF requirements

  - Specify RRM requirements

  - Specify UE demodulation requirements

- Specify RRM requirements for (e)RedCap for relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle

  - The relaxation solutions should be limited to solutions defined in Rel-17 NR_UE_pow_sav_enh WI

TU allocation

- Given that objectives include both RF/RRM/Demod scope, a dedicated WI is recommended.

- The workload is relatively small and consider 0.25RD / 0.25RF TUs with ~12 months duration

Proposal #1:	RRM objective “(e)RedCap UE enhancement”

- The work objectives shall include definition of requirements for

  - (e)RedCap RRM requirements for RLM/BFD relaxation based on solutions defined in Rel-17 NR_UE_pow_sav_enh WI

  - (e)RedCap UE RF / RRM / Demod requirements for 3 MHz operation with 15 PRB transmission bandwidth configuration and 12 PRB SSB

- Handle the objective as a separate work item with ~12 months duration

## Fast SCell activation

Views on objective scope/prioritization

A single objective is preferred to maintain a reasonable workload and propose to focus on “Scell activation reduction for intra-band co-located CA, incl. SSB-less cases and enhanced CSI reporting” and “A-TRS/TRS based unknown SCell activation” can be de-prioritized.

Below we provide further details of the possible Scell activation reduction objective scope:

- The work cover intra-band co-located CA scenarios including contiguous and non-contiguous CA scenarios. Further extension to inter-band CA can be helpful and should relatively straightforward, and as agreed it should be discussed at a later point once the progress of the WI is clear.

- Enhanced CSI reporting term is ambiguous should be clarified. In our understanding the underlying idea of CSI reporting enhancements is to allow the network to reliably schedule UE before the legacy ending point of the SCell activation procedures (i.e. before UE sends a valid CSI report). For instance, the network may be expected to apply the CSI report corresponding to the PCell. RAN4 is expected to assess feasibility of such solution and identify appropriate conditions, when such approach may be suitable.

- Per prior agreements the work should cover SSB-less SCell operation scenarios. For the specific case the SCell reductions can be achieved via scheduling of SSB-less SCell before (without) the reception of TRS signals at the UE side along with mentioned scheduling approach without CSI reporting.

Candidate objectives:

- Specify RRM requirements for SCell activation delay reduction for intra-band co-located based on

  - Scheduling of SCell before without a valid CSI report incl. scenarios with SCell with and without SSB transmissions

  - Scheduling SCell without the need for TRS reception for SSB-less SCell scenarios

TU allocation

- Expect limited workload and may consider shorter work duration (e.g., 12 months).

Proposal #2:	RRM objective “Fast SCell activation”

- The work objectives shall include definition of

  - RRM requirements for SCell activation delay reduction for intra-band co-located CA including the following solutions

    - Scheduling of SCell before without a valid CSI report incl. scenarios with SCell with and without SSB transmissions

    - Scheduling SCell prior without the need for TRS reception for SSB-less SCell scenarios

# Conclusions

In summary, we make the following proposals on the scope of RAN4 Rel-20 5G-A RRM topics:

Proposal #1:	RRM objective “(e)RedCap UE enhancement”

- The work objectives shall include definition of requirements for

  - (e)RedCap RRM requirements for RLM/BFD relaxation based on solutions defined in Rel-17 NR_UE_pow_sav_enh WI

  - (e)RedCap UE RF / RRM / Demod requirements for 3 MHz operation with 15 PRB transmission bandwidth configuration and 12 PRB SSB

- Handle the objective as a separate work item with ~12 months duration

Proposal #2:	RRM objective “Fast SCell activation”

- The work objectives shall include definition of

  - RRM requirements for SCell activation delay reduction for intra-band co-located CA including the following solutions

    - Scheduling of SCell before without a valid CSI report incl. scenarios with SCell with and without SSB transmissions

    - Scheduling SCell prior without the need for TRS reception for SSB-less SCell scenarios

# References

RP-251789 “WF for RAN4 led REL-20 topics”, RAN4 Chair (Apple), RAN #108, June 2025