3GPP TSG-RAN WG2 Meeting #131	R2-2506363

# Bangalore, India, Aug 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5439 | rev | 1 | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on configured grant power control in unified TCI framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ofinno |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2025-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In ConfiguredGrantConfig, pathlossReferenceIndex, p0-PUSCH-Alpha and powerControlLoopToUse fields are mandatory present and are used in UL power control during configured grant transmission. - According to 38.213, if a UE is configured with a unified TCI state framework (e.g., unifiedTCI-StateType is configured), the UE determines PUSCH transmission power based on power control parameters (e.g., p0, alpha, closedLoopIndex in p0AlphaSetforPUSCH) associated with an indicated TCI state and a pathloss reference signal (e.g., pathlossReferenceRS-Id-r17) associated with the indicated TCI state. Therefore, a UE in unified TCI state framework does not use the power control parameters (e.g., pathlossReferenceIndex, p0-PUSCH-Alpha and powerControlLoopToUse fields) of the the configuredGrantConfig and instead UE uses the power control parameters and the pathloss reference signal associated with the indicated TCI state for PUSCH power control. UE behavior may be unclear when UE is configured with a unified TCI state framework and a configured grant configuration having mandatory fields for UL power control. It needs to be clarified that the UE in unified TCI state framework does not use these mandatory fields for UL power control of the configured grant configuration. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: UL power control Inter-operability: If the network is implemented according to the CR and the UE is not, the network configures the UE to ignore (i.e., not use) UL power control parameter (pathlossReferenceIndex, p0-PUSCH-Alpha, powerControlLoopToUse) for unified TCI framework, while the UE uses it which may lead the UE to transmit with the wrong UL Tx power. If the UE is implemented according to the CR and the network is not, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that pathlossReferenceIndex that is mandatory present in ConfiguredGrantConfig is not used in unified TCI framework. Clarify that p0-PUSCH-Alpha that is mandatory present in ConfiguredGrantConfig is not used in unified TCI framework. Clarify that powerControlLoopToUse that is mandatory present in ConfiguredGrantConfig is not used in unified TCI framework. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may be configured with UL power control parameters for the unified TCI framework that are inconsistent from a UE perspective, possibly resulting in RRC reestablishment or unclear UE behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2505784 |  |  |  |  |  |  |  |  |

Start of Changes

#### –	ConfiguredGrantConfig

The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2). Multiple Configured Grant configurations may be configured in one BWP of a serving cell.

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                INTEGER (1..64)                                                         OPTIONAL,   -- Need R

rrc-ConfiguredUplinkGrant           SEQUENCE {

timeDomainOffset                    INTEGER (0..5119),

timeDomainAllocation                INTEGER (0..15),

frequencyDomainAllocation           BIT STRING (SIZE(18)),

antennaPort                         INTEGER (0..31),

dmrs-SeqInitialization              INTEGER (0..1)                                                         OPTIONAL,   -- Need R

precodingAndNumberOfLayers          INTEGER (0..63),

srs-ResourceIndicator               INTEGER (0..15)                                                        OPTIONAL,   -- Need R

mcsAndTBS                           INTEGER (0..31),

frequencyHoppingOffset              INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                          OPTIONAL,   -- Need R

pathlossReferenceIndex              INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...,

[[

pusch-RepTypeIndicator-r16          ENUMERATED {pusch-RepTypeA,pusch-RepTypeB}                             OPTIONAL,   -- Need M

frequencyHoppingPUSCH-RepTypeB-r16  ENUMERATED {interRepetition, interSlot}                                OPTIONAL,   -- Cond RepTypeB

timeReferenceSFN-r16                ENUMERATED {sfn512}                                                    OPTIONAL    -- Need S

]],

[[

pathlossReferenceIndex2-r17        INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1)                        OPTIONAL,   -- Need R

srs-ResourceIndicator2-r17         INTEGER (0..15)                                                         OPTIONAL,   -- Need R

precodingAndNumberOfLayers2-r17    INTEGER (0..63)                                                         OPTIONAL,   -- Need R

timeDomainAllocation-v1710         INTEGER (16..63)                                                        OPTIONAL,    -- Need M

timeDomainOffset-r17               INTEGER (0..40959)                                                      OPTIONAL,   -- Need R

cg-SDT-Configuration-r17           CG-SDT-Configuration-r17                                                OPTIONAL    -- Need M

]]

}                                                                                                           OPTIONAL,   -- Need R

...,

[[

cg-RetransmissionTimer-r16              INTEGER (1..64)                                                     OPTIONAL,   -- Need R

cg-minDFI-Delay-r16                     ENUMERATED

{sym7, sym1x14, sym2x14, sym3x14, sym4x14, sym5x14, sym6x14, sym7x14, sym8x14,

sym9x14, sym10x14, sym11x14, sym12x14, sym13x14, sym14x14,sym15x14, sym16x14

}                                                   OPTIONAL,   -- Need R

cg-nrofPUSCH-InSlot-r16                 INTEGER (1..7)                                              OPTIONAL,   -- Need R

cg-nrofSlots-r16                        INTEGER (1..40)                                             OPTIONAL,   -- Need R

cg-StartingOffsets-r16                  CG-StartingOffsets-r16                                      OPTIONAL,   -- Need R

cg-UCI-Multiplexing-r16                 ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

cg-COT-SharingOffset-r16                INTEGER (1..39)                                             OPTIONAL,   -- Need R

betaOffsetCG-UCI-r16                    INTEGER (0..31)                                            OPTIONAL,   -- Need R

cg-COT-SharingList-r16                  SEQUENCE (SIZE (1..1709)) OF CG-COT-Sharing-r16             OPTIONAL,   -- Need R

harq-ProcID-Offset-r16                  INTEGER (0..15)                                             OPTIONAL,   -- Need M

harq-ProcID-Offset2-r16                 INTEGER (0..15)                                             OPTIONAL,   -- Need M

configuredGrantConfigIndex-r16          ConfiguredGrantConfigIndex-r16                              OPTIONAL,   -- Cond CG-List

configuredGrantConfigIndexMAC-r16       ConfiguredGrantConfigIndexMAC-r16                           OPTIONAL,   -- Cond CG-IndexMAC

periodicityExt-r16                      INTEGER (1..5120)                                           OPTIONAL,   -- Need R

startingFromRV0-r16                     ENUMERATED {on, off}                                        OPTIONAL,   -- Need R

phy-PriorityIndex-r16                   ENUMERATED {p0, p1}                                         OPTIONAL,   -- Need R

autonomousTx-r16                        ENUMERATED {enabled}                                        OPTIONAL    -- Cond LCH-BasedPrioritization

]],

[[

cg-betaOffsetsCrossPri0-r17             SetupRelease { BetaOffsetsCrossPriSelCG-r17 }               OPTIONAL,   -- Need M

cg-betaOffsetsCrossPri1-r17             SetupRelease { BetaOffsetsCrossPriSelCG-r17 }               OPTIONAL,   -- Need M

mappingPattern-r17                      ENUMERATED {cyclicMapping, sequentialMapping}               OPTIONAL,   -- Cond SRSsets

sequenceOffsetForRV-r17                 INTEGER (0..3)                                              OPTIONAL,   -- Need R

p0-PUSCH-Alpha2-r17                     P0-PUSCH-AlphaSetId                                         OPTIONAL,   -- Need R

powerControlLoopToUse2-r17              ENUMERATED {n0, n1}                                         OPTIONAL,   -- Need R

cg-COT-SharingList-r17                  SEQUENCE (SIZE (1..50722)) OF CG-COT-Sharing-r17             OPTIONAL,   -- Need R

periodicityExt-r17                      INTEGER (1..40960)                                          OPTIONAL,   -- Need R

repK-v1710                              ENUMERATED {n12, n16, n24, n32}                             OPTIONAL,   -- Need R

nrofHARQ-Processes-v1700                INTEGER(17..32)                                             OPTIONAL,   -- Need M

harq-ProcID-Offset2-v1700               INTEGER (16..31)                                            OPTIONAL,   -- Need R

configuredGrantTimer-v1700              INTEGER(33..288)                                            OPTIONAL,   -- Need R

cg-minDFI-Delay-v1710                   INTEGER (238..3584)                                         OPTIONAL    -- Need R

]],

[[

harq-ProcID-Offset-v1730                INTEGER (16..31)                                            OPTIONAL,   -- Need R

cg-nrofSlots-r17                        INTEGER (1..320)                                            OPTIONAL    -- Need R

]]

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic                                 SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic                              BetaOffsets

}

CG-COT-Sharing-r16 ::= CHOICE {

noCOT-Sharing-r16                   NULL,

cot-Sharing-r16                     SEQUENCE {

duration-r16                       INTEGER (1..39),

offset-r16                         INTEGER (1..39),

channelAccessPriority-r16          INTEGER (1..4)

}

}

CG-COT-Sharing-r17 ::=  CHOICE {

noCOT-Sharing-r17                   NULL,

cot-Sharing-r17                     SEQUENCE {

duration-r17                       INTEGER (1..319),

offset-r17                         INTEGER (1..319)

}

}

CG-StartingOffsets-r16 ::= SEQUENCE {

cg-StartingFullBW-InsideCOT-r16         SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)             OPTIONAL,   -- Need R

cg-StartingFullBW-OutsideCOT-r16        SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)             OPTIONAL,   -- Need R

cg-StartingPartialBW-InsideCOT-r16      INTEGER (0..6)                                       OPTIONAL,   -- Need R

cg-StartingPartialBW-OutsideCOT-r16     INTEGER (0..6)                                       OPTIONAL    -- Need R

}

BetaOffsetsCrossPriSelCG-r17 ::= CHOICE {

dynamic-r17         SEQUENCE (SIZE (1..4)) OF BetaOffsetsCrossPri-r17,

semiStatic-r17      BetaOffsetsCrossPri-r17

}

CG-SDT-Configuration-r17 ::= SEQUENCE {

cg-SDT-RetransmissionTimer   INTEGER (1..64)                                                 OPTIONAL,   -- Need R

sdt-SSB-Subset-r17       CHOICE {

shortBitmap-r17          BIT STRING (SIZE (4)),

mediumBitmap-r17         BIT STRING (SIZE (8)),

longBitmap-r17           BIT STRING (SIZE (64))

}                                                                                            OPTIONAL,   -- Need S

sdt-SSB-PerCG-PUSCH-r17   ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight, sixteen}  OPTIONAL,   -- Need M

sdt-P0-PUSCH-r17         INTEGER (-16..15)                                                   OPTIONAL, -- Need M

sdt-Alpha-r17            ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need M

sdt-DMRS-Ports-r17       CHOICE {

dmrsType1-r17            BIT STRING (SIZE (8)),

dmrsType2-r17            BIT STRING (SIZE (12))

}                                                                                            OPTIONAL,  -- Need M

sdt-NrofDMRS-Sequences-r17  INTEGER (1..2)                                                   OPTIONAL   -- Need M

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

ConfiguredGrantConfig field descriptions

antennaPort Indicates the antenna port(s) to be used for this configuration, and the maximum bitwidth is 5. See TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1. The UE ignores this field in case of CG-SDT.

autonomousTx If this field is present, the Configured Grant configuration is configured with autonomous transmission, see TS 38.321 [3].

betaOffsetCG-UCI Beta offset for CG-UCI in CG-PUSCH, see TS 38.213 [13], clause 9.3

cg-betaOffsetsCrossPri0, cg-betaOffsetsCrossPri1 Selection between and configuration of dynamic and semi-static beta-offset for multiplexing HARQ-ACK in CG-PUSCH with different priorities. The field cg-betaOffsetsCrossPri0 indicates multiplexing LP HARQ-ACK in HP CG-PUSCH. This field is configured only if phy-PriorityIndex-r16 is configured with value p1. The field cg-betaOffsetsCrossPri1 indicates multiplexing HP HARQ-ACK in LP CG-PUSCH. This field is configured only if phy-PriorityIndex-r16 is configured with value p0.

cg-COT-SharingList Indicates a table for COT sharing combinations (see 37.213 [48], clause 4.1.3). One row of the table can be set to noCOT-Sharing to indicate that there is no channel occupancy sharing. If the cg-RetransmissionTimer-r16 is configured and the UE operates as an initiating device in semi-static channel access mode (see TS 37.213 [48], clause 4.3), then cg-COT-SharingList-r16 is configured.

cg-COT-SharingOffset Indicates the offset from the end of the slot where the COT sharing indication in UCI is enabled where the offset in symbols is equal to 14*n, where n is the signaled value for cg-COT-SharingOffset. Applicable when ul-toDL-COT-SharingED-Threshold-r16 is not configured (see 37.213 [48], clause 4.1.3).

cg-DMRS-Configuration DMRS configuration (see TS 38.214 [19], clause 6.1.2.3).

cg-minDFI-Delay Indicates the minimum duration (in unit of symbols) from the ending symbol of the PUSCH to the starting symbol of the PDCCH containing the downlink feedback indication (DFI) carrying HARQ-ACK for this PUSCH. The HARQ-ACK received before this minimum duration is not considered as valid for this PUSCH (see TS 38.213 [13], clause 10.5). The following minimum duration values are supported, depending on the configured subcarrier spacing [symbols]: 15 kHz: 7, m*14, where m = {1, 2, 3, 4} 30 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8} 60 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16} 120 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32} 480 kHz: m*14, where m = {2, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100, 104, 108, 112, 116, 120, 124, 128} 960 kHz: m*14, where m = {4, 8, 16, 24, 32, 40, 48, 56, 64, 72, 80, 88, 96, 104, 112, 120, 128, 136, 144, 152, 160, 168, 176, 184, 192, 200, 208, 216, 224, 232, 240, 248, 256}

cg-nrofPUSCH-InSlot Indicates the number of consecutive PUSCH configured to CG within a slot where the SLIV indicating the first PUSCH and additional PUSCH appended with the same length (see TS 38.214 [19], clause 6.1.2.3). The network can only configure this field if cg-RetransmissionTimer is configured.

cg-nrofSlots Indicates the number of allocated slots in a configured grant periodicity following the time instance of configured grant offset (see TS 38.214 [19], clause 6.1.2.3). cg-nrofSlots-r17 is only applicable for operation with shared spectrum channel access in FR2-2. When cg-nrofSlots-r17 is configured, the UE shall ignore cg-nrofSlots-r16. The network can only configure this field if cg-RetransmissionTimer is configured.

cg-RetransmissionTimer Indicates the initial value of the configured retransmission timer (see TS 38.321 [3]) in multiples of periodicity. The value of cg-RetransmissionTimer is always less than or equal to the value of configuredGrantTimer. This field is always configured together with harq-ProcID-Offset. This field is not configured for operation in licensed spectrum or simultaneously with harq-ProcID-Offset2. The network does not configure this field for CG-SDT.

cg-StartingOffsets This field is not applicable for a UE which is allowed to operate as an initiating device in semi-static channel access mode, i.e., not applicable for a UE configured with UE FFP parameters (e.g. period, offset) regardless whether the UE would initiate its own COT or would share gNB's COT.

cg-UCI-Multiplexing If present, this field indicates that in the case of PUCCH overlapping with CG-PUSCH(s) including CG-UCI within a PUCCH group, HARQ-ACK is multiplexed on the CG-PUSCH including CG-UCI (see TS 38.213 [13], clause 9).

configuredGrantConfigIndex Indicates the index of the Configured Grant configurations within the BWP.

configuredGrantConfigIndexMAC Indicates the index of the Configured Grant configurations within the MAC entity.

configuredGrantTimer Indicates the initial value of the configured grant timer (see TS 38.321 [3]) in multiples of periodicity. When cg-RetransmissonTimer is configured, if HARQ processes are shared among different configured grants on the same BWP, configuredGrantTimer * periodicity is set to the same value for the configurations that share HARQ processes on this BWP. The value of the extension configuredGrantTimer is 2 times the configured value.

dmrs-SeqInitialization The network configures this field if transformPrecoder is disabled or when the value of sdt-NrofDMRS-Sequences is set to 1. Otherwise, the field is absent.

frequencyDomainAllocation Indicates the frequency domain resource allocation, see TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1).

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured. The field frequencyHopping applies to configured grant for 'pusch-RepTypeA' (see TS 38.214 [19], clause 6.3.1).

frequencyHoppingOffset Frequency hopping offset used when frequency hopping is enabled (see TS 38.214 [19], clause 6.1.2 and clause 6.3).

frequencyHoppingPUSCH-RepTypeB Indicates the frequency hopping scheme for Type 1 CG when pusch-RepTypeIndicator is set to 'pusch-RepTypeB' (see TS 38.214 [19], clause 6.1). The value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, the frequency hopping is not enabled for Type 1 CG.

harq-ProcID-Offset For operation with shared spectrum channel access configured with cg-RetransmissionTimer-r16, this configures the range of HARQ process IDs which can be used for this configured grant where the UE can select a HARQ process ID within [harq-procID-offset, .., (harq-procID-offset + nrofHARQ-Processes – 1)]. harq-ProcID-Offset-v1730 is only applicable for operation with shared spectrum channel access in FR2-2. If the field harq-ProcID-Offset-v1730 is present, the UE shall ignore the harq-ProcID-Offset-r16. The network does not configure this field for CG-SDT.

harq-ProcID-Offset2 Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.4.1. This field is not configured together with cg-RetransmissionTimer-r16. If the field harq-ProcID-Offset2-v1700 is present, the UE shall ignore the harq-ProcID-Offset2-r16.

mappingPattern Indicates whether the UE should follow Cyclical mapping pattern or Sequential mapping pattern when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' for PUSCH transmission with a Type 1 configured grant and/or a Type 2 configured grant as described in clause 6.1.2.3 of TS 38.214 [19]

mcs-Table Indicates the MCS table the UE shall use for PUSCH without transform precoding. If the field is absent the UE applies the value qam64.

mcs-TableTransformPrecoder Indicates the MCS table the UE shall use for PUSCH with transform precoding. If the field is absent the UE applies the value qam64.

mcsAndTBS The modulation order, target code rate and TB size (see TS 38.214 [19], clause 6.1.2). The NW does not configure the values 28~31 in this version of the specification.

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321 [3], clause 5.4.1. If the UE is configured with nrofHARQ-Processes-v1700, the UE shall ignore nrofHARQ-Processes (without suffix).

pathlossReferenceIndex Indicates the reference signal index used as PUSCH pathloss reference (see TS 38.213 [13], clause 7.1.1). In case of CG-SDT or if unifiedTCI-StateType is configured, the UE does not use this field.

pathlossReferenceIndex2 Indicates the reference signal used as PUSCH pathloss reference for the second SRS resource set. When this field is present, pathlossReferenceIndex indicates the reference signal used as PUSCH pathloss reference for the first SRS resource set

p0-PUSCH-Alpha Index of the P0-PUSCH-AlphaSet to be used for this configuration. If unifiedTCI-StateType is configured, the UE does not use this field.

p0-PUSCH-Alpha2 Index of the P0-PUSCH-AlphaSet to be used for second SRS resource set. If this field is present, the p0-PUSCH-Alpha provides index for the P0-PUSCH-AlphaSet to be used for first SRS resource set.

periodicity Periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} 30 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} 60 kHz with normal CP 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 60 kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 120 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} 480 and 960 kHz: n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} In case of SDT, the network does not configure periodicity values less than 5ms.

periodicityExt This field is used to calculate the periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). If this field is present, the UE shall ignore field periodicity (without suffix). Network does not configure periodicityExt-r17 together with periodicityExt-r16. The following periodicites are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 640. 30 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 1280. 60 kHz with normal CP: periodicityExt*14, where periodicityExt has a value between 1 and 2560. 60 kHz with ECP: periodicityExt*12, where periodicityExt has a value between 1 and 2560. 120 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 5120. 480 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 20480. 960 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 40960. In case of SDT, the network does not configure periodicity values less than 5ms.

phy-PriorityIndex Indicates the PHY priority of CG PUSCH at least for PHY-layer collision handling. Value p0 indicates low priority and value p1 indicates high priority. The network does not configure this for CG-SDT.

powerControlLoopToUse Closed control loop to apply (see TS 38.213 [13], clause 7.1.1). If unifiedTCI-StateType is configured, the UE does not use this field.

powerControlLoopToUse2 Closed control loop to apply to second SRS resource set (see TS 38.213 [13], clause 7.1.1). If this field is present, the powerControlLoopToUse applies to the first SRS resource set.

precodingAndNumberOfLayers Indicates the precoding and number of layers (see TS 38.212 [17], clause 7.3.1.1.2, and TS 38.214 [19], clause 6.1.2.3). In case of CG-SDT, network sets this field to 1.

precodingAndNumberOfLayers2 Indicates the precoding and number of layers for the second SRS resource set. When this field is present, precodingAndNumberOfLayers indicated the precoding and number of layers for the first SRS resource set.

pusch-RepTypeIndicator Indicates whether UE follows the behavior for PUSCH repetition type A or the behavior for PUSCH repetition type B for each Type 1 configured grant configuration. The value pusch-RepTypeA enables the 'PUSCH repetition type A' and the value pusch-RepTypeB enables the 'PUSCH repetition type B' (see TS 38.214 [19], clause 6.1.2.3). The network does not configure this field if cg-RetransmissionTimer-r16 is configured for CG operation with shared spectrum channel access.

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent. Note: rbg-Size is used when the transformPrecoder parameter is disabled.

repK-RV The redundancy version (RV) sequence to use. See TS 38.214 [19], clause 6.1.2. The network configures this field if repetitions are used, i.e., if repK is set to n2, n4 or n8. This field is not configured when cg-RetransmissionTimer is configured. Otherwise, the field is absent.

repK Number of repetitions K, see TS 38.214 [19]. If the field repK-v1710 is present, the UE shall ignore the repK (without suffix).

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1. For Type 1 UL data transmission without grant, resourceAllocation should be resourceAllocationType0 or resourceAllocationType1.

rrc-ConfiguredUplinkGrant Configuration for "configured grant" transmission with fully RRC-configured UL grant (Type1). If this field is absent the UE uses UL grant configured by DCI addressed to CS-RNTI (Type2).

sequenceOffsetForRV Configures the RV offset for the starting RV for the first repetition (first actual repetition in PUSCH repetition Type B) towards the second 'SRS resource set' for PUSCH configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'.

srs-ResourceIndicator Indicates the SRS resource to be used. The network does not configure this for CG-SDT.

srs-ResourceIndicator2 Indicates the SRS resource to be used for the second SRS resource set. When this field is present, the srs-ResourceIndicator is used for the first SRS resource set.

startingFromRV0 This field is used to determine the initial transmission occasion of a transport block for a given RV sequence, see TS 38.214 [19], clause 6.1.2.3.1. The network does not configure this field if cg-RetransmissionTimer-r16 is configured for CG operation.

timeDomainAllocation, timeDomainAllocation-v1710 Indicates a combination of start symbol and length and PUSCH mapping type, see TS 38.214 [19], clause 6.1.2 and TS 38.212 [17], clause 7.3.1. If the field timeDomainAllocation-v1710 is present, the UE shall ignore timeDomainAllocation field (without suffix).

timeDomainOffset Offset related to the reference SFN indicated by timeReferenceSFN, see TS 38.321 [3], clause 5.8.2. timeDomainOffset-r17 is only applicable to 480 kHz and 960 kHz. If timeDomainOffset-r17 is present, the UE shall ignore timeDomainOffset (without suffix).

timeReferenceSFN Indicates SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration, see TS 38.321 [3], clause 5.8.2. If the field timeReferenceSFN is not present, the reference SFN is 0.

transformPrecoder Enables or disables transform precoding for type1 and type2. If the field is absent, the UE enables or disables transform precoding in accordance with the field msg3-transformPrecoder in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList), see TS 38.214 [19], clause 6.1.3.

uci-OnPUSCH Selection between and configuration of dynamic and semi-static beta-offset. For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic. The network does not configure this for CG-SDT.

CG-COT-Sharing field descriptions

channelAccessPriority Indicates the Channel Access Priority Class that the gNB can assume when sharing the UE initiated COT (see 37.213 [48], clause 4.1.3).

duration Indicates the number of DL transmission slots within UE initiated COT (see 37.213 [48], clause 4.1.3).

offset Indicates the number of DL transmission slots from the end of the slot where CG-UCI is detected after which COT sharing can be used (see 37.213 [48], clause 4.1.3).

CG-StartingOffsets field descriptions

cg-StartingFullBW-InsideCOT A set of configured grant PUSCH transmission starting offsets (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation includes all interlaces in the allocated RB set(s) and the CG PUSCH resource is inside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingFullBW-OutsideCOT A set of configured grant PUSCH transmission starting offset indices (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation includes all interlaces in the allocated RB set(s) and the CG PUSCH resource is outside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingPartialBW-InsideCOT A set of configured grant PUSCH transmission starting offset index (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation does not include all interlaces in the allocated RB set(s) and the CG PUSCH resource is inside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingPartialBW-OutsideCOT A set of configured grant PUSCH transmission starting offset index (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation does not include all interlaces in the allocated RB set(s) and the CG PUSCH resource is outside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

CG-SDT-Configuration field descriptions

cg-SDT-RetransmissionTimer Indicates the initial value of the configured grant retransmission timer used for the initial transmission of CG-SDT with CCCH message (see TS 38.321 [3]) in multiples of periodicity.

sdt-DMRS-Ports Indicates the set of DMRS ports for SSB to PUSCH mapping (see TS 38.213 [13]). The first (left-most / most significant) bit corresponds to DMRS port 0, the second most significant bit corresponds to DMRS port 1, and so on. A bit set to 1 indicates that this DMRS port is used for mapping. In case of a RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-NrofDMRS-Sequences Indicates the number of DMRS sequences for SSB to PUSCH mapping (see TS 38.213 [13]). In case of a RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-SSB-Subset Indicates SSB subset for SSB to CG PUSCH mapping within one CG configuration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not included in the SSB subset for SSB to CG PUSCH mapping while value 1 indicates that the corresponding SS/PBCH block is included in SSB subset for SSB to CG PUSCH mapping. If this field is absent, UE assumes the SSB set includes all actually transmitted SSBs. In case of a RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-SSB-PerCG-PUSCH The number of SSBs per CG PUSCH (see TS 38.213 [13]). Value one corresponds to 1 SSBs per CG PUSCH, value two corresponds to 2 SSBs per CG PUSCH and so on. In case of a RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-P0-PUSCH Indicates P0 value for PUSCH for CG SDT in steps of 1dB (see TS 38.213 [13]). When this field is configured, the UE ignores the p0-PUSCH-Alpha.

sdt-Alpha Indicates alpha value for PUSCH for CG SDT. alpha0 indicates value 0 is used alpha04 indicates value 4 is used and so on (see TS 38.213 [13]). When this field is configured, the UE ignores the p0-PUSCH-Alpha.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-BasedPrioritization | This field is optionally present, Need R, if lch-BasedPrioritization is configured in the MAC entity. It is absent otherwise. |
| RepTypeB | The field is optionally present if pusch-RepTypeIndicator is set to pusch-RepTypeB, Need S, and absent otherwise. |
| CG-List | The field is mandatory present when included in configuredGrantConfigToAddModList-r16, otherwise the field is absent. |
| CG-IndexMAC | The field is mandatory present if at least one configured grant is configured by configuredGrantConfigToAddModList-r16 in any BWP of this MAC entity, otherwise it is optionally present, need R. |
| SRSsets | This field is mandatory present when UE is configured with two SRS sets configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage codebook or non-codebook. Otherwise it is absent, Need R |

End of Changes