3GPP TSG RAN meeting #109										RP-252848

Beijing, China, September 15-18th, 2025

## Status Report to TSG

Agenda item:			9.7.1.1

| WI Name | Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_combinedLband |  |  |  |  |
| Unique ID | 1040134 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-250983 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 09/2025 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 100% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Luca Lodigiani |
|  | Company | Aalyria |
|  | Email | luca@aalyria.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

The Work Item is Complete in both Core and Performance parts and can be closed.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

### 2.4.2	Agreements after RAN4#116

The discussion in RAN4#116 focused on the remaining issues of the additional ETSI-specific emission requirements, protection of Radio-astronomy (RA) stations and finalizing A-MPR values for the UE.

An LS from ETSI TC SES was received (R4-2509023, SES(25)000071), and a reply was sent to ETSI ([5] R4-2511848), specifically in respect to the additional emission requirements.

The following Agreements were reached ([2] R4-2511751):

#### Issue 1-1-2: Handling of UE requirements based on ETSI EN standards

- Way Forward:

  - Capture two sets of additional UE emission requirements and associated A-MPR

    - Set 1 (NS_09N, NS_10N): covers emission requirements in accordance with EN 301 681 v2.1.2

    - Set 2 (NS_13N, NS_14N): aligned with the information in the LS from ETSI TC SES SCN (R4-2509023, SES(25)000071) and should be used to ensure UE support for the new requirements is in place, when appropriate (i.e. if/when the new ETSI requirements get ratified).

      - NS_13N and NS_14N will be captured without applicable bands, for the moment (as per below), until further revision

    - These requirements and corresponding A-MPR may be later revised in the maintenance phase, as necessary

  - Include NOTE5 in the spec, as follows, to clarify requirement applicability:

| NS_13N5 | 6.5.3.3.10 |  | 5 |  |  |
| --- | --- | --- | --- | --- | --- |
| NS_14N5 | 6.5.3.3.10 |  | 5, 10, 15, 20 |  | Clause 6.2.3.9 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. NOTE 4: The NS_07N and NS_08N is signalled based on coordination between operators and subject to regional/national regulation NOTE5: NS_09N (n253, n250), NS_10N (n251) are currently in force. NS_13N (n253, n250), NS_14N (n251) may supersede NS_09N, NS_10N when the new ETSI requirements get ratified. |  |  |  |  |  |

#### Issue 1-1-3: Cross-device GNSS Protection

- Way Forward:

  - It is RAN4’s understanding that requirements for GNSS protection are already covered by the additional ETSI out-of-band emission requirements.

#### Issue 1-1-4: Additional TX OFF Measurement

- Way Forward:

  - Any additional requirements for Tx OFF measurement in the radio astronomy frequency range, if necessary, will be handled in the maintenance phase.

### 2.4.3	Remaining Open issues after RAN4#116

None in either Core or Performance Parts.

The Work Item can be marked as Complete.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- RP-250983 - Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges – Aalyria, Inmarsat, Viasat

- R4-2511751	WF on NR IOT-NTN band emission requirements	Aalyria

- R4-2510922 - CR for TS 38.108, Introduction on band n253, n251 and n250		CATT, THALES, ZTE, Aalyria

- R4-2511631 - CR to TS 38.133: Introduction of new NTN Combined L-bands n253, n251, n250	Aalyria, ZTE Corporation, Sanechips, THALES

- R4-2511802 - CR to TR 38.863: Introduction of combined L-bands n253, n251, n250	Aalyria, THALES, Apple

- R4-2511848 - Reply LS on L-band NTN emission requirements	ViaSat

- R4-2511861 - CR to TS 38.101-5: Introduction of combined L-bands	Apple, Aalyria Technologies

- R4-2509023	LS on Harmonised Standard for NTN capable UE	ETSI TC SES