3GPP TSG RAN Meeting #109									RP-252886

Beijing, China, September 15-18, 2025

Agenda Item:		9.6.2.2

Source:			Thales, CATT

Title:	WI summary for WI Core part: Non-Terrestrial Networks (NTN) for NR Phase 3

WI code(s):		NR_NTN_Ph3

Leading WG:		RAN2

Release:			Rel-19

1	Introduction

Non-Terrestrial Networks (NTN) are a fundamental component of the 5G-Advanced, aiming to provide ubiquitous connectivity, enhance reliability, and enable new service paradigms. Release 17 specified the foundational support for NR NTN, primarily focusing on transparent payload Low Earth Orbit (LEO) and Geostationary Orbit (GEO) satellites. Release 18 is enhancing these capabilities by introducing support for regenerative payloads, mobility management, and co-existence with Terrestrial Networks (TN).

In accordance to the approved WID for NR NTN in Rel-19 [1], the WI has defined further enhancements for NG-RAN based Non-Terrestrial Networks in order to:

- Offer optimized performance especially when addressing handset terminals (including smartphones with -5.5 dBi antenna gain) w.r.t. downlink coverage considering the NTN deployment constraints such as payload power limitation, large satellite foot print and limited feeder link bandwidth.

- Offer optimized capacity performance on uplink through multiplexing techniques.

- Enables MBS feature, especially for the case the intended service area is expected to be smaller than the coverage of a Uu cell.

- Support of regenerative payload architecture with full gNB on-board the satellite.

- Support of RedCap devices (e.g. handheld and IoT) operating in FR1 band NR-NTN networks to offer enhanced service capabilities (wideband/broadband) compared to IoT-NTN while ensuring low-complexity devices.

2	Description

- To improve NR downlink coverage in NTN:

  - the following enhancements are supported for NTN operating in FR1-NTN bands:

- Repetition of PDCCH in common search space.

- Repetitions of PDSCH carrying Msg4.

- Repetitions of PDSCH carrying SIB1 within 20 ms.

  - To extend the NR downlink coverage for NTN operating in, for FR1-NTN and FR2-NTN bands:

- Extended periodicity of 160 ms (apart from the existing 20ms value) for the half frames with SS/PBCH blocks is assumed by UE during initial cell selection.

- To improve uplink capacity in NR-NTN, the following enhancements are supported for PUSCH transmission:

- Inter-slot orthogonal cover code with length 2 to multiplex up to two UEs.

- Inter-slot orthogonal cover code with length 4 to multiplex up to four UEs.

- To support broadcast service in NR-NTN through

- enhanced SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area.

- enhanced signaling between CN and NG-RAN, intended service area has been introduced in MBS Service Area Information.

- To support regenerative payload

- the support of gNB on board has been added in TS 38.300.

- the NG Removal procedure has been introduced.

- To support Reduced Capability (RedCap) UEs and enhanced RedCap (eRedCap) UEs in NR NTN operating in FR1-NTN bands, through

- Enhancements to address issues caused by Timing Advance (TA) mismatch for Half-Duplex UE in paired spectrum are specified. This mismatch occurs between the actual TA used by the UE and the assumed TA for the UE at the gNB.

- Enhancements specifically target the issues in half-duplex collision cases at the UE side, including scenarios where semi-statically configured downlink reception collides with semi-statically configured uplink transmission, and where dynamically scheduled downlink reception collides with dynamically scheduled uplink transmission (see clause 17.2 of TS 38.213).

3	References

Latest WID

- RP-251954, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

Last Status Report

- RP-2519 Status report for WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

RAN1 CRs

- R1-2506653 Corrections to NR_NTN_Ph3, Ericsson, 38.211_CR0159

- R1-2506662 Corrections on Non-Terrestrial Networks (NTN) for NR Phase 3, Samsung, 38.213_CR0723

- R1-2506672 Correction on specification support for NR_NTN_Ph3 functionality, Nokia, 38.214_CR0695

RAN2 CRs

- R2-2506592 Introduction of NR NTN phase 3, THALES, 38.300_CR1023

- R2-2506547 Introduction of Rel-19 NR NTN, ZTE Corporation, Sanechips, 38.304_CR0441

- R2-2506560 Introduction of NTN Phase 3 enhancements, InterDigital, 38.321_CR2121

- R2-2506522 Introduction of NTN Phase 3 enhancements, Ericsson, 38.331_CR5481

RAN3 CRs

- R3-256009 Introduce NG Removal procedure, CMCC, 38.410_CR0051

- R3-256010 Support for Regenerative Payload and MBS broadcast in NR NTN, CATT, 38.413_CR1212

RAN4 CRs

- R4-2511399 Big CR to TS 38.101-5: NR_NTN_PH3-Core UE RF, Qualcomm Incorporated, 38.101-5_CR0212

- R4-2509295 Big CR for RRM requirements of NR NTN Phase 3, CATT, 38.133_CR5738