3GPP TSG RAN Meeting #110	RP-253159

# Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.1

Source:	Samsung

Title:	Text Proposals for Requirements on 6G Radio KPIs

Document for:	Discussion and Decision

1	Introduction

In this contribution, we propose various text proposals to help finalize the sections related to requirements on 6G Radio Key Performance Indicators in TR 38.914.

2	Proposal

Proposal 1: TP for section 5.1.1 Peak data rate

In RAN#109 meeting, there was a view that the peak data rate requirement has to be defined while there were concerns on defining unrealistic requirements. Considering that the peak data rate could be derived from the peak spectral efficiency multiplied by the bandwidth, it is proposed to define the peak data rate requirement as follows.

- DL: 24 Gbit/s (60 bit/s/Hz x 400 MHz)

- UL: 12 Gbit/s (30 bit/s/Hz x 400 MHz)

The corresponding text proposal is as follows.

<<<<<< Text Proposal 1: Start >>>>>>

### 5.1.1	Peak data rate

Peak data rate is the theoretical maximum [achievable] data rate under ideal conditions, which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands, then the total peak data rate is

R=i=1Q  Wi × SEpi

where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is 24 Gbit/s.

–	Uplink peak data rate is 12 Gbit/s.

<<<<<< Text Proposal 1: End >>>>>>

Proposal 2: TP for section 5.1.2 Peak spectral efficiency

We propose 8 spatial layers in the downlink and 4 spatial layers in the uplink for consistency with the expected technology advancement and products towards 6G.

<<<<<< Text Proposal 2: Start >>>>>>

### 5.1.2	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is 60 bit/s/Hz.

–	Uplink peak spectral efficiency is 30 bit/s/Hz.

These values were defined assuming an antenna configuration to enable 8 spatial layers (streams) in the downlink and 4 spatial layers (streams) in the uplink.

<<<<<< Text Proposal 2: End >>>>>>

Proposal 3: TP for section 5.1.7 User plane latency

While it has to be studied whether 6GR will support a battery-efficient (e.g. inactive) state, the user plane latency should be the same in both active and battery-efficient state. Hence, we propose the following.

<<<<<< Text Proposal 3: Start >>>>>>

### 5.1.7	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active or, if supported, in a battery efficient (e.g. inactive) state.

The minimum requirements for user plane latency are:

-	4ms for IC

-	1ms for HRLLC

Assuming unloaded conditions for small IP packets, (e.g. 0 byte payload + IP header), for both downlink and uplink.

<<<<<< Text Proposal 3: End >>>>>>

Proposal 4: TP for section 5.1.11 Mobility

We propose to keep the consistency between all test environments in terms of the relative improvement (i.e., 1.5x) with respect to 5G.

<<<<<< Text Proposal 4: Start >>>>>>

Table 5.1.11-2: Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – IC | 2.25 | 10 |
| Dense Urban – IC | 1.68 | 30 |
| Rural – IC | 1.2 | 120 |
|  | 0.675 | 500 |

<<<<<< Text Proposal 4: End >>>>>>

Proposal 5: TP for section 5.1.14 Positioning

We propose to keep 90% confidence level and to specify both horizontal and vertical positioning accuracies for the standalone operation of 6G Radio taking into account the regulatory requirements. The corresponding text proposal is as follows.

<<<<<< Text Proposal 5: Start >>>>>>

### 5.1.14	Positioning

Positioning is the ability to estimate the position of connected devices.

The positioning accuracy is the 90% point of the CDF of the device positioning error for both horizontal and vertical directions.

Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude).

The minimum requirements for the device positioning accuracy for various test environments are summarized in Table 5.1.14-1.

Table 5.1.14-1: Positioning accuracy

| Test Environment | Horizontal Accuracy | Vertical Accuracy |
| --- | --- | --- |
| Indoor Factory - ISAC | 1m | 3m |
| Urban Macro – ISAC | 3m | 3m |

<<<<<< Text Proposal 5: End >>>>>>

Proposal 6: TP for section 5.1.17 Sensing

We propose the sensing-related requirements as follows.

- Detection/False alarm probabilities [%]: [95/5]

- Localization accuracy [m] @ 90% confidence level

  - Indoor Factory-ISAC (H/V): 3/3

  - Urban Macro-ISAC (H/V): 5/5

- Velocity accuracy [m/s] @ 90% confidence level

  - Indoor Factory-ISAC: 3

  - Urban Macro-ISAC: 5

We propose to remove sensing resolution to be in line with the recent decision in ITU-R WP5D.

The corresponding text proposal is as follows.

<<<<<< Text Proposal 6: Start >>>>>>

### 5.1.17	Sensing

Sensing-related capabilities are measured in terms of the following technical performance requirements:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error.

•	Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

•

Table 5.1.17-1: Sensing-related requirements

| Test Environment | Detection Probability | False Alarm Probability | Localization accuracy [m] @ 90% confidence level |  | Velocity accuracy [m/s] @ 90% confidence level |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal | Vertical |  |
| Indoor Factory - ISAC | 95% | 5% | 3 | 3 | 3 |
| Urban Macro – ISAC | 95% | 5% | 5 | 5 | 5 |

<<<<<< Text Proposal 6: End >>>>>>

3	Conclusion

We propose to agree on the text proposals in section 2.