3GPP TSG RAN #110	 	                                                                   RP-253170

Baltimore, USA, Dec 8-11, 2025

Source:	Intel Corporation

Title:	On coverage requirements for 3GPP Internal Study

Agenda item:	8.2.1.1

Document for:	Discussion and Decision

# Introduction

In the SID on Study on 6G Radio [1], the following coverage related objectives were agreed:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage. - … - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

In RAN#109, the following agreements were reached regarding 3GPP internal KPI on coverage [2]:

Agreements For 3GPP internal study, link budget is used as the evaluation methodology for coverage when applicable For 3GPP internal study, the target for coverage is to be determined by RAN. - FFS: Exact coverage target value(s). - FFS: Additional details considering control/data channel

In this contribution, we discuss link budget metrics and coverage target values for defining requirements on coverage for 3GPP internal KPI.

# Discussion

On link budget metrics, the following was identified in RAN1#122bis [3]:

- MCL, MIL, MPL as defined in the link budget template from TR 38.830

- MaxCL as defined in the link budget template from TR 38.913

Note that MaxCL in TR 38.913 and MCL in TR 38.830 are both calculation of maximum coupling loss. The MCL calculation in TR 38.830 additional counted receiver implementation margin and HARQ gain in receiver sensitivity calculation and Tx/Rx diversity gains. The MaxCL in TR 38.913 is MCL defined for a data rate of 160 bps. In the following discussion, we assume the general form of MCL as defined in TR 38.830.

MaxCL in uplink and downlink between device and Base Station site (antenna connector(s)) for a data rate of 160bps, where the data rate is observed at the egress/ingress point of the radio protocol stack in uplink and downlink.

Table 1 summarizes the calculations of MCL, MIL, MPL and their usages.

Table 1 MCL, MIL, MPL calculation and usages

|  | MCL | MIL | MPL |
| --- | --- | --- | --- |
| Calculation | MCLdB=Ptx-receiver sensitivity | MILdB=MCL+Txantenna directional gain+Rxantenna directional gain | MPLdB=MIL - Shadow fading margin + BS selection/macro diversity gain - Penetration margin |
| Usages | For unified coverage target setting independent of Tx/Rx antenna directional gains and channel propagation factors. | For evaluating coverage differences among different physical channels with different antenna gains. | For evaluating cell ISDs for different frequencies and antenna gains. |

Comparing among the metrics, we have the following observations:

- MCL provides a direct comparison with 5G coverage target as defined in TR 38.913. Comparing to MIL and MPL, MCL has less degrees of variations, since it is independent from channel variations and antenna gains.

- For evaluating bottleneck physical channels/signals using link budget analysis, MCL can be as effective as MIL. Since the Tx and Rx antenna directional gains are equally applied to all channels in link budget analysis and wouldn’t affect the relative differences among physical channels/signals.

- Regarding the requirement of reusing the existing 3.5 GHz cell grid, the issue is not about setting the coverage target (since existing 3.5GHz deployment is also in the field) but about identifying gaps in meeting the co-site deployment requirement. MPL can be used in this regard.

Based on the above observations, we believe MCL is the preferred coverage metric for RAN to define coverage requirement for 3GPP internal study. MIL and MPL can additionally be used in the Working Group level for studying physical channels and signals coverage in link-level or system-level simulations and for designs to meet cell ISD targets.

On MCL value, the following was defined for 5G in TR 38.913:

MaxCL in uplink and downlink between device and Base Station site (antenna connector(s)) for a data rate of 160bps, where the data rate is observed at the egress/ingress point of the radio protocol stack in uplink and downlink. The target for coverage should be 164dB. For a basic MBB service characterized by a downlink datarate of 2Mbps and an uplink datarate of 60kbps for stationary users, the target on maximum coupling loss is 140dB. For mobile users a downlink datarate of 384kbps is acceptable. For a basic MBB service characterized by a downlink datarate of 1Mbps and an uplink datarate of 30kbps for stationary users, the target on maximum coupling loss is 143dB. At this coupling loss relevant downlink and uplink control channels should also perform adequately.

For basic MBB service, we expect similar MCL target as in 5G can be applied in 6G with potentially 1 dB to 3 dB improvement, i.e., 144 dB to 146 dB with DL data rate of 1 Mbps and UL data rate of 30 kbps. For extreme coverage, we expect a relaxation on the target, e.g., 154 dB. This is based on following considerations:

- MCL is a function of transmission power, the receiver sensitivity and gNB beamforming gain. Under a given data rate, the improvement on transmission power and receiver sensitivity could come from RF component improvement, receiver sensitivity improvement and gNB beamforming gain improvement. For basic MBB service, 1 to 3 dB MCL improvement over 5G could be a reasonable assumption.

- Extreme low coverage can always be achieved by repetitive transmissions. However, repetition comes at the cost of data rate, e.g., 10x repetition and 10x lower data rate for 10 dB coverage gain. The 164 dB extreme coverage target in 5G was for NB-IoT type use cases. For 6G IoT, since the requirement is to not overlap with NB-IoT and be achieved by scalable design from MBB, a relaxation on the coverage target is necessary, e.g., target for 154 dB MCL.

Observation 1: MCL provides a direct comparison with 5G coverage target as defined in TR 38.913. For evaluating bottleneck physical channels/signals using link budget analysis, MCL can be as effective as MIL.

Proposal 1: Use MCL as the coverage metric in RAN to define coverage requirement for 3GPP internal study.

- MIL and MPL can additionally be used in the Working Group level for studying coverage of physical channels/signals and for designs to meet certain cell ISD targets.

Proposal 2: For basic MBB service, a MCL target similar to 5G can be applied in 6G with potentially 1 dB to 3 dB improvement, e.g., 144 dB to 146 dB with DL data rate of 1 Mbps and UL data rate of 30 kbps.

Proposal 3: For extreme coverage target, consider relaxation from the 5G target, e.g., target for 154 dB MCL.

# Conclusion

In this contribution, we put forward the following proposals on requirements for 6G architecture and migration:

Observation 1: MCL provides a direct comparison with 5G coverage target as defined in TR 38.913. For evaluating bottleneck physical channels/signals using link budget analysis, MCL can be as effective as MIL.

Proposal 1: Use MCL as the coverage metric in RAN to define coverage requirement for 3GPP internal study.

- MIL and MPL can additionally be used in the Working Group level for studying coverage of physical channels/signals and for designs to meet cell ISD targets.

Proposal 2: For basic MBB service, a MCL target similar to 5G can be applied in 6G with potentially 1 dB to 3 dB improvement, e.g., 144 dB to 146 dB with DL data rate of 1 Mbps and UL data rate of 30 kbps.

Proposal 3: For extreme coverage target, consider relaxation from the 5G target, e.g., target for 154 dB MCL.

# References

RP-251881, New SID: Study on 6G Radio

RP-252883, Moderator's summary for RAN led 6G SI: Key performance indicators

R1-2508301, Report of RAN1#122bis meeting

TR 38.914, “Study on 6G Scenarios and requirements”, 3GPP RAN

TR 38.913, “Study on scenarios and requirements for next generation access technologies”, 3GPP RAN