3GPP TSG RAN meeting #110 									RP-253339

Baltimore, USA, December 8-11, 2025

## Status Report to TSG

Agenda item:			9.3.1.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_AIML_air_Ph2 |  |  |  |  |
| Unique ID | 1080085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252445 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: March ‘27 | Performance part: Sept ‘27 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 20% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo (RAN1); Xiaofeng Liu (RAN4); Marco Belleschi (RAN2); Anna Pantelidou (RAN3). |
|  | Company | Qualcomm; CAICT; Ericsson; Nokia |
|  | Email | juanm@qti.qualcomm.com; liuxiaofeng1@caict.ac.cn; marco.belleschi@ericsson.com; anna.pantelidou@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.1.1 	RAN1#122bis

// CSI spatial/frequency compression without temporal aspects (“Case 0”): inference related aspects

Agreement:

Support precoding matrix with port-subband domain representation as target CSI type.

- For a certain layer l, a precoding matrix on N_sb subband and P CSI-RS ports is Winl=[Win,0l, Win,1l,⋯,Win, Nsb-1l] with Win,kl being a P×1 vector.

- Note: Only sub-band PMI reporting is supported.

Agreement:

For determining the payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and ranks, consider following general principles for precoding matrix-based target CSI

- Rank-common and layer-common model with payload-scalable design is considered as the baseline implementation for discussion purpose.

  - Note: layer-specific / rank-specific (if supported) payload parameters are supported by this rank-common and layer-common model with payload-scalable design.

- Limiting the total number of (per-layer) payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and all ranks.

- At least for medium to high per-layer payload regime, the total payload size of rank 2, rank 3 and 4 are comparable to each other.

- Spectral efficiency or system throughput and trade-off with feedback overhead are considered as the performance metric.

Agreement:

When the Target CSI is precoder matrix, in order to down-select to N per-layer payload parameter combinations (PC) of dl,ν,Ql,ν or dl,ν, L,Ql,ν based on performance, overhead and complexity trade-off results, consider the following candidate values:

- Candiate values for dl,ν: 32, 64, 96, 128, 192

- Candidate values for Ql,ν: 1, 2, 3, 4, (8, 10 only for VQ)

  - 2 is considered as the baseline assumption for SQ.

- Candidate values for L: L=2, 4,  dl,ν

- Candidate values for N: 4, 5, 6

- Note: These N payload PCs of dl,ν,Ql,ν or dl,ν, L,Ql,ν are considered as the pool for the payload PC for each layer and each rank.

- Note: The above is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: Rank-common and layer-common model with payload scalable design is considered.

- Note: The above does not mean support of both SQ and VQ.

- Note: The number of parameters combinations configurations (Y) does not exceed 8.

Agreement:

For payload parameter combinations (PC), consider followings

- Support Y configurable PCs (across layers and ranks) as below, where X_n, Xn_31, Xn_32, Xn_33, Xn_41, Xn_42, Xn_43, Xn_44 represent a pair of dl,ν,Ql,ν or a triplet of dl,ν, L,Ql,ν  chosen from the per-layer PC pool.

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| PC1 | X1 | X1, X1 | X1_31, X1_32, X1_33 | X1_41, X1_42, X1_43, X1_44 |
| PC2 | X2 | X2, X2 | X2_31, X2_32, X2_33 | X2_41, X2_42, X2_43, X2_44 |
| … |  |  |  |  |

- For rank 1 and rank 2, layer-common and rank-common payload is supported.

- For rank 3, consider alternatives of {Xn_31, Xn_32, Xn_33} = {a, a, a}, {a, a, b}, {a, b, b} for potential down selection.

- For rank 4, consider alternatives of {Xn_41, Xn_42, Xn_43, Xn_44} = {a, a, a, a}, {a, a, b, b}, {a, b, b, c}, {a, a, b, c} or {a, b, c, c} for potential down selection.

- Note: The table is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: a, b values in rank3 can be different from that in rank 4.

Agreement:

Consider following for evaluation:

- For down-selection of per-layer pairs of dl,ν,Ql,ν or a triplets of dl,ν, L,Ql,ν , average SGCS across per layer can be considered as performance metric.

- For down-selection of the rank-specific and layer-specific patterns (e.g., {Xn_31, Xn_32, Xn_33} and {Xn_41, Xn_42, Xn_43, Xn_44}), spectral efficiency or throughput should be considered.

Agreement:

For CSI feedback via two-sided AIML models, reuse legacy CSI reporting framework

- Support CSI report based on periodic, semi-persistent and aperiodic CSI-RS resources with legacy measurement resource configurations.

- Support aperiodic report and semi-persistent CSI report on PUSCH and reuse legacy triggering mechanism.

// CSI spatial/frequency compression without temporal aspects (“Case 0”): other aspects

Agreement:

For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantization), consider the following definition for evaluation:

- Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall 2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2.

  - Note: Companies to report whether  real value and imaginary value are quantized, or amplitude value and phase value are quantized.

- Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part. Uniform quantization is assumed.

  - Note: Companies to report whether/how the normalization is performed, and whether it is normalized within or across 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts.

- Fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values.

Note: It is up to company to use number of samples S>1 in their evaluations. In this case, companies need to provide information how S samples are quantized in their evaluation.

Agreement:

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection.

- L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits

  - Company to report the quantization step for reference amplitude for 6bits.

- No restriction to the total number of non-zero coefficients for a specific rank.

- Other combinations can be considered and reported by companies.

Agreement:

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer.

- Tx port: 32, 64, or 128 up to companies.

- Subband number: 13, or 19 up to companies.

- Performance metric:

  - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

  - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution

  - Overhead per sample and per layer SGCS results are reported per layer

- Note: Complexity should be analysed.

- For Option 1, consider k1+k2=4, 6 ,7, 8, 16, 32

- Other assumptions follow R18 study EVM

Agreement:

For CPU counting of UE side data collection,

- OCPU=1

- FFS: The CPU occupancy duration

// Inter-vendor training collaboration for two-sided AI/ML models

Conclusion:

For Option 4-1 under Direction A in AI/ML based CSI compression, there is no consensus to further study the necessity of sharing assisted information to align model design aspects.

Agreement:

- For Option 4-1 under Direction A in AI/ML based CSI compression, both average SGCS and average NMSE as performance target in the exchanged dataset are supported. Both average SGCS and average NMSE are provided.

  - NW side can optionally send NMSE and/or SGCS values at X-percentiles.

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, for the association between target CSI and CSI feedback in the exchanged dataset.

- Support 1:M mapping between one target CSI sample/set of samples to M CSI feedback samples/sets of samples associated with different CSI payload size configurations

- FFS: Association between Target CSI and CSI feedback, including scalability related information for different number of Tx port, sub bands related information.

- FFS: Whether/how Target CSI samples/set of samples within an exchanged dataset, with different Tx port and sub-band related information are organized.

- FFS: Whether the Target CSI is per layer or per rank when Target CSI is precoder matrix.

- FFS: Whether and how to support CSI feedback structure with different payload scalability options

- FFS: Other details

#### 2.1.1.1 	RAN1#123

// CSI spatial/frequency compression without temporal aspects (“Case 0”): inference related aspects

Agreement:

Support NW side optionally exchanges codebook to the UE side.

- If NW side exchanges a codebook, the exchanged codebook is associated to the pairing ID.

  - FFS SQ / VQ / both is supported

  - FFS the details, e.g., whether exchange the NW-defined codebook or the  ID of a standardized SQ codebook in case of multiple standardized codebooks.

- If NW side does not exchange a codebook or an ID of a standardized SQ codebook in case of multiple standardized codebooks, a standardized SQ codebook is associated to the pairing ID.

- Note: For the same quantization resolution, a common codebook is used for all the scalars/segment(s), all the layers of pre-coding matrix and all parameter pairs/triplets for the associated pairing ID.

Agreement:

If target CSI type is pre-coder matrix, at least support per-layer payload parameter pairs {32, 2}, {64, 2}, {128, 2}, {192, 2}

- FFS: Whether support per-layer payload parameter pairs {32, 1}, {48, 2}, {96, 1}, {96, 2}, {192, 1}, {128,10/128}

- Note: The above pair refers to the pair of latent dimension and quantization resolution, i.e., {d, Q} for SQ or {d, Q/L} for VQ.

Agreement:

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 128 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength.

| Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- |
| {64, 2} | {64, 2}, {64, 2} | {32, 2}, {32, 2}, {64, 2} | {32, 2}, {32, 2}, {32, 2}, {32, 2} |

Agreement:

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 256 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength.

| Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- |
| {128, 2} | {128, 2}, {128, 2} | {64, 2}, {64, 2}, {128, 2} | {64, 2}, {64, 2}, {64, 2}, {64, 2} |

Agreement:

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 64 bits, select one alternative from following alternatives in RAN1 #124.

- The layers are reported in the order of decreasing strength.

- Note: For the purpose of comparison, consider the following:

  - For comparing Alt.1 and Alt. 2, consider payload scalability.

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| Alt1 | {32, 2} | {32, 2}, {32, 2} | NA | NA |
| Alt2 | {32, 2} | {32, 2}, {32, 2} | {32, 1}, {32, 1}, {32, 2} | {32, 1}, {32, 1}, {32, 1}, {32, 1} |

Agreement:

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 384 bits, select one alternative from following alternatives in RAN1 #124.

- The layers are reported in the order of decreasing strength.

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| Alt1.1 | {192, 2} | {192, 2}, {192, 2} | {96, 2}, {96, 2}, {192, 2} | {96, 2}, {96, 2}, {96, 2}, {96, 2} |
| Alt1.2 | {192, 2} | {192, 2}, {192, 2} | {192, 1}, {192, 1}, {192, 2} | {192, 1}, {192, 1}, {192, 1}, {192, 1} |
| Alt2 | {192, 2} | {192, 2}, {192, 2} | {128, 2}, {128, 2}, {128, 2} | {64, 2}, {64, 2}, {128, 2}, {128, 2} |
| Alt3.1 | {192, 2} | {192, 2}, {192, 2} | {128, 2}, {128, 2}, {128, 2} | {96, 2}, {96, 2}, {96, 2}, {96, 2} |
| Alt3.2 | {192, 2} | {192, 2}, {192, 2} | {128, 2}, {128, 2}, {128, 2} | {192, 1}, {192, 1}, {192, 1}, {192, 1} |

// CSI spatial/frequency compression without temporal aspects (“Case 0”): other aspects

Agreement:

For NW side data collection with higher layer reporting, regarding Target CSI quantization, Option 1 (Scalar quantization) is supported.

Note: Supporting target CSI payload size greater than single RRC message may need RAN2 justification.

Agreement:

For Option 1 (scalar quantization) of NW side data collection with higher layer reporting, down select between Option 1a and Option 1b:

- Option 1a: Each complex element of the Target CSI sample is quantized to real value and imaginary value

  - Further study at least k1=k2=3, 4, 5, 6, and 8 to ensure the performance (e.g., the minimum value that achieves the close performance to upperbound).

  - Further study the quantized value of each codepoint.

- Option 1b: Each complex element of the Target CSI sample is quantized to amplitude value and phase value

  - Further study at least k1+k2=6, 7, 8, 12, and 16 with k1>k2, k1<k2 or k1=k2 to ensure the performance (e.g., the minimum value that achieves the close performance to upperbound).

  - Further study the quantized value of each codepoint.

- Further study the potential UE complexity between Option 1a and Option 1b, e.g., considering the potential conversion to amplitude/phase domain of Option 1b.

- Consider Metric#1 (SGCS for a trained model) for the evaluation.

- The study at least considers 32 ports, 64 ports, and 128 ports; 19 subbands; rank 4.

- It is up to companies to study mixed Target CSIs subject to multiple {k1, k2} values each corresponding to a specific port/subband/layer configuration, for training scalable model.

- Study the necessity of RRC signaling enhancement, e.g., for a large k1, k2 pair if needed.

- Further study whether/how to perform normalization for Option 1a and Option 1b, e.g., the ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1] for real/imag quantization, or [0, 1]/[0, 2π) for amplitude/phase quantization. Select one among the alternatives:

  - Alt.1: It is UE implementation to perform scale up/down to fit the fixed range.

  - Alt.2: Additional specification on the quantization, e.g.,

    - Alt.2-1: At least one element among all real and imag elements in the Target CSI matrix (FFS per layer and/or per subband) correspond to any of Tmin (where Tmin=T1min=T2min) and Tmax (where Tmax=T1max=T2max) in the quantization table or at least one element among all amplitude elements in the Target CSI matrix (FFS per layer and/or per subband) correspond to T1max

    - Alt.2-2: Reporting scaling factor and offset factor used for denormalization.

    - Alt.2-3: Fixed scaling factor used for normalization.

Agreement:

Support providing Pairing ID(s) in CSI-ReportConfig for UE side data collection

- FFS mandatory or optional

- FFS whether to configure Pairing ID for NW side data collection

- FFS whether multiple Pairing IDs provided in one CSI-ReportConfig for UE side data collection

// Inter-vendor training collaboration for two-sided AI/ML models

Agreement:

When the target CSI is precoding matrix with port-subband domain representation, for the association between target CSI and CSI feedback in the exchanged dataset,

    - A pairing ID has K target CSI sets,

      - Each target CSI set has Sk target CSI samples, 1â‰_kâ‰_K, Sk>1.

      - Each target CSI sample within the same target CSI set k has a port dimension of N1x N2x2, and the maximum number of subbands, and a rank Vk

      - Each target CSI sample within the same target CSI set k has Vk layers, ordered from the strongest to the weakest layer, where Vk is the rank

    - Target CSI set k is associated with M Mk CSI feedback sets,

      - Each CSI feedback set corresponds to a specific per layer CSI payload size configuration for a rank and subband configuration

  - FFS: rank (CSI feedback rank can be different compared to target CSI rank)

  - FFS: whether per layer CSI payload size is the same for all the layers of all the samples of the same CSI feedback set.

      - Each CSI feedback sample within the set includes CSI feedback per layer, ordered from the strongest to the weakest layer.

      - The target CSI set and each of the corresponding CSI feedback set has the same number of samples

    - Metadata:

      - Per Target CSI set metadata: for each 1â‰_kâ‰_K  set, number of samples Sk, the port configuration of N1 and N2, subband size and BWP info (NR ARFCN of the lowest PRB of the BWP and BWP size and the SCS), rank Vk, number of associated CSI feedback set  Mk

  - FFS: the number of aggregated CSI-RS resources for total number of antenna ports larger than 32, and the portMappingMethod applicable to aggregated resources

- For each CSI feedback set metadata: CSI payload size configuration, subband configuration

  - FFS: Additional CSI feedback metadata to indicate when specific scalability option is used in model design (e.g. truncation/masking) is used as CSI feedback payload scalability option.

  - FFS: rank

      - Other metadata are not precluded.

      - How the association between the kth target CSI set and Mk CSI feedback sets is indicated is up to SA WG.

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression:

- Discuss whether to introduce linkage between multiple data sets having their own pairing IDs.

- Discuss whether to introduce delivery of additional data set appending new samples on top of previously exchanged data set.

#### 2.1.2	Remaining Open issues

Specification support for CSI feedback enhancement (two-sided model):

Specification support for inter-vendor training collaboration for two-sided AI/ML models focusing on standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) and, subject to RAN4 progress, on specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account.

## 2.2	RAN2

#### 2.2.1	Agreements

Not started.

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

Not started.

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1 RAN4#116bis

Response to RAN checkpoint

Agreement:

Definitions:

Fully specified the model is defined as a specified structure with parameter in [TS].

Reference model means UE/NW is not mandated to implement it.

Test decoder means TE vendor needs to implement it for testing purpose

Direct C in RAN1: fully specified reference encoder and decoder

Option 3 in RAN4: fully specified test decoder and the reference encoder is under discussion (it is agreed to have a fully specified reference encoder)

Direction A 3a-1 in RAN1: specify the reference encoder structure. Presumably, NW will send the parameters to the UE.

Relation between RAN1 and RAN4:

•	The test decoder of option 3 is the same as the reference decoder of direction C

- Test decoder of option 3 will be specified by RAN4

- If RAN1 develops the reference decoder for direction C which is different from RAN4’s, further discussion across RAN4 and RAN1 is needed.

•	The reference encoder is the same for both option 3 and direction C

•	The option 3 reference encoder structure is the same as the encoder structure for direction A option 3a-1.

- Reference encoder of option 3 will be specified in RAN4.

How to respond to the RAN checkpoint

•	Option 3 is feasible for RAN4 to complete. RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model.

- RAN4 agrees that the test decoder of option 3 is the same as the reference decoder of direction C and the reference encoder of option 3 is the same as the reference encoder of direction C. If so, RAN4 believes that direction C should be feasible to complete too.

•	Direction A 3a-1 is feasible for RAN4 to complete from reference encoder perspective.

•	FFS on detailed steps to develop the model

•	RAN4 will send LS on RAN4’s agreement regarding the RAN checkpoint.

Steps to complete the WI objectives:

Agreement

Which SI option to use for the WI

Based on the current WID RP-251870, RAN4 only defines the requirements for the encoder.

RAN4 will discuss option 3 only this WI.

Test decoder and reference encoder backbone

The test decoder and the reference encoder in Option 3 will have the same backbone.

- It is FFS if a single test decoder will be specified.

- The test decoder and the reference encoder can have different structures, e.g. # of layers

Dataset for model evaluation

RAN4 will discuss how to introduce the new mixed dataset for model structure evaluation purpose at the beginning of this WI, by taking into consideration of the scalability and/or various configurations.

- Details are FFS including the channel model(s).

Scalability

Unless the significant issues are identified, RAN4 is going to only discuss the scalability issues and options, which have been agreed in RAN1.

Model backbone

Transformer is assumed as the baseline

Only Quantized parameters are specified

Steps to develop the test decoder and reference encoder:

- Step 1:  Create dataset

  - Step 1-1: Align on configurations for mixed dataset construction, taking factors such as scalability over Tx port number and subband size and payload size into account

  - Step 1-2 Decide how align and on which metrics can be used for evaluating dataset alignment, if any

  - Step 1-3 Companies create datasets

  - Step 1-4 Identify which company datasets to include in the mixed dataset(s)

  - Step 1-5 Create new dataset(s)

- Step 2: Align on the model backbone, model hyperparameters and the scalability solution.

  - Step 2-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution.

    - This can be in parallel with step 1.

    - Transformer is agreed as baseline for backbone

    - Same backbone for test decoder and reference encoder

  - Step 2-2: Preliminary selection of structures, taking into account complexity

  - [Step 2-2a Align on training parameters]

  - Step 2-3: Agree on metrics and steps for evaluation

  - Step 2-4: Check on performance alignment -> see simulation results from contributing companies.

  - Step 2-5: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed.

  - Step 2-6: Select one or more model structures considering complexity and robustness and decide how many models.

  - Step 2-7: Select one or more scalability solutions. This step can be done in parallel with Steps 2-2 to 2-4.

- Step 3: Specify the model for the following work in WID. Where and how to specify may need further discussion.

Note: At this stage, models using the agreed structures, dataset, scalability have already been created by most companies

  - Step 3-1 Decide how to select from the trained model(s) from each company

  - Step 3-2: Freeze model(s)

  - Step 3-3: Agree on how and where to capture the models, the dataset and relevant information

Recommendations for RAN4#117

The following are not agreements, but are recommendations for contributions and discussions at RAN4#117:

- Proposals for assumptions for the dataset including how to cover scalability, channel models, number of samples

- Proposals for how to evaluate the dataset including potential metrics to compare different dataset contributions

- Proposals on model hyperparameters, considering in particular complexity, considering also quantization size

- Proposals and reasoning for scalability solutions

- Proposals and reasoning for training parameters

- Proposals for performance metrics for evaluating the different potential model structures

- Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

##### 2.4.1.2 RAN4#117

Topic 1: Scalability

Scalability dimensions:

- Agreement:

  - The dimensions for scalability are agreed as follows:

    - Sub-band size/number of sub-bands

    - Number of TX ports

    - CSI payload size

Sub-band size:

There was no agreement on sub-band size. The following options were proposed but not agreed.

- Option 1: 6, 12 sub-bands

- Option 2: 13 (10MHz), 26 (20 MHz) sub-bands

- Option 3: 4 (5MHz), 8 (10 MHz) and 16 (20 MHz) sub-bands

(Further options not precluded for future meetings)

Number of TX ports:

- Agreement:

  - It was agreed that 32 and 16 ports are considered for the final test decoder and reference encoder (and direction C model), associated with RAN4 performance requirements

    - It is FFS whether to additionally consider 8 ports

    - It is FFS whether to additionally consider 64 and 128 ports

TX ports configuration:

- Agreement:

  - For the port configurations, consider typical configurations and also whether all configurations need to be captured in the test decoder.

    - BS vendors requested to provide input on typical configurations

    - It is for further discussion whether

      - there is a performance test per TX port number and configuration, or

      - there is not a performance test for every TX port number and configuration, but some TX port number and configurations that are not tested are included in the model training for direction C scalability

The following TX port configurations were proposed. It is for further discussion whether to take any of these options, or other options:

  - For 32 ports:

    - N1, N2

    - Option 1: Consider  [4, 4, 2] and [8, 2, 2] configurations

    - Option 2: [8, 2, 2] only

    - Option 3: …

  - For 16 ports

    - Option 1: Consider [4, 2, 2] and [8, 1, 2] configurations

    - Option 2: [8 1 2] only

    - Option 3: …

  - For 8 ports (if considered)

    - Option 1: Consider [2, 2, 2] configuration

    - Option 2: …

Payload size:

- Agreement:

  - For the purposes of studying scalability, the payload size will be assumed to be 128 bits. This may be revised for the final test decoder.

The following possible payload sizes were proposed for the final model; other options not precluded: 64, 128, 256 bits. Potentially the RAN4 decision may follow RAN1 decisions.

Token and feature dimension:

- Agreement:

  - Assuming <=32 ports supported, sub-band as token dimension and TX port as feature dimension

Method for scalability:

- Agreement:

  - For TX ports:

    - Option 1: use embedding layer for each feature set.

    - Option 2: use zero padding

  - For sub-bands

    - Option 1: use positional embedding specific to each token index.

    - Option 2: use zero padding

For CSI payload, there was no agreement, but the following options were proposed:

  - Option 1: Truncation

  - Option 2: Adaptation layer

  - Option 3: Vary quantization parameters

  - Option 4: Wait for RAN1 decision on Direction A sub-option 4-1

Other options are not precluded at this stage.

How to evaluate scalability methods:

- Agreement:

  - Evaluate the impact of scaling by comparing Case 1 (scalable model structure with joint training on a combined dataset on scalable scenarios) and Case2 (dedicated model structure trained on dedicated data)

Topic 2: Model design

Model structure:

- Agreement:

  - RAN1 transformer structure is taken as the baseline for simulation purpose and scalability analysis.

    - The scalability discussion and conclusion can be made based on RAN1 transformer structure.

    - Other model structure can be considered based on the analysis from the perspective of both performance and complexity.

Model hyperparameters to be discussed:

- Agreement:

  - The following hyperparameters should be agreed:

    - Number of transformer blocks N_TF

    - Dimension of transformer block d_model

      - N_head*d_head

    - Number of self-attention heads N_head

    - Dimension of attention head d_head

    - Dimension of latent space inside feedforward module d_FF

      - Multiple of d_model

    - Activation function choice

Quantization of model parameters and payload:

- Agreement:

  - Evaluations that decide on which structure to use should take into account quantization of model parameters and payload

    - Baseline for study of scalability and model structure study: Quantization aware training

  - Scalar quantization assumed for the payload as the assumption for the study on model structure and scalability

    - FFS quantization codebook

  - 8 bit quantization as default assumption for the parameters for the study on structure and scalability

    - 16 bit quantization if too much degradation seen with 8 bit

Complexity considerations:

- Agreement:

  - Show complexity for proposed model parameters, including FLOPS, number of parameters

  - Rule out encoder structures with > any of [8 million flops or 500,000 parameters]

  - Discuss complexity level for decoder

Number of decoders and encoders:

- Agreement:

  - Strive for one test decoder

  - Strive for one reference encoder per test decoder

Further assumptions for the study of model structure and scalability:

- Agreement:

  - Use the at least existing Rel-19 mixed dataset for evaluating model structure performance (in order that work may proceed)

    - Further analysis of structures with additional datasets to check generalization not precluded

  - Wider dataset needed for evaluating scalability; this should be generated and described by individual companies

  - For comparing model structures, consider:

    - SGCS (mean, percentile) per layer

  - Training assumptions for studying scalability and model selection are as follows:

    - Ratio of samples of total samples for training, validation, and testing: 80% / 10% / 10%

    - Optimizer: Adam

    - Learning rate: 0.0001

    - Batch size: 256

    - Max number of epochs: 150

    - Stopping criteria: No validation loss improvement over 25 epochs

    - Loss function: 1 - SGCS

    - Drop out: No drop out

  - Training method for final training can be further discussed

  - For study of scalability and model structure, do not consider N-1 training

    - FFS whether to consider N-1 training for the final test decoder training

The following was not agreed, but should be considered further as a method for selecting the final structure:

  - Downselect to a small number of model structures/parameter sets with good performance / complexity tradeoff, and then further downselect between these based on robustness.

    - Robustness means test decoder works well across different encoder structures

Topic 3: Creation of a dataset for the final model (test decoder and reference encoder)

Dataset contents:

- Agreement:

  - Dataset consists of Eigenvectors

  - At least Rank 2 considered

    - FFS for rank 3 and rank 4

  - Common procedure to select Eigenvectors:

    - Select the right eigenvectors corresponding to the top-maximum- layer-number eigen values

    - Perform column normalization by normalizing the L2 norm of each eigenvector in to 1

    - Perform phase normalization by subtracting the angle of the first element from the angles of all the elements in each eigenvector

How to generate the dataset (LLS, SLS etc.)

There was a discussion on whether to include SLS (Uma, Umi etc.) and/or LLS (TDL or CDL) in the dataset, but no conclusion.

- Agreement:

  - In RAN4#118, the principle on how to define the database for training purpose will be discussed. Interested companies are encouraged to propose their principles with justification. This includes, but not limited to,

    - How to enable RAN4 requirement

    - If and how to ensure the requirements related to the real deployment

    - What’s needed for direction-C model.

How to check dataset alignment

It may be necessary to check alignment between datasets of different companies. There was no agreement on this topic, but the following general options were presented:

  - Option 1: Do not align, just combine all companies data with the same simulation assumptions

  - Option 2: Use either Rel-19 models or make new models to compare database similarity using model performance

  - Option 3: Use a metric such as PSE, FC, SP, Samsung’s dissimilarity or NTU method

  - Other options are not precluded.

Topic 4: Design of the performance requirement

Performance requirement definition:

There was a discussion on how to define the performance requirement, but no conclusion. The following options were discussed:

- Option 1: Throughput ratio of follow PMI vs random PMI

- Option 2: Throughput ratio of follow PMI (AI) vs follow PMI (eTypeII codebook)

- Option 3: Additional metric on overhead reduction (ratio of overhead with AI vs overhead with eTypeII)

- Option 4: Absolute throughput

- Other options not precluded

Channel model for the performance requirement:

Agreement:

- TDL is used for performance requirement.

Topic 5: Lifecycle management

Agreement:

- For now, all of the following could be in scope:

  - Activation

  - De-activation

  - Switching

  - Fallback to non-AI

  - Performance monitoring

    - Reporting delay

    - Reporting accuracy

- More progress needed from RAN1/2

- Discuss whether it is useful and how to evaluate testability of RAN1 performance monitoring options.

  - Wait for RAN1 progress; contribution driven

Summary of issues to consider for RAN4#118

The following is not intended as an agreement, but is a summary of topics for which contributions are invited for RAN4#118 in order to make progress:

- Scalability options:

  - Whether to consider 8, 64, 128 ports

  - TX port configurations

  - Sub-band configurations

  - Eventually, CSI payload sizes for the final model

- Method to achieve scalability

- Evaluation of the model structure variations with swapped normalization and MLA/MLP and/or additional layers

- Proposals for model hyperparameters

- Further discussion of quantization for the final model

- Confirmation or proposals for encoder complexity limit

- Proposals for decoder complexity limit

- Whether to consider robustness when selecting the final hyperparameters set

- Whether to consider rank 3 and 4 in the dataset

- How to generate the dataset

- Dataset alignment (whether to do, which metrics)

- Performance requirement design

- Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

- LCM requirements in case there is sufficient RAN1/2 progress

#### 2.4.2	Remaining Open issues

- Scalability options:

  - Whether to consider 8, 64, 128 ports

  - TX port configurations

  - Sub-band configurations

  - Eventually, CSI payload sizes for the final model

- Method to achieve scalability

- Evaluation of the model structure variations with swapped normalization and MLA/MLP and/or additional layers

- Proposals for model hyperparameters

- Further discussion of quantization for the final model

- Confirmation or proposals for encoder complexity limit

- Proposals for decoder complexity limit

- Whether to consider robustness when selecting the final hyperparameters set

- Whether to consider rank 3 and 4 in the dataset

- How to generate the dataset

- Dataset alignment (whether to do, which metrics)

- Performance requirement design

- Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

- LCM requirements in case there is sufficient RAN1/2 progress

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

##### 4.1.1 RAN1#122bis

// CSI spatial/frequency compression without temporal aspects (“Case 0”): inference related aspects

R1-2506743	Discussion on inference related aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

R1-2506774	Discussion on inference related aspects of CSI compression	ZTE Corporation, Sanechips

R1-2506781	Discussion on interference related aspects for CSI compression	TCL

R1-2506792	AI/ML CSI Spatial/Frequency Compression: Inference Aspects	InterDigital, Inc.

R1-2506803	Discussion on AIML for CSI compression inference related aspects	Spreadtrum, UNISOC, BUPT

R1-2506832	Inference related aspects of AI/ML for CSI compression	Ericsson

R1-2506887	Discussion on inference related aspects for CSI compression	vivo

R1-2506931	Discussion on Inference related aspects for CSI compression	Huawei, HiSilicon

R1-2506978	Discussion on inference related aspects of two-sided AI/ML model based CSI feedback	Xiaomi

R1-2507006	Discussion on inference related aspects of CSI compression	CMCC

R1-2507108	Discussion on inference related aspects of AI/ML-based CSI compression	CATT

R1-2507165	Inference related aspects for AI/ML CSI compression	OPPO

R1-2507183	Discussion on inference related aspects for CSI compression	KT Corp.

R1-2507205	Inference related aspects for CSI compression	HONOR

R1-2507242	Views on inference related aspects of CSI compression	Samsung

R1-2507280	Discussion on inference related aspects in CSI compression with AI/ML	Fujitsu

R1-2507301	Discussion on inference aspects of CSI compression	NEC

R1-2507389	CSI Compression: Inference Related Aspects	Nokia

R1-2507396	Discussion on inference related aspects for CSI compression	LG Electronics

R1-2507415	Discussion on inference aspects for AI/ML-based CSI compression	Panasonic

R1-2507443	Inference related aspects for CSI compression	Lenovo

R1-2507462	Discussion on Inference related aspects for AI CSI compression	Ofinno

R1-2507496	Discussion on inference related aspects of CSI compression	ETRI

R1-2507517	Inference for AI/ML based CSI Compression	Google

R1-2507552	Discussion on Inference Related Aspects for CSI Compression	Sharp

R1-2507616	CSI spatial/frequency compression without temporal aspects (“Case 0”)- Inference related aspects			MediaTek Inc.

R1-2507630	Discussion on inference related aspects of CSI compression	Transsion Holdings

R1-2507663	On inference related aspects for AI based CSI spatial/frequency domain compression	Apple

R1-2507710	Specification of inference aspects of AIML CSI compression	Qualcomm Incorporated

R1-2507733	Inference Related Aspects of AI/ML for CSI Compression	TOYOTA Info Technology Center

R1-2507802	Discussion on the inference-related aspects of AI/ML CSI compression	NTT DOCOMO, INC.

R1-2507897	Discussion on AI/ML for CSI compression inference related aspects	CEWiT

R1-2508021	FL summary 1 on AIML inference related aspects		Moderator (Qualcomm)

R1-2508022	FL summary 2 on AIML inference related aspects		Moderator (Qualcomm)

R1-2508023	FL summary 3 on AIML inference related aspects		Moderator (Qualcomm)

// CSI spatial/frequency compression without temporal aspects (“Case 0”): other aspects

R1-2506744	Discussion on other aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

R1-2506775	Discussion on other aspects of CSI compression	ZTE Corporation, Sanechips

R1-2506782	Discussion on other aspects for CSI compression	TCL

R1-2506793	AI/ML CSI Spatial/Frequency Compression: Other Aspects	InterDigital, Inc.

R1-2506804	Discussion on AIML for CSI compression other aspects	Spreadtrum, UNISOC

R1-2506833	Other aspects of AI/ML for CSI compression	Ericsson

R1-2506888	Discussion on other aspects for CSI compression	vivo

R1-2506932	Discussion on other aspects for CSI compression	Huawei, HiSilicon

R1-2506979	Discussion on other aspects of CSI spatial/frequency compression	Xiaomi

R1-2507007	Discussion on other aspects of CSI compression	CMCC

R1-2507109	Discussion on other aspects of AI/ML-based CSI compression	CATT

R1-2507166	Other aspects for AI/ML CSI compression	OPPO

R1-2507184	Discussion on other aspects of CSI compression	KT Corp.

R1-2507206	Other aspects for CSI compression	HONOR

R1-2507243	Views on other aspects of CSI compression	Samsung

R1-2507281	Discussion on other aspects of CSI compression	Fujitsu

R1-2507303	Discussion on other aspects of CSI compression	NEC

R1-2507390	CSI Compression: Other Aspects	Nokia

R1-2507397	Discussion on other aspects for CSI compression	LG Electronics

R1-2507416	Discussion on other aspects for AI/ML-based CSI compression	Panasonic

R1-2507444	Other aspects for CSI compression	Lenovo

R1-2507478	Other aspects	Tejas Network Limited

R1-2507497	Discussion on other aspects of CSI compression	ETRI

R1-2507518	Other Aspects for AI/ML based CSI Compression	Google

R1-2507587	Discussion on other aspects for CSI compression	Sony

R1-2507617	CSI spatial/frequency compression without temporal aspects (“Case 0”)- Other aspects	MediaTek Inc.

R1-2507664	On other aspects for AI based CSI spatial/frequency domain compression	Apple

R1-2507711	Specification of other aspects of AIML CSI compression	Qualcomm Incorporated

R1-2507775	Discussions on other aspects for NR air interface enhancements	Sharp

R1-2507778	Discussion on performance monitoring for CSI compression	Shanghai Jiao Tong University, Toyota

R1-2507803	Discussion on other aspects of AI/ML CSI compression	NTT DOCOMO, INC.

R1-2507868	Discussion on AIML based CSI compression	ASUSTeK

R1-2508028	FL summary#1 for other aspects of CSI compression	Moderator (Huawei)

R1-2508030	FL summary#3 for other aspects of CSI compression	Moderator (Huawei)

R1-2508031	FL summary#4 for other aspects of CSI compression	Moderator (Huawei)

// Inter-vendor training collaboration for two-sided AI/ML models

R1-2506745	Discussion on inter-vendor training collaboration for two-sided AI/ML models	FUTUREWEI

R1-2506776	Discussion on inter-vendor training collaboration for two-sided AI/ML models	ZTE Corporation, Sanechips

R1-2506783	Discussion on inter-vendor collaboration for CSI compression	TCL

R1-2506794	AI/ML CSI Spatial/Frequency Compression: Inter-vendor Collaboration	InterDigital, Inc.

R1-2506805	Discussion on Inter-vendor training collaboration for two-sided AI/ML models	Spreadtrum, UNISOC

R1-2506834	Inter-vendor training collaboration for two-sided AI/ML models	Ericsson

R1-2506889	Discussion on inter-vendor training collaboration for two-sided AI/ML models	vivo

R1-2506933	Discussion on Inter-vendor training collaboration for two-sided AI/ML models	Huawei, HiSilicon

R1-2506980	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Xiaomi

R1-2507008	Discussion on inter-vendor training collaboration for CSI compression	CMCC

R1-2507110	Inter-vendor training collaboration for CSI compression	CATT

R1-2507167	Inter-vendor training collaboration for AI/ML CSI compression	OPPO

R1-2507244	Views on inter-vendor training collaboration for two-sided AI/ML models	Samsung

R1-2507282	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Fujitsu

R1-2507324	Discussion on Inter-vendor training collaboration for two-sided AI/ML models	China Telecom

R1-2507391	Inter-vendor training collaboration for two-sided AI/ML models	Nokia

R1-2507398	Discussion on Inter-vendor training collaboration for two-sided models	LG Electronics

R1-2507414	Discussion on Inter-vendor Training Collaboration for two-sided AI/ML models	NEC

R1-2507417	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Panasonic

R1-2507487	Inter-vendor training collaboration for two-sided AI/ML models	Lenovo

R1-2507498	Discussion on inter vendor training collaboration for two-sided AI/ML models	ETRI

R1-2507519	Inter-Vendor Collaboration for AI/ML based CSI Compression	Google

R1-2507553	Discussions on Inter-vendor training collaboration	Sharp

R1-2507588	Inter-vendor training collaboration for two-sided AI/ML models	Sony

R1-2507618	Inter-vendor training collaboration for two-sided AI/ML models	MediaTek Inc.

R1-2507665	Discussion on inter-vendor training collaboration for two sided AI/ML models	Apple

R1-2507666	FL summary # 1 for inter-vendor training collaboration	Moderator (Apple)

R1-2507667	FL summary # 2 for inter-vendor training collaboration	Moderator (Apple)

R1-2507668	FL summary # 3 for inter-vendor training collaboration	Moderator (Apple)

R1-2507712	Inter-vendor training collaboration for two-sided CSI compression use case	Qualcomm Incorporated

R1-2507732	Discussion on inter-vendor training collaboration for two-sided models	Indian Institute of Tech (M)

R1-2507764	Discussion on inter-vendor training collaboration	Continental Automotive

R1-2507804   Discussion on the inter-vendor training collaborations for two-sided AI/ML models	NTT DOCOMO, INC.

R1-2507828   Views on inter-vendor training collaboration for two-sided AI/ML models	BUPT, ZGC Institute of Ubiquitous-X Innovation and Application

R1-2507666	FL summary # 1 for inter-vendor training collaboration	Moderator (Apple)

R1-2507668	FL summary # 3 for inter-vendor training collaboration	Moderator (Apple)

##### 4.1.2 RAN1#123

// CSI spatial/frequency compression without temporal aspects (“Case 0)

R1-2509419   Inference Aspects Of AI/ML Assisted CS Compression in Spatial/Frequency Domain Without Temporal Aspects	TOYOTA Info Technology Center

// CSI spatial/frequency compression without temporal aspects (“Case 0”): inference related aspects

R1-2508325  Discussion on inference related aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

R1-2508344	AI/ML CSI Spatial/Frequency Compression: Inference Aspects	InterDigital, Inc.

R1-2508362	Inference related aspects of AI/ML for CSI compression	Ericsson

R1-2508374	Discussion on AIML for CSI compression inference related aspects	Spreadtrum, UNISOC

R1-2508418	Discussion on inference related aspects for CSI compression	vivo

R1-2508444	Discussion on inference related aspects of CSI compression	CMCC

R1-2508494	Discussion on Inference related aspects for CSI compression	Huawei, HiSilicon

R1-2508536	Discussion on interference related aspects for CSI compression	TCL

R1-2508561	Discussion on inference aspects of CSI compression	NEC

R1-2508583	Inference related aspects for AI/ML CSI compression	CATT

R1-2508601	Discussion on inference related aspects of AI/ML for CSI compression	China Telecom

R1-2508670	Discussion on inference related aspects of two-sided AI/ML model based CSI feedback	Xiaomi

R1-2508693	Discussion on inference related aspects of CSI compression	ZTE Corporation, Sanechips

R1-2508713	Inference related aspects for AI/ML CSI compression	OPPO

R1-2508749	Discussion on inference related aspects for CSI compression	LG Electronics

R1-2508762	Discussion on inference related aspects for CSI compression	KT Corp.

R1-2508788	Views on inference related aspects of CSI compression	Samsung

R1-2508876	Discussions on Inference Related Aspects for CSI Compression	Sharp

R1-2508884	Discussion on inference aspects for AI/ML-based CSI compression	Panasonic

R1-2509459	Discussion on inference aspects for AI/ML-based CSI compression	Panasonic

(Revision of R1-2508884)

R1-2508925	Discussion on inference related aspects in CSI compression with AI/ML	Fujitsu

R1-2508942	Inference for AI/ML based CSI Compression	Google

R1-2508949	CSI Compression: Inference Related Aspects	Nokia

R1-2508954	Inference related aspects for CSI compression	Lenovo

R1-2508962	Discussion on inference related aspects of CSI compression	ETRI

R1-2508985	Inference related aspects for CSI compression	HONOR

R1-2509009	Discussion on inference aspects of CSI compression	KAIST

R1-2509035	Discussion on Inference related aspects for AI CSI compression	Ofinno

R1-2509092	On inference related aspects for AI based CSI spatial/frequency domain compression	Apple

R1-2509150	Inference aspects of AI/ML-based CSI compression	MediaTek Inc.

R1-2509193	Discussion on inference related aspects in AI/ML based CSI compression	Quectel

R1-2509213	Specification of inference aspects of AIML CSI compression	Qualcomm Incorporated

R1-2509249	Discussion on inference related aspects of CSI compression	Transsion Holdings

R1-2509266	Discussion on the inference-related aspects of AI/ML CSI compression	NTT DOCOMO, INC.

R1-2509317	Discussion on Inference related aspects of Case-0 CSI Compression	Rakuten Mobile, Inc

R1-2509214   FL summary 1 on inference related aspects of CSI feedback via two-sided model	 	Moderator (Qualcomm Incorporated)

R1-2509215   FL summary 2 on inference related aspects of CSI feedback via two-sided model		Moderator (Qualcomm Incorporated)

R1-2509216   FL summary 3 on inference related aspects of CSI feedback via two-sided model 	Moderator (Qualcomm Incorporated)

R1-2509477   FL summary 4 on inference related aspects of CSI feedback via two-sided model		Moderator (Qualcomm Incorporated)

// CSI spatial/frequency compression without temporal aspects (“Case 0”): other aspects

R1-2508326  Discussion on other aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

R1-2508345	AI/ML CSI Spatial/Frequency Compression: Other Aspects	InterDigital, Inc.

R1-2508375	Discussion on AIML for CSI compression other aspects	Spreadtrum, UNISOC

R1-2508396	Other aspects of AIML for CSI compression	Ericsson

R1-2508419	Discussion on other aspects for CSI compression	vivo

R1-2508445	Discussion on other aspects of CSI compression	CMCC

R1-2508495	Discussion on other aspects for CSI compression	Huawei, HiSilicon

R1-2508537	Discussion on other aspects for CSI compression	TCL

R1-2508547	Discussion on other aspects of CSI compression	NEC

R1-2508584	Other aspects for AI/ML CSI compression	CATT

R1-2508671	Discussion on other aspects of CSI spatial/frequency compression	Xiaomi

R1-2508694	Discussion on other aspects of CSI compression	ZTE Corporation, Sanechips

R1-2508714	Other aspects for AI/ML CSI compression	OPPO

R1-2508750	Discussion on other aspects for CSI compression	LG Electronics

R1-2508763	Discussion on other aspects of CSI compression	KT Corp.

R1-2508789	Views on other aspects of CSI compression	Samsung

R1-2508828	Other Aspects	Tejas Network Limited

R1-2508861	Views on the lifecycle management for AI/ML-based CSI Compression	Southeast University

R1-2508885	Discussion on other aspects for AI/ML-based CSI compression	Panasonic

R1-2508926	Discussion on other aspects of CSI compression	Fujitsu

R1-2508943	Other Aspects for AI/ML based CSI Compression	Google

R1-2508950	CSI Compression: Other Aspects	Nokia

R1-2508955	Other aspects for CSI compression	Lenovo

R1-2508963	Discussion on other aspects of CSI compression	ETRI

R1-2508986	Other aspects for CSI compression	HONOR

R1-2509036	Discussion on other aspects for AI CSI compression	Ofinno

R1-2509051	Views on the lifecycle management for AI/ML-based CSI Compression	Southeast University

R1-2509066	Discussion on performance monitoring for CSI compression	Sony

R1-2509093	On other aspects for AI based CSI spatial/frequency domain compression	Apple

R1-2509151	Other aspects of AI/ML-based CSI compression	MediaTek Inc.

R1-2509183	Discussions on other aspects for NR air interface enhancements	Sharp

R1-2509218	Specification of other aspects of AIML CSI compression	Qualcomm Incorporated

R1-2509267	Discussion on other aspects of AI/ML CSI compression	NTT DOCOMO, INC.

R1-2509319	Discussion on other aspects of AIML-based CSI compression	Rakuten Mobile, Inc

R1-2509367	Discussion on AIML based CSI compression	ASUSTeK

R1-2509418	Other Aspects of AI/ML Assisted CSI Compression	TOYOTA Info Technology Center

R1-2509505	FL summary#1 for other aspects of CSI compression	Moderator (Huawei)

R1-2509506	FL summary#2 for other aspects of CSI compression	Moderator (Huawei)

R1-2509507	FL summary#3 for other aspects of CSI compression	Moderator (Huawei)

R1-2509508	FL summary#4 for other aspects of CSI compression	Moderator (Huawei)

R1-2509509	FL summary#5 for other aspects of CSI compression	Moderator (Huawei)

// Inter-vendor training collaboration for two-sided AI/ML models

R1-2508327	Discussion on inter-vendor training collaboration for two-sided AI/ML models	FUTUREWEI

R1-2508346	AI/ML CSI Spatial/Frequency Compression: Inter-vendor Collaboration	InterDigital, Inc.

R1-2508376	Discussion on Inter-vendor training collaboration for two-sided AI/ML models	Spreadtrum, UNISOC

R1-2508397	Inter-vendor training collaboration for two-sided AIML models	Ericsson

R1-2508420	Discussion on inter-vendor training collaboration for two-sided AI/ML models	vivo

R1-2508446	Discussion on inter-vendor training collaboration for CSI compression	CMCC

R1-2508496	Discussion on Inter-vendor training collaboration for two-sided AI/ML models	Huawei, HiSilicon

R1-2508538	Discussion on inter-vendor collaboration for CSI compression	TCL

R1-2508585	Views on inter-vendor training collaboration for CSI compression	CATT

R1-2508602	Discussion on inter-vendor training collaboration for two-sided AI/ML models	China Telecom

R1-2508630	Discussion on Inter-vendor Training Collaboration for two-sided AI/ML models	NEC

R1-2508672	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Xiaomi

R1-2508695   Discussion on inter-vendor training collaboration for two-sided AI/ML models	ZTE Corporation, Sanechips

R1-2508715	Inter-vendor training collaboration for AI/ML CSI compression	OPPO

R1-2508751	Discussion on Inter-vendor training collaboration for two-sided models	LG Electronics

R1-2508764	Discussion on inter-vendor training collaboration aspects for two-sided AI/ML models 	KT Corp.

R1-2508790	Views on inter-vendor training collaboration for two-sided AI/ML models	Samsung

R1-2508886	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Panasonic

R1-2508927	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Fujitsu

R1-2508944	Inter-Vendor Collaboration for AI/ML based CSI Compression	Google

R1-2508951	Inter-vendor training collaboration for two-sided AI/ML models	Nokia

R1-2508964	Discussion on inter vendor training collaboration for two-sided AI/ML models	ETRI

R1-2508979	Discussions on Inter-vendor training collaboration	Sharp

R1-2508987	Inter-vendor training collaboration for two-sided CSI compression	HONOR

R1-2509003	Inter-vendor training collaboration for two-sided AI/ML models	Lenovo

R1-2509037	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Ofinno

R1-2509067	Discussion on Inter-Vendor Training Collaboration for Two-Side AI/ML Models	Sony

R1-2509094	On Inter-vendor training collaboration for two sided AI/ML models	Apple

R1-2509152	Inter-vendor training collaboration for two-sided AI/ML models	MediaTek Inc.

R1-2509219   Inter-vendor training collaboration for two-sided CSI compression use case	Qualcomm Incorporated

R1-2509268   Discussion on the inter-vendor training collaborations for two-sided AI/ML models	NTT DOCOMO, INC.

R1-2509374	Discussion on inter-vendor training collaboration for two-sided models	Indian Institute of Tech (M)

R1-2509401   Inter-Vendor Training Collaboration for AI/ML Assisted CSI Compression in Spatial/Frequency Domain Without Temporal Aspects	TOYOTA Info Technology Center

R1-2509095	FL summary # 1 for inter-vendor training collaboration	Moderator (Apple)

R1-2509096	FL summary # 2 for inter-vendor training collaboration	Moderator (Apple)

R1-2509097	FL summary # 3 for inter-vendor training collaboration	Moderator (Apple)

R1-2509098	FL summary # 4 for inter-vendor training collaboration	Moderator (Apple)

## 4.2	RAN2

## 4.3	RAN3

## 4.4	RAN4

##### 4.4.1 RAN4#116bis

// Moderator summary and conclusions

R4-2514530	Topic Summary for [116bis][123] R20 NR_AIML_air_Ph2

Type: other		For: Information
					Source: Moderator

Decision:		Noted.

Minutes and agreements in the online session and ad hoc

Please refer to the hyperlink below for the detailed minutes of online session:

2. R4-2514530 [116bis][123] NR_AIML_air_Ph2 topic summary v00.docx

Minutes and agreements in the online session and ad hoc

Please refer to the hyperlink below for the detailed minutes of online session:

4. R4-2514530 [116bis][123] NR_AIML_air_Ph2 topic summary v00.docx

R4-2514614	WF on R20 NR_AIML_air_Ph2

Type: other		For: Approval
					Source: Ericsson

Decision:		Approved.

R4-2514633	AH minutes on R20 NR_AIML_air_Ph2

Type: other		For: information
					Source:

Decision:		Noted.

// General aspects and workplan

R4-2513425	Discussion on general aspects and workplan for AI-CSI compression

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2513541	On LCM and Performance Monitoring Requirement Scope

Type: discussion		For: Approval
					Source: NTU

Decision:		Noted.

R4-2513588	Discussion on General Aspects of AI/ML for NR Air Interface Phase 2

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Noted.

R4-2513654	Discussion on general aspects for enhancement on AI/ML for NR air interface

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2513784	On general aspects for Rel-20 AI/ML for NR air interface

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2513948	Discussion on General Aspects for Two-sided AI/ML Models

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2514253	Workplan for Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

Type: other		For: Information
					Source: Ericsson

Abstract:

Workplan for the RAN4 related work

Decision:		Noted.

// Inter-vendor training collaboration and interoperability for two-sided AI/ML models

R4-2513224	Discussion on inter-vendor training collaboration for two-sided AIML models

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2513417	Discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Noted.

R4-2513426	Discussion on inter-vendor training collaboration and interoperability for two-sided AIML models

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		Noted.

R4-2513499	Discussion on inter-vendor training collaboration and interoperability for two-sided models

Type: discussion		For: Discussion
					38.133 v..
					Source: LG Electronics Inc.

Decision: 		Noted.

R4-2513538	Discussion on Inter-vendor training collaboration and interoperability for two-sided AIML models

Type: discussion		For: (not specified)
					Source: CAICT

Decision: 		Noted.

R4-2513540	Reference Model Derivation and Selection for Inter-Vendor Training Collaboration and Interoperability for Two-Sided AI/ML Models

Type: discussion		For: Approval
					Source: NTU

Decision: 		Noted.

R4-2513589	Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Noted.

R4-2513655	Discussion on Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Noted.

R4-2513764	Discussion on training and interoperability for two-sided AIML models

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		Noted.

R4-2513785	On inter-vendor training collaboration and interoperability for two-sided models

Type: other		For: Approval
					Source: Samsung

Decision: 		Noted.

R4-2513808	Inter-vendor training collaboration and interoperability for two-sided AIML models

Type: discussion		For: Approval
					Source: OPPO

Decision: 		Noted.

R4-2513949	Discussion on Interoperability for Two-sided AI/ML Models

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Noted.

R4-2513975	Discussion on the 2-sided CSI

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses inter-vendor training collaboration and interoperability for 2-sided CSI compression model.

Decision: 		Noted.

R4-2514072	CSI compression on Low-Complexity Encoders under SGCS-4

Type: discussion		For: (not specified)
					Source: Korea Testing Laboratory

Decision: 		Noted.

R4-2514149	Discussion on inter-vendor training collaboration and interoperability for two-sided models

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		Noted.

R4-2514462	Inter-vendor Training Collaboration and Interoperability of CSF Compression

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Noted.

// RRM core requirement

R4-2513225	Discussion on the RRM core requirements of AIML air ph2

Type: discussion		For: Discussion
					Source: CATT

Decision: 		Noted.

R4-2513418	Discussion on RRM core requirement for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Noted.

R4-2513427	Discussion on RRM core requirement for AI-CSI compression

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		Noted.

R4-2513590	RRR Core Requirement

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Noted.

R4-2513656	Discussion on requirements for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Noted.

R4-2513795	Discussion on RRM impacts on Rel-20 AI/ML

Type: discussion		For: Discussion
					38.133 v..
					Source: Samsung

Decision: 		Noted.

R4-2513950	Discussion on Core Requirements for Two-sided AI/ML Models

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Noted.

R4-2514103	Discussion on RRM requirements on two-sided models

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		Noted.

R4-2514458	Core Requirements of AI-ML based CSF Compression

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Noted.

##### 4.4.2 RAN4#117

// Moderator summary and conclusions

R4-2522288	Topic Summary for [117][123] R20 NR_AIML_air_Ph2

Type: other		For: Information
					Source: Moderator

Decision:		Noted.

Minutes and agreements in the online session and ad hoc

Please refer to the hyperlink below for the detailed minutes of online session:

5.R4-2522288 Topic Summary for [117][123] R20 NR_AIML_air_Ph2_v03.docx

R4-2522432	WF on [117][123] R20 NR_AIML_air_Ph2

Type: other		For: Approval
					Source: Ericsson

Decision:		Approved.

R4-2522433	AH minutes on [117][123] R20 NR_AIML_air_Ph2

Type: other		For: Approval
					Source: Ericsson

Decision:		Noted.

R4-2522434	LS on RAN check point of R20 NR_AIML_air_Ph2

Type: other		For: Approval
					To: RAN, cc: RAN1

Source: vivo, Ericsson

Decision:		Approved.

// General aspects and workplan

R4-2520375	Discussion on general aspects for Rel-20 RAN1-led AI/ML WI

Type: other		For: Approval
					Source: Samsung

Decision: 		Note.

R4-2520449	Discussion on general aspects for enhancement on AI/ML for NR air interface

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Note.

R4-2520618	Discussion on General Aspects of AI/ML for NR Air Interface Phase 2

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Note.

R4-2521240	General issues on CSI compression

Type: discussion		For: Approval
					Source: OPPO

Decision: 		Note.

R4-2521386	Discussion on general aspects and workplan for AI-CSI compression

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

R4-2521528	Discussion on General Aspects for Two-sided AI/ML Models

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Note.

//Inter-vendor training collaboration and interoperability for two-sided AI/ML models

R4-2520148	Discussion on inter-vendor training collaboration for two-sided AIML models

Type: discussion		For: Discussion
					Source: CATT

Decision: 		Note.

R4-2520228	Discussion on inter-vendor training collaboration and interoperability for two-sided models

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		Note.

R4-2520233	Discussion on training and interoperability for two-sided AIML models

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		Note.

R4-2520311	Discussion on Inter-vendor training collaboration and interoperability for two-sided AIML models

Type: discussion		For: (not specified)
					Source: CAICT

Decision: 		Note.

R4-2520376	Discussion on Inter-vendor training collaboration and interoperability

Type: other		For: Approval
					Source: Samsung

Decision: 		Note.

R4-2520426	On Dataset Alignment and Reference Model Selection Criterion for Inter-Vendor Training Collaboration and Interoperability for Two-Sided AI/ML Models

Type: discussion		For: Approval
					Source: NTU

Decision: 		Note.

R4-2520450	Discussion on Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Note.

R4-2520619	Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Note.

R4-2521021	Discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Note.

R4-2521193	Discussion on inter-vendor training collaboration and interoperability for two-sided models

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		Note.

R4-2521234	Discussion on the 2-sided CSI

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses inter-vendor training collaboration and interoperability for 2-sided CSI compression model.

Decision: 		Note.

R4-2521241	Inter-vendor training collaboration and interoperability for two-sided AIML models

Type: discussion		For: Approval
					Source: OPPO

Decision: 		Note.

R4-2521387	Discussion on inter-vendor training collaboration and interoperability for two-sided AIML models

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

R4-2521529	Discussion on Test Decoder Specification

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Note.

R4-2522155	Inter-vendor Training Collaboration and Interoperability of CSF Compression

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Note.

// RRM core requirement

R4-2520149	Discussion on the RRM core requirements of AIML air ph2

Type: discussion		For: Discussion
					Source: CATT

Decision: 		Note.

R4-2520451	Discussion on requirements for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Note.

R4-2520620	RRR Core Requirement for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Note.

R4-2521022	Discussion on RRM core requirement for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Note.

R4-2521235	RRM requirements for two-sided AI/ML models

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the RRM requiremetns two-sided AI/ML models.

Decision: 		Note.

R4-2521388	Discussion on RRM core requirement for AI-CSI compression

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

R4-2521530	Discussion on Requirements for Two-sided AI/ML Models

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Note.

R4-2521538	Discussion on RRM requirements on two-sided models

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		Note.

R4-2522029	Discussion on RRM impacts on Rel-20 AI/ML

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		Note.

R4-2522151	Core Requirements of AI-ML based CSF Compression

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Note.

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template