3GPP TSG RAN #109			RP-252888

Beijing, China, Sept. 15th – 18th, 2025

Title:	pCR on Deployment scenarios for TR 38.914 (after 2. Offline)

Source:		Deutsche Telekom (Moderator)

Agenda Item: 	8.2.1.1

# Introduction

A total of 30 contributions has been submitted to the agenda item 8.2.1.1 on Deployment Scenarios for the RAN lead SI on 6G. Three documents were misplaced and have been moved to the appropriate agenda item.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4.	Deployment scenarios

Editor note: Sensing related aspects will be included in section 4

High-level descriptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD/ Orbit, BS / UE antenna elements, UE distribution / speed and service profile are proposed in this TR. It is assumed that more detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. should be defined in the new RAT study item.

The deployment scenarios in this section are defined for the sole purpose of evaluating RAN performance requirements or features, not intended to mandate actual deployments or network/device implementation.

Note: Regarding the various carrier frequency configurations and options listed in the tables below, only a sub-set of them will be required to be evaluated for ITU IMT-2030 self-evaluation and 3GPP internal evaluation, others may be optionally used for specific RAN features or requirements evaluation.

## 4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 2 GHz: Up to 200 MHz (DL+UL) NOTE3 . Around 4 GHz: Up to 300 MHz (DL+UL) NOTE3 Around 7 GHz Up to 400 MHz (DL+UL) NOTE3 Around 15 GHz Up to 400 MHz (DL+UL) NOTE3 Around 30 GHz: up to 1 GHz (DL+UL) NOTE3 |
| Layout | Single layer: - Indoor floor (Open office) , 120m x 50m |
| ISD | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | TBD |
| User distribution and UE speed | 100% Indoor, 3km/h, [10] users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| NOTE 1: TBD NOTE 2: TBD NOTE 3: TBD NOTE 4: TBD |  |

## 4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 4 GHz + Around 7 GHz Around 7 GHz + Around 4 GHz + Around 2 GHz + Around 700 MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | TBD |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10] users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| NOTE 1: TBD NOTE 2: TBD |  |

## 4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7 GHz (ISD 1) Around 700 MHz + Around 7 GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | TBD |
| User distribution and UE speed | [15% outdoor vehicles (120km/h), 20% outdoor (3 km/h) and 70% indoor (3 km/h) 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI |

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz Around 7GHz + Around 4 GHz + Around 2 GHz + Around 700MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 2 GHz + Around 30GHz Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to [400] MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | TBD |
| User distribution and UE speed | TBD [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.5	Sub-Urban macro

The suburban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 700 MHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 7 GHz + Around 30 GHz Around 2 GHz + Around 700 MHz Around 7GHz + Around 4 GHz + Around 2 GHz + Around 700 MHz |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer - Hex. Grid |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | TBD |
| User distribution and UE speed | [10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h] [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |