3GPP TSG-RAN Meeting #110	RP-253756

Baltimore, USA, 8 - 11 December 2025

Agenda item:	8.2.1.3.3

Source:	RAN3 Chair (Nokia)

Title:	pCR to TR 38.914 v0.2.0 with RAN #110 results for other operational requirements

Document for:	Approval

# 1	Introduction

A total of 21 papers is included in this agenda item (including 2 late submissions). Most of the proposals address topics with existing placeholders under section 5.3 of the TR (i.e., RAN sharing, ease of operation and self-organization, network resilience, and service awareness). However, several new topics are also presented.

The table below maps topics (column 1) to papers (column 2) that contain at least one proposal related to the topic.

| Topic | TDoc | Source |
| --- | --- | --- |
| RAN sharing | RP-253083 | Deutsche Telekom, Vodafone, Orange, Telecom Italia Turkcell, Spark NZ, KT Corp., Odido, BT, SK Telecom, Bouygues Telecom, Telefonica, Telia Company, KPN, Rakuten Mobile, CK Hutchison, Telenor, KDDI, Boost Mobile Network, Jio Platforms |
|  | RP-253101 | THALES, Sateliot, Viasat, ST Engineering iDirect, Novamint, Fraunhofer IIS, Fraunhofer HHI, EchoStar, Gatehouse Satcom, Airbus, SES, ESA, Hispasat |
|  | RP-253103 | Boost Mobile Network |
|  | RP-253164 | Samsung |
|  | RP-253241 | NEC |
|  | RP-253268 | CATT, China Unicom, China Telecom |
| Ease of operation and self-organization | RP-253078 | Deutsche Telekom, Vodafone, Orange, Telecom Italia, Turkcell, Spark NZ, KT Corp, Odido, BT, SK Telecom, Bouygues Telecom, Telefonica, Telia Company, KPN, Rakuten Mobile, CK Hutchison, Telstra, Telenor, KDDI, KT Corp., LG Uplus, Jio Platforms |
|  | RP-253081 | Deutsche Telekom, Vodafone, Orange, Telecom Italia, Turkcell, Spark NZ, Odido, BT, Bouygues Telecom, Telefonica, Telia Company, KPN, Rakuten Mobile, CK Hutchison, Telstra, Telenor, KDDI, SK Telecom, Jio Platforms |
|  | RP-253101 | THALES, Sateliot, Viasat, ST Engineering iDirect, Novamint, Fraunhofer IIS, Fraunhofer HHI, EchoStar, Gatehouse Satcom, Airbus, SES, ESA, Hispasat |
|  | RP-253127 | CMCC |
|  | RP-253164 | Samsung |
|  | RP-253268 | CATT, China Unicom, China Telecom |
|  | RP-253337 | Ericsson, NTT DOCOMO, CHTTL, China Unicom, T-Mobile USA, Deutsche Telekom, Jio Platforms, Verizon |
| Network resilience | RP-253088 | Ericsson, Airbus, BDBOS, Deutsche Telekom, DISA, Erillisverkot, FirstNet, LGUplus, Motorola Solutions, Nkom, Nokia, Softil, SyncTechno Inc, Telstra, Thales, The Netherlands Police, T-Mobile USA, Verizon |
|  | RP-253101 | THALES, Sateliot, Viasat, ST Engineering iDirect, Novamint, Fraunhofer IIS, Fraunhofer HHI, EchoStar, Gatehouse Satcom, Airbus, SES, ESA, Hispasat |
|  | RP-253164 | Samsung |
|  | RP-253241 | NEC |
|  | RP-253322 | DISA, FirstNet, Erillisverkot, ST Engineering iDirect, DeepSig Inc, DHS Science & Technology, Tiami Networks, Lockheed Martin |
|  | RP-253827 | Sharp, Thales, [Novamint], Eutelsat, [Tejas Networks] [Fraunhofer HHI], [Fraunhofer IIS], [Amazon], Gatehouse Satcom, [ST Engineering iDirect], [Sony], Skylo, Airbus |
| Service Awareness | RP-253803 | Nokia, T-Mobile USA, Deutsche Telekom, AT&T, BT, Verizon, SK Telecom, China Unicom, CHTTL, NVIDIA, LG Uplus |
| Other | RP-253126 | CMCC, China Unicom, ZTE |
|  | RP-253206 | ViaSat Satellite Holdings Ltd, Novamint, Airbus, Sateliot, Gatehouse Satcom, ESA, EchoStar, Thales, Iridium, Skylo, Terrestar |
|  | RP-253215 | KT Corp., Thales, Iridium, Viasat |
|  | RP-253228 | Qualcomm Incorporated |
|  | RP-253319 | Apple |
|  | RP-253323 | MediaTek Finland |
|  | RP-253385 | KDDI Corporation (TTC), Verizon |

# 2	Discussion

## 2.1	RAN sharing

TR 38.914 section 5.2 currently includes the following requirement related to RAN sharing:

-	“The 6G RAN architecture shall support sharing of the RAN between multiple operators.”

Also, TR 38.760-3 section 5.2.2 contains additional description and requirements for shared RAN deployments:

-	“6GR should support shared RAN deployments, including support of multiple participating operators. A shared RAN should be able to efficiently interoperate with a non-shared RAN. Mobility and service continuity between the non-shared RAN and the shared RAN shall be supported.”

| TDoc | Proposals and moderator’s comments |
| --- | --- |
| RP-253083 | 1. 6G RAT shall support Network Sharing (i.e. MOCN, MORAN and their enhancements) 2. 6G RAN architecture shall support network sharing of the RAN between multiple operators Comments: This seems already captured in section 5.2 of the TR (although absent specific references to MOCN or MORAN, see RP-253164). |
| RP-253101 | 1. The 6G Radio access network shall be able to support the sharing between several network operators (mobile and satellite) of a multi orbit space segment. Comments: A combination of existing requirements in section 5.2 (i.e., “support sharing of the RAN between multiple operators” + “be designed considering both TN and NTN”) may be seen as already addressing this proposal. |
| RP-253103 | 1. 6G RAN sharing shall support diverse participating Core network operators, enabling them to deliver 6G services across the shared RAN infrastructure. Comments: Similar proposal in RP-253164. 2. 6G RAN shall allow multiple RAN sharing mechanisms, e.g. MOCN, Network slicing, Cell based RAN sharing, and/or other enhanced mechanisms. Comments: To ensure the requirement remains well-bounded, it may be better to avoid using “e.g.” or else specify a closed list of mechanisms. 3. 6G RAN sharing shall support both UE-associated services and Non UE-associated services for participating operators. 4. 6G RAN sharing shall support UE-related and/or Non UE-related data collection of participating operators for new services (e.g. AI and Sensing). 5. 6G RAN sharing shall support standardized QoS mechanisms for both participating operators and the host MNO. Comments: Proposals 3-5 seem to extend into service/feature-level aspects, which may be too detailed in the context of RAN-sharing principles. |
| RP-253164 | 1. The 6G RAN design shall support shared RAN deployments, supporting multiple hosted Core Operators. The Shared RAN may operate either on shared spectrum or on the spectrum of each hosted Operator. Comments: This is a generic shared RAN requirement that appears to implicitly cover MOCN/MORAN (multiple hosted cores, shared or separate spectrum). This could be added to 5.3.3. |
| RP-253241 | 1. The shared RAN deployment refers to subclause 5.2.2 in TR38.760-3. Comments: The reference to the RAN sharing section in the RAN3 TR seems useful, to avoid duplication. The reference could be added to 5.3.3. |
| RP-253268 | 1. The RAN design for 6G shall support RAN sharing to enable the dynamic and intelligent utilization of pooled resources and reduce the capital and operational expenditures of network deployment. Comments: The requirement seems already covered, but the proposal includes a description for RAN sharing that could be added to 5.3.3, perhaps replacing “dynamic and intelligent” (which seems to go beyond what is required from RAN sharing) with “efficient”. |

Moderator’s proposed way forward:

Add the following to section 5.3.3 (wording may be further refined)

a. 	Description: “RAN sharing enables the efficient utilization of pooled resources and reduces the capital and operational expenditures of network deployment.” (based on RP-253268)

b. 	Requirement: “The 6G RAN design shall support shared RAN deployments, supporting multiple hosted Core Network operators. The Shared RAN may operate either on shared spectrum or on the spectrum of each hosted operator.” (based on RP-253164)

c. 	Reference: “Additional description and requirements for RAN sharing are provided in TR 38.760-3.” (based on RP-253241)

Based on discussion during the Tuesday offline session, the following is proposed:

Proposal 1: Capture the following text in section 5.3.3 of TR 38.914:

### 5.3.3 RAN sharing 6G RAT shall support Network Sharing (i.e., MOCN, MORAN, and Indirect Network Sharing). ### Additional description and requirements for RAN sharing are provided in TR 38.760-3 [x].

## 2.2	Ease of operation and Self Organization

TR 38.914 does not currently include any requirements related to this topic, while TR 38.760-3 section 5.1 contains the following general principle and FFS:

-	“The 6G RAN architecture shall support self-configuration and self-optimization. FFS on which features should be recommended to be standardized from day1.”

| TDoc | Proposals and moderator’s comments |
| --- | --- |
| RP-253078 | 1. 6G RAT & RAN architecture shall make use of AI where justified in terms of gains and energy efficiency versus classic non-AI based solutions Comments: The requirement does not seem specific to self-organization, but rather a general requirement for AI. |
| RP-253081 | 1. 6G RAT shall support automatic configuration and optimisation to the maximum extent possible 2. 6G RAN architecture (Including RAN-CN Interface) shall support automatic configuration and optimisation to the maximum extent possible Comments: This seems to reflect a basic requirement and is consistent with TR 38.760-3. Based on the online discussion, the bullets should be simplified and combined. |
| RP-253101 | - 6GR shall enable a migration path for 5G NTN (both NR-NTN and NB-IoT-NTN) prioritizing operator cost and ease. In particular, it shall be possible to deploy 6GR with in orbit satellites designed for 5G NTN. Comments: The scope of this requirement seems much broader than self-organization (e.g., 5G to 6G migration). |
| RP-253127 | Self-organizing function shall be designed to fulfill the following requirements: 1. RAN shall support the deployment of RAN SON functions in a hybrid manner (distributed and centralized). 2. Collaboration and coordination among RAN SON functions need to be addressed. 3. User/application level QoS and QoE monitoring capability by UEs and network elements shall be supported. 6G Day 1 supports ANR and MDT functions for network operation and self-organization. Comments: The first two requirements are supported by multiple companies and appear appropriate for inclusion in 5.3.4. The third requirement seems more closely related to service/application awareness. Regarding 6G Day 1 functions, ANR and MDT seem reasonable if Day 1 features are within the scope of this agenda item. |
| RP-253164 | The 6G RAN design shall be designed to fulfil the following requirements: 1. 6G RAN shall support the deployment of 6G RAN SON functions in a hybrid manner (distributed and centralized). 2. Collaboration and coordination among 6G RAN SON functions need to be addressed. 3. User / application level QoS and QoE monitoring capability by UEs and network elements shall be supported. Comments: These proposals are covered by RP-253127. |
| RP-253268 | The RAN design for 6G shall fulfill the following requirements: 1. RAN shall support self-configuration to enable the automatic plug-and-play deployment of network nodes, establishing their basic operational parameters without manual intervention. 2. RAN shall support SON/MDT features to provide the foundational, rule-based automation and data-driven visibility for robust and efficient network operation. 3. RAN shall support AI/ML-assisted optimization to empower 6G networks to autonomously predict issues, optimize for complex and conflicting goals, and continuously self-adapt in real-time. Comments: The requirements seem already covered, but the proposals include description of self-organization that could be added to 5.3.3. |
| RP-253337 | The 6G Radio and 6G RAN architecture shall be designed to fulfil the following requirements: 1. RAN shall support the deployment of RAN SON functions, including inter-vendor interoperability, in a hybrid manner (distributed and centralized). 2. Collaboration and coordination among RAN SON functions should be supported, when needed. 3. RAN SON functions design should be agnostic to the type of algorithm (including AI based and non-AI based) used to realise the needed functionality. 4. The data/customer privacy and integrity are important and should be ensured. Comments: The first two proposals are already covered by RP-253127. The 3rd proposal seems to reflect a long-standing principle in 5G for SON and AI/ML for NG-RAN, so appears appropriate for inclusion in 5.3.4. The 4th proposal seems to have wider scope than ease of operation and self-organization and is under discussion in another agenda item. |

Moderator’s proposed way forward:

Add the following to section 5.3.4 (wording may be further refined)

a. 	Description: “Self-organization facilitates the automatic plug-and-play deployment of network nodes, including the establishment of basic operational parameters without manual intervention. AI/ML capabilities can further enhance this by enabling 6G networks to predict issues, optimize for complex and sometimes conflicting objectives, and continuously self-adapt in real time.” (based on RP-253268)

b. 	Requirement: “6GR and 6G RAN architecture shall support automatic configuration and optimisation.” (based on RP-253081)

c. 	Requirement: “The 6G RAN architecture shall support the deployment of RAN SON functions in a hybrid manner (distributed and centralized). Collaboration and coordination among RAN SON functions should be supported, when needed.” (based on RP-253127, RP-253164, RP-253337)

d. 	Requirement: “RAN SON functions design should be agnostic to the type of algorithm (including AI based and non-AI based) used to realise the needed functionality.” (based on RP-253337)

e.	6G Day 1: “ANR and MDT are recommended to be standardized from Day 1.” (based on RP-253127)

Based on discussion during the Tuesday offline session, the following is proposed:

Proposal 2: Capture the following text in section 5.3.4 of TR 38.914:

### 5.3.4 Ease of operation and Self Organization Self-organization facilitates the automatic deployment of network nodes, including the establishment of basic operational parameters without manual intervention. 6GR and 6G RAN architecture shall support automatic configuration and optimisation of the RAN, including ANR and collection of MDT data.

Note: yellow highlighted text was added during post-offline email review.

## 2.3	Network Resilience

TR 38.914 section 5.2 currently includes the following requirement related to network resilience:

-	“The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.”

Also, TR 38.760-3 section 4.1 contains the following definition:

-	Resilience: refers to enabling the RAN to recover from failures and to ensure accessibility and service continuity.

| TDoc | Proposal(s) |
| --- | --- |
| RP-253088 | 1. The 6G Radio and 6G RAN architecture shall be designed to facilitate implementations minimizing occurrence and duration of interruption of the user plane and control plane caused by environmental factors (e.g., radio link issues, electromagnetic interference, etc.) as well as issues affecting the UE and the network infrastructure (e.g., power outages, failures in hardware or software, etc.). Comments: This requirement appears clear and suitable for inclusion in section 5.3.5. However, “issues affecting the UE” seems too vague (since it may be interpreted to include issues in the UE itself). 2. The 6G Radio and the 6G RAN architecture shall facilitate implementations suitable for varying levels of redundancy in the network deployment, avoiding in general specific assumptions on availability of redundancy options (e.g., redundant serving cells, etc.). Comments: It is not entirely clear what this requirement is attempting to say (suitable for varying levels of redundancy while generally avoiding assumptions on availability of redundancy?). The next requirement is also related to redundancy, so perhaps they can be merged and refined. 3. The 6G Radio procedures and the 6G RAN architecture (including TN and NTN deployments) shall be robust (e.g., to avoid failures) and shall allow for utilizing redundancy (e.g., overlapping radio coverage, duplicated hardware or software, etc.), whenever available, to recover from failures (e.g., link interruptions, radio unit failures, baseband failures, hardware/software failures, etc.) with a predefined behaviour (e.g., based on configuration) and a configurable recovery timeline (e.g., limit the service interruption time by handing over users to well-functioning cells, by re-establishing the connection, etc.) Comments: A requirement on redundancy seems suitable for inclusion in section 5.3.5. However, the text could be simplified, as it includes many examples and definitions. Also, the aspects of “predefined behaviour” and “configurable recovery timelines” don’t seem inherently tied to redundancy. 4. The 6G Radio procedures and the RAN architecture shall be designed to facilitate RAN maintenance with minimal service interruption times, including tools for recovering the connectivity. Comments: This requirement appears clear and suitable for inclusion in section 5.3.5. 5. The 6G Radio shall support anticipation and early detection of issues affecting user plane and control plane. Comments: This requirement seems to lack a clear linkage to resiliency, e.g., “issues” seems rather vague. Also, it may be covered already by proposal 7. 6. The 6G Radio and the 6G RAN shall know which users and services (e.g., QoS flows) have requirements regarding resilience. The 6G Radio and 6G RAN architecture shall be designed to ensure that features relevant for resilience can be configured and used for specific users and services (e.g., specific QoS flows) based on resiliency requirements. Comments: This requirement appears clear and suitable for inclusion in section 5.3.5. 7. The 6G Radio shall support collection of information, including UE reports, that allow for managing the network to address issues and provide the desired level of resiliency (e.g., information about coverage, failures, etc.), including automation. Comments: Clear, though it could alternatively fit under SON / self-organization. Would this also cover requirement 5, possible replacing “issues” with “(potential) issues” (where “potential” implies “anticipation and early detection”)? |
| RP-253101 | 1. Radio interface level a. Given that GNSS services may be unavailable or degraded, the operation over NTN access for communication service or for Positioning, Navigation and Timing (PNT) service should be possible independently from any service not under the control of the satellite network operator, e.g., external GNSS service. b. The 6G Radio design shall be able to support a GNSS-independent physical layer operation for NTN, to provide resilience in case of service degradation or unavailability of third-party systems, such as Global Navigation Satellite Systems (GNSS). 2. Radio access architecture level a. The support of mobility and simultaneous multi connectivity between NTN and TN components or between two NTN components (e.g. GSO and NGSO) is beneficial for ubiquitous coverage, service continuity and/or resiliency purposes. Multi-connectivity can also contribute to ensure a quasi-seamless continuity (no added data latency during handovers), robust mobility (connection failures minimized) and increased throughput. Note: Given that the NTN may feature a different bandwidth compared to the one offered by TN, only part of the traffic (most critical one) may be duplicated via the NTN connectivity. 3. Network level a. The 6G RAN shall enable NTN to act as a backup component for the terrestrial network, with capacity that can be prioritized for the most critical services. Voice services and low data rate connection suitable for messaging shall be supported. b. The 6G RAN shall enable GSO to act as a backup and/or capacity augmentation layer for a NGSO network component in a multi orbit NTN infrastructure. Comments: Some items appear more related to NTN design or NTN–TN interworking; further evaluation is needed to identify any resiliency-specific deltas that should be captured in 5.3.5. |
| RP-253164 | 1. The 6G RAN shall be designed to support enhanced ability to maintain services and recover quickly when facing various failure, such as equipment failures, overload. Comments: This appears largely aligned with the proposals captured from RP-253088. |
| RP-253241 | 1. Standardization shall support enhanced recovery mechanisms to ensure service continuity and accessibility. This includes the RAN network nodes and CN network nodes. Comments: This appears largely aligned with the proposals captured from RP-253088. |
| RP-253322 | 1. Network Resilience: A computing infrastructure that provides continuous business operation (i.e., highly resistant to disruption and able to operate in a degraded mode if damaged), rapid recovery if failure does occur, and the ability to scale to meet rapid or unpredictable demands [aa]. 2. The 6G Radio and 6G RAN architecture should be designed with resilient capabilities including high service availability, high reliability and high mission assurance. 3. The 6G Radio and 6G RAN architecture should support detection of RF interference. 4. The 6G Radio and 6G RAN architecture should support classification of RF interference where RF interference includes 3GPP interference and non-3GPP interference. 3GPP interference includes inter-cell interference within one Mobile Network Operator (MNO) and inter-cell cross boarder interference between two or more MNOs. Non-3GPP interference includes unintentional interference and intentional interference. Non-3GPP unintentional interference includes internal interference and external interference (e.g., tropospheric ducting). 5. The 6G Radio and 6G RAN architecture should support mitigation of RF interference including non-3GPP intentional interference. 6. The 6G Radio and 6G RAN architecture should support notification of RF interference for inter-cell cross boarder 3GPP interference between two or more MNOs and non-3GPP intentional interference via air interface directly to the subscribers for government public safety or law enforcements personnel on site impacted by RF interference as well as via wired means to public safety or law enforcements personnel offsite in control rooms by coordinating with 6G core. Comments: This is a late document, which may be further discussed if time allows. The delta to other papers is a definition of network resilience, and requirements for interference mitigation/coordination which may be seen as a separate topic. |
| RP-253827 | 1. The 6G Radio and 6G RAN architecture shall be designed to ensure GNSS-less/resilient operation for NTN to minimize service interruption/unavailability in case of degradation or unavailability of Global Navigation Satellite Systems (GNSS). |

Moderator’s proposed way forward:

Add the following to section 5.3.5 (wording may be further refined)

a. 	Requirement: “The 6G Radio and 6G RAN architecture shall be designed to facilitate implementations minimizing occurrence and duration of interruption of the user plane and control plane caused by environmental factors (e.g., radio link issues, electromagnetic interference, etc.) as well as network issues affecting the UE (e.g., power outages, failures in hardware or software, etc.).” (based on RP-253088)

b. 	Requirement: “The 6G Radio procedures and the 6G RAN architecture (including TN and NTN deployments) shall be robust (e.g., to avoid failures) and shall allow for utilizing redundancy (e.g., overlapping radio coverage, duplicated hardware or software, etc.), whenever available, to recover from failures (e.g., link interruptions, radio unit failures, baseband failures, hardware/software failures, etc.).” (based on RP-253088)

c. 	Requirement: “The 6G Radio procedures and the RAN architecture shall be designed to facilitate RAN maintenance with minimal service interruption times, including tools for recovering the connectivity.” (based on RP-253088)

d. 	Requirement: “The 6G Radio and the 6G RAN shall know which users and services (e.g., QoS flows) have requirements regarding resilience. The 6G Radio and 6G RAN architecture shall be designed to ensure that features relevant for resilience can be configured and used for specific users and services (e.g., specific QoS flows) based on resiliency requirements.” (based on RP-253088)

Discuss whether the following requirement falls under “Network Resilience” or “Ease of operation and Self Organization”.

Requirement: “The 6G Radio shall support collection of information, including UE reports, that allow for managing the network to address (potential) issues and provide the desired level of resiliency (e.g., information about coverage, failures, etc.), including automation.” (based on RP-253088)

Based on discussion during the Tuesday offline session, the following is proposed:

Proposal 3: Capture the following text in section 5.3.5 of TR 38.914:

### 5.3.5 Network Resilience ### The 6G RAN shall be designed to ensure accessibility and service continuity with minimum interruption time in any failure condition.

RP-253827 was submitted late to AI 8.2.1.3.3 and could not be treated due to time constraints. The topic was discussed under AI 8.2.2 (RP-253443), with guidance to continue discussion under “Other operation requirements”.

Proposal 4: Discuss whether the following text can be captured in section 5.3.5 of TR 38.914:

### The 6G Radio and 6G RAN architecture shall be designed to ensure GNSS-less/resilient operation for NTN to minimize service interruption/unavailability in case of degradation or unavailability of GNSS.

## 2.4	Service Awareness

TR 38.914 section 5.2 currently includes the following requirement related to service awareness:

-	“The 6G RAN architecture shall support enhanced service awareness in RAN.”.

Also, TR 38.760-3 section 4.1 contains the following definition:

-	Service awareness: refers to enabling the RAN to acquire and use relevant information to appropriately handle a service.

| TDoc | Proposal(s) |
| --- | --- |
| RP-253803 | 1. 6G radio access network (6G RAN) shall have means to address evolving and new services like immersive communication (XR) and AI effectively and become more adaptive to the application traffic. 6G RAN will need to obtain more visibility to application-level attributes (e.g., performance requirements, status, etc.) to effectively allocate radio resources and utilize the different RAN resources to meet the applications minimum requirements while maintaining optimal overall system efficiency and efficient utilization of RAN resources. It shall also be possible to provide related information from the 6G RAN to the 6G UE. Comments: The paragraph seems suitable as introductory text with some simplification and refinement (e.g., removal of shall statements, redundancies, etc). 2. 6G RAN shall be able to understand and be capable of adapting to traffic demands of applications while maintaining overall system efficiency in terms of radio resource utilisation and energy consumption. Comments: This requirement appears clear and suitable for inclusion in section 5.3.6. 3. 6G UE shall provide assistance information related to traffic and application-level attributes to the 6G RAN. Comments: This requirement appears clear, but perhaps rephrased as “shall be able to” (similar to the wording of requirement 2)? 4. 6G RAN shall provide configuration to the 6G UE based on network load, scheduling efficiency and application characteristics. Comments: This requirement appears clear, but perhaps rephrased as “shall be able to” (similar to the wording of requirement 2)? |

Moderator’s proposed way forward:

Add the following to section 5.3.5 (wording may be further refined)

a. 	Description: “To address evolving and new services such as immersive communication (XR) and AI, 6G RAN is expected to become more adaptive to application traffic. Allowing the 6G RAN to obtain greater visibility into application-level attributes (e.g., performance requirements, status, etc.) can enable it to allocate radio resources more effectively to meet applications’ minimum requirements while maintaining optimal overall system efficiency.” (based on RP-253803)

b. 	Requirement: “6G RAN shall be able to understand and be capable of adapting to traffic demands of applications while maintaining overall system efficiency in terms of radio resource utilisation and energy consumption.” (based on RP-253803)

c. 	Requirement: “6G UE shall be able to provide assistance information related to traffic and application-level attributes to the 6G RAN.” (based on RP-253803)

d. 	Requirement: “6G RAN shall be able to provide configuration to the 6G UE based on network load, scheduling efficiency and application characteristics.” (based on RP-253803)

Based on discussion during the Tuesday offline session, the following is proposed:

Proposal 5: Capture the following text in section 5.3.6 of TR 38.914:

### 5.3.6 Service awareness in RAN ### 6G RAN shall be able to understand and be capable of adapting to traffic demands of applications while maintaining overall system efficiency in terms of radio resource utilisation and energy consumption.

## 2.5	Other

There are several other papers which may or may not be in scope of this agenda item.

| TDoc | Proposal(s) |
| --- | --- |
| RP-253126 | [Relay] To support the resilience and extended connectivity, the design of the 6G RAN and Radio Interface Technology shall aim at supporting wireless relay-kind functions, e.g. Relay, (Network-controlled) Repeaters, reconfigurable intelligent surfaces. Comments: Appears to be outside the scope of this agenda item (e.g., deployment scenarios?) |
| RP-253206 | [Multi-RAT co-existence] 1. The 6G RAN shall support configuration and co-existence of in-band and guard band NB-IoT NTN channel(s) in a 6GR NTN channel. 2. The 6G RAN shall support co-existences of in-band NB-IoT NTN in one orbit (e.g., GSO) within a 6GR NTN channel deployed via another orbit (e.g., NGSO). 3. The 6G RAN shall support frequency shift and associated capability signalling to support co-existence between NB-IoT and 6GR. 4. To facilitate co-existence, NB-IoT channel raster values shall be aligned with 6GR channel raster to allow flexibility in placement of an NB-IoT channel within a 6GR channel while still maintaining orthogonality across the sub-carriers of NB-IoT and 6GR channels. Comments: Appears to be outside the scope of this agenda item. NTN is also being discussed under AI 8.2.1.4. |
| RP-253215 | [Change of preferred/priority radio access technology] Subject to operator’s policy, the 6G network shall be able to configure the UE to access the network via NTN as a preferred/priority radio access technology when TN coverage is not available due to disaster situations or TN temporary failure in a given area. Comments: NTN is also being discussed under AI 8.2.1.4. |
| RP-253228 | Group the initial access channels as - 1st group: Same design must be copy-pasted in all CBW for a given SCS - 2nd group: At least BW can be different for different CBW Comments: Appears to be outside the scope of this agenda item. |
| RP-253319 | 1. An enhanced UE capability report framework should be specified as 6G Day-1 operational feature, allowing the UE to dynamically update its capabilities in the RRC_CONNECTED state. 2. A general rule shall be defined to ensure that 6GR day-1 mandatory UE features are IoDTed and deployed in commercial network (e.g., by making any feature that is mandatory for the UE equally mandatory for the network). 3. A device type is determined by a core set of mandatory UE capabilities, with any additional UE capabilities serving as complementary features that build upon the mandatory features. Comments: Appears to be outside the scope of this agenda item. |
|  | [Fixed Wireless Access (FWA)] For FWA, improvements should be identified that can cater for deployment characteristics predominantly applicable for FWA. The following specific attributes should be taken into account: 1. CPE/FWA device capabilities, i.e. antennas and Tx power may be less restricted than for other devices due to less sensitivity to form-factor, power consumption and SAR considerations. 2. FWA device may be typically stationary. 3. FWA devices may typically be located at larger heights than general devices. a. The above may lead to higher inter-cell interference being observed by FWA devices. 4. FWA traffic patterns may be different compared with eMBB, e.g. more uplink heavy from media broadcasting services. Comments: FWA is also being discussed under AI 8.2.1.0 and 8.2.1.4. |
| RP-253385 | [Network Slicing] 1. 6G RAN shall support Network Slicing based on the requirements from SA. 2. 6G RAN shall support Network Slicing considering service continuity from 5G RAN. Comments: TR 38.914 section 5.2 already includes a requirement on slicing (“The 6G RAN architecture shall allow for the operation of network slicing”) and network slicing is also in the scope of SA2’s study. The proposed text could be considered in a new 5.3.x subclause for network slicing or deferred pending further progress in SA2. |

Moderator’s proposed way forward:

Postpone consideration of the above seven papers to a future meeting under this or another appropriate agenda item.

# 3	Conclusions

Proposals 1, 2, 3 and 5 were discussed during the Tuesday offline session and seem agreeable. The associated text is provided in the pCR in section 4.

Proposal 4 could not be discussed during the offline session due to time constraints. Given that the topic was discussed already under AI 8.2.2 (RP-253443), it is suggested to check online whether the text is agreeable for operational requirements. If so, the pCR in section 4 can be updated to include it.

# 4	pCR for TR 38.914 V0.2.0

*** start of changes ***

### 5.3.3	RAN sharing

6G RAT shall support Network Sharing (i.e., MOCN, MORAN, and Indirect Network Sharing).

### Additional description and requirements for RAN sharing are provided in TR 38.760-3 [x].

### 5.3.4	Ease of operation and Self Organization

Self-organization facilitates the automatic deployment of network nodes, including the establishment of basic operational parameters without manual intervention.

### 6GR and 6G RAN architecture shall support automatic configuration and optimisation of the RAN, including ANR and collection of MDT data.

### 5.3.5	Network Resilience

### The 6G RAN shall be designed to ensure accessibility and service continuity with minimum interruption time in any failure condition.

### 5.3.6	Service awareness in RAN

### 6G RAN shall be able to understand and be capable of adapting to traffic demands of applications while maintaining overall system efficiency in terms of radio resource utilisation and energy consumption.

*** end of changes ***