3GPP TSG RAN meeting #110										RP-253764

Baltimore, USA, Dec 8 – 12, 2025

## Status Report to TSG

Agenda item:			9.3.4.7

| WI / SI Name | Enhancement of low NR band carrier aggregation via switching |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LBCA_Sw_enh |  |  |  |  |
| Unique ID | 1090071 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252953 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2027 | Performance part: 09/2027 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 10% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ivo Maljevic |
|  | Company | TELUS |
|  | Email | ivo.maljevic@telus.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #116bis – RF (Core)

General aspects

-	The topic summary was prepared in [31]

-	The work plan was presented and discussed in [4]

UE RF aspects

-	A draft CR for 38.768 on enhanced LB CA via switching scenarios was endorsed in [34]

-	The following agreements were captured in [33]:

Issue 1: Work scope clarification - Do not consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19 - Restrict the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier Issue 2: Common understanding of the UE reference architecture RAN4 has reached the common understanding of the following aspects related to the UE reference architecture: - The main path antenna is shared across Case 1 and Case 3 switching states - The diversity path antenna is shared across Case 1 and Case 3 switching states- UE needs to switch duplex/RF filter in the front-end across Case 1 and Case 3 switching states - An example of the front end filter architecture for the case of adjacent bands is provided below for information Issue 3: Open issues related to the UE reference architecture - Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation - Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination - Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above - Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated Issue 4: Switching period for switching between Case 1 and Case 3 - Introduce new UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementations - Switching period capability values {35us, 70us, 140us} - The capability shall be per band pair per band combination Issue 5: Common understanding of the switching time mask for switching between Case 1 and Case 3 - Reuse the Rel-19 ON/OFF time mask figure for Rel-20 LB CA switching Issue 6: Open issues related to the switching time mask for switching between Case 1 and Case 3 - Whether/what additional clarifications are needed for the time mask considering DL CA configuration of Case 3 - Whether confirmation by RAN1 is needed on any assumptions made by RAN4 - How to verify for conformance that the UL transient period precedes the first UL symbol after the switch and that it does not include the switching period is challenging using the existing time-mask tests Issue 7: Possible co-existence issues - The preliminary analysis of possible co-existence issues is summarized in the table below for information only (the intention is to give companies a starting point for their analysis) - Companies are encouraged to provide their analysis of the issue in the future meetings to allow RAN4 to confirm the co-existence scenarios and to make progress toward identifying possible mitigation - It is recommended to arrange future discussion of this issue in the Rel-19 maintenance agenda for the NR_LBCA_Sw work item Scenario; Type of interference; CA_n12-n29; CA_n28-n67. UE to UE (UE “A” is transmitting FDD UL and UE “B” is receiving both SDL & FDD DL in dual DL operation); Co-channel; UE “B” may face heavy co-channel interference issue if UE “B” is receiving signal at the first half part of n29 spectrum and UE “A” is transmitting signal at the upper side of n12 UL spectrum. This could be acceptable, given that n29 is supplemental carrier.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. If, however, RAN4 assumes that n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz for the LB CA, it may be feasible to mitigate the situation thanks to 5 MHz guard band. This also mitigates the challenges in developing UE Rx filter to cover both the entire n28 and n67, since relative frequency ratio of around 8.4% (100*(65/770.5)) decreases to around 6.6% (100*(50/762)). Blocking; Since Rx filter covering both n29 and n12 DL cannot cut n12 UL power out if n12 UL is used at the upper side of n12, both n29 & n12 DL of UE “B” may face a blocking issue. Given that dual DL operation assumes that n12 DL (PCell) is always available, the blocking issue should be carefully handled, though those issues including co-channel are UE to UE so that the actual impact depends on various conditions such as distance between UEs, UL power, RBs position, received DL power, etc.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to Co-channel interference, 5 MHz guard band can help UE “B” cut the power of n28 UL out. BS to UE / UE to BS; Given that co-location is assumed, all those wouldn’t have an issue.. BS to BS; Co-channel interference and blocking; Since the assumption is collocated scenario, this will cause a big impact on BS performance to receive n12 UL from UEs due to co-channel and blocking from n29 BS DL transmission, given that the guard band is only 1MHz.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to UE to UE co-existence for CA_n28-n67, 5 MHz guard band can help n28 BS cut the power of n67 BS out and n67 BS cut the interference over n28 UL out.. Issue 8: Impact on RX requirements - Further discuss REFSENS, ΔRIB,c, ACS, in-band blocking, out of band blocking Issue 9: Related to configuration - Further discuss the relationship and/or fallback behavior between Rel-19 low band switching and Rel-20 low band switching

RAN4 #116bis – RRM

General aspects

-	The topic summary was prepared in [30]

RRM Core aspects

-	The following agreements were captured in [32]:

### Issue 1-1-1: RRM work scope Thursday online Agreement: - It is RAN4 common understanding that the following RRM requirements are defined in Rel-19 LB-CA switching SDL SCell related RRM requirement; Index; Requirements. 1; Deactivated SDL SCell measurement. 2; Interruption due to deactivated SCell measurement. 3; Activated SDL SCell L3 measurement. 4; BFD/CBD for SDL SCell. 5; L1-RSRP/SINR for SDL SCell. 6; SDL SCell activation/deactivation. 7; TCI state switch delay. FDD PCell related RRM requirement; 8; PCell L3 measurement. 9; BFD/CBD for PCell. 10; L1-RSRP/SINR for PCell. 11; RLM for PCell. 12; TCI state switch delay. 13; PL-RS switch delay for PCell. - For Rel-20, following the WID, RAN4 to Reuse Rel-19 LB-CA RAN4 requirements as much as possible. Session Chair: - Based on companies’ comments, the following difference between Rel-19 and Rel-20 can be further discussed in RAN4, for example and for information: - Potential requirement impact considering the DL reception in FDD PCell in Rel-20 (HW, OPPO) - Need of Tx timing requirement for PCell (ZTE) - Need of Interruption or scheduling restriction requirement for CC switching (QC) - Need of switching gap (Nokia). ### Issue 1-2-1: Switching pattern applicability Thursday online Agreement: - Switching pattern applicability for RRM requirement in Rel-20 LB-CA via switching: - For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is applied. - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied.. ### Issue 1-3-2: SDL SCell related requirement Thursday online Agreement: - For L3 measurement and L1 measurement on activated SDL SCell, reuse the Rel-19 requirement as starting point. - For TCI state switch delay requirements, reuse the Rel-19 requirement as starting point. - For SDL SCell activation/deactivation delay requirements, reuse the Rel-19 requirement as starting point. - No new RRM requirement is expected for deactivated SCell measurement period requirements. ### Issue 1-1-2: MRTD assumption Way forward: - FFS: Options in topic summary R4-2513530. ### Issue 1-1-4: SSB assumption Way forward: - FFS: Options in topic summary R4-2513530. ### Issue 1-2-2: Interruption or scheduling restriction requirement for CC switching Way forward: - FFS: Options in topic summary R4-2513530. ### Issue 1-3-1: Requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC Way forward: - FFS: Options in topic summary R4-2513530. ### Issue 1-3-3: Tx timing requirement for FDD PCell Way forward: - FFS: Options in topic summary R4-2513530. ### Issue 1-4-1: SDL SCell related RRM requirement Way forward: - FFS: Options in topic summary R4-2513530. ### Issue 1-4-2: FDD PCell related RRM requirement Way forward: - FFS: Options in topic summary R4-2513530. ### Issue 1-4-3: Active BWP switching for PCell and SCell Way forward: - FFS: Options in topic summary R4-2513530. ### Issue 1-5-1: Interpretation on switching pattern Way forward: - FFS: Options in topic summary R4-2513530.

RAN4 #117 – RF (Core)

General aspects

-	The topic summary was prepared in [58]

UE RF aspects

-	A CR for 38.768 on enhanced LB CA via switching scenarios was agreed in [61]

-	The following agreements were captured in [60]:

Issue 1: Common understanding of the UE reference architecture RAN4 has reached the common understanding of the following aspects related to the UE reference architecture: - For CA_n28A-n67A: - Specify requirements assuming a filter implementation that takes the Region 1 frequency range restriction for band n28 into account - The following note can be added to the specification in relation to this band combination: “the range of operation in band n28 is restricted to 703-733 MHz UL and 758-788 MHz DL” - For other potential NR_LBCA_Sw_Enh CA combinations where the frequency gap between DL bands is not small enough or there is potential interference in the gap, RAN4 assumes the following are needed for Case 3 operation: - A CR to TR38.768 is needed to capture these aspects, together with those agreed during RAN4 #116bis in R4-2515156, in the next meeting Issue 2: Switching time mask between Case 1 and Case 3 - The following updates to the big picture of the switching time mask (to be introduced into the TR) are needed: - ;Companies are encouraged to check the following proposals based on the paper R4-2520092. Figure 1 The whole picture on switching time mask for RTDS2P ≤ 0 Figure 2 The whole picture on switching time mask for RTDS2P > 0 1) From UL/DL FDD band to Dual-DL FDD+SDL bands: FFS 2) From Dual-DL FDD+SDL bands to UL/DL FDD band: Actual switching gap length ≥ ceil[ ( Tswitch + TTA * 106+ l * RTDS2P + FDD UL leading transient period ) / (1/14*103) ] UL transient period is assumed as 10us. Where RTDS2P is the receiving timing difference among the closest DL slot timing boundaries between SCell and PCell, with the range of negative MRTD to positive MRTD. And MRTD = 3us assuming that the FDD and SDL carriers are colocated. In addition, RTDS2P = - RTDP2S. If RTDS2P ≥ 0, then l = 1; if RTDS2P < 0, then l = 0.. - A CR to TR38.768 is needed to capture these aspects in the next meeting Issue 3: Open issues related to the switching time mask for switching between Case 1 and Case 3 - companies are encouraged to share views on if there is RAN1 impact to clarify the switching pattern for Case 3 in the next meeting - After approving the big picture of the switching time mask (see Issue 2), RAN4 may send a liaison with the information to RAN1 with a request to take the information into account during the next meeting. - RAN4 can further discuss whether an update of the RMC is needed to test switching between Case 1 and Case 3 with consideration of RAN1 impact conclusion. Issue 4: Summary of impacted Rx requirements - Inter-band CA operating bands (already defined in Rel-19) - NR CA configurations and bandwidth combinations (already defined in Rel-19) - ∆TIB,c for Inter-band CA (No need to define in Rel-20) - Transmit ON/OFF time mask for inter-band CA (reuse in Rel-19) - ∆RIB,c due to CA - Blocking requirement Issue 5: REFSENS and ΔRIB,c - For CA_n28A-n67A, specifying ΔRIB,c to accommodate potential higher insertion losses for the combined filter relative to the single-band filters; the relaxation value is FFS - For CA_n12A-n29A, FFS if we can reuse single-carrier REFSENS or not. - Consider the fact that if the case 3 operation is affected by blocker, both PCell (n12) and SCell (n29) are impacted, while for Rel-19 LBCA switching, only SCell is affected. Issue 6: ACS and blocking - For CA_n28A-n67A: - Considering the limitation of wider DL filters in suppressing closer UL, the performance of ACS can be further analyzed - Specify the IBB and OBB blocking requirements based on the Region 1 restricted operating range of band n28 - For CA_n12A-n29A: - Specify ACS, IBB, OBB based on unrestricted operation in both bands - NOTE: unrestricted operation subject to BS-BS coexistence discussion in maintenance

RAN4 #117 – RRM

General aspects

-	The topic summary was prepared in [57]

RRM Core aspects

-	The following agreements were captured in [59]:

### Issue 1-2-1: Requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC Tuesday online Agreement: - In Rel-20 LB-CA via switching, for gap-less based L3 measurement and L1 measurement of activated SDL SCC - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed. - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.. ### Issue 1-2-2: Application time of switching pattern in SCell activation/deactivation Tuesday online Agreement: - The application time of enabling/disabling switching pattern including TLBCA for the SCell which is in activation/deactivation procedure defined in Rel-19 can be reused, containing - MAC CE based SCell activation/deactivation - RRC based direct SCell activation - timer triggered SCell deactivation - For CQI/L1-RSRP reporting during SDL SCell activation in LB-CA requirement. - The issue can be closed (without any additional clarification or spec change in RAN4). ### Issue 1-2-3: Others for SDL SCell including measurements, TCI state switch delay and PL-RS switch delay Issue 1-2-3: Others for SDL SCell including measurements, TCI state switch delay and PL-RS switch delay Agreement: - Activated SDL SCell measurement requirements in Rel-20 LB-CA via switching - The requirements of L3 measurement and L1 measurement delay (including L1-RSRP/L1-SINR/BFD/CBD) on activated SDL SCell defined in Rel-19 LB-CA can be reused - TCI state switch delay requirement in Rel-20 LB-CA via switching - The TCI state switch delay requirement defined in Rel-19 LB-CA can be reused - Deactivated SCell measurement period requirement in Rel-20 LB-CA - The legacy deactivated SCell measurement period requirements are applied for UE to perform deactivated SDL SCell measurement. ### Issue 1-1-4: Tx timing requirement for FDD PCell Agreement: - Reuse the existing Tx timing requirement for PCell with the same prerequisite (SSB is available in last 160ms). - No spec impact for this agreement.. ### Issue 1-1-1: MRTD assumption Agreement: - The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching in Rel-20 LB CA is 3us. ### Issue 1-1-2: Band combination assumption Agreement: - For Rel-20 LB-CA via switching, RAN4 only defines requirements for CA with FDD PCC and one SDL SCC. ### Issue 1-1-3: SSB assumption Way forward: - For SSB assumptions in Rel-20 LB-CA via switching: - Reuse the SSB assumptions for Rel-19 LB-CA via switching as baseline, that is SDL SCell is transmitting SSB - FFS: SSB-less SCell ### Issue 1-3-1: SDL SCell activation/deactivation Way forward: - For the SCell activation/deactivation requirements identified for Rel-20 LB-CA via switching - FFS: Whether/How to reuse Rel-19 LB-CA requirements as starting point to define the delay requirement - Whether/How to clarify definitions of parameters (e.g., TfirstSSB/TfirstSSB_MAX) to reflect Rel-20 LB-CA enhancement - Whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed - Other aspects are not precluded - FFS: Whether/How to define interruption or scheduling restriction during SDL SCell activation/deactivation for Rel-20 LB-CA via switching - FFS: Wait for more conclusions on Rel-19 LB-CA ### Issue 1-3-2: Interruption due to deactivated SCell measurement Way forward: - For interruption requirement of deactivated SDL SCell measurement in Rel-20 LB-CA via switching - FFS: Wait for more RF progress on reference architecture for Case 3 to determine whether/how to define interruption requirement on PCell (i.e., interruption length on PCC) due to deactivated SCell measurement - FFS: How to define Missed ACK/NACK rate: - Option 1: X=2xA+BmeasCycleSCell ,where A = 2x retuning time (switch to and back), B= SMTC duration (i.e., time duration staying on SDL carrier) - Option 2: YUL+YDLmeasCycleSCell, where YUL is the interruption length on FDD UL, and YDL is the interruption length on FDD DL - Option 3: [(2 x retuning time + SMTC duration)]/measCycleSCell - Option 4: [2x[tuning + retuning] + SMTC duration]/ measCycleSCell - Other options are not precluded ### Issue 1-3-3: FDD PCell related RRM requirement Way forward: - FFS: RAN4 to define the following requirements for FDD PCell in Rel-20 LB CA via switching - Intra-frequency L3 measurement (gap-less only) - RLM - L1-RSRP/SINR/BFD/CBD - PL-RS switch delay - TCI state switch delay - FFS: Wait for more RF progress on reference architecture for Case 3 to determine how to define the above RRM requirements for FDD PCell - Option 1: Whether/How to reuse legacy requirements as starting point to define the requirement - Option 2: Whether/How to reuse Rel-19 LB-CA requirements as starting point to define the requirement - Option 3: Other options are not precluded ### Issue 1-3-4: Active BWP switching for PCell and SCell Way forward: - FFS: Options in topic summary R4-2520844. ### Issue 1-3-5: Related to configuration Way forward: - Relation to Rel-19 LB CA via switching in Rel-20 LB-CA RRM, postpone discussions and wait for Rel-20 RF LB-CA progress ## Sub-topic 1-5 Potential RAN1 impacts Way forward: - It is necessary to re-consider/modify interruption or scheduling restriction requirement for Rel-20 LB-CA switching in RAN1/RAN4. - Postpone the discussion on interruption or scheduling restriction requirement in RAN4. - Ask RAN1 on the applicability of the Rel-19 LB-CA collision handling rules between switching pattern and configured DL/UL resources, postpone sending the corresponding LS until more solid agreements are achieved in RAN4 or until RAN1 gets allocated TU - FFS: What information from RAN4 should to inform RAN1 in the LS

#### 2.4.2	Remaining Open issues

Core part:

-	Specify the necessary UE RF core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band

-	Specify the necessary UE RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band

Performance part:

-	Specify RRM test case(s) corresponding to the RRM core requirements.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

RAN4 #116bis

- R4-2515093	Moderator (Apple)	Way Forward for [116bis][320] NR_low_band_CA_switching

- R4-2513536	Huawei, HiSilicon	General views on NR_LBCA_Sw_enh

- R4-2513769	Nokia	Considerations for NR_LBCA_Sw_enh

- R4-2514294	TELUS	Work plan for enhancement of low NR band carrier aggregation via switching

- R4-2513176	CATT	Discussion on R20 low bands CA via switching

- R4-2513448	Xiaomi	UE RF requirements on LB CA with switching

- R4-2513537	Huawei, HiSilicon	Views on UE RF requirements for NR_LBCA_Sw_enh

- R4-2513563	vivo	Discussion on UE RF requirements for Rel-20 LBCA

- R4-2513641	Spreadtrum, UNISOC	Discussion on UE RF requirements for R20 LB-LB CA switching

- R4-2513735	Samsung	Views on UE RF requirements for LB CA switching

- R4-2513810	Skyworks Solutions, Inc.	Feasibility of LB switching FDD+SDL example combinations

- R4-2513813	Ericsson	On low-band CA via switching with dual downlink

- R4-2513855	Apple	Initial Views on RF requirements for LB CA via switching with 2 DLs

- R4-2513993	OPPO	Discussion on R20 low NR band carrier aggregation via switching

- R4-2513997	ZTE Corporation, Sanechips	Discussion on R20 low-low band CA via switching

- R4-2514254	LG Electronics	Discussion on low NR band carrier aggregation via switching

- R4-2514371	Google	Discussion on enhancement of low band carrier aggregation via switching

- R4-2513101	OPPO	Discussion on RRM core requirements for R20 LB CA

- R4-2513223	CATT	Discussion on the RRM core requirements of LBCA enhancements

- R4-2513446	Xiaomi	Discussion on LB-CA RRM requirements

- R4-2513495	Huawei, HiSilicon	Discussion on enhancement of low NR band carrier aggregation via switching

- R4-2513632	Apple	On RRM requirement for R20 LB CA enhancement

- R4-2513731	LG Electronics	Discussion on RRM requirements for enhanced Low Band CA operation

- R4-2513923	ZTE Corporation, Sanechips	Discussion on RRM aspects of R20 Low band carrier aggregation

- R4-2513952	Nokia	Discussion on RRM aspects for enhanced LBCA switching

- R4-2514082	vivo	Discussion on LB CA via switching enhancement

- R4-2514207	Samsung	General discussion on RRM requirements for LB CA switching enhancement

- R4-2514302	Ericsson	RRM requirements for low band carrier aggregation via switching

- R4-2514463	Qualcomm Incorporated	RRM requirements for Rel-20 low NR band carrier aggregation via switching

- R4-2513530	Samsung	Topic summary for [116-bis][224] NR_LBCA_Sw_enh_RRM

- R4-2514443	Moderator (Apple)	Topic summary for [116bis][320] NR_low_band_CA_switching

- R4-2514824	Moderator (Samsung)	WF for [116bis][224] NR_LBCA_Sw_enh_RRM

- R4-2515156	Moderator (Apple)	Way Forward for [116bis][320] NR_low_band_CA_switching

- R4-2514405	Apple	Draft CR for 38.768 on enhanced LB CA via switching scenarios

RAN4 #117

- R4-2520092	CATT	Discussion on R20 low bands CA via switching

- R4-2520253	Huawei, HiSilicon	On remaining UE RF issues for NR_LBCA_Sw_enh

- R4-2520582	Apple	On RF architectures and requirements for LB CA via switching with 2DL

- R4-2520737	vivo	Discussion on LBCA R20

- R4-2520754	Samsung	Views on UE RF requirements for LB CA switching

- R4-2521133	Ericsson	More on low-band CA via switching with dual downlink

- R4-2521276	ZTE Corporation, Sanechips	On R20 low-low band CA via switching

- R4-2521519	LG Electronics	Discussion on low NR band carrier aggregation via switching

- R4-2521572	Nokia	On the issues for NR_LBCA_Sw_enh

- R4-2522254	Skyworks Solutions, Inc.	On band n28 and n67 co-banding for CA_n28-n67 LBLB switching

- R4-2520147	CATT	Discussion on the RRM core requirements of LBCA enhancements

- R4-2520481	Xiaomi	Discussion on LB-CA RRM requirements

- R4-2520665	Apple	On RRM requirement for R20 LB CA enhancement

- R4-2520887	LG Electronics	Discussion on RRM requirements for enhanced Low Band CA operation

- R4-2520938	Huawei, HiSilicon	Discussion on enhancement of low NR band carrier aggregation via switching

- R4-2521010	vivo	Further discussion on LB CA via switching enhancement

- R4-2521143	Nokia	Discussion on NR LBCA enhancements Core Requirements

- R4-2521337	OPPO	Discussion on RRM core requirements for R20 LB CA

- R4-2521681	Samsung	Discussion on RRM requirements for LB CA switching enhancement

- R4-2521925	Ericsson	RRM requirements for Rel-20 low band carrier aggregation via switching

- R4-2521926	ZTE Corporation, Sanechips	Discussion on RRM aspects of R20 Low band carrier aggregation

- R4-2522156	Qualcomm Incorporated	RRM requirements for Rel-20 low NR band carrier aggregation via switching

- R4-2520844	Samsung	Topic summary for [117][219] NR_LBCA_Sw_enh_RRM

- R4-2522232	Moderator (Apple)	Topic summary for [117][320] NR_low_band_CA_switching

- R4-2522653	Moderator (Samsung)	WF on NR_LBCA_Sw_enh_RRM

- R4-2522978	Moderator (Apple)	Way Forward for [117][320] NR_low_band_CA_switching

- R4-2520600	Apple	(NR_LBCA_Sw_enh-Core) CR for 38.768 on enhanced LB CA via switching scenarios

04.02.2025		minor adaptations for RAN #107

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template