3GPP TSG RAN #110			RP-253782

Baltimore, USA, December 8th – 11th 2025

Title:	Moderator’s summary for AI 8.2.1.5 Key technical principles

Source:	RAN2 Chair (InterDigital)

# Introduction

This document summarizes the discussion in Agenda item 8.2.1.5 Key technical principles, with the aim to agrees to a set of proposals for pCR in TR 38.913.

- RP-253047	Discussion on 6G Key technical principles	OPPO	(not available)

- RP-253065	Key Technical Design Principles for 6G	AT&T	(not available)

- RP-253082	TP on key technical principles for 6G for TR 38.914	Deutsche Telekom, Vodafone, Orange, Telecom Italia Turkcell, Spark NZ, Odido, BT, Bouygues Telecom, Telefonica, Telia Company, SK Telecom, KPN, Rakuten Mobile, CK Hutchison, Telenor, KT Corp., LG Uplus, Boost Mobile Network, Jio Platforms

- RP-253136	Network and UE mutual beneficial feature combination framework for 6G	CMCC

- RP-253149	Key Technical Principles	MediaTek Inc.

- RP-253188	Requirements for AI enablers in 6G radio and RAN	Nokia

- RP-253190	on 6GR key technical principles	Apple

- RP-253196	Consideration on 6G AI/ML functional framework	ZTE Corporation, CAICT, China Unicom, NEC, Jio Platforms Ltd

- RP-253270	Discussion on Key technical principles	CATT

- RP-253320	TP on Key technical principles	Amazon Web Services

- RP-253415	Requirements for AI model transfer and delivery in 6G radio and RAN	Nokia, T-Mobile USA, Ericsson

- RP-253419	Flexible Architecture considerations	Reliance Jio	(Not Available)

# Summary of proposals

## General Aspects

Energy efficiency

- 6G RAT & RAN shall be energy efficient [3]

Spectrum efficiency

- 6G RAT shall provide significant gains in terms of spectrum efficiency and energy efficiency against 5G-Advanced in existing bands [3]

  - The gains shall be assessed in the RAN WG SI based on the NGMN RPAF (https://www.ngmn.org/publications/ngmn-radio-performance-assessment-framework.html)

Moderator proposal 1: Energy efficiency and Spectrum efficiency principles are not to be captured in the key technical principle section, already in scope section

Security

- 6G RAT & RAN shall provide enhanced security, integrity and privacy, e.g. quantum-safe [3]

Moderator proposal 2: Security, privacy principles can be place in operational requirements: 6G RAT & RAN shall provide security, ensuring user privacy, integrity, and confidentiality.

Frequency resource re-use

- Universal frequency resource reuse instead of per base station is considered [9]

Moderator proposal 3: Frequency re-source re-use should not to be captured in the key technical principle section, already in scope section

Moderator would like to get some initial input on this during the offline

User experience/traffic characterization/architecture/simple-single framework

- 6G RAT and RAN shall support a simple framework for user and traffic flow differentiation that improves user experience given network resource constraints, while allowing flexibility, service and user customization, and service exposure (e.g. with APIs) [3]

- 6G RAT and RAN shall support native IMS Voice, incl. emergency calls [3]

- 6G RAT and RAN shall be designed to take into account the bursty nature of the traffic and the large amount of small data transmissions on the air interface [3]

- RAN shall support customized service delivery capability with user centric oriented design, to ensure better coverage, mobility and user experienced data rate compared to 5G. [9]

- RAN shall support functionality decoupling to facilitate seamless introduction of new features in future, targeted to minimize backward compatibility limitation. [9]

- RAN shall support standalone operation for operators, aiming to support self-contained design by including IMS voice, full coverage and other basic service capabilities, with optional MRSS capability. [9]

- The 6GR and 6G RAN architecture shall be defined as a single technology framework based on a standalone architecture. [5]

  - The design shall:

    - Ensure an appropriate set of functionalities; and

    - Minimize the adoption of multiple options for the same functionality; and

    - Avoid excessive configurations; and

    - Avoid excessive UE capabilities and UE capabilities reporting.

- A single common 6GR design shall be defined to meet mobile broadband service requirements [5]

  - This design is expected to also meet vertical needs

- Mutual beneficial feature combination framework is considered for 6GR. Two features that are mutual beneficial for both network and UE respectively can be considered as a feature combination, in which the following rules are followed:

-	Neither feature can be enabled by the network unless the UE declares support for both features.

-	The network can activate either of the two features only if the UE supports both features simultaneously.

-	If the UE supports only one of the features, both features remain deactivated [4].

Moderator proposal – consider the following

- Stand-alone, simplified architecture: 6GR shall use a single and simple design framework minimizing optionality/configurations/UE capability reporting, enabling standalone operation and decoupled functionality to ease future feature introduction

- User/service differentiation & customization: Support simple, flexible traffic differentiation, customized service delivery, service exposure APIs

- Discuss [mutual beneficial feature combination framework]

TN and NTN

- 6G radio design shall aim for common design and UE implementation for TN and NTN. And with such common design, NTN based access (basic feature) should be a mandatory feature for all 6G UE. [10]

Moderator proposal: Support a unified 6G radio design supporting both TN and NTN (when possible)

## Bandwidth & Numerology

- For guidance to WG 6G study regarding numerology, only single numerology (SCS) for the same band / sub-range and avoid ECP. [7]

- For Minimum Spectrum Allocation, RAN#110 to down-select to either 3MHz or 5MHz for 15kHz SCS as Minimum Spectrum Allocation. [7]

- For smallest maximum BW for both UE RF and BB, the smallest maximum BW for both UE RF and BB shall not be less than the Minimum Spectrum Allocation (Smallest maximum BW for both UE RF and BB ≥ Minimum Spectrum Allocation).

  - Focus on the 6GR design requirement that is needed to support diverse device types.

  - Strive to avoid different smallest maximum BW for UE RF and UE BB.

  - As one example, different UE RF and BB BW was introduced in eRedCap which complicates the design without justifiable benefit.

- For guidance to WG 6G study regarding CBW, Maximum CBW from NW perspective Up to 400MHz contiguous spectrum. Maximum CBW from UE perspective Up to 200MHz CBW (30kHz SCS + 8K FFT). [7]

Moderator thinks some of these details are already captured in other sections of the TR:

1.	Maximum NW and UE CBW guidance in spectrum discussion.

2. 	Minimum spectrum allocation in device type discussion

Moderator proposal 4:  Bandwidth and numerology aspects are not captured in the key technical principle section

## Coverage

- 6G RAT and RAN shall support improvements for the Uplink concerning data rate performance (especially at cell edge) and coverage [3]

- For 6GR coverage target [7],

  - Comprehensively address the coverage for (1) All channels/signals and PHY procedures in all RRC states, (2) All services, use cases, verticals, (3) All supported device types

  - It is important to avoid negative impact to the eMBB devices, e.g., avoid over-design of coverage which requires excessive repetition in time domain that leads to low spectral efficiency and higher latency

  - Consider [154dB] MCL (Maximum Coupling Loss) as coverage target.

    - Note: 154dB MCL is needed at least for the following scenarios (Antenna gain loss due to the form factor limitation for wearable devices, e.g., watches NTN deployment, especially GEO).

    - Note: The lowest UL data rate that can be supported at 154dB MCL is expected to be ~1-4 kbps depending on the UE Power Class, e.g., for GEO

Moderator notes that many of these requirements captured in other sections of the TR in KPI.

Moderator proposal 5: Coverage related aspects are not captured in the key technical principle section.  Already in the TR in KPI sections

## Multi-Antenna

- 6G RAT and RAN shall support enhancements of MIMO technology for the scenario of passive antennas with up to 8T8R [3]

  - Note: commercially other lower configurations might be relevant depending on the deployment scenario

- 6G RAT and RAN shall support massive MIMO with active antennas with up to at least 1024 antenna elements and up to at least 256T/256R [3]

  - Note: commercially other lower configurations might be relevant depending on the deployment scenario

- 6G RAT and RAN shall support improvements of massive MIMO technology for both TDD and FDD [3]

- 6G RAT and RAN shall support efficient SU-MIMO and MU-MIMO CSI acquisition techniques including low and high resolution CSI codebook techniques and channel reciprocity based CSI acquisition (e.g. SRS based) techniques from day 1 [3]

- 6G RAT & RAN architecture shall support connectivity through multiple transmission points (multiple TRPs), either collocated or non-collocated [3]

- The following technical aspects shall be considered at least [9]:

  - Single or multiple TRPs connection dynamically configured

Moderator notes that many of these requirements are captured in other sections of the TR including deployment scenarios and device type for UEs.

Also some aspects like “•6G RAT and RAN shall support efficient SU-MIMO and MU-MIMO CSI acquisition techniques including low and high resolution CSI codebook techniques and channel reciprocity based CSI acquisition (e.g. SRS based) techniques from day 1” seems to be too detailed and are still need to be discussed in RAN1

Moderator proposal 6:  Multi-antenna related aspects are not captured in the key technical principle section.  Already in the TR in KPI sections

## Mobility

- 6G RAT & RAN architecture shall support optimised RAN and CN mobility management and paging, and fast state transition to connected mode [3]

- 6G RAT & RAN architecture shall support fast network-based mobility in CONNECTED mode with small interruption time and a minimum number of options [3]

- 6G RAT and RAN shall support IDLE and CONNECTED mode mobility to and from 5G NR (TN & NTN) [3]

  - There is no need to support radio level mobility (i.e. cell reselection or handover) to 3G or 2G – interworking can be done via PLMN/Cell Selection

  - Radio level inter-working (i.e. cell reselection or handover) with 4G is FFS

  - Note: “Mobility chaining” from 6G to 5G to 4G and back from 4G to 5G to 6G shall be supported by the respective CN procedures to be studied in SA2

- 6G RAT and RAN shall support UE-based mobility in IDLE (incl. redirection by the network) [3]

- Consider Mobility transparent to user without interruption impact [9]

Moderator proposal: Consider the following

- Support optimized RAN/CN mobility, fast transitions, and fast network-based connected-mode mobility with minimal options [3].

- Support UE-based mobility in IDLE (incl. redirection by the network)

- Support mobility to/from 5G NR for both TN and NTN.

- Mobility should avoid perceptible interruption to users [9].

## AI for 6G Radio and RAN

Data collection

6G radio and 6G RAN shall be designed to fulfil the following requirements for UE side data collection and transfer:

- Data Security and Integrity: The collected data collected shall be secured, with full guarantees of data integrity and confidentiality during transfer. [6]

- User Privacy and Consent: The system shall respect user privacy, ensuring anonymity and compliance with user consent requirements. [6]

- MNO Control Over Data Transfer: MNOs shall have full control over the data collection process, including initiating, terminating, and managing the transfer of data to the server for UE-side data collection. This control shall not require a Service Level Agreement (SLA) to manage the transfer process. [6]

- Visibility and Transparency: MNOs shall have full visibility over the standardized data, ensuring monitoring and management of the data collection process. [6]

- Future-proof and Extensible Design: The design shall be flexible and scalable to adapt to future 6G AI use cases and evolving technology. [6]

- Design a uniform and efficient data collection and transmission mechanism among network entities (e.g., UE, RAN, CN) in 6G era to support various AI/ML use cases. [8]

- A unified RAN intelligence functional framework is defined to support data collection and model management across all involved WGs. [8]

- Incorporate Agentic AI into the 6G functional framework for RAN intelligence. [8]

- Study the functional framework for RAN intelligence for 6G AI RAN above as starting point. [8]

- Cross-WGs collaboration is required to jointly investigate the impact of Agentic AI on the overall 6G network architecture and study a uniform AI/ML functional framework in 3GPP. Such work can be triggered in RAN3 first and coordinate with other TSG WGs when needed. [8]

- Consider self-learning and optimization by AI/ML at the UE and the network [9]

Moderator notes that the following was agreed in RAN2 – consider similar wording as below as WF

Agreements in RAN2

1.  	Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

    - The data collected is secured and data integrity and confidentiality for that data is ensured.

    - User data privacy, anonymity and user consent is respected.

    - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

    - This includes initiating, terminating, and fully managing data transfer.

    - MNO has full visibility for standardized data.

    - The design is future-proof and extendable.

    - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

    - UE data collection should minimize impact to user traffic transmission and power saving features

These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

Model transfer

AI enablers for 6G radio and 6G RAN shall be designed to fulfil the following requirements for model transfer and delivery [11]:

- Differentiation of Model Transfer Traffic: It shall be possible to differentiate traffic for model transfer and from regular user traffic. The priority for model transfer shall be configurable by the network, allowing for prioritization without impacting user traffic.

- Security of Model Transfer: Model transfer shall be done in a secure and network-aware manner, ensuring that untrusted or malicious models are prevented from being downloaded onto the network or UE.

- Initiation of Model Transfer: The UE shall be able to initiate model transfer while the network shall make the decision of when to transfer the model, providing flexibility and control over the process.

- Addressability of Models: Trained models shall be addressable so that the UE can request models to be transferred or delivered, enabling flexibility in model management. The network shall be able to enable or disable trained models on demand, based on operational needs.

- Network Control of Model Delivery: The network shall retain full control over when and how models are transferred or delivered to the UE.

- Monitoring of models in use: Sufficient monitoring and management information shall be available for each trained model, ensuring that MNOs can assess and manage the performance of models effectively.

Other

- Design a uniform and efficient data collection and transmission mechanism among network entities (e.g., UE, RAN, CN) in 6G era to support various AI/ML use cases. [8]

- A unified RAN intelligence functional framework is defined to support data collection and model management across all involved WGs. [8]

- Incorporate Agentic AI into the 6G functional framework for RAN intelligence. [8]

- Study the functional framework for RAN intelligence for 6G AI RAN above as starting point. [8]

- Cross-WGs collaboration is required to jointly investigate the impact of Agentic AI on the overall 6G network architecture and study a uniform AI/ML functional framework in 3GPP. Such work can be triggered in RAN3 first and coordinate with other TSG WGs when needed. [8]

- Consider self-learning and optimization by AI/ML at the UE and the network [9]

# Discussions

Based on some offline discussions some concerns were raised that a lot of the proposals are already covered in the scope section and we shouldn’t duplicate.  A proposal was made to either 1) expand the section on scope to include technical principles or 2) copy some of the aspects in scope section to the technical principle section.  Some concerns were raised for both options, for 1 one concern was that it may become too large if we start adding more and for 2 that we spent a lot of time drafting the scope and not ideal to start moving things.

Moderator proposal:

- Take scope section as a baseline (i.e. the content of what’s in the scope aren’t duplicated)

- Identify what is missing from the scope that is a technical principle and agree what should be added to the TR

- After we have identified all technical principles that need to be added on top of what’s already in the scope, we can discuss and agree where to put them in the TR (e.g. in the scope section that is extended to include technical principles or in the technical principle section that contains additional principles not already in the scope).

# Conclusions

Agreed to not include the following aspects from the key technical principle sections

- Proposal 1: Energy efficiency and Spectrum efficiency principles are NOT to be captured in the key technical principle section, already in scope section

- Proposal 2: Security, privacy principles can be placed in operational requirements: 6G RAT & RAN shall provide security, ensuring user privacy, integrity, and confidentiality.

- Proposal 3: Frequency resource re-use should NOT to be captured in the key technical principle section, already in scope section

- Proposal 4:  Bandwidth and numerology aspects are NOT captured in the key technical principle section

- Proposal 5: Coverage related aspects are NOT captured in the key technical principle section.  Already in the TR in KPI sections

- Proposal 6:  Multi-antenna related aspects are NOT captured in the key technical principle section.  Already in the TR in KPI sections

Moderator proposal to discuss:

  - Take scope section as a baseline (i.e. the content of what’s in the scope aren’t duplicated)

  - Identify what is missing from the scope that is a technical principle and agree what should be added to the TR

  - After we have identified all technical principles that need to be added on top of what’s already in the scope, we can discuss and agree where to put them in the TR (e.g. in the scope section that is extended to include technical principles or in the technical principle section that contains additional principles not already in the scope).