3GPP TSG RAN meeting #110										     RP-253794

Baltimore, USA, December 8-11, 2025

## Status Report to TSG

Agenda item:			10.5.1

| WI / SI Name | Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_Ph3 |  |  |  |  |
| Unique ID | 1020096 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252504 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% RAN1: 100% RAN2: 100% RAN3: 100% RAN4: 100% | Performance Part: 85% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Chun-Fan (Felix) Tsai |
|  | Company | MediaTek Inc. |
|  | Email | Chun-Fan.tsai@mediatek.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

NA (Core part is ended)

#### 2.1.2	Remaining Open issues

No open issues remaining.

## 2.2	RAN2

#### 2.2.1	Agreements

NA (Core part is ended)

#### 2.2.2	Remaining Open issues

No remaining open issues.

## 2.3	RAN3

#### 2.3.1	Agreements

NA (Core part is ended)

#### 2.3.2	Remaining Open issues

No remaining open issues.

## 2.4	RAN4

#### 2.4.1	Agreements

###### RAN4#117, Nov’25

RRM

No open issues

## 2.1 NPUSCH Aspects

#### Margin

Agreement:

Additional Margin unlikely to be required as simulation results close to alignment and 1 meeting cycle left.

Companies are encouraged to provide updated results in the next meeting.

#### Transmit Timing Drift

Agreement:

Apply the additional transmitting timing drift to NPUSCH format 1, capturing in 36.181.

| Channel | Step size Δt |
| --- | --- |
| NPUSCH format 1, 3.75KHz, 1 tone | [0.32] µs per RU |
| NPUSCH format 1, 15KHz, 1 tone | [0.08] µs per RU |

Companies to check the values in square brackets ahead of next meeting.

SAN Demod

###### RAN4#116bis, Oct’25

#### Issue 3-1-1: Test purpose

Agreement

- The RAN4 test case for CB-Msg3 shall test the following functionalities:

a.	Reception of an incorrect message over CB-RNTI

b.	Reception of a correct message of CB-RNTI

c.	No CB-Msg3 response within CB-Ms3ResponseTimer

d.	The calculated UE Tx power at every CB-Msg3 retransmission according to the respective formula

e.	FFS The right amount of transmission of replicas.

#### Issue 3-1-2: Test configuration for CB-Msg3-EDT

Agreement

- The CB-Msg3-EDT configuration, as configured in CB-Msg3-ConfigSIB-NB-r19 should be provided.

- The legacy RSRP threshold used in the random access test can be reused for cb-Msg3-MinRSRP-Threshold-NB-r19.

#### Issue 3-1-3: Test configuration for HARQ

Agreement

- The HARQ feedback upon reception of Msg4 shall be configured in the configuration of gNB parameters in the test case.

#### Issue 3-1-4: Ensure CB-MSG3 transmission

WayForward

- Leave the decision to R5

RRM

## Sub-topic 2-1: OCC Aspects

#### OCC Length

Agreement

RAN4 shall configure the length of OCC operation, and add the note following when specifying the NPUSCH 1 requirement into specification:

  - Note: The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU,  where NslotsUL is number of UL slot and NRU is the number of RU.

#### How to model two UEs

Agreement

RAN4 shall specify the SNR to achieve 70% of the maximum throughput of NPUSCH format 1 for OCC sequence indexes 0 and 1, which correspond to one UE and another UE, respectively.

#### SNR with two UEs

Agreement

RAN4 to set the same SNR for NPUSCH format 1 with OCC2 from two UEs.

## Work Split

Specification – TS 36.108

| Big CR | ZTE |
| --- | --- |
| NPUSCH requirements (8.5.x, 11.5.x) | Ericsson |
| FRC for 15 kHz | Samsung |

Specification – TS 36.181

| Big CR | Ericsson |
| --- | --- |
| NPUSCH requirements (8.5.x) | Samsung |
| FRC for 15 kHz (11.5.x, FRC) | ZTE |

SAN Demod

###### RAN4#116, Aug’25

Topic #2: LS on Harmonised Standard for NTN capable UE (R4-2509023/SES(25)000071)

### Introduce Nominated BW concept to 3GPP NTN requirements

Issue 2-0: Whether the discussion or agreements for the nominated bandwidth concept affects closure of WI IoT_NTN_Ph3.

- Agreement: Issues 2-1 to 2-3 are out of WI IoT_NTN_Ph3 scope and shall not affect the closure of WI IoT_NTN_Ph3.

#### Issue 2-1: Regarding meeting the ETSI SEM requirement for NB-IoT/NR NTN, whether the “nominated BW (Bn) concept” (e.g., guard-band, extended BW) is useful from 3GPP perspective.

- Agreement:

  - The nominated channel bandwidth concept is considered to be useful to minimize NB-IoT/NR NTN UL power backoff (i.e., A-MPR) for meeting ETSI SEM requirements. The nominated channel bandwidth concept can be adopted to the 3GPP specifications with modifications, such as restriction on the possible extended guard-band size(s) and fixed symmetric extended guard-band values.

  - The utilization of the above concept would be under network's control to use it depending on the deployment.

  - 3GPP RAN WG4 aims to work on the design of this concept as a general framework.

  - Introduction of the concept is considered earliest in Release 19.

#### Issue 2-2: If nominated BW concept (e.g., extended guard-band/extended BW) can be applicable to NB-IoT/NR NTN, then how to specify it.

- Agreement:

  - For meeting ETSI SEM requirements, 3GPP can use fixed extended guard-band value such as 400kHz for meeting nominated BW concept for FR1 NB-IoT NTN band, e.g., apply additional 100 kHz guard band on both sides.

  - FFS on NR-NTN.

Topic #1: IoT_NTN_Ph3 RF core requirements

### CFO grouping to avoid frequency inaccuracy

#### Issue 1-1: Regarding NW using CFO grouping to avoid frequency inaccuracy between OCC UEs, whether this could result in RF requirements impact.

- Agreement:

  - There is no RF requirements impact when NW uses CFO grouping to avoid frequency inaccuracy between OCC UEs.

### Phase continuity and power consistency for the duration of OCC group

#### Issue 1-2: Whether UE is required to maintain phase continuity and power consistency for the duration of OCC group.

- Agreement:

For NB-IoT NTN OCC feature, phase continuity is expected for OCC2 feature, and the phase continuity and related side conditions for OCC2 are UE implementation specific.  UE RF requirements are not specified.

RF

#### Issue 2-3: Introduction of extended guard bands for NTN IOT.

- Agreement:

  - Introduce the following UE capability to be added to Rel-19 capability feature list based on RAN guidance:

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 36.331 | Parent IE in TS 36.331 | Need of FDD/TDD differentiation | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBD |  | Extended guard band for A-MPR reduction | 1. Support of extended guard-band feature |  | [NTN-extendedGuardBand-A-MPR-r19] | [FeatureSetUplink-v1900] | TBD | The feature can be supported in below scenarios: RAN4 intends for the NTN IoT bands. | Optional with capability signalling |

Issue 1-1: LS reply for TAC in CB-Msg3-EDT procedure

Agreement:

- RAN4 has no concern on the agreements regarding TAC in MSG4.

  - Capture this sentence in the reply LS to RAN2.

Issue 2-1: Test cases list

Agreement:

- Define only TC1.

Background:

| Random Access (A.13.3.2) | A.13.3.2.X1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-msg3-EDT | TC1 |
| --- | --- | --- |

RRM

# Topic #1 General

Issue 1-1: Requirements applying under standalone and in-band

- Agreement:

  - The same requirement for IOT NTN NPUSCH applies for both standalone and in-band operation.

# Topic #2 SAN demodulation requirements

Issue 2-1: DMRS pattern for NPUSCH 3.75kHz SCS

- Agreement:

  - Sequential mapping of samples of the original DMRS sequence to active DMRS slots.

Issue 2-2: Whether needs to configure the co-schedule UE

- Agreement:

  - Configure the co-scheduled UE for OCC2 NPUSCH format 1 demodulation requirements.

Issue 2-3: Number of Repetition

- Agreement:

  - Reuse the last meeting agreement: 2 for single tone with 3.75kHz and 8 for single tone with 15kHz.

  - And discuss how to capture the number of repetitions into specification during CR drafting stage.

Issue 2-4: Power assumption for two co-schedule UEs

- Agreement:

  - Assuming the same transmission power for two UEs.

  - Assuming the same receiving power for two UEs at gNB side.

Issue 2-5: Test metric

- Agreement:

  - Measure the NPUSCH format 1 throughput per UE.

    - UE1 uses OCC sequence index 0

    - UE2 uses OCC sequence index 1

  - Set the same SNR for NPUSCH format 1 from two UEs if there is no performance difference.

  - FFS how to write the specification, define NPUSCH format 1 demodulation requirements for each UE

Issue 2-6: Tx Duration

- Agreement

  - Tx Duration = 256ms for 15kHz.

Issue 2-7: Frequency offset

- Agreement:

  - 128Hz for 15kHz.

SAN Demod

# Topic #3 Simulation assumptions

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D)) | FRC | Frequency offset | TxDuration | Repetition number(NRep in 36.213 Table 16.5.1.1-3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 128 Hz | 256 | 2 |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 128 Hz | 256 | 2 |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 128 Hz | 256 | 8 |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 128 Hz | 256 | 8 |

###### RAN4#115, May’25

#### Issue 1-1: CR to TS 36.108

- Proposal:

  - CR to update TS 36.108 clauses 4.3.1 and 4.3.2 to capture correctly the support of regenerative payload with the satellite payload based on RAN4#114bis endorsed draft CR R4-2504715.

- Agreement:

  - Agree CR R4-2508637.

#### Issue 2-1: Whether UE is required to maintain phase continuity and power consistency for the duration of OCC group.

- Agreement:

  - FFS on UE RF requirements and whether related UE capabilities would be needed.

    - Take the NR-NTN DMRS bundling requirements as a starting point to evaluate the phase continuity requirements for IoT-NTN with slot level OCC.

#### Issue 3-1: Regarding UE requirements of NB-IoT NTN operating in NR NTN in-band, study the principle of selected specification series.

- Agreement:

  - It is proper to put UE requirement for NB-IoT NTN in-band operation in 36 series.

  - There is no need to create new 37 specification for NB-IoT NTN in-band operation within NR NTN at current stage unless request on additional case.

#### Issue 3-2: Regarding SAN requirements of NB-IoT NTN operating in NR NTN in-band, study the principle of selected specification series.

- Agreement:

  - It is proper to put SAN requirements for NB-IoT NTN in-band operation with NR NTN in 36 and 38 series.

  - There is no need to create new 37 specification for NB-IoT NTN in-band operation within NR NTN at current stage unless request on additional case.

RF

RRM

Issue 2-1: Test cases for CB-Msg3 - General

Agreement:

- Define test cases for random access for CB-msg3-EDT, and the correct behaviour and transmit timing can be verified in the same test case.

- Further discuss whether to down-select the test cases:

| Random Access (A.13.3.2) | A.13.3.2.X1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-msg3-EDT |
| --- | --- |
|  | A.13.3.2.X2 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage for CB-msg3-EDT |
|  | A.13.3.2.X3 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage for CB-msg3-EDT |

Issue 2-2: Test cases for CB-Msg3 for eMTC

Agreement:

- Not to introduce test cases for CE Mode A and mode B for CB-msg3-EDT

Demod

Sub-topic 1 Work plan

- Agreement:

| Meeting | Target |
| --- | --- |
| RAN4 115 (May 2025) | Discuss work scope and agree with initial simulation assumptions. |
| RAN4 116 (Aug 2025) | Further discuss general test cases and simulation assumptions for each test case. Collect the initial simulation results for alignment. |
| RAN4 116-bis (Oct 2025) | Finalize the simulation assumption. Collect the simulation results. Decide the CR work split. |
| RAN4 117 (Nov 2025) | Collect the simulation results. Review the draft CRs. |
| RAN4 118 (Feb 2026) | CR review and agree on the formal CR. |

Sub-topic 2-1 NPUSCH format 1 demodulation requirements

Whether to define NPUSCH format 1 demodulation requirements with OCC for Rel-19 IoT-NTN

- Agreement:

  - Introduce NPUSCH format 1 demodulation requirements with OCC for Rel-19 IoT-NTN.

SCS

- Agreement:

  - Both 15kHz and 3.75kHz SCS with OCC.

Number of allocated subcarriers

- Agreement:

  - Prioritize the single tone for specifying NPUSCH format 1 requirement with OCC.

OCC length

- Agreement:

  - OCC length = 2.

DMRS pattern

- Agreement:

  - CDM DMRS is assumed for 15kHz SCS OCC and TDM work assumption is assumed for 3.75kHz SCS OCC.

  - RAN4 needs to follow RAN1’s conclusion of DMRS sequence mapping for TDM DMRS slots.

Antenna configuration

- Agreement:

  - 1Rx and 2Rx.

Number of repetitions

- Agreement:

  - 2 for single tone with 3.75kHz and 8 for single tone with 15kHz

MCS

- Agreement:

  - BPSK MCS 0 in Table 16.5.1.2-1 of TS 36.213.

Propagation conditions

- Agreement:

  - Only consider NTN TDLA100-1.

Frequency offset

- Agreement:

  - 128Hz for 3.75kHz SCS and 15kHz SCS.

Tx Duration

- Agreement:

  - 256 for 3.75kHz SCS and 128 for 15kHz.

Test metric

- Agreement:

  - 70% maximum throughput.

Whether to define NPRACH demodulation requirements for Rel-19 IoT-NTN

- Agreement:

  - No need to define NPRACH demodulation requirements for Rel-19 IoT-NTN.

Sub-topic 2-3 Simulation assumptions

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D)) | FRC | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 128 Hz | 256 | 2 | 70% |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 128 Hz | 256 | 2 | 70% |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 128 Hz | 128 | 8 | 70% |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 128 Hz | 128 | 8 | 70% |
| Note 1: The MCS is defined in the Annex A.7.1 of TS 36.108. Note 2: The MCS is defined in the Annex A.16.1 of TS 36.104. |  |  |  |  |  |  |  |  |  |

###### RAN4#114bis, April’25

Issue 1-1: Draft CR to TS 36.108 for supporting regenerative payload with satellite payload

- Agreement:

  - Agree the draft CR R4-2504715.

#### Issue 2-1: Is it feasible to introduce support of TDM DMRS  as per the description in R1-2501512?

- Agreement:

  - It is feasible for the UE to maintain the phase continuity across the blank DMRS symbol in TDM DMRS scheme.

  - Further discuss potential need for requirements if TDM DMRS would be specified (e.g., the measurement time for frequency error requirement).

#### Issue 2-2: Whether NW would need to use CFO grouping or resynchronization schemes to avoid frequency inaccuracy between a pair of OCC UEs and whether this will result in UE RF specification impacts.

- Agreement:

  - FFS CFO group requirements for UE and gNB.

  - Other methods to ensure the CFO grouping performance are not precluded if time allowed.

#### Issue 3-1: Whether any of the following measurement quantities, described in clause 5.2 of TS 36.214, are applicable for NB-IoT-based NTN and/or eMTC-based NTN:

  - DL RS TX power

  - Received Interference Power

  - Thermal Noise Power

- Agreement:

  - All the quantities (i.e., DL RS TX power, received interference power, thermal noise power) above are applicable to NB-IoT/eMTC NTN SAN.

#### Issue 3-2 The reference points for DL RS Tx power, received interference power and thermal noise power.

- Agreement:

  - Agree the conclusion of R4-2504718.

#### Issue 4-1: Regarding NB-IoT NTN operating in NR NTN in-band, study the principle of selected specification series:

- Agreement:

  - The conclusion about the selected specification would be decided in RAN4#115.

RF

RRM

Issue 1-1: Subclauses in Random Access requirement for CB-Msg3

Agreement:

- Include the following subclauses in Random Access requirement for CB-Msg3 in IoT NTN

  - Correct behavior when transmitting CB-Msg3.

  - Correct behavior when receiving CB-Msg4

  - Correct behavior when not receiving CB-Msg4

- The exact titles of the subclauses can be further updated to align with RAN2 specifications.

Issue 1-4: CR work split

Agreement:

Table 1. CR work split for CB-Msg3

| Core requirement | Volunteer | Comment |
| --- | --- | --- |
| 6.6A Random Access for UE category NB-IoT for Satellite Access | Nokia | New subclause for CB-Msg3 |
| 6.2.4A Random Access Requirements for Cat-M1 UEs with CB-Msg3 for Satellite Access | CMCC | New subclause for CB-Msg3 It appears that a separate new subclause is necessary. |
| 7.20A UE transmit timing for NB-IoT for Satellite Access | MediaTek | Revised to apply to CB-Msg3 |
| 7.24A UE transmit timing for Category M1 for Satellite Access |  |  |

###### RAN4#114, Feb’25

Issue 1-1: Draft CR to TS 36.108 for supporting regenerative payload

- Agreement:

  - Postpone the draft CR R4-2502286.

#### Issue 2-1: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - No RF impact on OCC2 for IoT NTN.

#### Issue 2-2: Whether there would be UE RF requirement impact when OCC schemes would cross gap in UL transmission.

- Agreement:

  - No RF impact for <1ms gap length.

  - FFS for longer gaps.

#### Issue 2-3: Whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

- Agreement:

  - Further discuss whether solutions (e.g., CFO grouping) for solving frequency inaccuracy between OCC UEs would have potential RF impact.

RF

Issue 1-1-1: Procedural requirements for CB-MSG3 transmission

Agreement:

- Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133.

  - Further discuss the content to include in the next meeting.

RRM

###### RAN4#113, Nov’24

#### Issue 1-1: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with symbol-level OCC2

- Agreement:

  - Keep following the RAN4#112 agreement of “No UE RF requirement impact from symbol-level for NPUSCH”.

#### Issue 1-2: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with slot-level OCC2/OCC4

- Agreement:

  - RAN4 stops evaluating UE RF requirement impacts for slot-level OCC2/OCC4 of 3.75 kHz single-tone NPUSCH format 1 since RAN1 agreed to remove the options.

#### Issue 1-3: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC4

- Agreement:

  - RAN4 stops evaluating UE RF requirement impacts for slot-level OCC4 of 15 kHz single-tone NPUSCH format 1 since RAN1 agreed to remove this option.

#### Issue 1-4: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - Pending on RAN1 progress of slot-level OCC2 resource mapping.

#### Issue 1-5: Whether there would be UE RF requirement impact when OCC shemes would cross gap in UL transmission.

- Agreement:

  - Whether to study the case that gap between OCC slots may affect RF requirement pending on RAN1.

#### Issue 1-6: Whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

- Agreement:

  - Further discuss whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

RF

Issue 1-1-1: RRM impact from CB-msg3 EDT

Agreement:

- Clarify the “N_TA” mentioned in the agreement R4-2416867 refers to “N_TA_Ref” as specified in clause 7.20A.2 and 7.24A.2 of TS 36.133 v18.7.0.

RRM

###### RAN4#112bis, Oct’24

#### Issue 1-1: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with symbol-level OCC4

- Agreement:

  - Encourage companies to reconfirm whether previous agreement would be still valid.

    - RAN4#112 R4-2413530: “Agreement: No UE RF requirement impact from symbol-level for NPUSCH”

#### Issue 1-2: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with slot-level OCC2/OCC4

- Agreement:

  - Wait for further progress in RAN1 before further work on inter-slot OCC, if any.

#### Issue 1-3: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - Encourage companies to study the potential RF impact.

#### Issue 1-4: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC4

- Agreement:

  - Encourage companies to study the potential RF impact.

#### Issue 1-5: UE RF requirement impact from NPRACH with OCC2/OCC4 feature

- Agreement:

  - Wait for further progress in RAN1 before further work on inter-symbol/inter-slot OCC, if any.

RF

RRM

Topic #2: LS reply for R1-2407548

Issue 2-1: Reply regarding the Note 1

Agreement:

- Reply to RAN1 that RAN4 assumed that N_TA is equal to 0, if there is no other procedure for NTA determination introduced by other WG. RAN4 will follow the conclusion of the other working groups in the future, if any.

Issue 2-2: Reply regarding the Note 2

Agreement:

- Confirm to RAN1 that NOTE 2 in R1-2407548 is valid.

Besides, the following LS is approved.

- R4-2416918 Reply LS to RAN1 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

###### RAN4#112, Aug’24

# Topic #1: Work plan for Rel-19 IoT_NTN_Ph3

Issue 1-1: Work plan for Rel-19 IoT_NTN_Ph3

- Approve the workplan in R4-2413529

# Topic #2: RF core requirements

NOTE1: Enhancements to enable multiplexing of multiple UEs in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH.

#### Issue 2-1: UE RF requirement impact for NPUSCH with OCC feature

- Agreement:

  - No UE RF requirement impact from symbol-level for NPUSCH

  - FFS on UE RF requirement impact from slot-level for NPUSCH

#### Issue 2-2: UE RF requirement impact for NPRACH with OCC feature

- Agreement:

  - Wait RAN1 reach conclusions on OCC feature before RAN4 evaluates the UE RF impact.

#### Issue 2-3: SAN RF requirement impact for NPUSCH/NPRACH with OCC feature

- Agreement:

  - Wait RAN1 reach conclusions for OCC feature before RAN4 evaluates the SAN RF impact.

RF

### Sub-Topic 1-1: RRM impact

The work scope of RRM for Rel-19 IoT NTN is to discuss whether and how to define timing requirements for Msg3 transmission without msg1/ Random Access Response (RAR).

- Note: It can be revisited if the RRM impact from other objectives has been identified.

RRM

Besides, the following LS is approved.

- R4-2414114 Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE

#### 2.4.2	Remaining Open issues

- No remaining open issues in core part.

- Performance part:

  - To confirm the step size regarding the additional transmitting timing drift to NPUSCH format 1, capturing in 36.181.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

List of all related Tdocs in the affected WGs since last TSG.

## 4.1	RAN1

NA (Core part is ended)

## 4.2	RAN2

NA (Core part is ended)

## 4.3	RAN3

NA (Core part is ended)

## 4.4	RAN4

RAN4#117, Nov’25:

| R4-2520037 | Introduction of test case for CB-Msg3 procedure for NB-IoT with Satellite Access | MediaTek inc. |
| --- | --- | --- |
| R4-2521182 | Simulation results on SAN demodulatoion requirements for IoT-NTN | ZTE Corporation, Sanechips |
| R4-2521183 | Simulation summary on SAN demodulatoion requirements for IoT-NTN | ZTE Corporation, Sanechips |
| R4-2521185 | Big CR on 36.108 for Rel-19 IoT-NTN Phase 3 demodulation requirements | ZTE Corporation, Sanechips |
| R4-2521226 | SAN demodulation requirements for IoT-NTN | Ericsson |
| R4-2521228 | Big CR: Introduction of SAN demodulation requirements for IoT-NTN Phase 3 | Ericsson |
| R4-2521358 | Discussion on IoT NTN Phase3 demodulation performance requirements | Huawei,HiSilicon |
| R4-2521472 | Discussion and simulation results on demodulation requirement for IoT NTN | Samsung |
| R4-2521473 | Draft CR on FRC for 15KHz for IoT NTN phase 3 in TS 36.108 | Samsung |
| R4-2523072 | Draft CR on 36.181 for Rel-19 IoT-NTN Phase 3 demodulation requirements | ZTE Corporation, Sanechips |
| R4-2523073 | draft CR: NPUSCH format 1 requirements for NTN | Ericsson |
| R4-2523075 | Draft CR on NPUSCH requirement for IoT NTN phase 3 in TS 36.181 | Samsung |
| R4-2522219 | Topic summary for [117][310] Rel-19 RRM perf_Part2_NTN | Moderator (MediaTek) |
| R4-2522222 | Topic summary for [117][312] Rel-19 Demod_Part2_NTN | Moderator (Nokia) |
| R4-2522936 | Way Forward for [117][310] Rel-19 RRM perf_Part2_NTN | MediaTek |
| R4-2523085 | Way Forward for [117][312] Rel-19 Demod_Part2_NTN | Nokia |

RAN4#116bis, Oct’25:

| R4-2513638 | Discussion on RRM performance requirements for IoT NTN phase 3 | MediaTek inc. |
| --- | --- | --- |
| R4-2513395 | Discussion and simulation results on demodulation requirement for IoT NTN | Samsung |
| R4-2513441 | Discussion on IoT NTN Phase3 demodulation performance requirements | Huawei,HiSilicon |
| R4-2513970 | SAN demodulation requirements for IoT-NTN | Ericsson |
| R4-2514151 | Simulation results on SAN demodulatoion requirements for IoT-NTN | ZTE Corporation, Sanechips |
| R4-2514152 | Simulation summary on SAN demodulatoion requirements for IoT-NTN | ZTE Corporation, Sanechips |
| R4-2514433 | Topic summary for [116bis][310] Rel-19 RRM performance_Part2 | Moderator (MediaTek) |
| R4-2514435 | Topic summary for [116bis][312] Rel-19 Demodulation_Part2 | Moderator (Nokia) |
| R4-2515055 | Way Forward for [116bis][310] Rel-19 RRM performance_Part2 | MediaTek |
| R4-2515160 | Way Forward for [116bis][312] Rel-19 Demodulation_Part2 | Nokia |