3GPP TSG RAN Meeting #110										       RP-253835

Baltimore, US, 8th – 11th Dec 2025

## Status Report to TSG

Agenda item:			9.3.4.6

| WI / SI Name | Enhanced requirements for NR NTN and IoT NTN Phase 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IoT_NTN_req_Ph2 |  |  |  |  |
| Unique ID | 1090070 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252962 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2027 | Performance part: 09/2027 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 15% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Hsieh, Daniel, Yang, Yoonoh |
|  | Company | Mediatek, LGE |
|  | Email | Daniel.Hsieh@mediatek.com, yoonoh.yang@LGE.COM |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

In last RAN Meeting, the TU excel file was missing in the WID. So, Rapporteur would like to add TU excel sheet in the SR. This TU budget already assumed in RAN4 Total TU budget.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

Summary

Based on the WID, the group had agreed the work plan and started work on the PC1.5 for NB-IoT NTN and the HD-FDD VSAT and small size antenna phase 1.

The group had discussed the UE requirements for PC1.5 NB-IoT NTN, and agreed several Tx requirements, including SAR solutions, ACLR, SEM, while the MPR and A-MPR will require further input to finalize the values. Regarding the Rx requirements, the group agreed that there is no impact when introducing PC1.5 NB-IoT NTN.

Regarding the RF impact on the phase 1 HD-FDD VSAT and small size antenna, the group had agreed to re-use the existing FR1 EIRP, EIS and ON/OFF time mask requirement of the FD-FDD VSAT for HD-FDD. The group will further study the potential new VSAT type(s) for small antenna, including VSAT that support LEO only and VSAT that support both GSO and LEO. For small size VSAT that support LEO only, it was agree to focus on 20x20 cm size as the starting point, and several related parameters had been discussed. For small size VSAT that support both GSO and LEO, the size with 30x30 cm is considered as the starting point.

For the RRM impact on the phase 1 HD-FDD VSAT and small size antenna, the group had agreed to reuse the existing handover, conditional handover, satellite switch and UL timing accuracy requirements for HD-FDD.

#### 2.4.1	Agreements

##### Core part:

###### RAN WG4 Meeting #116bis, Prague, Czech Republic, 13th – 17th October, 2025:

The work plan of Enhanced requirements for NR NTN and IoT NTN Phase 2 in [48] was approved.

The following agreements documented in the Way Forward for NR_IoT_NTN_Ph2 in [47] were approved:

WF for PC1.5 IoT NTN

Sub-topic 2-1: General issues

Issue 2-1-1: Scope clarification, feasibility

Agreement:

The feasibility of PC 1.5 single Tx NB-IoT NTN UE in both handhelds and non-handhelds is confirmed.

Both category NB1 and NB2 are in the scope of this WI.

The eMTC-based IoT-NTN is out of scope of this release.

Issue 2-1-2: Work plan on the TR

Agreement:

Suggest to update the WID in the next RAN meeting to include a TR for NTN HPUE

Issue 2-1-6: Capability signalling for PC1.5 NB-IoT NTN UE

Agreement:

The signaling to indicate support for PC1.5 is needed to be added to LTE RRC signalling with preference on early implementation.

Issue 2-1-7: Power boosting for PC1.5 NB-IoT NTN UE

Agreement:

Power boosting is not considered for PC1.5 NB-IoT NTN UE.

Sub-topic 2-2: Tx requirements for PC1.5 NB-IoT NTN UE

Issue 2-2-1: The tolerance of maximum output power

Agreement:

Apply +2/-3 dB tolerance for the 29 dBm NB-IoT based NTN PC1.5 power class.

Issue 2-2-2: SAR solution for PC1.5 NB-IoT NTN UE

Agreement:

Re-use existing P-MPR method for PC1.5 NB-IoT NTN UE.

Issue 2-2-3: UE RF front end assumptions and related accommodations

WayFoward:

FFS on power handling challenges when NB-IoT NTN UE is scheduled for long duration e.g. 256ms in UL with high power e.g. +29dBm. (e.g. potential accommodations to UE maximum output power requirements)

The timeline of the PC1.5 IoT NTN work shall not be impacted by this issue, even if no solution is agreed, no discussion after RAN4#118.

Issue 2-2-4: ACLR requirement for PC1.5 NB-IoT NTN UE

Agreement:

Apply UTRA ACLR value of 37 dBc for PC1.5 single Tx NB-IoT NTN UE.

Issue 2-2-5: SEM requirement for PC1.5 NB-IoT NTN UE

Agreement:

Reuse the existing SEM requirement in TS 36.102 clause 6.5B.3.2

Apply the following for the additional SEM requirement for NS_02N (for band 255)

4 dBm when ΔfOOB is ± 0.09–0.28 MHz

-6 dBm when ΔfOOB is ± 0.28–0.85 MHz

-13 dBm when ΔfOOB is larger than ± 0.85 MHz

Issue 2-2-6: MPR requirements for PC1.5 NB-IoT NTN UE

Agreement:

Take the following tables as starting point for the MPR for PC1.5 NB-IoT NTN UE (Based on MPR = maximum {0, PC1 MPR - 2dB})

Table 2-2-6-1 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ [0] dB | [0] dB | ≤ [0] dB | ≤ [1.5] dB | [0] dB | ≤ [1.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table 2-2-6-2 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [0] dB | ≤ [0] dB |  | ≤ [0] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [0.5]dB |  | ≤ [0.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [2] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Interested companies can provide further evaluation in the next RAN4 meeting based on following assumptions to check whether modifications on the above two tables are needed

Option 1

Calibration points for PC1.5 (based on NR PC2 PA):

DFT-s-OFDM QPSK 20 MHz

100RB0

4dB post PA loss /or [5] dB

1dB MPR

Carrier leakage: 28 dBc

IQ image: 28 dBc / or [31] dBc

CIM3: 60 dBc

NR ACLR: 31dBc

Option 2

DFT-s-OFDM 1RE (Single Tone)

Centre Single Tone [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK]

[3] or [4] dB post PA loss

0dB MPR

Carrier leakage 28dBc

IQ image:28 dBc

CIM3: 60 dBc

The result from companies provided in the next RAN4 meeting will be taken into account when deciding the final value

Issue 2-2-7: A-MPR requirement for band 256 PC1.5 NB-IoT NTN UE

Agreement:

Take the following as the starting point for the A-MPR for band 256 PC1.5 NB-IoT NTN UE with NS_24, depending on the final MPR value

Table 2-2-7.1 Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_24 | 6.5B.4.4.3 | 256 | ≤ [8~8.5] |

Interested companies can provide further evaluation in the next RAN4 meeting based on the assumptions in issue 2-2-6, including whether the above A-MPR is applicable for all frequencies of band 256 or a subset of frequencies of band 256.

The result from companies provided in the next RAN4 meeting will be taken into account when deciding the final value

Issue 2-2-8: A-MPR requirement for band 255 PC1.5 NB-IoT NTN UE

Agreement:

A-MPR is not needed for band 255 PC1.5 NB-IoT NTN UE with NS_02N

FFS on A-MPR evaluation for band 255 PC1.5 single Tx NB-IoT NTN UE with additional ETSI requirements associated with NS_10N and NS_14N

Sub-topic 2-3: Rx requirements for PC1.5 NB-IoT NTN UE

Issue 2-3-1: RSD requirement

Agreement:

No RSD requirement is needed for NB-IoT based NTN PC1.5 UE.

Issue 2-3-2: Other Rx requirement

Agreement:

Confirm there is no Rx requirement impact for PC1.5 NB-IoT based IoT-NTN.

The following agreements documented in the Way Forward for NR_IoT_NTN_Ph2_Part2 in [46] were approved:

WF for UE RF requirements for HD-FDD for Ku band with FR1 numerology

Sub-topic 1-1: Issues for HD-FDD requirements

Issue 1-1-1: Antenna type assumption for HD-FDD VSAT

WF

The working assumption for Phase 1 is that the HD-FDD VSAT antenna type is a single physical aperture with Tx/Rx switching capability.

Other implementations are not precluded.

Issue 1-1-2: RF requirements for fixed VSAT for HD-FDD

WF

For FR1-NTN Ku band n248, n247, the FDD requirements for Type 2 VSAT and Type 3 VSAT are based on the Type 5 and Type 6 requirements regarding maximum output power, OTA reference sensitivity level, and maximum input level. The same principle should be applied to HD-FDD.

Further check the regulation related requirements related to NS_203N~NS_208N for the fixed VSAT operating in n248 and n247.

Issue 1-1-3: Transmit ON/OFF time mask for HD-FDD

WF

Consider reusing existing Transmit ON/OFF time mask requirement as starting point.

Issue 1-1-4: Treatment on HD-FDD specifications.

WF

Start with FDD requirements as the baseline and further check if any RF requirements need to be modified for HD-FDD.

Update the SAN RF requirements if necessary.

Issue 1-1-5: EIRP requirements for HD-FDD

WF

Consider the existing FDD VSAT EIRP requirements for HD-FDD as the starting point

FFS on whether to modify.

Issue 1-1-6: EIS requirements for HD-FDD

WF

Consider the existing FDD VSAT EIS requirements for HD-FDD as the starting point

FFS on whether to modify.

Sub-topic 1-2: Issues for VSAT with small antennas (e.g., 20x20cm)

Issue 1-2-1: New VSAT type

WF

Study potential new VSAT type(s) for small antenna with the following candidates:

VSAT that support LEO only

VSAT that support both GSO and LEO

Issue 1-2-2: Assumptions for VSAT with small antenna

WF

Conduct implementation and feasibility studies for the small antenna considering, but not limited to, the following aspects:

Antenna size, number of antenna elements, antenna gain, Tx power per element and link budget evaluation for supporting LEO only or supporting both GSO and LEO.

Issue 1-2-3: ACLR and ACS for the new VSAT with small antenna

WF

Consider the existing Ku-band ACLR and ACS as the starting point

FFS on whether to modify.

Encourage companies to provide justifications for whether to re-use the existing specifications.

Issue 1-2-4: Specifications impact for HD-FDD VSAT with small antenna

WF

FFS in the next meeting.

WF for RRM core requirements for HD-FDD for Ku band with FR1 numerology

Sub-topic 2-1: HD-FDD with FR1 numerology

Issue 2-1-1: RRM requirements for HD-FDD with FR1 numerology

Agreement

Define the HD FDD VSAT with FR1 numerology RRM requirements based on the Rel-19 Ku band NTN FDD requirements, and incorporate updates to address HD FDD impacts, using the RedCap NTN approach as the baseline.

The following RRM requirements scope for HD FDD VSAT in Ku band with FR1 numerology

Cell reselection, including paging reception

Handover and CHO

Satellite switching

Random access

UE UL timing accuracy

RLM, including scheduling availability and L1 indication

intra/inter-frequency measurements, including scheduling availability

L1-RSRP measurement, including scheduling availability

Issue 2-1-3: Handover interruption time

WayForward

Option 1: HD-FDD has no impact on NR handover.

Option 2: FFS impact due to reduced Rx time

Further discuss the following conditions for HD-FDD

For RACH-based HO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

SSB is available at the UE once every SMTC period during Tsearch

One SSB is available during T∆

One SSB is available during TIU

For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

One SSB is available during T∆

One SSB is available during TIU

Issue 2-1-5: UE UL timing accuracy

WayForward

Further discuss the following options

Option 1: HD-FDD has no impact on UE UL timing accuracy.

Option 2: FFS impact due to Tx/Rx pattern.

Issue 2-1-2/4/6/7: other RRM requirement impact due to HD-FDD

WayForward

RAN4 to further discuss if any RRM requirement impact due to HD-FDD for the following aspects

Maximum interruption in paging reception

initial access/random access

inter and/or intra-measurement requirement

scheduling availability

Issue 2-1-8: HD-FDD Mobile VSAT with small antennas

WayForward

wait for further results from the RF discussions.

Issue 2-1-9: UE capability

WayForward

RAN4 to assess the need for new UE capabilities for HD-FDD VSAT UE with NTN.

Issue 2-1-10: Collision handling

Agreement

reuse the existing collision handling approach as the baseline unless any issues are identified.

Issue 2-1-11/12:

WayForward

RAN4 can further study the following aspects

RAN4 to discuss recommending to RAN1 that TA reporting should become a mandatory feature for Ku Bands supporting HD-FDD.

RAN4 to study potential enhancements in intra-satellite measurements to support mobility measurements in HD-FDD mode.

Issue 2-1-13: spec structure

WayForward

Defer the discussion to the CR drafting phase.

Sub-topic 2-2: HD-FDD with FR2 numerology

Issue 2-2-1: HD-FDD with FR2 numerology

WayForward

postpone this until phase 2 is triggered.

Sub-topic 2-3: performance part

Issue 2-3-1: Measurement accuracy and test configurations

WayForward

Defer the discussion to the performance part.

Endorsed draft CRs in RAN4#116bis:

None

###### RAN WG4 Meeting #117, Dallas, USA, 17th – 21th December, 2025:

The following agreements documented in the Way Forward for NR_IoT_NTN_Ph2 in [95] were approved:

WF for PC1.5 IoT NTN

Sub-topic 2-1: General issue for PC1.5 NB-IoT NTN UE

Issue 2-1-1: Clarification on the scope of the TR

WF:

Suggest to update the WID in the next RAN meeting to include a TR(s) for the whole WI.

Issue 2-1-2: LS on the capability for PC1.5 NB-IoT NTN

WF:

Target on sending an LS to RAN2 together with completion of PC1.5 NB-IoT NTN requirements.

Sub-topic 2-2: Tx requirements for PC1.5 NB-IoT NTN UE

Issue 2-2-1: Discussion on power handling challenges on PC1.5 NB-IoT NTN UE

WF:

Companies are encouraged to analysis whether there exist challenges for UE transmitting high power (e.g. 29 dBm) with long duration, with consideration of aspects below.

NR PC2 and NB-IoT PC1.5 supported via the same Tx path as one implementation option

Impact to supporting voice over NB-IoT NTN under coverage-limited conditions shall be minimized.

Issue 2-2-2: MPR requirements for PC1.5 NB-IoT NTN UE

WayForward:

Target on finalized MPR requirements in next RAN4 meeting, with consideration of further evaluation input based on the same simulation assumption in R4-2515157.

Consider the following tables as tentative values (numbers can be further adjusted based on the input in the next RAN4 meeting)

Table 2-2-2.11 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0 | 1-10 | 11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ [0 or 0.5] dB | [0] dB | ≤ [0 or 0.5] dB dB | ≤ [1.5] dB | [0] dB | ≤ [1.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table 2-2-2.12 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [0] dB | ≤ [0] dB |  | ≤ [0] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [0.5or 1]dB |  | ≤ [0.5 or 1] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [2] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Issue 2-2-3: A-MPR requirement for band 256 PC1.5 NB-IoT NTN UE

WF:

A-MPR value for NS_24 as [8] dB

Issue 2-2-4: A-MPR requirement for band 255 PC1.5 NB-IoT NTN UE

WayForward:

Companies are encouraged to study the A-MPR for NS_10N and NS_14N, provide measurements/simulations results at the next meeting to finalize the work for PC1.5 UE, together with the work for PC3/PC2/PC1 in the maintenance part.

The following agreements documented in the Way Forward for NR_IoT_NTN_Ph2_Part2 in [92] were approved:

Sub-topic 1-1: Issues for HD-FDD requirements

Issue 1-1-1: Regulation requirements for fixed VSAT (type 2 and 3)

WF

Reflect the 50 dBW maximum EIRP limit for NTN VSAT operating in NTN CEPT Ku-band(s) 14.25 – 14.5 GHz from ECC Decision(03)04 in the Maximum output power limits for Fixed VSAT table.

The requirements related to NS_203N, NS_204N, NS_207N, and NS_208N apply to fixed VSAT.

The off-axis EIRP requirements related to NS_205N apply to NTN Ku fixed VSAT operating in CEPT regions.

Further investigate the on-axis spurious, off-axis spurious and receiver off-axis gain pattern requirements for NTN Ku fixed VSAT operating in CEPT regions, with consideration of EN 301 428, EN 302 448 at least.

Issue 1-1-2: Transmit ON/OFF time mask for HD-FDD

Agreement

Reuse the existing Ku-band FR1 FDD Transmit ON/OFF time mask requirement for HD-FDD

Issue 1-1-3: Treatment on HD-FDD specifications.

Agreement

No SAN RF impact has been observed by introducing HD-FDD VSAT.

Issue 1-1-4: EIRP requirements for HD-FDD

Agreement

Reuse the existing FDD VSAT minimum EIRP requirements for HD-FDD for VSAT types 2, 3, 5 and 6

Reuse the existing FDD VSAT maximum EIRP requirements for HD-FDD for VSAT types 2, 3, 5 and 6

Issue 1-1-5: EIS requirements for HD-FDD

Agreement

Reuse the existing FDD VSAT EIS requirements for HD-FDD for VSAT types 2, 3, 5 and 6

Reuse the existing FDD VSAT maximum input level for HD-FDD for VSAT types 2, 3, 5 and 6

Sub-topic 1-2: Issues for VSAT with small antennas (e.g., 20x20cm)

Issue 1-2-3: New VSAT type

WF

Study potential new VSAT type(s) for small antenna with the following candidates:

VSAT that support LEO only

VSAT that support both GSO and LEO

Issue 1-2-4: System parameters alignments for VSAT with small antenna

WF

Take the following VSAT parameters as a starting point for 20x20cm electronically steered panel, other values are not precluded

| 1 | VSAT Antenna Characteristics |  |
| --- | --- | --- |
| 1.1 | Antenna pattern | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 1) | [3.5] |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization (Note 5) | Circular (RHCP or LHCP) |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | [15x15], [16x16], [17x17] elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | [0.5] lambda |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | [4-15] |
| 1.11 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | Normal direction | Toward X+ axis |
| 1.14 | Noise Figure | [6] dB |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: Antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization evaluations are conducted by converting to linear polarization |  |  |

Issue 1-2-5: System parameters alignments for SAN aspects

WF

Take the following satellite parameters as a starting point, other values are not precluded

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite EIRP density | Ku-band (i.e. 11 GHz for DL) | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite RX max Gain | Ku-band (i.e. 14 GHz for UL) | 44 dBi | 38.5 dBi | 38.5 dBi |

Issue 1-2-6: Assumptions for VSAT with small antenna for LEO

WF

Study antenna size with 20×20cm with the following candidates:

17 × 17 antenna elements

16 × 16 antenna elements

Other antenna elements are not precluded.

Issue 1-2-7: New VSAT with small antenna to support LEO

WF

Encourage more companies to analyze the following RF requirements based on the updated system parameters (Issue 1-2-4 & 5):

Minimum peak EIRP

Maximum EIRP

Maximum TRP

EISREFSENS_50MHz

Issue 1-2-8: New VSAT with small antenna to support both GSO and LEO

WF

Further study on how to support both GSO and LEO for VSAT with small antennas size with the following consideration as the starting point

Antenna size around 30cm × 30cm

Consideration of small channel BW support (e.g. 3, 5MHz)

Issue 1-2-9: ACLR and ACS for the new VSAT with small antenna

Agreement

Reuse the existing Ku-band ACLR and ACS requirements for small antenna VSAT.

Issue 1-2-10: Regulations for the new VSAT with small antenna

Agreement

Consider whether to exclude the non-conformance testable “off-axis gain limit“ and “Rx antenna off-axis performance” requirements for new VSAT types with small antenna for LEO only and for both GSO and LEO or not.

The following agreements documented in the Way Forward for NR_IoT_NTN_Ph2_Part2: RRM_part in [93] were approved:

Sub-topic 2-1: HD-FDD with FR1 numerology

Issue 2-1-1: RRM requirements scope for HD-FDD VSAT with FR1 numerology

Agreement

RAN4 to confirm that VSATs in Ku bands with FR1 numerology does not take inter-satellite measurements into this WI scope.

Discuss whether to capture requirement applicability rule regarding the VSAT types,

Option 1: clarify the HD-FDD VSAT requirements applicability in clause 3 in CR

Option 2: no applicability rule for VSAT type needed.

Issue 2-1-2: Handover and satellite switch

Agreement

For HD-FDD VSAT, reuse the existing handover, conditional handover, satellite switch requirements defined for NTN VSAT

FFS whether to capture the following applicability conditions in CR

For RACH-based HO,

SSB is available at the UE once every SMTC period during Tsearch

One SSB is available during T∆

One SSB is available during TIU

For Time/location-based CHO,

One SSB is available during T∆

One SSB is available during TIU

For satellite switching requirements,

SSB is available at the UE once every SMTC period during Tsearch

One SSB is available during T∆

One SSB is available during TIU

Issue 2-1-3: UE UL timing accuracy

Agreement

For HD-FDD VSAT, reuse the existing timing requirements defined for NTN VSAT.

Issue 2-1-4: Scheduling availability and collision handling

Agreement

For HD-FDD VSAT, reuse the existing scheduling availability for NTN FR1 Redcap as the baseline.

This WI applies the HD-FDD collision rule for HD-FDD VSATs without TA reporting as the baseline.

Issue 2-1-5: Other RRM requirement impact due to HD-FDD

Agreement

For HD-FDD VSAT, reuse the legacy applicability conditions for HD-FDD for the following existing VSAT requirements

Maximum interruption in paging reception

Random access requirements

RLM and L1-RSRP requirements, including CSI-RS based measurement and L1 indication.

FFS whether to add condition as at least one SSB is available.

Issue 2-1-6: TA report

WayForward

Further discuss the following options

Option 1: keep it optional

Option 2: make technical recommendations if appropriate

Issue 2-1-8: New UE capability

WayForward

Postpone the discussion

Issue 2-1-9: CR structure

Recommended WF:

In Phase 1, HD-FDD related requirements should be defined under the clauses defined for NTN RRM requirements for Ku band in Rel-19.

Issue 2-1-10: CR work split for phase 1

Agreement

| VSAT requirement in 38.133 | Volunteer |
| --- | --- |
| Applicability | Xiaomi |
| Cell reselection, including paging reception |  |
| Handover and CHO | ZTE |
| Satellite switching |  |
| Random access | CATT |
| UE UL timing accuracy | Ericson |
| RLM, including scheduling availability and L1 indication |  |
| intra/inter-frequency measurements, including scheduling availability | Huawei |
| L1-RSRP measurement, including scheduling availability | MTK |

Sub-topic 2-2: HD-FDD Mobile VSAT with small antennas

Issue 2-2-1: HD-FDD Mobile VSAT with small antennas

WayForward

Postpone the discussion

Sub-topic 2-3: performance part

Issue 2-3-1: Measurement accuracy and test configurations

WayForward

Defer the discussion to the performance part.

Endorsed draft CRs in RAN4#117:

[94]   R4-2523087	draft CR for Introduction of PC1.5 for NB-Iot based Iot-NTN	vivo, MediaTek

##### Performance part:

Not started.

#### 2.4.2	Remaining Open issues

##### Core part:

Remaining RF requirements including MPR, A-MPR requirements for PC1.5 NB-IoT NTN,

Remaining RF and RRM requirements for HD-FDD NTN phase 1

RF and RRM requirements for HD-FDD Mobile VSAT with small antennas with FR1

RF and RRM requirements for HD-FDD NTN phase 2

RF and RRM requirements for DL intra-band intra-SAN CA for NTN

##### Performance part:

RRM performance and demodulation requirements for HD-FDD for NTN

SAN conformance, RRM performance and demodulation requirements for intra-SAN DL CA

## 2.5	RAN5

#### 2.5.1	Agreements

The work plan was updated in [1]. The work plan shows the current completion status in detail.

- Overall completion is 100% (+5%).

#### 2.5.2	Remaining Open Issues

N/A

#### 2.5.3	Remaining Open issues with cross-WG dependencies

N/A

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## Not applicable

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

Contributions submitted to RAN4#116bis:

| Ref | Tdoc | Subject | Source | Status |
| --- | --- | --- | --- | --- |
| [1] | R4-2513202 | Discussion on RRM requirements for HD-FDD for Ku band with FR1 numerology | CATT | noted |
| [2] | R4-2513232 | Discussion on general aspect for NR NTN and IoT NTN Phase 2 | CATT | noted |
| [3] | R4-2513233 | Discussion on RF requirements for IoT NTN support PC 1.5 | CATT | noted |
| [4] | R4-2513234 | Discussion on UE RF requirements for Ku band HD-FDD VSAT | CATT | noted |
| [5] | R4-2513355 | On IoT NTN PC1.5 Single Tx | Samsung | noted |
| [6] | R4-2513356 | On HD-FDD VSAT RF requirements | Samsung | noted |
| [7] | R4-2513379 | Views on general aspects of NTN topics | CSCN | noted |
| [8] | R4-2513462 | Discussion on RRM core requirements of HD-FDD for Ku band with FR1 numerology | Xiaomi | noted |
| [9] | R4-2513463 | Discussion on R20 HPUE NTN TX requirement | Xiaomi | noted |
| [10] | R4-2513464 | Discussion on R20 HPUE NTN RX requirement | Xiaomi | noted |
| [11] | R4-2513465 | Discussion on R20 HD-FDD NTN RF requirement | Xiaomi | noted |
| [12] | R4-2513560 | HD-FDD implications for Ku and Ka band VSAT RF | Eutelsat, Thales | noted |
| [13] | R4-2513564 | Discussion on PC1.5 HPUE Requirements for Rel-20 NTN | vivo, MediaTek | noted |
| [14] | R4-2513565 | draft CR for Introduction of PC1.5 for NB-Iot based Iot-NTN | vivo, MediaTek | postponed |
| [15] | R4-2513568 | Initial views on Enhanced requirements for NR NTN and IoT NTN Phase 2 | vivo | noted |
| [16] | R4-2513579 | HD-FDD implications for Ku and Ka band RRM | Eutelsat, Thales | noted |
| [17] | R4-2513640 | Discussion on RRM core requirements for NTN requirement enhancement phase 2 | MediaTek inc. | noted |
| [18] | R4-2513678 | Work plan for Enhanced requirements for NR NTN and IoT NTN Phase 2 | CHTTL, MediaTek Inc., LG Electronics | revised |
| [19] | R4-2513711 | On Ku-band VSAT Half-Duplex RF requirements with FR1- Numerology | LG Electronics Finland | noted |
| [20] | R4-2513732 | Discussion on RRM requirements for Ku-band NTN with HD-FDD | LG Electronics Inc. | noted |
| [21] | R4-2513733 | Discussion on Enhanced requirements for NR NTN and IoT NTN Phase 2 in Rel-20 | LG Electronics France | noted |
| [22] | R4-2513840 | Assumptions, Proposals and Work Plan for IoT-NTN PC1.5 single Tx related work | ViaSat Satellite Holdings Ltd | noted |
| [23] | R4-2513866 | UE RF requirements to support HD-FDD for Ku/Ka bands with FR1 numerologies | Sharp | noted |
| [24] | R4-2513868 | Discussion on RF requirements for NB-IoT based NTN PC1.5 | China Telecom Corporation Ltd. | noted |
| [25] | R4-2513903 | Initial views on NTN HD-FDD VSAT RF Requirements for Ku-band | MediaTek (Wuhan) Inc. | noted |
| [26] | R4-2513913 | Genernal discussion on RRM impact for HD-FDD for Ku band with FR1 numerology | vivo | noted |
| [27] | R4-2513940 | (NR_IoT_NTN_req_Ph2-Core) Discussion on HD-FDD for NTN Core Part | Samsung | noted |
| [28] | R4-2513992 | Discussion on R20 PC1.5 with 1Tx for IoT NTN HPUE | OPPO | noted |
| [29] | R4-2514031 | Discussion on IoT-NTN PC1.5 single Tx | ZTE Corporation, Sanechips | noted |
| [30] | R4-2514032 | Discussion on UE RF requirements for HD-FDD for Ku band | ZTE Corporation, Sanechips | noted |
| [31] | R4-2514070 | Discussion on RRM requirements for enhanced requirements and test methodology for NR and IoT NTN | Ericsson | noted |
| [32] | R4-2514104 | Discussion on RRM requirements for HD-FDD VSAT on Ku band | ZTE Corporation, Sanechips | noted |
| [33] | R4-2514115 | R4-2513810 1Tx PC1.5 for NB-IoT-NTN based on Release 19 PC1 work | Skyworks Solutions Inc. | noted |
| [34] | R4-2514126 | Initial discussion on the Rel.20 FR1-NTN Ku band VSAT enhancement | CHTTL, SKY Perfect JSAT Corporation | noted |
| [35] | R4-2514131 | On IoT-NTN PC1.5 single Tx | Huawei, HiSilicon | noted |
| [36] | R4-2514183 | Discussion on RRM impact of Rel-20 NTN enhancement | Huawei, HiSilicon | noted |
| [37] | R4-2514349 | On introducing HD-FDD on the Ku Band | Nokia | noted |
| [38] | R4-2514370 | VSAT RF requirement for HD-FDD | Ericsson | noted |
| [39] | R4-2514383 | NB-IoT NTN PC1.5 UE RF requirements | Qualcomm Incorporated | noted |
| [40] | R4-2514407 | Views and proposals related to MPR and A-MPR for bands 255 and 256 for IoT-NTN PC1.5 single Tx | ViaSat Satellite Holdings Ltd | noted |
| [41] | R4-2514441 | Topic summary for [116bis][318] NR_IoT_NTN_Ph2 | Moderator (CHTTL) | noted |
| [42] | R4-2514503 | On HPUE for NTN UE RF requirements | Ericsson | noted |
| [43] | R4-2514537 | Topic summary for [116bis][330] NR_IoT_NTN_Ph2_Part2 | Moderator (MediaTek) | noted |
| [44] | R4-2515080 | Work plan for Enhanced requirements for NR NTN and IoT NTN Phase 2 | CHTTL, MediaTek Inc., LG Electronics | revised |
| [45] | R4-2515081 | Way Forward for [116bis][318] NR_IoT_NTN_Ph2 | CHTTL | revised |
| [46] | R4-2515082 | Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2 | MediaTek | approved |
| [47] | R4-2515157 | Way Forward for [116bis][318] NR_IoT_NTN_Ph2 | CHTTL | approved |
| [48] | R4-2515163 | Work plan for Enhanced requirements for NR NTN and IoT NTN Phase 2 | CHTTL, MediaTek Inc., LG Electronics | approved |

Contributions submitted to RAN4#117:

| Ref | Tdoc | Subject | Source | Status |
| --- | --- | --- | --- | --- |
| [49] | R4-2520039 | Discussion on RRM core requirements for NTN requirement enhancement phase 2 | MediaTek inc. | noted |
| [50] | R4-2520119 | Discussion on RRM requirements for HD-FDD for Ku band with FR1 numerology | CATT | noted |
| [51] | R4-2520160 | Discussion on RF requirements for IoT NTN support PC 1.5 | CATT | noted |
| [52] | R4-2520161 | Discussion on UE RF requirements for Ku band HD-FDD VSAT | CATT | noted |
| [53] | R4-2520227 | (NR_IoT_NTN_req_Ph2-Core) Discussion on RRM requirements for HD-FDD for Ku band with FR1 numerology | Samsung | noted |
| [54] | R4-2520295 | UE RF requirements to support HD-FDD for Ku/Ka bands with FR1 | Sharp | noted |
| [55] | R4-2520402 | HD-FDD RRM | Eutelsat Group, THALES | noted |
| [56] | R4-2520485 | Discussion on R20 HD-FDD NTN RF requirement | Xiaomi | noted |
| [57] | R4-2520500 | Discussion on RRM core requirements of HD-FDD for Ku band with FR1 numerology | Xiaomi | noted |
| [58] | R4-2520581 | On NB-IoT PC1.5 A-MPR | Apple | noted |
| [59] | R4-2520688 | On Ku-band VSAT Half-Duplex RF requirements with FR1-Numerology | LG Electronics Finland | noted |
| [60] | R4-2520733 | Discussion on Enhanced requirements for NR NTN and IoT NTN Phase 2 | vivo | noted |
| [61] | R4-2520741 | draft CR for Introduction of PC1.5 for NB-Iot based Iot-NTN | vivo, MediaTek | revised |
| [62] | R4-2520742 | Further Discussion on PC1.5 HPUE Requirements for Rel-20 NTN | vivo, MediaTek | noted |
| [63] | R4-2520760 | Discussion on Ku-band HD-FDD VSAT | Murata Manufacturing Co Ltd. | noted |
| [64] | R4-2520780 | Proposed A-MPR for NS_10N and NS_14N for PC1.5 NB-NTN UE for band 255 | ViaSat Satellite Holdings Ltd, Thuraya | noted |
| [65] | R4-2520822 | Views on NTN HD-FDD VSAT RF Requirements for Ku-band | MediaTek (Wuhan) Inc. | noted |
| [66] | R4-2520888 | Discussion on RRM requirements for Ku-band NTN with HD-FDD | LG Electronics Inc. | noted |
| [67] | R4-2521061 | Discussion on IoT-NTN PC1.5 single Tx | ZTE Corporation, Sanechips | noted |
| [68] | R4-2521062 | Discussion on UE RF requirements for HD-FDD for Ku band | ZTE Corporation, Sanechips | noted |
| [69] | R4-2521063 | LS on RRC signalling for PC1.5 IoT-NTN | ZTE Corporation, Sanechips | Postponed |
| [70] | R4-2521222 | Discussion on RRM requirements for enhanced requirements and test methodology for NR and IoT NTN | Ericsson | noted |
| [71] | R4-2521260 | Views on UE RF requirements for HD-FDD for Ku-band | Qualcomm Incorporated | noted |
| [72] | R4-2521422 | Discussion on IoT-NTN PC1.5 single Tx | Samsung | noted |
| [73] | R4-2521423 | Discussion on UE RF requirements of HD-FDD for Ku band with FR1 numerology | Samsung | noted |
| [74] | R4-2521442 | Genernal discussion on RRM impact for HD-FDD for Ku band with FR1 numerology | vivo | noted |
| [75] | R4-2521501 | Discussion on NTN enhancement HD-FDD for Ku band small-type Antenna | SKY Perfect JSAT Corporation, CHTTL, SES | noted |
| [76] | R4-2521516 | Discussion for Rel-20 IoT-NTN HPUE | LG Electronics France | noted |
| [77] | R4-2521521 | Views on general aspects of NTN topics | CSCN | noted |
| [78] | R4-2521531 | Discussion on the Rel.20 FR1-NTN Ku band VSAT enhancement | CHTTL, SKY Perfect JSAT Corporation | noted |
| [79] | R4-2521539 | Discussion on RRM requirements for HD-FDD VSAT on Ku band | ZTE Corporation, Sanechips | noted |
| [80] | R4-2521655 | Discussion on RRM impact of Rel-20 NTN enhancement | Huawei, HiSilicon | noted |
| [81] | R4-2521658 | On IoT-NTN PC1.5 single Tx | Huawei, HiSilicon | noted |
| [82] | R4-2521746 | Discussing the impact of introducing HD-FDD on the Ku Band | Nokia | noted |
| [83] | R4-2521907 | 1Tx PC1.5 for NB-IoT-NTN based on Release 19 PC1 work | Skyworks Solutions Inc. | noted |
| [84] | R4-2521947 | NB-IoT NTN PC1.5 requirements | Qualcomm Incorporated | noted |
| [85] | R4-2522005 | NTN UE RF requirment for HD-FDD | Ericsson | noted |
| [86] | R4-2522172 | On HPUE for NTN UE RF requirements | Ericsson | noted |
| [87] | R4-2522194 | UE HD-FDD RF | THALES, EUTELSAT Group, Mediatek | noted |
| [88] | R4-2522229 | Topic summary for [117][318] NR_IoT_NTN_Ph2 | Moderator (CHTTL) | noted |
| [89] | R4-2522230 | Topic summary for [117][330] NR_IoT_NTN_Ph2_Part2 | Moderator (MediaTek) | noted |
| [90] | R4-2522968 | Ad-hoc meeting minutes for [117][318] NR_IoT_NTN_Ph2 | CHTTL | noted |
| [91] | R4-2522969 | Way Forward for [117][318] NR_IoT_NTN_Ph2 | CHTTL | revised |
| [92] | R4-2522970 | Way Forward for [117][330] NR_IoT_NTN_Ph2_Part2 | MediaTek | approved |
| [93] | R4-2523030 | Way Forward for [117][330] NR_IoT_NTN_Ph2_Part2: RRM part | MediaTek | approved |
| [94] | R4-2523087 | draft CR for Introduction of PC1.5 for NB-Iot based Iot-NTN | vivo, MediaTek | endorsed |
| [95] | R4-2523106 | Way Forward for [117][318] NR_IoT_NTN_Ph2 | CHTTL | approved |

12.08.2024		minor adaptations for RAN #105

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v01	13.11.2009		First version of the template