3GPP TSG RAN Meeting #110							RP-253845

Baltimore, USA, December 8-11, 2025

Status Report to TSG

Agenda item:			9.3.4.2

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_BS_RF_req_evo_Ph2 |  |  |  |  |
| Unique ID | 1091066 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252905 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2027 | Performance part: 09/2027 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 5% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Wubin Zhou |
|  | Company | ZTE Corporation |
|  | Email | zhou.wubin@zte.com.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

The progress in RAN4 #116bis meeting is summarized below:

The WF R4-2515091 for both objectives and the TR skeleton R4-2515090 for SBFD BS to SBFD BS adjacent channel co-existence are approved.

Enhancement of co-location requirements:

Agreements:

- At this point give equal priority to both option 1 and option 2.

- As starting point consider side-by-side co-location scenario with 10 cm separation. Further investigate typical scenarios for bands in upper region of FR1.

- At this point focus on licensed bands when coupling loss is evaluated.

SBFD BS to SBFD BS adjacent channel co-existence:

Agreements:

- For next meeting further consider following options, with the intention to down-select:

Option 1: UD (Operator 1) and DU (Operator 2)

Option 2: DUD (Operator 1 and 2)

Option 3: UD and DUD

Provide more description on each specific case.

- As baseline use simulation parameters defined in TR 38.858. For 6 GHz, new parameters for BS antenna should be established for SBFD.

Additionally, the initial work plan is agreed and incorporated in the approved WF R4-2515091.

The progress in RAN4 #117 meeting is summarized below:

The WF R4-2522977 for both objectives is approved.Enhancement of co-location requirements:

As overview, the outcome of Rel-19 coupling loss evaluation is captured in table below.

| Band regions | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- |
| 3.5 GHz | A-to-SA: M: 35 (R4-2507633) S: 55±15 (R4-2506307) A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB | A-to-SA: S: 57±10 (R4-2506307) A-to-A: S: 54±10 (R4-2506307) |
| 4.9 GHz | -- | A-to-SA: M: 51 (R4-2507445) M: 69 (R4-2507511) A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB |
| 7 GHz | -- | -- | A-to-SA: M: 55 (R4-2507633) S: 70±15 (R4-2506307) A-to-A: S: 77±15 (R4-2506307) |

For coming meetings focus on cases marked FFS.

Agreement:

Consider side-by-side 10 cm separation as baseline for coupling loss evaluation.

SBFD BS to SBFD BS adjacent channel co-existence:

Agreements:

- For co-existence evaluation consider following configurations:

  - UD (Operator 1) and DU (Operator 2)

  - DUD (Operator 1) and DUD (Operator 2).

- Companies are encouraged to indicate ACIR model assumption.

- For co-existence evaluation adopt 100 MHz carrier bandwidth.

Additionally, the updated work plans for both objectives are agreed and incorporated in the approved WF R4-2522977.

2.4.2	Remaining Open issues

Core requirement:

Enhancement of co-location requirements:

- The additional coupling loss evaluations  of BS-to-BS coupling loss, on top of what was done in Rel-19 NR_BS_RF_req_evo BS RF WID.

- The impact of the updated coupling-loss results on the OTA co-location requirements needs to be evaluated.

- The OTA co-location core requirements improvement with reasonable expected BS-to-BS isolation for the upper FR1 frequencies including CLRA/CLTA concept (e.g. alternative testing antenna or other feasible testing method) and/or the possibility to define requirements using the existing OTA metrics.

SBFD BS to SBFD BS adjacent channel co-existence:

- Co-existence study for SBFD BS coexisting with SBFD BS.

- The necessity, feasibility and definition of BS Tx unwanted emission and Rx in-band blocking RF requirements.

Perf requirement:

- All issues are open.

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A.

4.	References

RAN4 #116bis meeting:

R4-2513207	Discussion on SBFD BS to SBFD BS adjacent channel coexistence	CATT

R4-2513238	Discussion on enhanced BS co-location requirements	CATT

R4-2513744	Discussion on R20 SBFD general aspects		CMCC

R4-2513772	General aspects on BS RF requirement evolution		Huawei, HiSilicon

R4-2513773	On SBFD BS to SBFD BS adjacent channel coexistence		Huawei, HiSilicon

R4-2513791	On SBFD BS to SBFD BS Adjacent Channel Coexistence	Samsung

R4-2513942	On enhancement of BS co-location requirements		Ericsson

R4-2513943	Overview of Rel-20 BS RF evolution WI phase 2		Ericsson

R4-2513962	SBFD BS to SBFD BS adjacent channel coexistence	Tejas Network Limited

R4-2513969	On SBFD-SBFD co-existence	Ericsson

R4-2513994	Views on SBFD-to-SBFD coexistence	Qualcomm Germany

R4-2513998	Work plan for Rel-20 BS RF requirement evolution for FR1 and testing, phase 2	ZTE Corporation,Sanechips

R4-2513999	TR38.8xx v0.0.1 skeleton on NR; SBFD BS to SBFD BS adjacent channel co-existence	ZTE Corporation,Sanechips

R4-2514089	On general aspects and work plan of Rel-20 BS RF	Nokia

R4-2514090	On enhancement of co-location requirements	Nokia

R4-2514278	Discussion on Rel-20 Co-location requirement	ZTE Corporation, Sanechips

R4-2514279	Discussion on Rel-20 SBFD BS coexisting with SBFD BS	ZTE Corporation, Sanechips

R4-2514440	Topic summary for [116bis][317] NR_BS_RF_Ph2	Moderator (Ericsson)

R4-2514454	On SBFD BS to SBFD BS adjacent channel coexistence		Nokia

R4-2514468	Scenarios and general consideration on OTA co-location requirements		Huawei, HiSilicon

R4-2515090	TR38.8xx v0.0.1 skeleton on NR; SBFD BS to SBFD BS adjacent channel co-existence	ZTE Corporation,Sanechips

R4-2515091	Way Forward for [116bis][317] NR_BS_RF_Ph2		Ericsson

R4-2515092	Ad-hoc meeting minutes for [116bis][317] NR_BS_RF_Ph2	Ericsson

RAN4 #117 meeting:

R4-2520132	Discussion on the feasibility of adjacent channel coexistence between SBFD BS and SBFD BS	CATT

R4-2520256	Discussion on enhanced BS co-location requirements	CATT

R4-2520318	SBFD BS to SBFD BS adjacent channel coexistence	Tejas Network Limited

R4-2520756	Discussion on R20 SBFD adjacent channel coexistence	CMCC

R4-2521048	Update to workplan for NR BS RF requirement evolution WI	Ericsson

R4-2521424	On SBFD BS to SBFD BS Adjacent Channel Coexistence	Samsung

R4-2521454	On SBFD BS to SBFD BS adjacent channel coexistence		Nokia

R4-2521583	On enhancement of co-location requirements	Nokia

R4-2521697	On enhancement of BS co-location requirements		Ericsson

R4-2521698	On SBFD-SBFD co-existence	Ericsson

R4-2521701	SBFD BS to SBFD BS adjacent channel coexistence	Huawei, HiSilicon

R4-2521755	Discussion on Rel-20 Co-location requirement	ZTE Corporation, Sanechips

R4-2521756	Discussion on Rel-20 SBFD BS coexisting with SBFD BS	ZTE Corporation, Sanechips

R4-2521881	Simulation Study on Adjacent-Channel Coexistence between SBFD Systems for commercial deployment	ISSDU, NYCU

R4-2521897	Views on SBFD-to-SBFD coexistence	Qualcomm Germany

R4-2522228	Topic summary for [117][317] NR_BS_RF_Ph2	Moderator (Ericsson)

R4-2522976	Ad-hoc meeting minutes for [117][317] NR_BS_RF_Ph2	Ericsson

R4-2522977	Way Forward for [117][317] NR_BS_RF_Ph2	Ericsson

17.10.2025		minor adaptations for RAN #110

10.08.2025		minor adaptations for RAN #109

06.04.2025		minor adaptations for RAN #108

04.02.2025		minor adaptations for RAN #107

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template