3GPP TSG RAN Meeting #109                                     RP-252802

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.2

Source:	Moderator (CMCC)

Title:	Moderator's summary for RAN led 6G SI: Key performance indicators

Document for:	Report

- Introduction

This summary includes two parts:

- 3GPP internal KPIs, including:

- Energy efficiency

- Coverage

- Sensing

- AI

- Positioning

- NTN

- IMT-2030 TPRs

- In RAN#108, 3GPP TSG RAN discussed the IMT-2030 TPRs and the sent LS to ITU WP5D in [RP-251825].

- According to the workplan, RAN#109 is the last meeting for IMT-2030 TPR discussion. The aim of this meeting is to agree on the values of TPRs as much as possible and sent LS to ITU-R.

- 3GPP internal KPIs

1.1	Energy efficiency

1) Background

ITU already define energy efficiency as a quantitative TPR. Following is the definition of energy efficiency for sustainability agreed in ITU-R WP5D #49.

## 4.15 Energy Efficiency for sustainability Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving [in relation to the traffic characteristics]. The requirement is defined as the relative energy [savings/consumption] (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies. The minimum requirements for Energy efficiency are summarized in Tables below. [Editor’s note: 5D#49 meeting agreed that Network and UE Energy efficiency in unloaded case compared to fully loaded case is evaluated analytically using power models. On the evaluation of partial loaded case(s) (low load, light load and/or medium load), there are divergent views including mandatory, optional or not included, as well as the evaluation method of analysis or simulations. Proponents are encouraged to submit more details including load level (i.e. A/B/C in tables), requirement value (i.e. X/Y in tables) and the associated evaluation method, to facilitate the discussion at 5D#50 meeting.] Table: The minimum requirements for Network Energy efficiency [Test environment(s)]**; Selected load case(s)*; The reference case; Relative energy [saving/consumption] (%); [Editor notes: Mandatory, optional or not included – this is not part of the final document. N. A; empty load; Fully loaded case*; [X1%]; Mandatory. [Dense urban-IC]**; [A%]; [X2%]; TBD. [Rural-IC]**; [B%]; [X3%]; TBD. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Table: The minimum requirements for Device Energy efficiency [Test environment(s)]**; Selected load case(s)*; The reference case; Relative energy [saving/consumption] (%); [Editor notes: Mandatory, optional or not included - this is not part of the final document]. N. A; empty load; Fully loaded case; [Y1%]; Mandatory. [Dense urban-IC]**; [C%]; [Y2%]; TBD. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

RAN1 discussed energy efficiency in August meeting and reached following agreements.

Agreement in RAN1#122 Study how to reuse and update reference configurations in TR 38.864 for 6G BS. Agreement in RAN1#122 Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS. Agreement in RAN1#122 - Study metric(s) for UE energy efficiency. - Study metric(s) for BS energy efficiency. Agreement in RAN1#122 Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following: - TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations - TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models Agreement in RAN1#122 - Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s) - Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

| [RP-251960 Ericsson] | Proposal 1: Define energy efficiency as a quantitative KPI in 3GPP for both network and UE, evaluated via system-level simulations (SLS). Other evaluation methods, such as numerical, are not precluded when relevant. Proposal 2: Consider QoS-based energy saving metrics depending on the service and scenario under evaluation, instead of relying solely on throughput-based metrics. Proposal 3: Adopt NR’s relative energy saving evaluation methodology, comparing new techniques against baseline scenarios over the same duration. Proposal 4: Adopt the UE power model from TR 38.840 as the baseline, with refinements from TR 38.869 and TR 38.875 where appropriate. Proposal 5: Adopt the future-oriented Category 1 BS model from TR 38.864 as the baseline for 6G BS evaluations. Proposal 6: Study updates to BS/UE power parameters to support 7–15 GHz operation, building on the 5G baseline models. |
| --- | --- |
| [RP-251994 DOCOMO] | Proposal 2: For the evaluation of energy efficiency in 3GPP, it should be discussed how to clarify the improvement from IMT-2020 to IMT-2030. |
| [RP-252000 OPPO] | Proposal 2: For 6G Energy efficiency, use power/energy saving percentage over 5G as the baseline KPI. - Study the feasibility to define energy efficiency formulation. |
| [RP-252002 Vivo] | Proposal 2: Both unloaded case and loaded case should be considered for energy efficiency. Proposal 3: The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as data rate, and latency for control signaling/paging or data. Proposal 4: For unloaded case, two options can be considered: - Option 1: EE=VP , where P is the power consumption, v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver. - Option 2: EE is the relative power consumption with tolerable/less than [TBD] latency deterioration. Proposal 5: For loaded case, two options can be considered: - Option 1: EE=VP or E=PV , where P is the power consumption, v is the user perceived throughput (UPT). - Option 2: EE is the relative power consumption with tolerable/less than [TBD] data rate deterioration. |
| [RP-252061 SK Telecom] | Support unloaded case, both network/device gains |
| [RP-252085 CMCC] | Proposal 1: Define energy efficiency as a quantitative KPI in TR 38.914. Proposal 2: For energy efficiency, support the following definition to be captured in TR 38.914: 5.1.15 Energy efficiency Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. The requirement is the relative energy savings between the selected load case(s) and a reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. Proponents should report the power model used for evaluation (e.g. BS power model in 3GPP TR 38.864, UE power model in 3GPP TR 38.840, TR 38.869 or TR 38.865), the number of UE under each BS and the traffic model. The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant that certain percent (e.g. 90%) of UE fulfils the target should be achieved while evaluating the relative power saving for each load case. The certain metric can be selected according to the service type: - For Vo6G (i.e. VoNR-like) service, the certain metric is latency - For burst-buffer service, the certain metric is QoS or UPT - For XR service, the certain metric is latency Proposal 3: RAN plenary further discuss and give some guidance to RAN1 on the design principle of for 6G power model and energy efficiency in 6G from the following aspects: - How to handle the parallel discussion on the definition of energy efficiency between RAN and RAN1. - Design principle on power model for both 6G BS and 6G UE, including: - General structure for sleep mode, transition time and additional transition energy - Settings on specific values of relative power, transition time and additional transition energy - Scaling method of UE power model, e.g. how to jointly consider time/bandwidth/antenna/other aspects Proposal 4: RAN plenary wait for RAN1 progress on the design of 6G BS and 6G UE power model, and then discuss the specific value of NW EE and UE EE to avoid the duplicate work. |
| [RP-252129 Huawei] | Proposal 3: Energy saving should be a 6G quantitative KPI for both network and UE, and should be evaluated by system simulation methodology. Proposal 4: The power consumption model/assumptions need to be studied considering: • BS and UE power consumption model for FR1, FR2 and for frequency range between 7~24GHz. • Energy saving metric based on QoS satisfaction |
| RP-252167 Xiaomi] | Proposal 4: Energy efficiency can be evaluated in terms of power consumption per transmitted bit, which can be defined by the following formula for base station and UE respectively: EE=vE；E=i=0nEiTtotal=（Eon+Eoff+Etransition）Ttotal |
| [RP-252170 MediaTek] | Proposal 1: For 3GPP internal Energy Efficiency (EE) requirements, allow for a different metric than the metric recommended to ITU-R for IMT-2030. Proposal 2: Within the EE requirements framework, ensure that improved energy savings does not cause noticeable degradation to user experience. Proposal 3: For the 3GPP internal EE requirements framework, adopt 6G power/energy saving gain vs a baseline with same loading as the primary energy efficiency metric for BS and UE evaluations, and where the average data rate served by 6G and baseline are similar/equal. Proposal 4: For the 3GPP internal EE requirements framework, include NR features with real-world deployments as the baseline for 6G energy efficiency comparison. NR Release 17 UE/network power/energy saving features can serve as the initial reference. Proposal 5: Give time for RAN1 to provide feedback on the BS and UE power consumption models to support final requirements, taking into account considerations on practical deployment and realistic device implementation. Proposal 6: Consider the suggested ITU-R requirements framework for Sustainability in this document as the recommendation towards ITU-R. Proposal 7: Consider the 3GPP internal requirements framework for EE in this document as a starting point for further elaboration. |

2) Discussion

Regarding company views on energy efficiency,

- For definition of metric,

  - [RP-251960 Ericsson], [RP-252129 Huawei], [RP-252002 Vivo], [RP-252085 CMCC], [RP-252170 MediaTek] support to define the metric as relative energy saving with baseline when certain constraint is fulfilled (e.g. certain percent of UE fulfils the target, average data rate served by 6G and baseline are similar/equal, with tolerable/less than xx% latency/data rate deterioration, QoS-based satisfaction, etc.), which is similar as the current definition on ‘Energy Efficiency for sustainability’ in ITU-R WP5D.

  - [RP-252002 Vivo], [RP-252167 Xiaomi] support to define the metric as V/E or E/V, where V is the average rate/user perceived throughput (UPT) and E is the energy consumption.

  - [RP-251994 DOCOMO], [RP-252000 OPPO] support to define the metric as the improvement against NR or IMT-2020.

- For the power model.

  - [RP-251960 Ericsson] suggest to adopt the existing NR model as baseline.

  - [RP-251960 Ericsson], [RP-252085 CMCC], [RP-252129 Huawei] suggest to further study power model for existing frequency band (FR1, FR2) or new frequency band (7-24 GHz).

  - [RP-252170 MediaTek], [RP-252085 CMCC] suggest RAN plenary to wait for RAN1 progress, and then discuss the value of EE based on the defined power model.

Given consideration on the situation, moderator suggests to discuss the following proposals:

Proposal 1-1: Define energy efficiency as a quantitative KPI in 3GPP for both network and device.

Proposal 1-2: Definition and metrics of ‘Energy Efficiency for sustainability’ in ITU-R WP5D is used as baseline, FFS on other metrics for 3GPP internal evaluation.

Proposal 1-3: Both empty load and partial load cases should be evaluated in 6GR study.

- Evaluate analytically for empty load case

- Evaluate via system level simulations (SLS) for partial load cases

- Other evaluation methods, such as numerical, are not precluded when relevant.

Proposal 1-4: Power model is to be discussed in RAN1.

1.2	Coverage

| [RP-251988 T-Mobile USA] | ## 5.x Enhanced/similar coverage in a multiple band scenario (UL/DL decoupling) In multi-band configurations, the band on which the downlink performs the best and the band on which the uplink performs the best can be different. In multi-band configurations, the 6G specification should not restrict the allocation of bands for downlink and uplink so that the performance in any direction is worse than the best single-band performance in that direction. |
| --- | --- |
| [RP-252024 ZTE] | Proposal 1: Do not define new KPIs for coverage, i.e. re-use the definition in section 7.10 in TR 38.913. |
| [RP-252129 Huawei] | Proposal 5: Coverage shall be considered as a general requirement to support the following: - ~7GHz shall reach same coverage as existing ~4GHz C-band to enable co-site deployment - Coverage shall also support certain cell edge data rates to ensure user experience corresponding to diverse device types - Link budget evaluation methodology is used to address different deployment scenarios |
| [RP-252141 CATT] | Proposal 1: For 6G coverage, MCL (Maximum Coupling Loss) is used as the metric for coverage performance. Proposal 2: For 6G coverage, • For MBB UE, target at MCL=144 dB, with higher target data rate at cell edge compared to 5G • For IoT UE, target at MCL=149~154 dB, with lower data rate compared to MBB UE • Coverage enhancement method(s) for IoT UE should also be applicable to MBB UE |
| [RP-252169 MediaTek] | Proposal 1: RAN plenary to define targets for 6G coverage. RAN plenary can task RAN WGs to provide further input/analysis where needed to help RAN plenary make its decision. Proposal 2: Consider the following framework and tentative targets as a starting point for defining requirements for 6GR terrestrial 6GR coverage: • Target a [10] dB Maximum Coupling Loss improvement vs 5G NR Rel-16 for basic connectivity for all types of devices in sub-6GHz bands. The achievable QoS should be identified further by RAN1, with inputs also from RAN4. • Re-use of existing 5G mid-band (~3.5GHz) site grid shall be targeted for 6G deployments in around 7 GHz and comparable coverage and corresponding cell-edge user QoS to 5G mid-band should be targeted. Handheld devices should be considered as the reference in both cases, with 5G NR based on Rel-16. The exact 5G mid-band site grid assumptions to be used as a reference are TBD. |

Agreement in RAN1#122 - On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

Definition in TR38.913 7.10 Coverage MaxCL in uplink and downlink between device and Base Station site (antenna connector(s)) for a data rate of 160bps, where the data rate is observed at the egress/ingress point of the radio protocol stack in uplink and downlink. The target for coverage should be 164dB. Link budget and/or link level analysis are used as the evaluation methodology.

Regarding company views on coverage,

- For the definition on the KPI.

  - [RP-251988 T-Mobile USA] support to introduce enhanced/similar coverage in a multiple band scenario as the enhancement of coverage.

  - [RP-252141 CATT], [RP-252024 ZTE], [RP-252129 Huawei], [RP-252169 MediaTek] support to reuse the existing evaluation method (e.g. link budget, MCL).

- For the target of KPI,

  - [RP-252141 CATT], [RP-252129 Huawei], [RP-252169 MediaTek] support that ~7GHz shall at least reach same coverage as NR (e.g. MCL=144dB) for handheld/MBB device. [RP-252141 CATT] also think that higher target data rate at cell edge shall be met.

  - [RP-252141 CATT], [RP-252169 MediaTek] support that 5~10 dB extra margin shall be met to ensure service for all device types (e.g. IoT device).

Proposal 1-5: Link budget is used as the evaluation methodology for coverage

Proposal 1-6: MCL (Maximum Coupling Loss) is used as the metric for coverage performance.

Proposal 1-7: The target for coverage is FFS.

1.3	Sensing

| [RP-252002 Oppo] | Sensing-related capabilities are measured in terms of the following technical performance requirements: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. • Micro-Doppler accuracy: It is defined as the difference between the estimated doppler and the actual doppler due to the sensing object motion. The required value of micro-doppler accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the micro-doppler estimation error. This requirement is defined for the purpose of evaluation in the Integrated Sensing and Communication usage scenario. This requirement is evaluated by simulation approach. |
| --- | --- |
| [RP-252061 SK-Telecom] | SK Telecom supports the definition of 6G KPIs in alignment with ITU-R IMT-2030 TPRs, emphasizing realistic deployment feasibility and operator-centric service enablement. KPI Category; WP5D WD (5D/792 Annex 5.14) [1]; ITU-R WP5D Korea Contribution (5D/658) [2]; 3GPP TP (RP-251880) [3]; SKT ‘s View. Sensing KPIs; Defined as detectability, localization, velocity; resolution TBD; Same, resolution optional; -; Support Korea proposal. |
| [RP-252088 CMCC] | Proposal 1: KPIs on 6G sensing should consider at least the following use cases: - UAV - Automotive - Human - Environment awareness/reconstruction to assist communication Proposal 2: RAN should identify the communication services or functions which can be assisted by sensing, including as least the following： - Beam management - Channel estimation - Power saving Proposal 3: Support the metrics listed in Table 1 for 6G sensing. Metric; Definition. Miss detection probability; Miss detection probability is the probability that a true target is not associated with a detected target. From the aspect of evaluation, it is defined as Pmd=n=0N-1DnMnN Where, Dn is the number of missed targets in the drop n, i.e., the true target not associated with any detected object Mn is the number of true targets in the drop n. N is total number of drops with at least one target per drop. False alarm probability; Definition Type 1 (no target dropped in simulation area): False alarm probability is defined as the probability that an object is detected when there is no target present in simulation area is considered a false alarm. From the aspect of evaluation, it is defined as Pf1=n=0N-1QnN Where, Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0. N is the total number of drops without targets in the simulation area. Definition Type 2 (targets dropped in simulation area): False alarm probability is defined as the probability that an object is detected but not associated with any true targets in the simulation area is considered as a false alarm. From the aspect of evaluation, it is defined as Pf2=0≤n<NMn'≠0Dn'Mn'K Where, Dn' is the number of detected objects but not associated with any true targets in the drop n. Mn' is the total number of detected objects in the drop n. K is number of drops (N). Horizontal/Vertical Positioning Accuracy; Horizontal/vertical positioning accuracy is defined as the 95th percentile point of the cumulative distribution function (CDF) of the horizontal/vertical position estimation error. From the aspect of evaluation, the horizontal/vertical position estimation error is the norm of the difference between the estimated horizontal/vertical position and the corresponding true position of a sensing target.. Velocity Accuracy; Velocity accuracy is defined as the 95th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. From the aspect of evaluation, the velocity estimation error is the norm of the difference between the estimated velocity and the corresponding true velocity of a sensing target.. Sensing Capacity Note 1; Sensing capacity is defined as the maximum number of the targets per sector when sensing results of all targets in observation zone fulfil QoS requirements with 95% probability. The QoS requirements are defined by other KPIs (except sensing capacity itself) on sensing.. Max sensing service latency; Max sensing service latency is the time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface.. Refreshing rate; Refreshing rate is the rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results.. Note 1: An intuitive evaluation methodology for sensing capacity can be found in Annex A.. Proposal 4: Support to further study the metrics for sensing assisting communication in 6G. Proposal 5: Support the KPIs listed in Table 2 for 6G sensing. Use case; Positioning Accuracy; Velocity Accuracy [m/s]; Max sensing service latency [ms]; Refreshing rate [Hz]; Sensing Capacity [per sector]; Missed detection [%]; False alarm [%]. Horizontal [m]; Vertical [m]. UAV detection; 10; 10; 1; 1000; 1-10; 100; 1; 1. UAV tracking; 1; 1; 1; 100-1000; 1-10; 100; 0.1-1; 1. Automotive detection and tracking; 1; N/A; 1; 1000-5000; 10; 100; 1; 3. Human detection; 1; N/A; N/A; 1000 -5000; 10; 100; 5; 2. Note 1: Metrics for tracking can be different with detection. And metrics for tracking should be evaluated independently.. |
| [RP-252129 Huawei, HiSilicon] | Proposal 6: Reconstruction accuracy shall be defined as a 6G quantitative KPI for sensing, and the target value should be [submeter level – meter level]. Proposal 7: Horizontal localization accuracy is 1m and velocity accuracy is 1m/s shall be defined as a 6G quantitative KPI for static or moving target up to 5km/h detection use case. |
| [RP-252167 Xiaomi] | According to the TPR discussions, the following performance metrics can be considered for sensing. - Missed detection probability and False alarm probability Missed detection probability is defined as the probability of not detecting the presence of a target object when the target object is present, and False alarm probability is defined as the probability of falsely detecting the presence of a target object when the target object is not present. - Position accuracy The accuracy on position is defined as the difference of the sensed position of the target object to its true position value. Sensing accuracy on position can be further derived into a horizontal accuracy and a vertical accuracy. - Velocity accuracy The accuracy on velocity is defined as the difference of the sensed velocity of the target object to its true velocity value. Sensing accuracy on velocity may be further derived into a horizontal accuracy and a vertical accuracy too. - Sensing resolution Sensing resolution can be defined as the minimum difference in the measured magnitude of target objects to be allowed to detect objects in different magnitude, e.g., range resolution (m), velocity resolution (m/s). KPI value set 1. Position accuracy; Horizontal: [2]m Vertical: [5]m. Velocity accuracy; Horizontal: [1]m/s Vertical: [1]m/s. Resolution; Range: [1]m Velocity: [1]m/s. Missed detection probability / False alarm probability; [5]% / [5]%. |

Following is the definition of Sensing-related capabilities agreed in ITU-R WP5D #49.

## 4.13 Sensing-related capabilities Sensing-related capabilities are measured in terms of the following technical performance requirements: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error. • [Sensing Resolution: TBD]

Regarding company views on sensing KPIs for 3GPP,

- For the KPI metrics, almost all companies support the following:

  - Detection Probability (OR Missed Detection Probability) and False Alarm Probability

  - Positioning Accuracy (OR Horizontal/Vertical Positioning Accuracy)

  - Velocity Accuracy

  - [Resolution]

- For the KPI metrics additional to ITU,

  - [RP-252002 Oppo] suggests micro-Doppler accuracy.

  - [RP-252088 CMCC] suggests sensing capacity, max sensing service latency and refreshing rate.

  - [RP-252129 Huawei, HiSilicon] suggests reconstruction accuracy.

  - [RP-252167 Xiaomi] suggests sensing resolution.

- For the definitions of KPI metrics related to ITU document,

  - [RP-252002 Oppo] suggests definitions same as those of the latest ITU working document.

  - [RP-252088 CMCC] suggests definitions similar to RAN1 agreement for NR ISAC evaluation.

Given consideration of the situation, moderator suggests discussing the following proposals:

Proposal 1-8: Following metrics of Sensing in ITU-R WP5D is used as baseline

  - Missed Detection Probability and False Alarm Probability

  - Horizontal/Vertical Localization Accuracy

  - Velocity Accuracy

  - [Sensing Resolution]

Proposal 1-9: Further discuss the following metrics for 3GPP internal sensing KPIs, also take the conclusion of sensing use cases into consideration

  - Micro-Doppler accuracy

  - sensing capacity

  - sensing service latency

  - refreshing rate

  - reconstruction accuracy

  - Others are not precluded

1.4	AI-related capabilities

| [RP-252002 Vivo] | 5.1.18 AI Applicable AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These capabilities include: - Data collection - Data/Model exchange - AI model training - AI model inference - AI model monitoring - AI computing resource management This requirement is defined for the purpose of evaluation in the Artificial Intelligence and Communication usage scenario. This requirement is evaluated by inspection approach. |
| --- | --- |
| [RP-252061 SK Telecom] | For AI-related KPIs, only qualitative definitions are appropriate. Support for distributed learning, inference, and AI-native RAN functionalities should be ensured, but no quantitative targets should be defined, as performance heavily depends on implementation and hardware. |
| [RP-252129 Huawei] | Proposal 8: 6G RAN shall support AI capabilities to enhance communication performance and user experience. Proposal 9: 6G RAN shall support efficient transmission for traffics relevant for AI applications. Proposal 10: 6G RAN shall enable efficient utilization of data produced by sensing and AI capabilities to support new services. |

Following is the definition of AI-related capabilities agreed in ITU-R WP5D #49.

## 4.14 AI-related capabilities AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications, including. • RIT/SRIT facilitating AI applications i.e. “Network for AI” • AI to improve RIT/SRIT performance, i.e. “AI for network” The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling/ related to AI functionalities. This should include support for one or more of the following: – Data collection – Distributed processing – Distributed learning – AI model training – AI computing – AI model execution – AI model inference – or other AI capabilities of the RITs/SRITs (to be reported by the proponent) Option A RIT/SRIT should have the capability to enable and disable AI functionalities within the network (AI for network scenario only) Option B The proponent should report about mechanisms, e.g, related to enabling or disabling the supported AI functionalities, where applicable.

Regarding company views on AI-related capabilities,

- For description of KPI,

  - [RP-252002 Vivo] support to introduce Data/Model exchange, AI model monitoring, AI computing resource management as the capabilities on top of the definition in ITU-R.

  - [RP-252129 Huawei] support to add that 6G RAN shall support AI capabilities to enhance communication performance and user experience, efficient transmission for traffics relevant for AI applications and enable efficient utilization of data produced by sensing and AI capabilities to support new services in the definition.

Given consideration on the situation, moderator suggests to discuss the following proposals:

Proposal 1-10: Define AI as qualitative KPI in 3GPP. Definition of “AI-related capabilities” in ITU-R is used as starting point for 3GPP KPIs

Proposal 1-11: Further discuss whether to introduce following qualitative KPIs in 3GPP:

  - Data/Model exchange

  - AI model monitoring

  - AI computing resource management

  - AI capabilities to enhance communication performance and user experience

  - Efficient transmission for traffics relevant for AI applications

  - Efficient utilization of data to support new services

1.5	NTN

| [RP-251936 THALES] | Text proposal for TR 38.914 for NTN performance requirements |
| --- | --- |
| [RP-251945 Ericsson] | Proposal 1 The definition of the TPRs for satellite IMT-2030 should be based on those for the terrestrial component of IMT-2030, with the values adapted to satellite conditions, following the example from IMT-2020. Proposal 2 TPRs inherited from IMT-2020 with unchanged evaluation methodologies should also become TPRs for satellite IMT-2030. Proposal 3 The reliability TPR for satellite IMT-2030 should consist of the success probability of a data transmission without a simultaneous requirement on transmission time, as in the IMT-2020 satellite component. Proposal 4 Energy efficiency should be part of the satellite IMT-2030 TPRs, and should be evaluated analytically, following the model of the terrestrial requirement. Details are FFS. Proposal 5 Sensing-related capabilities and the “Joint requirement” should not become TPRs for the IMT-2030 satellite component. Proposal 6 The inclusion of Positioning as a TPR for the IMT-2030 satellite component needs further discussion. Proposal 7 We support the inclusion of Resilience, Interoperability, and AI-related capabilities as TPRs for satellite IMT-2030, to be evaluated by inspection. The details are FFS. |

Need to discuss first how to treat the TPRs for satellite in general and interaction with ITU-R WP4B.

1.6 Positioning

| [RP-252002 Vivo] | 5.1.14 Positioning Positioning is the ability to calculate the approximate position of connected devices. Positioning accuracy describes the closeness of the measured position of the device to its true position value. The accuracy can describe the accuracy either of an absolute position or of a relative position. It can be further broken into a horizontal position accuracy – referring to the position error in a 2D reference or horizontal plane, and into a vertical position accuracy – referring to the position error on the vertical axis or altitude. This requirement is defined for the purpose of evaluation in the Immersive Communication and Integrated Sensing and Communication usage scenarios. This requirement is evaluated by simulation approach. The preliminary requirements could be [10cm]. |
| --- | --- |
| [RP-252024 ZTE] | Proposal 6: Define both horizontal and vertical positioning accuracy. - Horizontal positioning accuracy: 20cm-1m (indoor), 3m (outdoor) for 90% of UEs - Vertical positioning accuracy: 5m for 90% of UEs |
| [RP-252061 SK Telecom] | For positioning, ambitious targets are desirable, such as 1 m horizontal / 3 m vertical accuracy for indoor, and 3 m / 3 m for outdoor. However, it should be recognized that such accuracies may not be achievable across all scenarios. Operators see this KPI as important for UAV, IoT, and location-based monetization, but deployment feasibility must be considered. |

Regarding company views on positioning,

- For the target value of KPI,

  - [RP-252002 Vivo] support that the preliminary requirements could be 10cm.

  - [RP-252024 ZTE] support the horizontal positioning accuracy is 20cm-1m (indoor) and 3m (outdoor) for 90% of UEs, and the vertical positioning accuracy is 5m for 90% of UEs.

  - [RP-252061 SK Telecom] support 1 m horizontal / 3 m vertical accuracy for indoor, and 3 m / 3 m for outdoor.

Given consideration on the situation, moderator suggests to first discuss the proposals for positioning in IMT-2030 TPR, and then look back to see whether the definition in 3GPP KPI need to be further modified.

1.7 Others

Discuss following KPIs if time allows.

| [RP-251988 T-Mobile USA] | ## 5.x Enhanced/similar coverage in a multiple band scenario (UL/DL decoupling) In multi-band configurations, the band on which the downlink performs the best and the band on which the uplink performs the best can be different. In multi-band configurations, the 6G specification should not restrict the allocation of bands for downlink and uplink so that the performance in any direction is worse than the best single-band performance in that direction. ## 5.x Reduced scheduling latency For reasons of reducing delay and improving coverage, it is important that when data is available for transmission, that data can be transmitted with a minimum of delay and interruptions. Delays and interruptions due to other flows or users are scheduled, or for reasons of energy efficiency must be accepted, but delays and interruptions, e.g. due to control signaling, should be minimized. ## 5.x Reuse of existing site grids It is important that 6G can be introduced and perform well using existing site grids, without the need for densification. This requirement can be divided into two parts: - When 6G is introduced in existing bands, using existing sites and antennas, the coverage of 6G should be at least as good as that of 5G. - When 6G is introduced on new frequency bands, higher than those used for 5G on the site grid in question, larger antennas can be used to compensate for the increased loss (between isotropic antennas), 6G should support the use of antenna sizes required to make higher frequencies useful on exiting site grids. Larger here refers to the number of antenna elements. Useful does not necessarily mean full coverage or coverage equal to that of lower bands, but that a significant fraction of users is covered, directly benefiting those users and providing offloading gains when deployed together with lower band frequency layers. The former requirement was also used when 5G was introduced (with 4G as a reference), with the name ‘extreme coverage’. For 6G the similar requirement is referred to as ‘coupling loss’. ### 5.x Coupling loss The coupling loss is defined as the total long-term channel loss over the link between the UE antenna ports and the eNode B antenna ports, and includes in practice antenna gains, path loss, shadowing, body loss, etc. The maximum coupling loss (MaxCL) is the limit value of the coupling loss at which the service can be delivered and therefore defines the coverage of the service. MaxCL is independent of the carrier frequency. It is defined in the UL and DL as: - UL MaxCL = UL Max Tx power - eNB Sensitivity - DL MaxCL = DL Max Tx power - UE Sensitivity The MaxCL is evaluated via link budget analysis (supported by link level simulations). The proposed MaxCL calculation template is given in following table 7.10.1-1: Table 7.10.1-1: MaxCL calculation template Item; Value. Transmitter. (1) Tx power (dBm). Receiver. (2) Thermal noise density (dBm/Hz). (3) Receiver noise figure (dB). (4) Interference margin (dB). (5) Occupied channel bandwidth (Hz). (6) Effective noise power = (2) + (3) + (4) + 10 log(5) (dBm). (7) Required SINR (dB). (8) Receiver sensitivity = (6) + (7) (dBm). (9) MaxCL = (1) - (8) (dB). The following assumptions are used. Table 7.10.1-2: MaxCL calculation assumptions UE Tx power; [23dBm]. DL Tx power; 46dBm. eNB receiver noise figure; 5dB. UE receiver noise figure; 9dB. Interference margin; 0dB. |
| --- | --- |

- IMT-2030 TPRs

2.1 Legacy TPRs

| Proposed TPR items | Agreed values in RAN#108 (RP-251825) | Moderator Proposal |
| --- | --- | --- |
| - Peak Data Rate | TBD | Background - For the peak data rate, the following two alternatives were provided during the CC#1: - Alternative 1: Not to define a requirement, e.g. leave the value for peak data rate for the technology proponent to provide - Alternative 2: Define a requirement with reasonable value Proposal 2-1: - Alternative 1: Not to define a requirement, e.g. leave the value for peak data rate for the technology proponent to provide - Alternative 2: [3-5]x IMT-2020 |
| - Peak Spectral Efficiency | TBD on the values | Background: - Agreement in CC#1: [1.5-2.5x] of IMT-2020 Proposal 2-2: - [2]x of IMT-2020 |
| - 5th percentile user data rate (Mbit/s) (Name changed from User Experienced Data Rate) | TBD on the values 3GPP RAN agreed to use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. Due to the different evaluation methodology, a new title can be considered to avoid the confusion with previous IMT-2020 values. | Proposal 2-3: - DL: [3-5]x of IMT-2020 - UL: same as IMT-2020, i.e. 50Mbps |
| - 5th percentile user Spectral Efficiency | Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx Form factor and other operational considerations may limit the number of receiver antennas of UE | Proposal 2-4: DL: - Rural-IC: [3]x of IMT-2020 for DL UL: Indoor hotspot-IC/Dense urban-IC/Rural-IC: - [3]x of IMT-2020 |
| - Average Spectral Efficiency | Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx Form factor and other operational considerations may limit the number of receiver antennas of UE | Proposal 2-5: DL: - Rural-IC: [3]x of IMT-2020 for DL UL: Indoor hotspot-IC/Dense urban-IC/Rural-IC: [3]x of IMT-2020 |
| - Energy Efficiency for sustainability (name changed from sustainability/energy efficiency) | TBD | To be discussed in Proposal 2 |
| - Area Traffic Capacity | TBD on values | Proposal 2-6: - [3-5]x of IMT-2020 |
| - User Plane Latency | IC: 4ms HRLLC: 1ms | N/A |
| - Control Plane Latency | 20ms | N/A |
| - Connection Density | 10⁶ (devices/km2) for massive communication | N/A |
| - Reliability | 1-10-5 with same IMT-2020 assumptions | N/A |
| 12. Mobility | Indoor Hotspot-IC: 2.25 (bit/s/Hz) @10 km/h Dense Urban- IC: 1.68 (bit/s/Hz) @30 km/h Rural-IC: [0.8~1.2] (bit/s/Hz)@120 km/h [0.45~0.675] (bit/s/Hz) @500 km/h NOTE1: Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. NOTE2: For Rural-IC, TBD on whether to include other velocities, e.g. 30km/h, 60km/h | Proposal 2-7: - Rural-IC: [1.5]x of IMT-2020, i.e. - [1.2] (bit/s/Hz)@120 km/h - [0.675] (bit/s/Hz) @500 km/h |
| - Mobility Interruption Time | TBD | Proposal 2-8: - Same as IMT-2020, i.e. 0ms |
| - Bandwidth | 400MHz (same definition as IMT-2020) | N/A |
| - Positioning | Quantitative TPR, TBD on the values | To be discussed in proposal 3 |
| - Sensing-related capabilities | TBD | To be discussed in proposal 4 |
| - AI-related capabilities | Qualitative TPR | To be discussed in proposal 5 |
| - Composite requirement | TPR related to data rate, latency, reliability, capacity etc. Applicable usage scenario: immersive communication | To be discussed in proposal 6 |

2.2 Energy efficiency for sustainability

RAN#107 agreement: - Proposal 4: Define energy efficiency as a TPR.

Following is the definition of energy efficiency for sustainability agreed in ITU-R WP5D #49.

## 4.15 Energy Efficiency for sustainability Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving [in relation to the traffic characteristics]. The requirement is defined as the relative energy [savings/consumption] (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies. The minimum requirements for Energy efficiency are summarized in Tables below. [Editor’s note: 5D#49 meeting agreed that Network and UE Energy efficiency in unloaded case compared to fully loaded case is evaluated analytically using power models. On the evaluation of partial loaded case(s) (low load, light load and/or medium load), there are divergent views including mandatory, optional or not included, as well as the evaluation method of analysis or simulations. Proponents are encouraged to submit more details including load level (i.e. A/B/C in tables), requirement value (i.e. X/Y in tables) and the associated evaluation method, to facilitate the discussion at 5D#50 meeting.] Table: The minimum requirements for Network Energy efficiency [Test environment(s)]**; Selected load case(s)*; The reference case; Relative energy [saving/consumption] (%); [Editor notes: Mandatory, optional or not included – this is not part of the final document. N. A; empty load; Fully loaded case*; [X1%]; Mandatory. [Dense urban-IC]**; [A%]; [X2%]; TBD. [Rural-IC]**; [B%]; [X3%]; TBD. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Table: The minimum requirements for Device Energy efficiency [Test environment(s)]**; Selected load case(s)*; The reference case; Relative energy [saving/consumption] (%); [Editor notes: Mandatory, optional or not included - this is not part of the final document]. N. A; empty load; Fully loaded case; [Y1%]; Mandatory. [Dense urban-IC]**; [C%]; [Y2%]; TBD. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

## Proposal 2-9:

- Network and UE Energy efficiency in unloaded case compared to fully loaded case is evaluated analytically using power models.

- Network and UE Energy efficiency in partial loaded case(s) are evaluated via simulation

  - FFS on the load cases for evaluation, e.g. low load (0%-15%) , light load (15%-30%) and/or medium load (30%-50%)

- The requirements of energy efficiency are based on proponent report.

2.3 Positioning

RAN#107 agreement: - Proposal 1: Define positioning accuracy as a quantitative TPR

Following is the definition of Positioning agreed in ITU-R WP5D #49.

## 4.11 Positioning Positioning is the ability to estimate the position of connected devices. The positioning accuracy is the [90%] point of the CDF of the device positioning error for [both] horizontal [and vertical] direction[s]. Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). [Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude)]. This requirement is defined for the purpose of evaluation in the [Immersive Communication, and Integrated Sensing and Communication usage scenario[s]]. The minimum requirements for the device positioning accuracy for various test environments are summarized in Table 9. Table 9 Positioning accuracy Test Environment; Horizontal Accuracy; Vertical Accuracy. Indoor Factory - ISAC. [Urban Macro – ISAC / Urban – ISAC].

## Proposal 2-10:

- Define requirements for both outdoor and indoor scenarios

- Usage scenario: ISAC

- Target value

  - Outdoor: meter level requirements, [3-5]m @ 90% UE for horizontal, [1-3]m @ 90% UE for vertical (if defined)

  - Indoor: sub-meter level requirements, [20-50]cm @90% UE

2.4 Sensing-related capabilities

RAN#107 agreement: - Proposal 2: Define Sensing-related capabilities as a TPR.

Following is the definition of Sensing-related capabilities agreed in ITU-R WP5D #49.

## 4.13 Sensing-related capabilities Sensing-related capabilities are measured in terms of the following technical performance requirements: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error. • [Sensing Resolution: TBD]

## Proposal 2-11: Define following TPRs and target values for sensing-related capabilities:

  - Detection Probability and False Alarm Probability: [90% or 95%] for detection probability, [10% or 5%] for false alarm probability

  - Horizontal/Vertical Localization Accuracy:

    - Horizontal: [1-5]m

    - Vertical: [1-5]m

  - Velocity Accuracy: [1m/s -5m/s]

  - [Sensing Resolution: TBD]

2.5 AI-related capabilities

RAN#107 agreement: - Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection.

Following is the definition of AI-related capabilities discussed in ITU-R WP5D #49.

## 4.14 AI-related capabilities AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications, including. • RIT/SRIT facilitating AI applications i.e. “Network for AI” • AI to improve RIT/SRIT performance, i.e. “AI for network” The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling/ related to AI functionalities. This should include support for one or more of the following: – Data collection – Distributed processing – Distributed learning – AI model training – AI computing – AI model execution – AI model inference – or other AI capabilities of the RITs/SRITs (to be reported by the proponent) Option A RIT/SRIT should have the capability to enable and disable AI functionalities within the network (AI for network scenario only) Option B The proponent should report about mechanisms, e.g, related to enabling or disabling the supported AI functionalities, where applicable.

Moderator note: No need to discuss the qualitative TPR as input to ITU, AI-related capabilities can be discussed in 6G SI part2.

2.6 Composite requirement

RAN#107 agreement: - Proposal 7: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR. - Applicable usage scenario: immersive communication RAN#108 agreement: - TPR related to data rate, latency, reliability, capacity etc. - Applicable usage scenario: immersive communication

Following is the definition of Composite requirement captured in ITU-R WP5D #49.

## [4.9 Composite requirement Editor’s note: there is concern expressed by a sector member and administration on this TPR and that it should be part of the submission template and will depend on the deployment scenarios. The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per [km2 or TRxP]. This requirement is defined as the number of users per [km2 or TRxP] of which at least [90%/95%] are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface. This requirement is defined for the purpose of evaluation in the Immersive Communication [and Hyper-Reliable Low Latency Communication] usage scenario. # The minimum performance requirement of composite requirement is summarized in Table X. TABLE X Composite requirement Test environment; Data rate (Mbits/s); Latency (ms); Packet success probability; Number of users ([/km2 or /TRxP]). Dense Urban - Immersive Communication; [DL = 30 Mbps/UL=10 Mbps]; [DL=10 ms / UL=30 ms / In total 40 ms]; [99%]; TBD. [Indoor Factory – HRLLC]. Others if any, are FFS. ]

## Proposal 2-12:

- The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP.

- This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency.

- Define the composite requirement as following

| Test environment | Data rate (Mbits/s) | Latency (ms) | Packet success probability | Number of users (TRxP) |
| --- | --- | --- | --- | --- |
| Dense Urban - Immersive Communication | [DL = 30 Mbps/UL=10 Mbps] | [DL=10 ms / UL=30 ms / In total 40 ms] | [99%] | [5-10] |

2.7 Others

Agreement in RAN#108 - Do not define security and interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030. - Sustainability is covered by energy efficiency, do not define separate TPR for sustainability. - For resilience, whether to define a qualitative TPR or to use the submission template for IMT-2030 is TBD.

2.7.1 Resilience

Following is the definition of Resilience captured in ITU-R WP5D #49.

## 4.17 Resilience (Editor’s note: This may also include Extended Connectivity) Resilience refers to capabilities of the networks and systems to continue operating correctly during and after a natural or man-made disturbance, such as the loss of primary source of power, etc. The RIT/SRIT should support functionalities over the radio interface that enable continuous operation or rapid temporary restoration during and after disturbance to radio infrastructure. These functionalities may include, but are not limited to, support for connecting to one or more of the following network infrastructures within the terrestrial component of IMT, which are temporarily operated to enhance connectivity during these situations: ‒ HIBS ‒ [Advanced] relays and repeaters ‒ Base station onboard a vehicle, airship and/or vessel ‒ or other mechanisms for resilience using network infrastructures within the terrestrial component of IMT

## Proposal 2-13: For Resilience, discuss following:

- whether to merge Resilience TPR with Extended Connectivity, creating a single qualitative TPR: “Resilience and Extended Connectivity”

- whether to adopt following changes:

‒ HIBS ‒ [Advanced] relays and repeaters ‒ Base station onboard a vehicle, airship and/or vessel Shared network infrastructures - ‒ or other mechanisms for resilience using network infrastructures within the terrestrial component of IMT

2.7.2  Coverage

Following is the definition of Coverage captured in ITU-R WP5D #49.

## [4.10 Coverage Coverage refers to the ability to provide access to communication services for users in a desired service area. In the context of this capability of the RIT, coverage is defined as the cell edge distance of a single cell through link budget analysis. This requirement is defined for the purpose of evaluation in the Ubiquitous Connectivity Usage Scenario. This requirement is defined for the purpose of evaluation in the Rural-UC test environment. Test Environment; Cell Edge1 Distance. Rural-UC; [3-7] km for a single cell. 1 Depends on UE height (to be specified in the corresponding evaluation configuration). Lower cell edge distance corresponds to a typical UE at a height of 1.5 m and higher cell edge distance corresponds to an outdoor UE at a height of 5m.. This capability does not require direct evaluation and could be described and reported by (S)RIT proponents as part of the technical characteristics template(s). ]

## Proposal 2-14: For coverage, further discuss the following options:

- Option 1: define coverage as a quantitative TPR

- Option 2: do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020)

- Conclusion

TBA

# Reference

- RP-251936	Text proposal for TR 38.914 for NTN performance requirements	THALES

- RP-251952	Technical Performance Requirements (TPR) for IMT-2030 related to WP5D		Ericsson

- RP-252533	Samsung Views on IMT-2030 TPRs	Samsung

- RP-251994	Views on 6G key performance indicators	NTT DOCOMO, INC.

- RP-252000	Discussion on 6G Key performance indicators	Guangdong OPPO Mobile Telecom.

- RP-252002	Discussion on 6G Key performance indicators	vivo

- RP-252014	Proposals on Technical Performance Requirements (TPRs) of IMT-2030	SoftBank Corp.

- RP-252041 (REVISION FROM RP-252023)	Views on IMT-2030 TPRs	ZTE Corporation, T-Mobile USA, SoftBank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Charter Communications, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University,nghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH

- RP-252024	Views on RAN KPIs for IMT-2030	ZTE Corporation, Sanechips

- RP-252025	Text proposal to draft TR 38.914 on KPIs	ZTE Corporation, Sanechips

- RP-252086	Discussion on IMT-2030 TPRs	CMCC

- RP-252087	Text proposal for TR 38.914 on KPIs	CMCC

- RP-252122	Continued operator’s view on realistic TPR definition for IMT-2030 / “6G”	Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telenor, SK Telecom

- RP-252128	Consideration on TPRs relating to ITU-R IMT-2030	Huawei, HiSilicon

- RP-252143	Views on IMT-2030 TPR	CATT

- RP-252167	Discussion on 6G Key performance indicators	Xiaomi

- RP-252172	IMT2030 TPRs and revisions to TR38.914		MediaTek Inc.

- RP-252181	IMT-2030 TPRs – Nokia Views	 Nokia

- RP-252214	Views on 6G Key Performance Indicators	Indian Institute of Tech (M), Tejas Networks, CEWiT

- RP-252215	Views on IMT-2030 TPR targets	Qualcomm

- RP-252317	On IMT-2030 Technical Performance Requirements	 Apple Inc.

- RP-251945	Technical Performance Requirements (TPR) for the IMT-2030 Satellite Component	Ericsson

- RP-251960	Network and UE energy efficiency beyond ITU-R	Ericsson Canada Inc.

- RP-251988	Internal 3GPP Requirements on Coverage and Efficiency 	T-Mobile USA Inc. Ericsson, Nokia

- RP-252061	Discussions on 6G KPI	  SK Telecom

- RP-252085	Discussion on 3GPP energy efficiency requirement	CMCC

- RP-252088	Discussion on 3GPP KPIs on sensing	CMCC

- RP-252129	Consideration on 3GPP requirements and KPIs in addition to ITU-R IMT-2030 TPRs	Huawei, HiSilicon

- RP-252135	Consideration on spectrum requirements	Huawei, HiSilicon

- RP-252141	Views on KPI of 6G coverage	CATT

- RP-252169	Coverage Requirements	MediaTek Inc.

- RP-252170	Energy Efficiency Requirements	MediaTek Inc.

Late Tdocs

- RP-252151	Discussion on 6G key performance indicators	Spreadtrum, UNISOC

- RP-252341	Key Performance Indicators for 6G	Google

# Annex 1: Summary of proposed TPR values

| ITU IMT-2030 TPR |  | IMT-2020 Value | Proposed TPR values | Assumptions/Comments |
| --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | - MTK: [1.5x] IMT-2020 - Nokia: NOT to define a requirement: - AT&T: Alt. 1 or Alt. 2 with [1x] of IMT-2020 - Apple: NOT to define a requirement or [1x] of IMT-2020 - Verizon: Define maximum achievable (not “theoretical”) values - OPPO, Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: Same as IMT-2020 - SKT: Conservative uplift, demo metric only - NTT DOCOMO, Samsung: [1.5x] IMT-2020 - KT: [1.5x] IMT-2020 for non-mmWave, [1x] is acceptable for mmWave - IIT (Madras), CEWiT, Tejas Networks: 22 Gbps for an aggregated bandwidth of 400 MHz and 45 Gbps for an aggregated bandwidth of 800 MHz. The proponent shall report the bandwidth used in the calculation. - LGE: [1.5-2x] IMT-2020 - Ericsson: [2.5x] IMT-2020 - CATT: [1.25x] or [2.5x] IMT-2020, “Reported by proponent” is also acceptable - Xiaomi: [1.5-2x] IMT-2020 for DL, [2x] for UL - CMCC: [3-5x] IMT-2020 - Vivo: [4x] IMT-2020 - Qualcomm, Spreadtrum, UNISOC, ZTE, T-Mobile USA, Softbank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Sanechips, Charter, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH, CHTTL, China Telecom (a reasonable value is needed): [5x] IMT-2020 - Huawei: [5-10x] IMT-2020 | - Samsung: It is preferred not to overemphasize this item considering practicality and the absence of commercial use cases requiring high peak data rate. - OPPO: Peak data rate is not a requirement from market, but only a theoretical calculation. Peak data rate in NR has been 200x of user experienced data rate. - Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: 200 MHz bandwidth (~7 GHz) - MTK: 7GHz. DL: 8Rx + 1024QAM, UL: 4Tx + 256QAM - NTT DOCOMO: [1.5x] is achieved assuming 1024QAM / 256QAM, 8 layers for DL / UL and bandwidth of 500 MHz. - Qualcomm: DL/UL Layers: Up to 16/8; Mod: Up to 1024QAM - China Telecom: The peak spectral efficiency has consensus with 1.5-2.5 times higher than IMT-2020. Based on the RAN#107 agreement, the target value of bandwidth is 400MHz |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | Agreement in CC#1: [1.5-2.5x] of IMT-2020 - SKT: Support modest gain, [1.5x] IMT-2020 - Samsung: [1.5x] IMT-2020 - KT, MTK, Nokia: [1.5x] IMT-2020 - Apple: [1.5x] IMT-2020 - LGE, Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: [1.5-2x] IMT-2020 - China Telecom: [1.5-2.5x] IMT-2020 - IIT (Madras), CEWiT, Tejas Networks: DL: 55; UL: 20 - Ericsson: [1.7x] IMT-2020, i.e., DL: 50, UL: 25 - CMCC, NTT DOCOMO, Vivo: [2x] IMT-2020 - Xiaomi: [1.5-2x] IMT-2020 for DL, [2x] for UL - OPPO: [2x] IMT-2020 for DL, [1.5x] IMT-2020 for UL - Huawei: [2-3x] IMT-2020 - Qualcomm, ZTE, T-Mobile USA, Softbank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Sanechips, Charter, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH, CHTTL: [2.5x] IMT-2020 - CATT: UL: [3x] IMT-2020; DL: Without layer increase: [1.875-2x] IMT-2020; With layer increase: 2x-3.75x of IMT-2020 TPR, depending on consensus to UE antenna - AT&T: [1.5x] IMT-2020 | - OPPO: From the requirement perspective, the Peak spectral efficiency should not be theoretically calculated based on number of MIMO layer and modulation order. - SKT: 1024-QAM, 8/4 layers - MTK: - 7GHz with practical Bandwidths, not 5MHz. - DL: 8Rx + 1024QAM, UL: 4Tx + 256QAM - Samsung: Assuming 1024 QAM (~25% over 256 QAM), 8/4 layers for DL/UL, overhead reduction compared to IMT 2020 (~30% to 20%). - Nokia: [1.5x] can be achieved in DL with 256-QAM and 8-layer MIMO, and in the UL with 256-QAM and 4-layer MIMO. - Apple: For 700 MHz, 4 GHz and 7 GHz the number of UE antenna elements is up to 8 Tx /Rx. - Vivo: assume an antenna configuration to enable eight spatial layers (streams) in both downlink and uplink. Higher modulation (e.g. 1024QAM, 4096 QAM) for downlink is possible. - NTT DOCOMO: Assuming 1024QAM / 256QAM and 8 layers for DL / UL - Qualcomm: DL/UL Layers: Up to 16/8; Mod: Up to 1024QAM - ZTE: NR can already achieve 1.6x with 256 QAM and 8 MIMO layers. 1024 QAM + 16 layers = 4x - Huawei: DL 8 layers/1024QAM, UL 8 layer/256QAM, and overhead optimization - Xiaomi: 1024QAM/256QAM for DL/UL, 8 layers for DL/UL |
| 5th percentile user data rate (Mbit/s) |  | DL: 100 UL: 50 | - Ericsson: - DL: 100Mbps - UL: 30Mbps - A lower requirement for e.g. the Rural test environment also can be considered - SKT: Support modest gain, DL: [4x] IMT-2020; UL: TBD - CMCC: [3x] IMT-2020 (DL), [1x] IMT-2020 (UL) - NTT DOCOMO: DL: [2.5x] IMT-2020; UL: TBD - KT, OPPO: [2x] IMT-2020 - MTK: [3x] IMT-2020 for DL; [2x] for UL in up to 40MHz - CATT: [2-3x] IMT-2020 (DL) - IIT (Madras), CEWiT, Tejas Networks: - For Dense Urban-IC: DL: [1.5x] IMT-2020; UL: [1.5-2x] IMT-2020 - For Rural-UC: DL: 1-2 Mbit/s, UL: 1-2 Mbit/s - LGE, China Telecom: [3x] IMT-2020 - Xiaomi: TBD - Spreadtrum, UNISOC: [3x] IMT-2020 (DL) - vivo : [4-6] IMT-2020 (DL) - ZTE, T-Mobile USA, Softbank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Sanechips, Charter, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH, CHTTL: [5x] IMT-2020 (DL) - ZTE: [1x] IMT-2020 (UL) - Huawei: DL: [5-10x] IMT-2020; UL: [1x] IMT-2020 - Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: - DL: [0.5-5x] IMT-2020 - UL: [0.04-4x] IMT-2020 - Apple: - DL: [3x] IMT-2020 - UL: [4x] IMT-2020 - Qualcomm: - DL: [5x] IMT-2020 - UL: TBD - Samsung: - DL: [4x] IMT-2020 - UL: TBD. Set target values after discussion on UL evaluation configurations. - propose ‘5 th percentile user data rate’ as a new title to distinguish it from IMT 2020 user experienced data rate | - Samsung: DL: Enhanced average spectral efficiency and/or wider bandwidth - MTK: For DU: 7GHz with DL 256Tx/6Rx, UL 4Tx/256Rx (assuming no BW increase) - Vivo: - DL: Analytical in case of single layer. Simulation in case of multiple layers; UL: Simulation - 7GHz, BS: 4x/8x/12x of 128 elements, UE: 4RX and 2/4TX - Ericsson: We are open to to reduce the number of simultaneously active users from 10 to a lower number, e.g. 2. - NTT DOCOMO: 2.5x for DL is calculated based on the target value of 5th percentile user spectral efficiency (i.e., 3x of IMT-2020) assuming bandwidth of 400 MHz - Qualcomm: For Dense Urban. See Antenna assumptions below Evaluation Freq; UE (Tx/Rx); BS (Tx/Rx). 700 MHz; Up to 4/4; Up to 64/64. 4 GHz; Up to 8/8; Up to 256/256. 7HGz; Up to 8/16; Up to 1024/1024. 30 GHz; Up to 32/32; Up to 2048/2048. - Xiaomi: 7GHz carrier frequency is assumed for 6G system. At each TRxP, each antenna port corresponds with 4 antenna elements in macro layer of dense urban and 1 antenna element in indoor hotspot, respectively; At each UE, 4 antenna elements driven by 4 TXRUs are assumed. 1024 antenna elements in macro layer of dense urban or 32 antenna elements in indoor hotspot. - Nokia: Details of the eval methodology should be clarified before setting the requirements. - Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: Note 1: Maximum available spectrum bandwidths in different bands: 2x20 MHz @ <1 GHz, 2x40 MHz @ ~2 GHz, 100 MHz @ ~4GHz, 200MHz @ ~7 GHz - Huawei: - DL: 3x IMT-2020 SE + wider bandwidth; - UL: 7GHz, UE transmission power 26dBm, different uplink bandwidth 20/100/200 in Dense Urban-IC, MIMO configuration is 768 antenna elements (24,16,2,1,1,8,16) for BS and 4 antenna elements for UE |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | Agreement in RAN#108: Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx To be discussed in RAN#109: Indoor Hotspot-IC/Dense Urban-IC (UL), Rural-IC (DL, UL) - IIT (Madras), CEWiT, Tejas Networks: - For Dense Urban-IC: 0.2 - 0.3 for UL - For Indoor Hotspot-IC: 0.3-0.4 for UL - For Rural-UC: DL: 0.1 bps/Hz, UL: 0.1 bps/Hz - Apple: [1.5x] IMT-2020 - SKT: Support DL uplift, UL constrained - Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: [1.5-2x] IMT-2020 - China Telecom: [1.5-3x] IMT-2020 - KT, LGE: [2x] IMT-2020 - Xiaomi: [2.5x] for UL - OPPO: prefer [2x] IMT-2020, but agree with [3x] for DL - Ericsson: [2-3x] IMT-2020 - vivo: [3x] for DL, [2.5x] for UL - Samsung: [3x] for DL, [3x] for UL - ZTE, T-Mobile USA, Softbank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Sanechips, Charter, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH, CHTTL: [3x] IMT-2020 - Nokia, Qualcomm, NTT DOCOMO: [3x] IMT-2020 - CMCC: [3x] IMT-2020 (UL), assuming 7GHz with UE 4Tx - Huawei: [3-5x] IMT-2020 - CATT: - DL: [2-3x] IMT-2020 - UL: [1.5-2x] IMT-2020 - MTK: [2x] for UL (Dense Urban/InH), Rural TBD | - Ericsson: This is enabled with more antenna elements than used for IMT-2020. Different factors of improvements could be considered in different test environments. - MTK: For DU/InH: 7GHz with DL 256Tx/6Rx, UL 4Tx/256Rx - . - HW: 7GHz, BS up to 1536 Tx/Rx antenna elements, UE up to 16 Tx/Rx antenna elements; TE: Dense urban, Rural and Indoor - CMCC: 7GHz, 64 TXRUs with 128/256/512 antenna elements and 128 TXRUs with 512/1024 antenna elements - Samsung: Assuming a larger number of antenna elements than those used for IMT 2020 - OPPO: Antenna assumption for 3x of IMT-2020 in DL: Up to CSI-RS 256T4R@7GHz - CATT: DL: to reach 3x, at 4GHz, max 256 antenna; at 7GHz, max 1024 antenna, 8 antenna at UE; UL:1.5x (assuming 4Tx UE antenna) ~ 2x (assuming 8Tx UE antenna) of IMT-2020 target - NTT DOCOMO: - Assuming up to 256QAM and 4 layers for DL / UL - Indoor Hotspot: 7GHz, 30GHz; Dense Urban: 4 GHz, 7GHz; Rural: 700 MHz, 4 GHz - Qualcomm: Antenna assumptions are same as user experienced data rate - Nokia: Assumption: 1024 BS antenna elements; 8 UE antenna elements - Apple: UE antenna elements up to 8 Tx/Rx for 700 MHz, 4 GHz and 7 GHz - Huawei: 7GHz, MIMO configuration is BS up to 1536 Tx/Rx antenna elements, UE up to 16 Tx/Rx antenna elements. The total overhead (control channel, reference signal etc.) is counted as 20% |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | Agreement in RAN#108: Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx To be discussed in RAN#109: Indoor Hotspot-IC/Dense Urban-IC (UL), Rural-IC (DL, UL) - IIT (Madras), CEWiT, Tejas Networks: - For Dense Urban-IC: 7-10 bps/Hz for UL - For Indoor Hotspot-IC: 9-12 bps/Hz for UL - For Rural-UC: DL: =2 bps/Hz, UL: =1 bps/Hz - Apple: [1.5x] IMT-2020 - AT&T, Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: [1.5-2x] IMT-2020 - China Telecom: [1.5-3x] IMT-2020 - Xiaomi: [1.5x] for UL - KT, LGE: [2x] IMT-2020 - OPPO: [2x] IMT-2020, but can agree with [3x] - Ericsson: [2-3x] IMT-2020 - vivo: [3x] for DL, [2.5x] for UL - ZTE, T-Mobile USA, Softbank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Sanechips, Charter, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH, CHTTL: [3x] IMT-2020 - Nokia, NTT DOCOMO, Qualcomm: [3x] IMT-2020 - CMCC: [3x] IMT-2020 (UL), assuming 7GHz with UE 4Tx - Samsung: [3x] IMT-2020. Assuming a larger number of antenna elements than those used for IMT 2020. - SKT: Accept [3x] if deployment density realistic - Huawei: [3-5x] IMT-2020 - CATT: - DL: [2-3x] IMT-2020 - UL: [1.5-2x] IMT-2020 - MTK: [2.8x] for UL (Dense Urban/InH), Rural TBD | - Ericsson: This is enabled with more antenna elements than used for IMT-2020. Different factors of improvements could be considered in different test environments. - MTK: For DU/InH: 7GHz with DL 256Tx/6Rx, UL 4Tx/256Rx - NTT DOCOMO: Assuming up to 256QAM and 4 layers for DL / UL - Qualcomm: Antenna assumptions are same as user experienced data rate - OPPO: feasible with AI/ML-based enhancements - Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: For ~7 GHz with ≤[1024] AEs - Xiaomi: 2TX antenna UEs and 1024 BS antenna elements - Nokia: Assumption: 1024 BS antenna elements; 8 UE antenna elements - Apple: UE antenna elements up to 8 Tx/Rx for 700 MHz, 4 GHz and 7 GHz |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | - MITRE: at least same as IMT-2020 - Apple, Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: Same as IMT-2020 - LGE: [1.5-2x] IMT-2020 - OPPO: prefer [3x] IMT-2020, but agree with [5x] - CATT: [2-3x] IMT-2020 - CMCC, Ericsson, Qualcomm: [3x] IMT-2020 - Xiaomi: [4x] IMT-2020 - Samsung: [4x] IMT-2020 - SKT: Support modest gain, [3-5x] IMT-2020 - Vivo: [4-6x] IMT-2020 - China Telecom, KT, Nokia, ZTE, T-Mobile USA, Softbank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Sanechips, Charter, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH, CHTTL: [5x] IMT-2020 - Huawei: [5-10x] IMT-2020 - IIT (Madras), CEWiT, Tejas Networks: [2-10x] IMT-2020 - NTT DOCOMO: - [2-5x] IMT-2020 @ Indoor Hotspot - IC - 0.8 Mbps/m2 @ Dense Urban – IC - MTK: - DL: [3x] IMT-2020 (for InH) - 7GHz - UL: [2.8x] IMT-2020 (for InH) - 7GHz | - MTK: For InH: 7GHz with DL 256Tx/6Rx, UL 4Tx/256Rx (assuming no BW increase) - NTT DOCOMO: Assuming up to 256QAM and 4 layers. For Indoor Hotspot-IC, 12-36 TRxPs and bandwidth of 400 MHz are assumed. Dense Urban-IC also should be considered. Target value is calculated based on the target value of average spectral efficiency (i.e., 3x of IMT-2020) assuming bandwidth of 400 MHz and ISD = 200 m. - Qualcomm: For In-Hotspot, derived from Avg SE. - Xiaomi: Assuming that the same TRxP density as 5G, 2 times of improvements on average spectral efficiency, and 2 times increase on channel bandwidth, totally 4 times improvements on area traffic capacity can be expected. - CATT: Indoor hotspot assuming 12TRP only to define value - Samsung: Assuming enhanced average spectral efficiency and wider bandwidth. |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | Agreement in RAN#108: - IC: 4ms - HRLLC: 1ms - AT&T, Qualcomm, KT, LGE, Nokia, OPPO, Samsung, Ericsson, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: Same as IMT-2020 - SKT: Same as or more than IMT-2020 - NTT DOCOMO, CMCC: - IC: 2ms - HRLLC: 0.5ms - China Telecom: - HRLLC: 0.1-0.5ms - Apple: - IC: 1ms - HRLLC: 1ms - Xiaomi, ZTE: - IC: 1ms - HRLLC: 0.5ms - Huawei: - IC: 1ms - HRLLC: 0.5-1ms - Vivo: - IC: 1-2ms - HRLLC: 0.5-1ms - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: - IC: 2-4 ms - HRLLC: 0.5-1ms - MTK: - IC: Unclear that we need an “eMBB” target - HRLLC: 1ms - CATT: IC: 2ms; No strong view to HRLLC while 0.5ms is challenging - Spreadtrum, UNISOC - IC: 2-4 ms - HRLLC: 0.5 ms | - MTK: - 4GHz and 7GHz TDD, with option to combine with 700MHz in uplink - SBFD and UL/DL band decoupling can be considered. - eMBB: If included, depends on assumptions vs HRLLC - NTT DOCOMO: For user plane, SCSs of 15 / 30 kHz for IC and 60 / 120 kHz for HRLLC are assumed. |
|  | Control plane latency (ms) | 20 | Agreement in RAN#108: 20ms - AT&T, KT, MTK, LGE, Ericsson, OPPO: Same as IMT-2020 - SKT: 10ms - Qualcomm, Apple: 15ms - Xiaomi: 5-10ms - Spreadtrum, UNISOC, NTT DOCOMO, CMCC, China Telecom, Huawei, CATT: 10ms - Samsung: 10ms considering 2 step RACH in Rel 16 or potential enhancements such as RACH-less approach for IMT 2030 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 10-20ms - Vivo, 10-15ms - Nokia: 15ms - ZTE: 10ms, consider 5ms | - MTK: - 4GHz and 7GHz TDD, with 700MHz option for UL - SBFD and UL/DL decoupling can be considered. - NTT DOCOMO: For control plane, enhanced procedure may be required (e.g., 2-step RACH) |
| Connection density (devices/km²) |  | 10⁶ | Agreement in RAN#108: 10⁶ (devices/km2) for massive communication - MTK, Samsung, Ericsson (OK with [10x]), Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: Same as IMT-2020 - SKT: Same as or more than IMT-2020 - AT&T, KT, OPPO, Nokia, Qualcomm, Apple, CATT: [10x] IMT-2020 - LGE: [1x] for normal devices; [10x] for IoT devices - Xiaomi, NTT DOCOMO: [1-10x] IMT-2020 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: [5-10x] IMT-2020 - Vivo, Huawei, China Telecom: [10-100x] IMT-2020 - ZTE: [100x] IMT-2020 - CMCC: 108-109 connections per km2 per day | - NTT DOCOMO: Assuming bandwidth of 180 kHz, SCS of 15 kHz and packet size of 32 bytes within packet delay of 10s. Assumption of bandwidth should be discussed. |
| Reliability |  | 1 - 10⁻⁵ | Agreement in RAN#108: 1-10-5 with same IMT-2020 assumptions - AT&T, Qualcomm (Or 10x, with latency >1ms), KT, MTK, LGE, OPPO, Samsung, Ericsson, Nokia, vivo, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM, Xiaomi: 1-10−5 - SKT: Same as or more than IMT-2020 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 1-10-5 ~ 1-10-6 - Huawei, CMCC, China Telecom: 1-10-6 ~ 1-10-7 - Apple: 1-10−6 - ZTE, NTT DOCOMO: 1-10-7 - MITRE: Consider higher reliability for some use cases (industrial, mission-critical, etc.) | - NTT DOCOMO: Assuming packet size of 32 bytes, required maximum time of 1 ms and SU-MIMO with rank 1. Assumptions of packet size and required maximum time should be discussed. |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | Agreement in RAN#108: Indoor Hotspot-IC: 2.25 (bit/s/Hz) @10 km/h Dense Urban- IC: 1.68 (bit/s/Hz) @30 km/h Rural-IC: [0.8~1.2] (bit/s/Hz) @120 km/h [0.45~0.675] (bit/s/Hz) @500 km/h NOTE1: Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. NOTE2: For Rural-IC, TBD on whether to include other velocities, e.g. 30km/h, 60km/h - Mobility class: - AT&T, Qualcomm, KT, SKT, Samsung, MTK, NTT DOCOMO, Nokia, Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom, Apple: same as IMT-2020 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: Divide Vehicular into two categories 10-30/60 km/h for Low Speed Vehicular and 30/60-120 km/h for Medium Speed Vehicular - ZTE, LGE, OPPO, CMCC, Xiaomi, China Telecom propose to introduce a new class of mobility (up to 1000/1200 km/h) for ATG. - Spectral efficiency for Rural-IC: - CATT, CMCC, Samsung: [1.5x] @120, [1.5x] @500 - SKT, AT&T, OPPO, MTK: Same as IMT-2020 - Vivo: [1.5x] IMT-2020 - LGE: [1.5-2x] IMT-2020 - NTT DOCOMO: [2x] IMT-2020 - Qualcomm, Nokia: the associated data rates should be increased - Vivo suggest to add the practical Urban mobility deployment scenario as a test environment to define the mobility classes and minimum requirement. | - NTT DOCOMO: Assuming up to 256QAM and 2 layers. - Qualcomm: Focus on terrestrial UEs. |
| Mobility interruption time (ms) |  | 0 | - SKT, AT&T, Qualcomm, KT, NTT DOCOMO, OPPO, Samsung, CMCC, Xiaomi, China Telecom, Huawei, CATT, ZTE, T-Mobile USA, Softbank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Sanechips, Charter, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH, , CHTTL, IIT (Madras), CEWiT, Tejas Networks: 0ms - LGE: Close-to-0 ms (to avoid mandating the support of concurrent connection of source cell and target cell) - MTK: 10ms - Apple: not to define IMT-2030 mobility interruption time requirement - Nokia: A qualitative requirement - Vivo: TBD | - MTK: Handover with single TRP/carrier operation. If 0ms is agreed, restrict to multi-carrier/TRP operation. - Nokia: A qualitative requirement such as “Low mobility interruption time” is proposed, and 3GPP may provide in its self-evaluation analysis of the achievable values. |
| Bandwidth (MHz) |  | 100 | RAN#107 agreement: 400MHz (same definition as IMT-2020) - China Telecom, MTK, Ericsson, CMCC, Huawei, Qualcomm, KT, SKT, MITRE, NTT DOCOMO, OPPO, Samsung, Nokia, vivo, China Telecom, Apple, ZTE: 400MHz - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 200MHz | - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: Indian regulator is unlikely to give 400 MHz per operator in 7GHz band - Qualcomm: up to 2 GHz (including freqs > 6GHz) |
| Positioning |  |  | Samsung: - Indoor Factory-ISAC (H/V): [0.5-1] / [1-3] for 90% of UEs - Urban Macro-ISAC (H/V): [3] / [3] for 90% of UEs NTT DOCOMO: - Indoor: < 2 m for 90% of UEs/devices (Horizontal accuracy) - Outdoor: < 6 m for 90% of UEs/devices (Horizontal accuracy) - discuss whether the evaluation of vertical accuracy is necessary or not Vivo: Positioning accuracy [10cm] (IC and ISAC) ZTE: - Horizontal positioning accuracy: 20cm-1m (indoor), 3m (outdoor) for 90% of UEs - Vertical positioning accuracy: 5m for 90% of UEs SKT: Indoor 1 m horizontal / 3 m vertical accuracy, Outdoor 3/3 m CMCC: - Horizontal accuracy: 20-100cm@90% for indoor, 3-5m@90% for outdoor - FFS the necessity of vertical accuracy Huawei: Horizontal accuracy: [20-50cm] @90% for indoor, [3-5m] @ 90% for outdoor Deutsche Telekom, Orange, Spark NZ, Telia Company, Bouygues Telecom, BT, Telefonica, Odido, Vodafone, KPN, Telecom Italia, Telenor, SK Telecom: 10 cm an optional capability for indoor MTK: - Basic performance: Horizontal accuracy of <50 metres for 80% of users and Vertical accuracy of <5 metres for 80% of users. - improved performance requirements: - For indoor deployments: Horizontal accuracy of 1 metre for 90% of users and vertical accuracy of 3 metres for 90% of users. - For outdoor deployments: Horizontal accuracy of 10 metres for 80% of users and vertical accuracy of 3 metres for 80% of users. Nokia: Indoor: sub-meter; Outdoor: a few meters IIT (Madras), Tejas Networks, CEWiT: - Indoor Factory ‒ HRLLC: - Horizontal accuracy (m): 0.1-3 - Vertical accuracy (m): 0.1-3 - Dense Urban-IC: - Horizontal accuracy (m): 1-5 - Vertical accuracy (m): 1-3 Apple: - Urban Macro: positioning accuracy requirements are 1m (horizontal) and 3m (vertical) - Indoor Factory: positioning accuracy requirements are 0.2m horizontal and 1m vertical Spreadtrum, UNISOC: | Samsung: - Target positioning accuracies may not necessarily be reached for all scenarios and deployments. - Necessity of vertical accuracy needs further discussion. Nokia: Separate requirements for vertical and horizontal positioning accuracy. The requirements need to take into account the deployment scenarios, and same accuracy cannot be assumed in all cases. Huawei: - InF-SH (Indoor Factory with Sparse clutter and High base station height), Indoor office (or hotspot as called in ITU), and Urban Macro. - The positioning method of UL-TDOA and single-BS positioning is used in the indoor and outdoor simulation, respectively, with 200MHz SRS bandwidth at carrier frequency around 7GHz. |
| Sensing |  |  | Samsung: - Detection/False alarm probabilities [%]: [95/5] - Localization accuracy [m]: [1-5] @ [95%] confidence level - Velocity accuracy [m/s]: [1-5] @ [95%] confidence level - Resolution: TBD NTT DOCOMO: ZTE: - Detection probability: 95% - False alarm probability: 5% - Horizontal localization accuracy: 2m - Vertical localization accuracy: 5m - Velocity accuracy: 1m/s CMCC: - Detection Probability: ≥90% - Probability and False Probability: ≤10% - Horizontal Localization Accuracy: ≤3-5m - Vertical Localization Accuracy: ≤3-5m - Velocity Localization Accuracy: ≤3-5m/s Huawei: For Outdoor ISD=500m: - Localization horizontal accuracy: 3m - Localization vertical accuracy: 3m - Velocity accuracy: 5m/s CATT: - Detection probability/False alarm: 90% /10% - Localization accuracy: Urban Macro - horizontal 3-5m, Vertical 3-5m - Velocity accuracy：Urban Macro – 5m/s Xiaomi: Nokia: value TBD - Sensing resolution - Detection probability and false alarm probability - Localization accuracy - Velocity accuracy Apple: Spreadtrum, UNISOC: - Detectability: - Detection probability: [95% – 99.9%] - False alarm probability: [5% – 1%] - Sensing Accuracy - Distance accuracy: [10 m -1 cm] - Angle accuracy: [2°– 0.1°] - Velocity accuracy: [10 m/s – 10 cm/s] Sensing Resolution - Distance resolution: [10 m – 1cm] - Angle resolution: [3°– 0.1°] - Velocity resolution: [10 m/s – 20 cm/s] | Samsung: - ITU-R WP 5D agreed to define target values for detectability/false alarm probability, localization accuracy, velocity accuracy, keeping resolution TBD. - It should be noted that the target sensing-related requirements may not necessarily be reached for all scenarios and deployments, depending on assumptions for sensing resources (e.g., bandwidth, power, number of antennas, and overhead, etc.) Nokia: Some of the KPIs listed here may be also 3GPP-internal. Huawei: - Sensing localization accuracy and velocity accuracy are simulated based on the urban grid geometry defined in TR37.885 as below. There are 9 buildings (the green blocks in the figure) with four lanes in between, the lane width is 3.5m. The size of each buildings is 413m (length) x 230m (width) x 20m (height). All BSs are located on the rooftop of the buildings, all sensing targets (vehicles) are located on the street, and the UEs are located on the sidewalk road. - The simulations are conducted at 7 GHz carrier frequency and 400MHz bandwidth, including 2 cases of BS-UE bistatic LOS and BS-UE bistatic LOS+NLOS. The cooperation among multiple BSs of same or different sites is applied. |
| Energy efficiency |  |  | Samsung: - Relative network energy saving gain: [TBD] % - Relative device energy saving gain: [TBD] % Vivo: - Network EE: TBD - Device EE SKT: Support unloaded case, both network/device gains MTK: No target value for ITU-R, just report what is achieved. Nokia: Requirements on relative energy savings between different transmission states. | Samsung: - Metric: Relative energy saving gain (in %) for unloaded case. - Evaluation: Analytical evaluation for the unloaded case only is preferred. - Power model: Proponents should report the power model used for the evaluation. |
| Composite requirement |  |  | Ericsson: - For XR, 30Mbps DL, 10Mbps UL, 40ms two-way delay (DL + UL <= 40ms, e.g. DL = 10ms & UL = 30ms) with 99% reliability, capacity average of 5 simultaneous users per TRxP, or 433 per sqkm for 200m ISD at 95% satisfaction ratio. - For video conferencing, 1Mbps DL, 1Mbps UL, 100ms two-way delay with 98% reliability. Samsung: Quantitative TPR for immersive communication (Dense Urban-IC) - Data rate (DL/UL): [30/10] Mbps - Latency (DL/UL): [10/30] ms - Reliability (DL/UL): [99/99] % - Number of users per TRxP: [10] users w/ 90% satisfaction ratio NTT DOCOMO: Vivo: ZTE: SKT: DL/UL 30/10 Mbps, latency 10/30 ms, 10 users/TRxP CMCC: 10 user/TRxP Huawei: 6-12 XR users per TRxP for Dense urban ISD=200m MTK: Add “…for Immersive Communication” to the TPR name. “The connections per TRxP, determined by 95% of satisfied users determined in DL and UL independently, where each satisfied user fulfils the requirements of the following traffic profiles: - For DL: 30Mbps data rate (single stream), 60 frames/second, 62.5kB packet size, 10ms packet delay budget, 1% PER - For UL: 10Mbps data rate (single stream), 60 frames/second, 20.833kB packet size, 30ms packet delay budget, 1% PER” | Ericsson: - Note: the proponent may evaluate DL and UL separately, with delay requirements that add to <=40ms. OPPO: Proposal 3: Consider composite requirement combining data rate, PDB (Packet Delay Budget), PER (Packet Error Rate) and capacity (number of UEs/cell). - Adopt XR evaluation methodology in TR 38.838 as reference. - At least used for Immersive and Massive communication application scenarios. Huawei: Using the evaluation methodology of 3GPP study on XR (Extended Reality) evaluations for NR in TR38.838, XR capacity is evaluated for eMBB dense urban test environment at 4GHz and 7 GHz. - BS 64: (M, N, P, Mg, Ng; Mp, Np) = (12,8,2,1,1,4,8), 192 antenna elements - BS 256: (M, N, P, Mg, Ng; Mp, Np) = (24,16,2,1,1,8,16), 768 antenna elements |

# Annex 2: Summary of energy efficiency proposals

| [RP-251956 Samsung] | Metric: Relative energy saving gain (in %) for unloaded case. Evaluation: Analytical evaluation for the unloaded case only is preferred. Power model: Proponents should report the power model used for the evaluation. |
| --- | --- |
| [RP-252002 Vivo] | ### 5.1.15.1 Energy efficiency for ITU IMT-2030 Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device power saving in relation to the traffic characteristics. The requirement is defined as the relative energy savings (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. For unloaded case, this requirement is evaluated by analytical approach, for loaded case, this requirement is evaluated by simulation approach. Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies. The minimum requirements for Energy efficiency are summarized in Tables below. Table: The minimum requirements for Network Energy efficiency [Test environment(s)]**; Selected load case(s)*; The reference case; Relative energy saving (%). N. A; Empty load; Fully loaded case*; TBD. Dense urban-IC; TBD; TBD. Rural-IC; TBD; TBD. Table: The minimum requirements for Device Energy efficiency [Test environment(s)]**; Selected load case(s)*; The reference case; Relative energy saving (%). N. A; Empty load; Fully loaded case; [90%]. Dense urban-IC; [<5%] NOTE; [90%]. NOTE: assume one active UE in one TRxP with FTP3 traffic (such as 0.5MB packet size, 200ms mean inter-arrival time) |
| [RP-252086 CMCC] | Proposal 3: Support to define energy efficiency as a quantitative TPR (i.e. the evaluation method is simulation). Proposal 4: For energy efficiency, support the following definition to be captured in TR 38.914 and the LS reply: 5.1.15 Energy efficiency Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics. The requirement is defined as the relative energy savings (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies. |
| [RP-252122 Deutsche Telekom] | A quantified target value for Energy Efficiency is recommended. Other KPIs/KVIs also TBD. |
| [RP-252172 MediaTek] | Network EE: “Quantify the relative improvement in energy consumption for a given system loading (unloaded, partially loaded) vs the energy consumption in fully loaded scenario.” Device EE: “Quantify the relative improvement in Device EE for a given traffic loading (zero load, partial load) vs energy consumption in fully loaded scenario. Proponents to report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving operation.” No target value for ITU-R, just report what is achieved. |
| [RP-252181 Nokia] | Relative energy savings may be analyzed using a power model, e.g. energy consumption in unloaded case is x % of that in fully loaded case. |

# Annex 3: Summary of positioning proposals

| [RP-251956 Samsung] | - Target positioning accuracies may not necessarily be reached for all scenarios and deployments. - Necessity of vertical accuracy needs further discussion. Target values: - Indoor Factory-ISAC (H/V): [0.5-1]/[1-3] for 90% of UEs - Urban Macro-ISAC (H/V): [3]/[3] for 90% of UEs |
| --- | --- |
| [RP-251994 DOCOMO] | Table 2: Target values for positioning accuracy Test environment; Horizontal accuracy; Vertical accuracy. Indoor; < 2 m for 90% of UEs/devices; [meter-level]. Outdoor; < 6 m for 90% of UEs/devices; [meter-level]. Proposal 3: To adopt the target values in Table 2 for positioning of IMT-2030 and discuss whether the evaluation of vertical accuracy is necessary or not. |
| [RP-252122 Deutsche Telekom] | 10 cm an optional capability for indoor |
| [RP-252122 Huawei] | Horizontal accuracy: [20-50cm]@90% for indoor, [3-5m]@90% for outdoor |
| [RP-252172 MediaTek] | “The RIT shall support the following basic performance: - Horizontal accuracy of <50 metres for 80% of users and Vertical accuracy of <5 metres for 80% of users. Proponents shall also describe how the RIT [is forwards compatible / may be evolved in future to support the following improved performance requirements: - For indoor deployments: Horizontal accuracy of 1 metre for 90% of users and vertical accuracy of 3 metres for 90% of users. - For outdoor deployments: Horizontal accuracy of 10 metres for 80% of users and vertical accuracy of 3 metres for 80% of users.” Scenarios: Reuse evaluation scenarios from TR 38.855 and Rel-17 TR 38.857. |
| [RP-252181 Nokia] | Separate requirements for vertical and horizontal positioning accuracy. The requirements need to take into account the deployment scenarios, and same accuracy cannot be assumed in all cases. Indoor: sub-meter Outdoor: a few meters |
| [RP-252214 IIT] | Indoor Factory ‒ HRLLC: Horizontal accuracy (m): 0.1-3 Vertical accuracy (m): 0.1-3 Dense Urban-IC: Horizontal accuracy (m): 1-5 Vertical accuracy (m): 1-3 |
| [RP-252317 Apple] | Proposal 5: positioning accuracy requirements for the evaluation in the Urban Macro TE are 1m (horizontal) and 3m (vertical), whereas for Factor TE it is 0.2m horizontal and 1m vertical. |

# Annex 4: Summary of sensing-related proposals

| [RP-251956 Samsung] | Detection/False alarm probabilities [%]: [95/5] Localization accuracy [m]: [1-5] @[95%] confidence level Velocity accuracy [m/s]: [1-5] @[95%] confidence level Resolution: TBD It should be noted that the target sensing-related requirements may not necessarily be reached for all scenarios and deployments, depending on assumptions for sensing resources (e.g., bandwidth, power, number of antennas, and overhead, etc.) |
| --- | --- |
| [RP-251994 Docomo] | Table 4: Target values of sensing-related capabilities Sensing task; Test environment; Detection probability (%); False alarm probability (%); Localization accuracy (m); Velocity accuracy (m/s); [Sensing resolution (m)]. Object detection and tracking; Indoor Outdoor; 95~99.9; 2~5; 0.5~10; 0.1~15; [0.5~10]. Environment monitoring; TBD; 95~99.9; 3; Horizontal: 10 Vertical: 0.2; -; [-]. Motion monitoring; TBD; 95; 5; 0.2; 0.1; [0.375]. Proposal 4: For sensing-related capabilities, sensing task, test environment, and target values of the requirements should be discussed based on TS 22.137 as starting point and should also be discussed whether to include sensing resolution as a performance requirement. |
| [RP-252000 Oppo] | KPIs for both basic and advanced physical parameters, such as accuracy of model parameters. Comparing with option 1, option 2 extends to some advanced parameters to cover some popular use cases recent years, such as motion monitoring. For motion monitoring, accuracy of model parameter, i.e. repetition rate can be defined as KPI. Proposal 4: For 6G Sensing, KPIs for both basic and advanced physical parameters can be studied. |
| [RP-252002 Oppo] | Sensing-related capabilities are measured in terms of the following technical performance requirements: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. This requirement is defined for the purpose of evaluation in the Integrated Sensing and Communication usage scenario. This requirement is evaluated by simulation approach. |
| [RP-252024 ZTE, Sanechips] | Sensing-related capabilities are measured in terms of the following KPIs: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Estimation Accuracy: Velocity estimation accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. Proposal 5: Define detection probability and false alarm probability, localization accuracy and velocity estimation accuracy as sensing KPIs. - Detection probability: 95% - False alarm probability: 5% - Horizontal localization accuracy: 2m - Vertical localization accuracy: 5m - Velocity accuracy: 1m/s |
| [RP-252061 SK-Telecom] | SK Telecom supports the definition of 6G KPIs in alignment with ITU-R IMT-2030 TPRs, emphasizing realistic deployment feasibility and operator-centric service enablement. KPI Category; WP5D WD (5D/792 Annex 5.14) [1]; ITU-R WP5D Korea Contribution (5D/658) [2]; 3GPP TP (RP-251880) [3]; SKT ‘s View. Sensing KPIs; Defined as detectability, localization, velocity; resolution TBD; Same, resolution optional; -; Support Korea proposal. |
| [RP-252086 CMCC] | Proposal 6: For the sensing TPRs, the definition captured in the latest ITU working document of 49th WP5D meeting shall be modified as follows. Metric; Definition; Proposal on IMT-2030 KPIs. Detection Probability and False alarm probability; Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. For cases with multiple objects, association between the detected sensing objects and true sensing objects is required.; Detection Probability: ≥90% False Alarm Probability: ≤10%. Horizontal/Vertical Localization Accuracy; Horizontal/vertical localization accuracy is defined as the 90th percentile point of the cumulative distribution function (CDF) of the horizontal/vertical localization estimation error. Localization estimation error is the norm of estimated horizontal/vertical position and the corresponding true position of a sensing target.; Horizontal Localization Accuracy: 3-5m Vertical Localization Accuracy: 3-5m. Velocity Accuracy; Velocity accuracy is defined as the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. Velocity estimation error is the norm of the estimated velocity and the corresponding true velocity of a sensing target.; Velocity Localization Accuracy: 3-5m/s. |
| [RP-252128 Huawei, HiSilicon] | TPR items; Proposed requirement values on remaining IMT-2030 items; Comments. 16. Sensing-related capabilities; For Outdoor ISD=500m: - Localization horizontal accuracy: 3m - Localization vertical accuracy: 3m - Velocity accuracy : 5m/s; See simulation in Annex. |
| [RP-252143 CACT] | Candidate IMT-2030 KPIs; Usage Scenario; Possible values in consideration. Sensing(detection probability， localization accuracy and velocity accuracy); Indoor Factory-SH & Urban Macro - AV; Detection probability/False alarm: 90% /10% Localization accuracy: Urban Macro - horizontal 3-5m, Vertical 3-5m Velocity accuracy：Urban Macro – 5m/s. |
| [RP-252151, Spreadtrum, UNISOC] | Detectability Detection probability: [95%-99.9%] Note: Detection probability the probability of correctly detecting the presence of the object False alarm probability: [5%-1%] Note: False alarm probability is the associated probability that an object is detected given that the object is not present Sensing Accuracy Distance accuracy: [10 m – 1 cm] Note: Distance accuracy is the difference between the calculated distance and the actual distance of the sensing target Angle accuracy: [2° - 0.1°] Note: Angle accuracy is the difference between the calculated angles and the actual angles of the sensing target Velocity accuracy: [10 m/s – 10 cm/s] Note: Velocity accuracy is the difference between the estimated velocity and the actual velocity of the sensing target. Sensing Resolution Distance resolution: [10 m – 1cm] Note: Distance resolution is the minimum difference of the measured distance value of two sensing targets Angle resolution: [3° - 0.1°] Note: Angle resolution is the minimum difference of the measured angle value of two sensing targets Velocity resolution: [10 m/s – 20 cm/s] Note: Velocity resolution is the minimum difference of the measured velocity value of two sensing targets |
| [RP-252172 MediaTek] | Proposal 1: Agree to the proposals in Table 1 and 2 in this document for the still undecided IMT-2030 TPRs. TPR; Proposed Definition. Sensing capabilities; Qualitative (note that 3GPP could provide quantitative performance results based on its own target metrics in addition to description). “Describe the ability of the RIT to support Sensing capability, considering design metrics such as Detection Probability, Object Location Accuracy and False Alarm Probability for a selected use case/environment.”. |
| [RP-252181 Nokia] | TPR; IMT-2030 Requirement; Comments. Sensing (New); - Sensing resolution - Detection probability and false alarm probability - Localization accuracy -Velocity accuracy; Some of the KPIs listed here may be also 3GPP-internal. The values of KPIs are TBD.. |
| [RP-252317 Apple] | Proposal 4: adopt the following IMT-2030 ISAC TPRs: Urban Macro; Indoor Factory. Missed detection probability; 5%; 5%. False alarm rate; 2%; 2%. Horizontal position accuracy; 10m@95%; 2m@95%. Vertical position accuracy; 10m@95%; 2m@95%. Velocity accuracy; 1 m/s; 1 m/s. |

# Annex 5: Summary of AI proposals

| [RP-251956 Samsung] | Fine with RAN agreement |
| --- | --- |
| [RP-251994 DOCOMO] | Proposal 5: For AI-related capabilities as qualitative TPR, its definition should be discussed based on the latest draft in the ITU-R WP 5D working document as starting point. |
| [RP-252086 CMCC] | Proposal 7: It is proposed to adopt the following definition for AI-related capabilities: AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications, including. • RIT/SRIT facilitating AI applications i.e. “Network for AI” • AI to improve RIT/SRIT performance, i.e. “AI for network” The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling/ related to AI functionalities. This should include support for one or more of the following: – Manageable AI data collection – Distributed processing – Distributed learning – AI model training – Efficient AI computing processing – AI model execution – AI model inference – Low total cost of ownership (TCO) - Decoupling Computing Module with Legacy Communication Module - Openness of Hardware Infrastructure - Flexible Scheduling of Computing Resources – or other AI capabilities of the RITs/SRITs (to be reported by the proponent) Option A RIT/SRIT should have the capability to enable and disable AI functionalities within the network (AI for network scenario only) Option B The proponent should report about mechanisms, e.g, related to enabling or disabling the supported AI functionalities, where applicable.. |

# Annex 6: Summary of composite requirement proposals

| [RP-251952 Ericsson] | For XR, 30Mbps DL, 10Mbps UL, 40ms two-way delay (DL + UL <= 40ms, e.g. DL = 10ms & UL = 30ms) with 99% reliability, capacity average of 5 simultaneous users per TRxP, or 433 per sqkm for 200m ISD at 95% satisfaction ratio. For video conferencing, 1Mbps DL, 1Mbps UL, 100ms two-way delay with 98% reliability. Note: the proponent may evaluate DL and UL separately, with delay requirements that add to <=40ms. |
| --- | --- |
| [RP-251956 Samsung] | - Data rate (DL/UL): [30/10] Mbps - Latency (DL/UL): [10/30] ms - Reliability (DL/UL): [99/99] % - Number of users per TRxP: [10] users w/ 90% satisfaction ratio - Quantitative TPR for immersive communication (Dense Urban-IC) |
| [RP-251994 DOCOMO] | Proposal 6: To adopt the target values in Table 6 and 7 for composite requirement in IC usage scenario. Table 6: DL target values for composite requirement for IC usage scenario Test environment; Data rate (Mbps); Latency (ms); Packet success probability; Number of satisfied users (/TRxP). Dense Urban-IC; 30; 10; 99%; 4~11. 15; 5~15. 45; 10; 2~7. 15; 6. Indoor Hotspot-IC; 30; 10; 99%; 4~9. 15; 8~11. 45; 10; 4~6. 60; 10; 2~4. Table 7: UL target values for composite requirement for IC usage scenario Test environment; Data rate (Mbps); Latency (ms); Packet success probability; Number of satisfied users (/TRxP). Dense Urban-IC; 0.2; 10; 99%; 20~240. 10; 15; 5. 30; 2~9. 20; 30; 3. Indoor Hotspot-IC; 0.2; 10; 99%; 20~240. 10; 30; 4~9. |
| [RP-251952 OPPO] | Proposal 3: Consider composite requirement combining data rate, PDB (Packet Delay Budget), PER (Packet Error Rate) and capacity (number of UEs/cell). - Adopt XR evaluation methodology in TR 38.838 as reference. - At least used for Immersive and Massive communication application scenarios. |
| [RP-252002 vivo] | Data rate: DL: 30Mbps UL: 10Mbps. Latency: DL: 10ms UL: 30ms. Reliability: DL: 99% UL: 99%. Capacity: number of users per km2 Dense urban-IC: 1000. |
| [RP-252024 ZTE] | Observation 5: Multi-modality (mixed traffic model of video, haptic, etc) is a nature of XR services that distinguishes them from traditional video services. Traffic model; DL Video: 45Mbps@60FPS, reliability = 99%, PDB = 10ms Haptic: fixed 100 Byte/s, reliability = 99.9%, PDB=5ms, pareto distribution arrival UL Video: 20Mbps@60FPS, PER=99%, PDB = 10ms. Proposal 4: Define a multi-modal traffic model (corresponding to data rate, latency and reliability) for composite requirement as in the table above, which comprises of video and haptic data. |
| [RP-252061 SKT] | Proposal 2. Introduce a Composite KPI as a central metric for immersive and XR services. |
| [RP-252086 CMCC] | Proposal 5: For the composite requirement, suggest the following definition to be captured in TR 38.914 based on that captured in the latest ITU working document of 49th WP5D meeting: 5.1.16 Composite requirement The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP. This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface. This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. The minimum performance requirement of composite requirement is summarized in Table 5.1.16-1. Table 5.1.16-1: Composite requirement Test environment; Data rate (Mbits/s); Latency (ms); Packet success probability; Number of users ( /TRxP). Dense Urban - Immersive Communication; DL = 30 Mbps/UL=10 Mbps; DL=10 ms / UL=30 ms / In total 40 ms; 99%; 10. Others if any, are FFS.. |
| [RP-252128 Huawei] | 6-12 XR users per TRxP for Dense urban ISD=200m - Using the evaluation methodology of 3GPP study on XR (Extended Reality) evaluations for NR in TR38.838, XR capacity is evaluated for eMBB dense urban test environment at 4GHz and 7 GHz. - The simulation results are in figures below for DL and UL respectively. |
| [RP-252129 Huawei] | Proposal 2: The evaluation of composite requirement for immersive communication should be supported by system simulation, with composite requirement of the number of satisfied users, by meeting the different QoS of data rate, latency and reliability per user. |
| [RP-252172 MTK] | Add “…for Immersive Communication” to the TPR name. “The connections per TRxP, determined by 95% of satisfied users determined in DL and UL independently, where each satisfied user fulfils the requirements of the following traffic profiles: • For DL: 30Mbps data rate (single stream), 60 frames/second, 62.5kB packet size, 10ms packet delay budget, 1% PER • For UL: 10Mbps data rate (single stream), 60 frames/second, 20.833kB packet size, 30ms packet delay budget, 1% PER” |
| [RP-252181 Nokia] | The requirements are defined for a limited number of scenarios to reflect the required values for key use cases such as XR, AI, and videoconferencing. Value: TBD |

# Annex 7: Summary of Resilience and coverage proposals

| [RP-251956 Samsung] | - Define as a qualitative TPR. - Proposed revision (yellow-highlighted) based on the latest text from ITU-R WP 5D#49 (Annex 5.14 to Document 5D/792). 4.17 Resilience (Editor’s note: This may also include Extended Connectivity) Resilience refers to capabilities of the networks and systems to continue operating correctly during and after a natural or man-made disturbance, such as the loss of primary source of power, etc. The RIT/SRIT should support functionalities over the radio interface that enable continuous operation or rapid temporary restoration during and after disturbance to radio infrastructure. These functionalities may include, but are not limited to, support for connecting to one or more of the following network infrastructures within the terrestrial component of IMT, which are temporarily operated to enhance connectivity during these situations: ‒ HIBS ‒ [Advanced] relays and repeaters ‒ Base station onboard a vehicle, airship and/or vessel ‒ Shared network infrastructures ‒ or other mechanisms for resilience using network infrastructures within the terrestrial component of IMT |
| --- | --- |
| [RP-251994 DOCOMO] | Proposal 7: RAN should discuss how to capture resilience, as a qualitative TPR similar to AI-related capabilities or some other way. |
| [RP-252014 SoftBank] | Proposal 1: If RAN#109 agrees to send an LS to WP 5D, the Resilience TPR should be merged with Extended Connectivity, creating a single qualitative TPR: “Resilience and Extended Connectivity”. |
| [RP-251956 Samsung] | It is preferred not to define a requirement but to use the link budget template similar to IMT-2020 submission. (same as Option 2 in RP-251394) |
| [RP-252122 Deutsche Telekom] | Integration of TN and NTN is required to improve coverage. Common methods for extending coverage at a lower data rate should be considered for various use cases |
| [RP-252172 MediaTek] | Link Budget evaluation – same as IMT2020. No target value. |
| [RP-252214 IIT] | 3-8 km for 2-3 Mbps data rate Proposal 5: Single cell rural deployments should be included in a new test environment called Rural-UC for the evaluation of TPRs related to Ubiquitous Connectivity. The Rural-UC test environment should include the sparse deployment of large cells without contiguous multi-cell coverage, as shown in Figure 1. Proposal 6: The evaluation parameters for the Rural-UC test environment should include different UE types. For example, a CPE type device with 5m height and 31dBm power class should also be considered in addition to the 1.5m handheld UE. Proposal 7: The evaluation of coverage (maximum link distance) in Rural-UC can done with the following steps: Step 1: Run link level simulations for downlink and uplink data channels with the given configuration as per section 8.4.X. Obtain the SNR that corresponds to a minimum data rate of [2-3] Mbps. Step 2: Perform a link budget analysis for uplink and downlink using the SNR obtained from Step 1, along with other parameters specified in section 8.4.X. Obtain the MCL values from the link budget. Step 3: Calculate the corresponding distance for NLOS, for the associated channel model for this test environment. |