# 3GPP TSG RAN Meeting #109	RP-252844

# Beijing, CN, 15-18, September 2025

## Status Report to TSG

Agenda item:			9.7.4.2

| WI / SI Name | NR FR1 7 MHz Channel Bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR1_7MHz_BW |  |  |  |  |
| Unique ID | 1060083 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251453 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% (+45%) | Performance Part: 80% (+60%) | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Lei GAO, Bill Shvodian |
|  | Company | China Telecom, T-Mobile USA |
|  | Email | gaol8@chinatelecom.cn, bill.shvodian@t-mobile.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

N/A.

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

N/A.

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

N/A.

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

## In RAN4#114, the following agreements were made.

#### Scope

Agreement:

- The 7 MHz channel bandwidth is only applicable to FDD bands that also support 5 MHz channel bandwidth.

NB-IoT support

Agreement:

- In-band NB-IoT can be supported in 7MHz channel BW

#### System parameters – Release independence

Agreement:

- 35 RBs will be supported.

Release independence

Agreement:

- Support of 7 MHz channel BW could be Release independent from Rel-15

  - FFS on if the maximum initial BWP size shall then be restricted to 5 MHz.

  - Support of 7MHz is optional in Rel-19 and in the previous releases

#### Channel spacing, channel raster and sync raster

Agreement:

- Introducing 7 MHz channel BW has no impact on channel spacing, channel raster nor sync raster.

#### UE RF

NS_100

Agreement:

- There is no impact on A-MPR requirements for NS_100.

NS_12, NS_13, NS_14, NS_15

Agreement:

- For n26

  - NS_12 and NS_15 shall be supported for 7 MHz channel BW.

  - To be further discussed if NS_13 and NS_14 shall also be supported.

SEM

Agreement:

- Same SEM requirement for 7 MHz and 10 MHz channel BW.

MPR

Agreement:

- No MPR impact when introducing 7 MHz channel bandwidth.

Minimum Output power, Transmit Off power, Occupied bandwidth, ACLR, general spurious

Agreement:

- The requirement for 7 MHz is the same requirement than the requirement for 3-20 MHz. 7 MHz shall be added to:

  - Table 6.3.1-1 (minimum output power).

  - Table 6.3.2-1(Transmit Off power).

  - Table 6.5.1-1 (occupied bandwidth).

  - Table 6.5.2.4.1-1 (ACLR).

  - Table 6.5.3.1-1 (general spurious).

Maximum input level

Agreement:

- The requirement for 7 MHz is the same requirement than the requirement for 3-20 MHz. 7 MHz shall be added to Table7.4-1.

ACS, In-band blocking, Out of band blocking, Narrow Band blocking, Intermodulation, Spurious response

Agreement:

- The requirement for 7 MHz is the same requirement than the requirement for 5 and 10 MHz. 7 MHz shall be added to:

  - Tables 7.5-1, 7.5-3 and 7.5-4 (ACS).

  - Table 7.6.2-1 (In-band blocking).

  - Table 7.6.3-1 (Out of band blocking).

  - Table 7.6.4-1 and evaluate Pw (Narrow band blocking).

  - Table 7.8.2-1(Intermodulation) and Table 7.7-1(Spurious response).

#### BS RF

BS type

Agreement:

- Conducted requirements shall be covered within this WI

Open issue:

- FFS if BS OTA requirements should also be covered within this WI.

Multi-carrier and CA

Agreement:

- Muti-carrier (including MSR) and CA related requirements are considered for new added 7MHz WID but no new BCS is specified within this WI.

IAB and NCR

Agreement:

- 7 MHz should not be supported by IAB

- 7 MHz should not be supported by NCR

#### Work effort split

According to the Work Plan, only Draft CRs for BS and UE specifications for generic requirements (TS 38.1010-1, TS 38.104, TS 37.104 and [TS 37.105]) should be provided for the RAN4#114bis meeting.

|  | Companies |
| --- | --- |
| TS 38.104 | Ericsson |
| TS 38.141-1 | Nokia |
| [TS 38.141-2] | Huawei |
| TS 38.101-1 | CR responsible: Qualcomm Sub-parts split (draft CRs only): System parameters: Nokia Tx: Huawei Rx and Annexes (if needed): Qualcomm |
| TS 38.101-4 | Qualcomm |
| TS 38.133 | ZTE |
| TS 38.307 | China Telecom |
| TS 37.104 | ZTE |
| TS 37.141 | ZTE |
| TS 38.106 (*) | Ericsson |
| TS 38.115-1 (*) | CATT |
| [TS 38.115-2] (*) | CATT |
| TS 38.174 (*) | Nokia |
| TS 38.176-1 (*) | Nokia |
| [TS 38.176-2] (*) | Huawei |
| TS 38.113 | ZTE |
| TS 37.113 | Ericsson |
| [TS 37.105] | Huawei |
| [TS 37.145-1] | Huawei |
| [TS 37.145-2] | Huawei |
| (*) The NCR and IAB specs would need to be updated to clarify 7 MHz channel BW is not supported. Note: TS in [] are pending on agreement on OTA BS requirements to be in the scope or not. |  |

## In RAN4#114-bis, the following agreements were made.

#### General

Initial BWP

Agreement:

- There is no restriction on the initial BWP size.

#### UE RF

NS_12, NS_13, NS_14 and NS_15 – A-MPR and frequency carrier Fc

Agreement:

- Evaluate A-MPR for:

  - NS_12 considering 7 MHz deployed within 814-824 MHz

  - NS_13 considering 7 MHz deployed in 817-824 MHz (Fc = 820.5 MHz).

  - NS_14 considering 7 MHz deployed within 824-849 MHz

  - NS_15 considering 7 MHz deployed within 814-845 MHz or within 841.5-849 MHz.

REFSENS for n5

Agreement:

- 7 MHz channel BW REFSENS for n5: -96.5 dBm value (25 RBs uplink configuration).

REFSENS for n26

Agreement:

- 7 MHz channel BW REFSENS for n26: -96.0 dBm value (25 RBs uplink configuration).

Remaining open issue:

- A-MPR requirement

#### BS RF

OTA requirements

Agreement:

- Specify OTA requirements for 7 MHz channel BW.

FRCs

Agreement:

- Reuse existing FRC(s) for 7MHz BS testing.

Output power dynamics

Agreement:

- Output power dynamics

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 7 | 15.4 | N/A | N/A |

Output power dynamics – NB-IoT in-band

Agreement:

- Output power dynamics – NB-IoT in-band

| BS channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 7, 10 | Any | +6 |

ACLR

Agreement:

- For 7 MHz channel BW, the ACLR limit in non-contiguous spectrum or multiple bands and the CACLR limit shall be the same ones as for 5 MHz channel BW.

REFSENS

Agreement:

- Specify REFSENS for 7 MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -101.7 | -96.7 | -93.7 |

Dynamic range

Agreement:

- Specify Dynamic Range for 7MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA | WA | MR | LA |
| 7 | 15 | G-FR1-A2-1 (Note 1) | -70.7 | -65.7 | -62.7 | -81.0 | -76.0 | -73.0 |

Dynamic range for NB-IoT operation in NR in-band

Agreement:

- Specify Dynamic Range for NB-IoT operation in NR in-band and for 7MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 7 | FRC A15-1 in Annex A.15 in TS 36.104 [13] | -99.7 | -81 | AWGN |
| 7 | FRC A15-2 in Annex A.15 in TS 36.104 [13] | -105.6 | -81 | AWGN |

ACS

Agreement:

- Specify ACS for 7MHz channel BW according to the following tables:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3 | PREFSENS + 8 dB | Wide Area BS: -52 Medium Range BS: -47 Local Area BS: -44 |
| 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB |  |

Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 7, 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |

In-band general blocking

Agreement:

- Specify In-band general blocking for 7MHz channel BW according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20 | PREFSENS + x dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |

Narrowband blocking - limit

Agreement:

- Specify narrowband blocking limit for 7MHz channel BW according to the following tables:

Table 7.4.2.2-2: Base Station narrowband blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area BS: -49 Medium Range BS: -44 Local Area BS: -41 |

Table 7.4.2.2-2a: Base Station narrowband blocking requirement for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50 | PREFSENS + x dB (Note 2) | Wide Area: -49 Medium Range: -44 Local Area: -41 |

Narrowband blocking – interferer

Agreement:

- Specify narrowband blocking interferer for 7MHz channel BW according to the following table:

Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 7, 10 | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |

Intermodulation interferer

Agreement:

- Specify intermodulation interferer for 7MHz channel BW according to the following table:

Table 7.7.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 7 | ±7.45 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

Narrowband intermodulation interferer

Agreement:

- Specify narrowband intermodulation interferer for 7MHz channel BW according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 7 | ±355 | CW |
|  | ±1240 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

EVM

Agreement:

- Specify EVM window length for 7MHz channel BW according to the following table:

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 7, 10 | 1024 | 72 | 28 | 40 |

Remaining Open issues:

- ICS requirement

## In RAN4#115, the following agreements were made.

#### UE A-MPR

NS_12: A-MPR regions

Agreement:

- The following NS_12 A-MPR regions definition was agreed. The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤1.8 | ≤0.72 | [A2] |
|  |  | ≤1.8 | >0.72 | [A1] |
|  |  | >1.8 | >1.44 | [A1] |

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

NS_13: A-MPR regions

Agreement:

- The following NS_13 A-MPR regions definition was agreed. The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | [A1] |
|  | 820.5 = Fc | ≤1.8 | ≥1.44 | [A2] |
|  |  | >1.8 | ≥1.8 | [A7] |

| Modulation/Waveform | A1 | A2 | A7 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 4.5 |

NS_14: A-MPR

Agreement:

- No A-MPR is needed for NS_14 and 7 MHz channel bandwidth.

NS_15: A-MPR regions

Agreement:

- The following NS_14 A-MPR regions definition was agreed. The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | [A1 or A7] |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | [A2] |
|  |  | ≤ 1.44 | ≤ 0.36 | [A3] |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | [A2] |
|  |  | ≥ 5.94 | ≤ 2.7 | [A3 or A8] |

| Modulation/Waveform | A1 | A2 | A3 | A7 | A8 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 7 | ≤ 2.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 8.5 | ≤ 3 |

A-MPR regions for PC2

Agreement:

- The following PC2 NS_12 A-MPR regions definition was agreed as starting point, meaning the exact borders might be further tuned in next meeting but the general shape of each region should be unchanged.

- The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | [A3] |
|  |  | ≤1.8 | >0.72 | [A1] |
|  |  | >1.8 | >1.44 | [A1] |

Table 3: A-MPR for NS_12 (PC2), from [3]

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.5 | 3 | ≤ 5.5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 | 3.5 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 | 4 | ≤ 6.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 5.0 | ≤ 6.5 |
| CP-OFDM QPSK | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 16 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 64 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 256 QAM | ≤ 9.5 | 6.5 | ≤ 7.5 |

NS_13: A-MPR regions for PC2

Agreement:

- The following PC2 NS_13 A-MPR regions definition was agreed as starting point, meaning the exact borders might be further tuned in next meeting but the general shape of each region should be unchanged.

- The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | [A1] |
|  | 820.5 ≤ Fc | ≤1.8 | ≥1.44 | [A2] |
|  |  | >1.8 | ≥1.8 | [A1] |

Table 6: A-MPR for NS_13 (Power Class 2), from [3]

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 5 | ≤ 7 |  |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

NS_14: A-MPR for PC2

Agreement:

- No A-MPR is needed for NS_14 PC2 and 7 MHz channel bandwidth.

NS_15: A-MPR regions for PC2

Agreement:

- The following PC2 NS_15 A-MPR regions definition was agreed as starting point, meaning the exact borders might be further tuned in next meeting but the general shape of each region should be unchanged.

- The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.68 | >0 | [A1] |
|  |  | <4.68, ≥2.52 | ≥2.16 | [A2] |
|  |  | <4.68, ≥2.52 | <2.16, ≥1.44 | [A2] |
|  |  | <1.8 | ≤0.36 | [A3] |
|  | 837 < Fc ≤ 841.5 | ≥0 | ≥2.7 | [A2] |
|  |  | ≥5.94 | <2.7 | [A3] |

Table 10: A-MPR for NS_15 (Power Class 2), from [3]

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 |

#### BS RF

Narrowband blocking – interferer offset

Agreement:

- The narrowband blocking interferer offset for 7MHz channel BW is 400kHz.

ICS

Agreement:

- The size of the interferer for ICS and 7MHz channel BW is15 RBs

#### RRM performance requirements:

Agreement:

- No need to define new RRM performance requirements to support 7MHz CBW.

- No RRM specification impacts for this WI.

#### UE Demodulation performance requirements

PDSCH requirements:

Agreement:

- Don’t define 7MHz for PDSCH requirements for single carrier

- Define normal PDSCH type A CA requirements for 7MHz

- Not to consider HST-SFN and HST-DPS scenario.

- Interested companies are welcome to bring results for 7MHz.

- FFS how to define requirements:

  - Option 1: Define new requirements for 7MHz CC based on the simulations

  - Option 2: Reuse existing 5MHz/10MHz requirements

Simulation assumptions for 7MHz CC for CA requirements

Agreement:

- Duplex mode: Only FDD

- Bandwidth/SCS: 7MHz/15kHz

- Number of RB for CORESET:30

- Number of RB: 35PRBs

- MCS: 13

- Rank: Rank2

- MCS: 16QAM,0.48

- Propagation conditions: TDLA30-10

- Antenna configuration: 2T2R, 2T4R ULA Low

- Other parameters: Reuse Table 5.2.1 in TS 38.101-4

SDR requirements

Agreement:

- Define SDR requirements for 7MHz bandwidth

  - For Table 5.5A-1 in TS 38.101-4, add the following in TS38.101-4 for Number of PDCCH candidates and aggregation levels: “2/AL2 for 15kHz / 7MHz”.

  - For Table 5.5A-4 in TS 38.101-4, set the number of CORESET PRBs to ‘30’.

CSI requirements

Agreement:

- Don’t define CQI requirements for 7MHz for single carrier

- Define CQI requirements for 7MHz for CA

- Set CQI subbandSize to ‘8’ for 7MHz CBW in TS 38.101-4 Table 6.2A.3.1.1-2.

PDCCH and PBCH requirements

Agreement:

- Don’t define PDCCH and PBCH requirements for 7MHz

Release independence

Proposal

- Option 1: RAN4 to follow the discussion on release independent for support of 7MHz in RF session, once it is agreed, it should also apply to demodulation and CSI requirements.

#### BS demodulation requirements

Agreement:

- Don’t introduce BS demodulation requirements for 7MHz

- Don’t update applicability rules for BS supporting different bandwidth.

CR split

Companies are welcome to bring the CRs according to following CR splitting table next meeting.

| CR/simulation summary | Content | Company |
| --- | --- | --- |
| PDSCH requirements with CA for 7MHz | Capture the requirements table for 7MHz for both 2Rx and 4Rx. Capture the number of RB for CORESET in common parameter table | Nokia |
| FRC for 7MHz | Capture the 7MHz FRC | Apple |
| CQI and SDR requirements for 7MHz | Update the test setup for CQI and SDR requirements for 7MHz | CTC |
| Simulation summary for 7MHz | Spread sheet for simulation summary | ZTE |
| Big CR on introducing demodulation and CSI requirements for 7MHz | Big CR | Huawei |

## In RAN4#116, the following agreements were made

#### System parameters – Release independence

Agreement:

- In single carrier, 7 MHz can’t be the lowest channel BW in a band.

- In CA, 7MHz can be min/max CBW of one band in the combination supporting BCS5.

- In MSD, 7MHz cannot be a minimum channel BW in CA.

Agreement:

- Remove the texts “Outer/Inner”, “Outer” and “Edge/Inner” from the NS_12, NS_13, NS_14 and NS_15 A-MPR tables

- RAN4 can further discuss clarification on the general statement on RB allocation for the aforementioned NS values.

#### UE A-MPR

NS_12: A-MPR regions

Agreement:

- The following NS_12 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤1.8 | ≤0.72 | A3 |
|  |  | ≤1.8 | >0.72 | A4 |
|  |  | >1.8 | >1.44 | A5 |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | ≤ 3.5 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | ≤ 4 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 4 | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 | ≤ 4.5 | ≤ 5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 | ≤ 5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | ≤ 5.5 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | ≤ 5.5 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 | ≤ 6 | ≤ 7 | ≤ 6.5 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |  | ≤ 7 | ≤ 6.5 |

NS_13: A-MPR regions

Agreement:

- The following NS_13 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A5 |
|  | 820.5 = Fc | ≤1.8 | ≥1.44 | A6 |
|  |  | >1.8 | ≥1.8 | A7 |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 | ≤ 3.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 | ≤ 3.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 | ≤ 4.5 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  | ≤ 5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 6 | ≤ 6 | ≤ 5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  | ≤ 8 | ≤ 8 | ≤ 6.5 |

NS_15: A-MPR regions

Agreement:

- The following NS_15 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A7 |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | A2 |
|  |  | ≤ 1.44 | ≤ 0.36 | A8 |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | A9 |
|  |  | ≥ 5.94 | ≤ 2.7 | A10 |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner |  |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | ≤ 8.5 | ≤ 5 | ≤ 4 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | ≤ 10 | ≤ 9 | ≤ 5 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 4 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  | ≤ 11 | ≤ 9.5 | ≤ 6.5 | ≤ 5 |

A-MPR regions for PC2

Agreement:

- The following PC2 NS_12 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | A4 |
|  |  | ≤1.8 | >0.72 | A5 |
|  |  | >1.8 | >1.44 | A6 |

Table 3: A-MPR for NS_12 (PC2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.5 | 3 | ≤ 5.5 | ≤ 4 | ≤ 6 | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | 3 | ≤ 6 | ≤ 4.5 | ≤ 6 | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 | 3.5 | ≤ 6 | ≤ 4.5 | ≤ 6.5 | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 | 4 | ≤ 6.5 | ≤ 5 | ≤ 6.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 5.0 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM QPSK | ≤ 8.5 | 4.5 | ≤ 7.5 | ≤ 6 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 | ≤ 6 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 | ≤ 6 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM 256 QAM | ≤ 9.5 | 6.5 | ≤ 7.5 | ≤ 7 | ≤ 8 | ≤ 7.5 |

NS_13: A-MPR regions for PC2

Agreement:

- The following PC2 NS_13 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A5 |
|  | 820.5 ≤ Fc | ≤1.8 | ≥1.44 | A6 |
|  |  | >1.8 | ≥1.8 | A7 |

Table 6: A-MPR for NS_13 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 5.5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 5.5 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 | ≤ 4.5 | ≤ 5.5 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 | ≤ 4.5 | ≤ 6 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 5 | ≤ 7 |  |  | ≤ 5.5 | ≤ 6 | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 | ≤ 6 | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 | ≤ 6.5 | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  | ≤ 8 | ≤ 8 | ≤ 8 |

NS_15: A-MPR regions for PC2

Agreement:

- The following PC2 NS_15 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.68 | >0 | A8 |
|  |  | <4.68, ≥2.52 | ≥2.16 | A9 |
|  |  | <4.68, ≥2.52 | <2.16, ≥1.44 | A10 |
|  |  | <1.8 | ≤0.36 | A11 |
|  | 837 < Fc ≤ 841.5 | ≥0 | ≥2.7 | A12 |
|  |  | ≥5.94 | <2.7 | A13 |

Table 10: A-MPR for NS_15 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 | A12 | A13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |  |  |  |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 0 | ≤ 3.5 | 11 | 6 | 4.5 | 7.5 | 5.5 | 3.5 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 | ≤ 4 | 11.5 | 6.5 | 5 | 8.5 | 5.5 | 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4.5 | ≤ 4.5 | 11.5 | 6.5 | 5 | 8.5 | 5.5 | 4 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 5 | ≤ 5 | 11.5 | 6.5 | 5 | 8.5 | 5.5 | 4 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 | ≤ 5 | 12.5 | 6.5 | 5 | 9 | 5.5 | 5.5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 | ≤ 6 | 13 | 8 | 6 | 10.5 | 7 | 5 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 | ≤ 6 | 13 | 8.5 | 6 | 10.5 | 7 | 5 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 | ≤ 6 | 13 | 8.5 | 6 | 10.5 | 7 | 5 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 | ≤ 7 | 13.5 | 8.5 | 7 | 10.5 | 7.5 | 7 |

#### UE demodulation and CSI requirements

How to define requirements

Agreement:

- RAN4 to define new requirements for 7MHz for CA with new simulation results.

Release independency

Agreement:

- 7MHz performance requirements are released independent from Rel-15.

CR split

| CR/simulation summary | Content | Company |
| --- | --- | --- |
| PDSCH requirements with CA for 7MHz | Capture the requirements table for 7MHz for both 2Rx and 4Rx. Capture the number of RB for CORESET in common parameter table | Nokia |
| FRC for 7MHz | Capture the 7MHz FRC | Apple |
| CQI and SDR requirements for 7MHz | Update the test setup for CQI and SDR requirements for 7MHz | CTC |
| Simulation summary for 7MHz | Spread sheet for simulation summary | ZTE |
| Introduction of release independency for 7MHz performance requirements | Introduce release independency for 7MHz requirements in TS 38.307 | Ericsson |
| Big CR on introducing demodulation and CSI requirements for 7MHz | Big CR | Huawei |

## 2.5	RAN5

N/A.

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

N/A.

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

#### 4.1	List of RAN WG1 contributions

#### 4.2	List of RAN WG2 contributions

#### 4.3	List of RAN WG3 contributions

#### 4.4	List of RAN WG4 contributions

#### 4.4.1	RAN4#114

R4-2500103	On BS specification impact due to introduction of 7MHz channel bandwidth	Nokia

R4-2500310	On the introduction of 7MHz CBW	Apple

R4-2500408	Introduction of 7MHz channel bandwidth: System parameters and RF requirements	Huawei, HiSilicon

R4-2500420	Inputs to 7MHz CBW	Skyworks Solutions Inc.

R4-2500496	7 MHz Channel Bandwidth for Bands n26 and n5	Qualcomm Incorporated

R4-2500557	Discussion on system parameters and UE RF requirements to introduce 7MHz for n5 and n26	ZTE Corporation, Sanechips

R4-2500558	Discussion on BS RF core requirements for supporting 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2500559	Draft CR to TS38.104 Introduction of 7 MHz NR FR1 channel bandwidth for NR bands n26 and n5	ZTE Corporation, Sanechips

R4-2500560	Draft CR to TS38.141-1 Introduction of 7 MHz NR FR1 channel bandwidth for NR bands n26 and n5	ZTE Corporation, Sanechips

R4-2500561	Draft CR to TS38.141-2 Introduction of 7 MHz NR FR1 channel bandwidth for NR bands n26 and n5	ZTE Corporation, Sanechips

R4-2500679	Topic summary for [114][127] NR_FR1_7MHz_BW	Moderator(Ericsson)

R4-2500695	Work plan on 7MHz Channel Bandwidth for n26 and n5	China Telecom,T-Mobile USA

R4-2501059	Discussion on release independence for 7MHz channel bandwidth	CATT

R4-2501060	Discussion on spectrum utilization and UE RF requirements for 7MHz channel bandwidth	CATT

R4-2501061	Discussion on BS RF requirements for 7MHz channel bandwidth	CATT

R4-2501361	7 MHz channel BW: system parameters and UE RF impacts	Ericsson

R4-2501363	7 MHz channel BW: BS RF impacts	Ericsson

R4-2501479	Discussion on UE RF requirements and system parameters for 7MHz 	MediaTek (Wuhan) Inc.

R4-2501824	Discussion on system parameter and UE RF aspects of 7 MHz channel bandwidth	Nokia

R4-2502133	BS specifications missing in the WID	Huawei, HiSilicon

R4-2502134	BS RF impact for 7 MHz Channel Bandwidth for n26 and n5	Huawei, HiSilicon

R4-2502199	Release Independence of the 7 MHz channel bandwidth	T-Mobile USA, China Telecom, Ericsson

R4-2502200	Spectrum Utilization for the 7 MHz channel bandwidth	T-Mobile USA

R4-2502870	WF on requirements for 7MHz channel bandwidth	Ericsson

R4-2503017	LS on Signalling for 7 MHz Channel Bandwidth	T-Mobile USA

#### 4.4.2	RAN4#114-bis

R4-2503192	On the introduction of 7MHz CBW	Apple

R4-2503232	n26 7 MHz A-MPR simulations and proposal	Nokia

R4-2503234	On BS specification impact due to introduction of 7MHz channel bandwidth	Nokia

R4-2503262	Discussion on initial BWP size restriction to 5 MHz for 7MHz Channel Bandwidth	China Telecom

R4-2503276	draftCR for Tx RF requirements for introduction of 7MHz	Huawei Technologies France

R4-2503473	Clarification on the usage of 7MHz channel bandwidth	CATT

R4-2503474	Further discussion on UE RF requirements for 7MHz channel bandwidth	CATT

R4-2503475	Further discussion on BS RF requirements for 7MHz channel bandwidth	CATT

R4-2503642	Discussion on A-MPR of 7MHz for band n26	ZTE Corporation, Sanechips

R4-2503643	Discussion on BS RF requirements for supporting 7 MHz channel bandwidth	ZTE Corporation, Sanechips

R4-2503950	7 MHz channel BW: UE RF and system parameters	Ericsson

R4-2503951	7 MHz channel BW: BS RF impacts	Ericsson

R4-2504261	Discussion on UE RF requirements for 7MHz CBW	MediaTek (Wuhan) Inc.

R4-2504484	Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth	Qualcomm Incorporated

R4-2504588	OTA requirements consideration for network-nodes specifications for 7MHz channel introduction	Huawei, HiSilicon

R4-2504680	Topic summary for [114bis][122] NR_FR1_7MHz_BW	Moderator(Ericsson)

R4-2505098	WF on UE and BS RF requirements for 7MHz channel bandwidth	Ericsson

R4-2505136	Introduction of 7 MHz channel bandwidth for n26 and n5	Nokia

R4-2505137	Draft CR to TS37.104: Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2505138	Draft running CR to TS 38.104 - Introduction of 7 MHz channel BW	Ericsson

R4-2505233	Draft CR to TS 37.105: 7MHz channel bandwidth introduction	Huawei, HiSilicon

#### 4.4.3	RAN4#115

R4-2505387	Discussion on demodulation aspects for NR FR1 7 MHz Channel Bandwidth	China Telecom

R4-2505428	On the introduction of 7MHz CBW	Apple

R4-2505496	Views on NR FR1 7MHz CBW for UE Demod	Qualcomm Incorporated

R4-2505557	Topic summary for [115][208] NR_FR1_7MHz_BW	Moderator (China Telecom)

R4-2505719	A-MPR requirements for 7MHz channel bandwidth	Huawei Technologies France

R4-2505803	On BS RF requirements due to introduction of 7MHz channel bandwidth	Nokia

R4-2505900	On UE Demodulation and CSI Reporting Requirements for 7MHz	Apple

R4-2505972	View on demodulation requirement for 7MHz	Samsung

R4-2506062	Further discussion on UE RF requirements for 7MHz channel bandwidth	CATT

R4-2506063	Further discussion on BS RF requirements for 7MHz channel bandwidth	CATT

R4-2506149	7 MHz channel BW: UE RF and system parameters	Ericsson

R4-2506150	7 MHz channel BW: BS RF impacts	Ericsson

R4-2506151	Running CR to TS 38.104 - Introduction of 7 MHz channel BW	Ericsson

R4-2506153	CR to TS 37113 - Introduction of 7 MHz channel BW	Ericsson

R4-2506318	Discussion on A-MPR of 7MHz for band n26	ZTE Corporation, Sanechips

R4-2506323	Discussion on BS RF requirements for supporting 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2506325	Draft CR to TS38.113: Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2506330	Discussion on RRM performance requirements for NR FR1 7 MHz Channel Bandwidth	China Telecom

R4-2506444	Discussion on UE_BS demodulation performance requirements for 7 MHz CBW	Nokia

R4-2506518	n26 7 MHz A-MPR simulations and proposals	Nokia

R4-2506634	Discussion on Rel-19 NR FR1 7 MHz Channel Bandwidth performance requirements	Ericsson

R4-2506675	Discussion on UE demodulation and CSI performance requirements for 7MHz CHBW	Samsung

R4-2506780	Discussion on performance requirements for 7MHz	Huawei, HiSilicon

R4-2506952	Discussion on UE and BS demodulation and CSI performance requirements for 7MHz channel bandwidth	ZTE Corporation, Sanechips

R4-2507031	Discussion on UE and BS performance requirements for 7MHz	Huawei, HiSilicon

R4-2507032	Work plan on UE and BS performance requirements for 7MHz	Huawei, HiSilicon

R4-2507157	RRM impact of 7 MHz channel bandwidth	Ericsson

R4-2507437	Discussion on BS RF in-channel selectivity requirement for 7 MHz channel bandwidth	Huawei, HiSilicon

R4-2507566	Topic summary for [115][127] NR_FR1_7MHz_BW	Moderator (Ericsson)

R4-2507651	Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth	Qualcomm Incorporated

R4-2507652	7 MHz Channel Bandwidth for n5 and n26	Qualcomm Incorporated

R4-2507653	PC2 UE : Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth	Qualcomm Incorporated

R4-2507867	Topic summary for [115][335] NR_7MHz_demod	Moderator (Huawei)

R4-2507880	RRM requirements for 7MHz Channel BW	Qualcomm Incorporated

R4-2507918	draftCR to TS 38.115-1 on 7MHz channel bandwidth	CATT

R4-2507920	CR to TS 38106 - 7 MHz channel BW clarification	Ericsson

R4-2507922	Draft CR to TS37.141: Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2507923	Draft CR to TS 37.105: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507924	Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507925	Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507926	Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507927	Draft CR to TS 38.176-2: restriction of 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507954	WF on NR_FR1_7MHz_BW	Ericsson

R4-2508088	Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth	T-Mobile USA

R4-2508260	WF on RRM requirements for NR_FR1_7MHz_BW	China Telecom

R4-2508722	Offline meeting minutes for [115][335] NR_7MHz_demod	Huawei

R4-2508757	Way Forward for [115][335] NR_7MHz_demod	Huawei

#### 4.4.4	RAN4#116

R4-2509094	Topic summary for [116][127] NR_FR1_7MHz_BW	Moderator (Ericsson)

R4-2509143	Draft CR on CQI and SDR requirements for 7MHz	China Telecom

R4-2509144	Discussion and simulation results for UE demodulation requirements for 7MHz	China Telecom

R4-2509389	CR to TS 38.115-1 on 7MHz channel bandwidth	CATT

R4-2509390	draftCR to TS 38.104 on ACLR and blocking requirements for 7MHz channel bandwidth	CATT

R4-2509476	On UE Demodulation and CSI Reporting Requirements for 7MHz	Apple

R4-2509477	Draft CR on FRC for 7MHz CBW	Apple

R4-2509544	On the introduction of 7MHz CBW	Apple

R4-2509578	Big CR on 7 MHz Channel Bandwidth for bands n5 and n26	Qualcomm Incorporated

R4-2509579	7 MHz Channel Bandwidth A-MPR Analysis for band n26	Qualcomm Incorporated

R4-2509857	Views on NR FR1 7MHz CBW for UE Demod	Qualcomm Incorporated

R4-2509942	n26 7 MHz A-MPR proposals	Nokia

R4-2510405	CR to TS37.104 Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2510406	CR to TS37.141 Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2510407	CR to TS38.113 Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2510427	Discussion on UE_BS demodulation performance requirements for 7 MHz CBW	Nokia

R4-2510436	Discussion on UE demodulation and CSI performance requirements for 7MHz CHBW	Samsung

R4-2510464	PC2 A-MPR requirements for 7MHz channel bandwidth	Huawei, HiSilicon

R4-2510510	7 MHz channel BW: UE A-MPR results	Ericsson

R4-2510512	CR to TS 38106 -  7 MHz channel BW clarification	Ericsson

R4-2510513	CR to TS 37113 - Introduction of 7 MHz channel BW	Ericsson

R4-2510832	UE demodulation requirements for 7MHz CBW	Ericsson

R4-2510892	Discussions on demodulation requirements for 7 MHz	Huawei,HiSilicon

R4-2510981	Discussion on UE and BS demodulation and CSI performance requirements for 7MHz channel bandwidth	ZTE Corporation, Sanechips

R4-2510982	Simulation summary for 7MHz	ZTE Corporation, Sanechips

R4-2511315	Release independence of 7 MHz channel bandwidth	Nokia

R4-2511479	Topic summary for [116][333] NR_7MHz_demod	Moderator (Huawei)

R4-2511540	CR to TS 37.105: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2511541	CR to TS 37.145-1: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2511542	CR to TS 37.145-2: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2511544	CR to TS 38.176-2: restriction of 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2512618	(NR_FR1_7MHz_BW-Perf) DraftCR on 38.101-4 for adding UE performance requirements for 7 MHz CBW	Nokia

R4-2512619	Way Forward for [116][333] NR_7MHz_demod	Huawei

R4-2512824	Big CR on 7 MHz Channel Bandwidth for bands n5 and n26	Qualcomm

R4-2512829	CR to 38.141-1 on introduction of RF requirements for 7MHz channel bandwidth	Nokia

R4-2512830	Running CR to TS 38.104 - Introduction of 7 MHz channel BW	Ericsson

R4-2512831	CR to TS 38.141-2: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2512850	Release independence of 7 MHz channel bandwidth	Nokia