3GPP TSG RAN #109			RP-252900

Beijing, China, Sep 15th – 18th, 2025

Title:	Moderator's summary for RAN4 REL-20 5G-Adv: UE RF topics

Source:	Moderator (vivo)

Introduction

This document summarizes the discussion of RAN4 candidate UE RF topics for R20 under agenda 9.1.4.1 in RAN #109 meeting.

In RAN #108 meeting, a high-level scope of the WI was endorsed in RP-251879 as below

The high-level scope of the WI on UE RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - FR1 HPUE enhancement - Further consider/discuss until RAN#109 on the detailed scope - This work is to focus on the TN application - 6MHz channel bandwidth - It can be prioritized to FDD band <1GHz - Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for5GNRhigher modulation orders, i.e., 64QAM, 256QAM - BS demod scope is included. - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact - The details for study is to be discussed until RAN#109 - This work will be started in Q1’26. If the similar scope (The examples include the enhancements enabled by BS advanced receiver and UE Tx signal modifications. It is noted that referring to these examples in this WF should not be deemed as taking these examples as the baseline or starting point in 6G study) is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, it is possible that 6G related conclusion can be applied to 5G. - No RAN1/2 scope is expected - Target 0.5 RF TU in Q4’25 and 1.25 TU (0.75 RF and [0.5] Demod for EVM relaxation and 6MHz) from Q1’26.

Submitted contributions in RAN #109 for agenda 9.1.4.1 are listed as below:

| RP-252033 | Views on Rel-20 UE RF topics for 5G-A | ZTE Corporation, Sanechips |
| --- | --- | --- |
| RP-252066 | Views on 5G-A Rel-20 UE RF topics | Samsung |
| RP-252155 | Scope consideration on some R20 UE RF centric items | Spreadtrum, UNISOC |
| RP-252189 | Views on Rel-20 UE RF in 5G-A | vivo |
| RP-252369 | RAN4 R20 UE RF enh | OPPO |
| RP-252451 | Views on the UE RF enhancements for Rel-20 5G-A | Apple |
| RP-252483 | Qualcomm views on UE RF topics for 5GA Rel-20 | Qualcomm Incorporated |
| RP-252484 | General enhancements for high power UE | Qualcomm Incorporated |
| RP-252512 | Rel-20 RAN4 UE RF | MediaTek Inc. |
| RP-252524 | Views on 5G-A Rel-20 UE RF topics | LG Electronics Deutschland |
| RP-252534 | Views on UE RF topics for RAN4 Rel-20 | China Telecom |
| RP-252554 | Views on Rel-20 UE RF topics for 5G-A | China Unicom |
| RP-252567 | Proposal on HPUE scope in UE RF project | Xiaomi, Oppo, Qualcomm, ZTE, China Telecom, China Unicom, CMCC, T-Mobile USA, Telus, Verizon |
| RP-252570 | Views of RAN4 5G-A UE RF topics | Ericsson |
| RP-252609 | Views on Rel-20 UE RF Topics | CMCC |
| RP-252616 | Relaxed Tx EVM requirement under network control | Vodafone Romania S.A. |
| RP-252658 | Considerations on RAN4-led Rel-20 5G-Adv UE RF topics | Nokia |
| RP-252703 | Views on the objectives of the RAN4 Rel.20 UE RF centric evolution WI | CHTTL |
| RP-252710 | Views on UE RF works for Rel-20 5G-A | CATT |
| RP-252738 | Proposals for RAN4-led Rel-20 UE RF topics | Huawei, HiSilicon |
| RP-252744 | Proposals for power enhancement with relaxed UE Tx EVM and advanced BS receiver | Huawei, HiSilicon, Telefónica, Verizon, Telus, CMCC, China Telecom, China Unicom, CBN, China Broadnet, Deutsche Telekom AG, Türk Telekom, OPPO, vivo, Xiaomi, Samsung, Spreadtrum, Spark NZ, Telia Company, CHTTL |
| RP-252749 | Views on RAN4 Rel-20 5G-A - UE RF topics | Intel Corporation |
| RP-252760 | Views on RAN4 Rel-20 5G-A UE RF topics | T-Mobile USA Inc. |

FR1 HPUE enhancement

Summary of FR1 HPUE

|  | Proposed objectives for Rel-20 | Supportive companies |
| --- | --- | --- |
| 1 | PC1 TDD with 4Tx | - Specify the requirements for PC1 single carrier with 4Tx for FWA devices (ZTE, Samsung, Spreadtrum, vivo, OPPO, QC, MTK, LG, CMCC, Nokia(32dBm), CATT (32dBm), Intel, T-Mobile USA) - Co-existence study for ACLR (QC, Nokia) - Example bands: n41 (Samsung, Spreadtrum), n77/n78(ZTE, Spreadtrum), n79(ZTE), n104 (ZTE) |
| 2 | PC1.5 TDD with 1Tx | - Study and specify if the feasibility is confirmed, PC1.5 TDD band single carrier with 1Tx for both handheld and FWA device (ZTE, vivo, OPPO, Apple, QC, MTK, Xiaomi, Oppo, China Telecom, China Unicom, CMCC, T-Mobile USA, Telus, Verizon, Huawei (FWA/CPE)) - Whether to extend PC1.5 with 2Tx requirement (without change) to PC1.5 with 1Tx (MTK) - Example bands: n41 n77/n78, n79, n104 (ZTE, CATT) |
| 3 | PC1 TDD with 2Tx | - Study and specify if the feasibility is confirmed, PC1 TDD band single carrier with 2Tx for FWA device (ZTE, OPPO, QC, MTK, T-Mobile USA) - Co-existence study for ACLR (QC) - Whether to extend PC1 with 4Tx requirement (without change) to PC1 with 2Tx (MTK) - Example bands: n41, n77/n78, n79, n104 (ZTE) |
| 4 | PC1.5 FDD with 2Tx | - Specify the requirements for PC1.5 FDD band single carrier with 2Tx for both FWA and handheld (ZTE, Samsung, vivo, OPPO, Apple, MTK, CMCC, Nokia, Huawei, Intel (Handheld), T-Mobile USA, ) - No co-existence study needed (vivo) - Example bands: n28 (ZTE, Samsung), n1(ZTE, China Unicom, Huawei), n3 (ZTE, Huawei), one band around 2GHz (CATT) |
| 5 | PC1.5 TDD/FDD with 3Tx | - Specify the requirements for PC1.5 TDD/FDD with 3Tx (OPPO, CMCC, T-Mobile USA) |
| 6 | PC1 FDD with 4Tx | - Specify the UE RF requirements to support PC1 4Tx for CPE/FWA UE with 4x26dBm PA configurations (Huawei) - Example band: n1 |
| 7 | PC1.5 FDD with 1Tx | - PC1.5 enhancements should be common for both TDD and FDD bands so that it can be enabled for terrestrial and satellite communication (Apple) |
| 8 | PC1.5 for intra-band UL CA with 2T/3T | - PC1.5 for intra-band UL CA with 2T/3T (OPPO, T-Mobile USA) - If included in Rel-20, intra-band contiguous UL CA is preferred considering the high workload for NCCA MPR/AMPR simulations and possibility of eating up all the Tus reserved for FR1 UE RF. (OPPO) |
| 9 | 2Tx + 2Tx with inter-band UL CA | - Straightforward definition based on UL CA and 2L UL MIMO (QC, LG) |
| 10 | PC2 and PC1.5 for intra-band CA with 4Tx | - Focus on maximum output power (MOP), MPR/A-MPR requirements (LG) |
| 11 | PC2 and PC1.5 for inter-band EN-DC | - Include LTE PC2 and NR carrier with UL-MIMO or Tx diversity for handheld and FWA (LG, CHTTL) |
| 12 | HPUE for CA/DC | - HPUE for CA/DC based on high power for single carrier (China Unicom) |
| 13 | PC1 with 8Tx for FWA | - PC1bis (32dBm) with 8Tx with 8-layer UL-MIMO for FWA devices |
| 14 | PC1.5 with 4Tx or 8Tx | - PC1.5 with 4Tx or 8Tx for UL intra-band and inter-band CA (T-Mobile USA) |
| 15 | PC1 with DL CA | T-Mobile USA |

Moderator’s observations

It is Moderator observation that majority companies suggest to focus on the single carrier HPUE in Rel-20 considering the overall TU budget as approved in previous meeting. Moderator suggest to discuss whether to focus on single carrier case first, i.e., objective 1-7 in above table.

Among objective for single carrier, it is also moderator observation that objective 1-4 has gathered more companies supports. Moderator suggest to confirm the guideline and focus on these objectives.

For objective 1, the value of maximum output power, i.e., existing value for PC1 31dBm or 32dBm assuming 4x26 dBm PA architecture assumption, has to be discussed if we need to proceed objective 1. Also, whether to include the co-existence study to investigate the needs of change of ACLR requirements also has to be discussed.

For both Objective 2 and 3, majority companies believe a study phase is required to focus on feasibility study. Also, RAN/RAN4 chair suggest to start the feasibility from Q4 2026 taking PC1.5 NR NTN discussion outcome into account. Moderator suggest confirming such understanding if we proceed with objective 2 and 3.

For Objective 2, moderator suggest to discuss the device type, i.e., FWA or both FWA and Handheld. To be noted, FWA is suggested in Sep RAN plenary guideline. As suggested by MTK, moderator think maybe we can discuss how to consider to extend the requirements of PC1.5 with 2Tx for PC1.5 with 1Tx in WID. Another issue is the example bands. Majority companies suggest to consider n41, n78/n77, n79. For n104, moderator think we may need to discuss it targeting on reducing the number of example bands from workload management perspective.

For objective 3, similar as objectives 1, the needs of co-existence study and example bands can be discussed.

For objective 4, device type, i.e., both FWA and handheld or Handheld only can be discussed. Also, whether to skip co-existence study can be discussed.

Offline discussions on HPUE including the following topics

- Whether to focus single carrier in R20?

- Confirm objective 1-4 to be included in R20 RAN4 package in Sep

- Detailed description of the objectives if group agree to proceed.

Summary/conclusions for HPUE topics: On objective 1 Maximum value of output power Option 1: 31dBm Option 2: 32dBm Option 3: PC1 with 32dBm Needs of co-existence study in WID. Option 1: Yes Option 2: No - Add co-existence study objective in the WID. Whether to adjust the ACLR requirements shall be decided in RAN4. - Example bands: n41, n77/n78, n79 - FFS for n104 For objective 2 and objective 3 - Example bands: Example bands: n41 n77/n78, n79 - RAN4 is supposed to study the possibility of apply the PC1.5 with 2Tx A-MPR/MPR requirements to the PC1.5 with 1Tx - FFS for n104 pending on the ongoing ACLR relaxation discussions for PC3. For objective 4 - No co-existence study needed - Example bands: n1, n3 - FSS on n28, n25, n8 Further suggestions for WID objectives based on email discussion - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, [n104] - PC1.5 1Tx for TDD bands for FWA [and handheld] UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79[, n104] - This work is to start in 2026 Q4 - PC1 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79[, n104] - This work is to start in 2026 Q4 - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n3, [n8, n25, n28] Agreement - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, [n104] - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4 - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25

6MHz Channel bandwidth

Summary of contributions on 6MHz topics

| Supporting companies | Details of objectives |
| --- | --- |
| ZTE (2033) | - Similar wording as 7MHz by the removal of demod requirement |
| Samsung (2066) | - R-19 7MHz WI as baseline |
| vivo (2189) | - Introduce 6MHz for band n5, n12, n85, n29 |
| Qualcomm (2483) | - Not define any other channel BWs up to 14MHz |
| MediaTek (2512) | - NR Example band: n5, n12, n29 and n85 - Limited 15KHz SCS |
| China Telecom (2534) | - Introduce new 6MHz bandwidth for band n5 including both RRM and Demod requirements - Introduce 11MHz as next-larger CBW |
| China Unicom (2554) | - Introduce 6MHz single carrier channel bandwidth for band n8 |
| Ericsson (2570) | - Adding 6MHz bandwidth is reasonable |
| Nokia (2658) | - As the 6 MHz channel bandwidth will impact UE RF, BS RF, RRM and demod, it would be best handled in a dedicated WI. |
| Huawei (2738) | - 6MHz CBW is considered as a separate WI, rather than part of UE RF enhancement WI for Rel-20 |
| Intel (2749) | - Take 7MHz WI as baseline - Limit to 15KHz SCS - As an optional feature from R15 |

Moderator’s Observations

It has been observed that introduction of 6MHz taking the R19 7MHz work as baseline is supported by every company in RAN #109. First of all, moderator suggest to discuss the detailed objectives based on 7MHz WID in Ran #109 meeting with the following detailed aspects

- Example bands (reference of 7MHz TR38.844/RP-242873)

- Whether to include RRM/Demod requirements -> whether a separate WI is required.

- Release independent

There are some conflicting proposals on whether to include additional BWs, e.g., 11 MHz (China Telecom), or stopping any other BW work up to 14MHz in 5G-A (QC, Intel). Moderator also suggest to leave this to further guideline from RAN/RAN4 chair without any offline discussions.

Offline discussions on 6MHz topics

Moderator suggest to focus on drafting of detailed objections for 6MHZ based on R19 7MHz work - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the RRM/Demod requirement - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Release independence shall be from Rel-15 - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] Note This should be similar to the signalling for 7 MHz channel bandwidths - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] Note This should be similar to the signalling for 7 MHz channel bandwidths NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. Further suggestion on the WID objectives based on email discussions - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the Demod requirement - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] Note This should be similar to the signalling for 7 MHz channel bandwidths - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] Note This should be similar to the signalling for 7 MHz channel bandwidths NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. Agreement - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements (OTA and conductance) for UL and DL - Specify the demod requirement - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] Note This should be similar to the signalling for 7 MHz channel bandwidths - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] Note This should be similar to the signalling for 7 MHz channel bandwidths NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

Tx EVM relaxation

Summary of contributions on Tx EVM relaxation topics

For reduce UE MPR value with relaxed Tx EVM, the further guideline from Sep RAN is as below

- Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details in 2025.Q4. - Aspects relevant for only 5G-Adv are constrained by the boundary conditions included in the endorsement from RAN#108. Additional boundary conditions may be introduced in RAN#109. - Start study on both 5G-Adv/6G UE TX EVM relaxation in 2026.Q1 and target 2026.Q3 for completion of the study aspects relevant for 5G-Adv - Decide in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv - Further details on above to be discussed in RAN#109

| Supporting companies | Details of proposals |
| --- | --- |
| ZTE (2033) | - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact - The non-linearity issue from UE transmitter side should be investigated including the realistic PA modelling, memory effect, IQ image, DC leakage, CFR, DPD etc; - The type of non-AI based receiver for EVM compensation and the BS implementation complexity; - Performance gain for this non-AI based approach compared with the legacy receiver; |
| Samsung (2066) | - The study is performed based on non-AI-based approach at BS receiver as baseline under existing and relaxed UE TX EVM requirements [RAN4] - Non-linearity models of transmission signals is studied to capture PA non-linearity and I/Q imbalance. - Specify reduced MPR with relaxed EVM requirements for high order modulations (64QAM, 256QAM) [RAN4] - The requirements for other gating factors to impact on MPR including ACLR, SEM and SE will be kept unchanged. |
| vivo (2189) | - BS demodulation requirement shall be studied to guarantee the sufficient performance gain - New MPR requirement for 64QAM and 256QAM, based on the outcome of EVM relaxation study |
| OPPO (2369) | - Tx EVM relaxation should base on 5G waveform and requirements |
| Qualcomm (2483) | - Study would have to take into account many different aspects to conclude on feasibility and possible gains - Different UE PA and nonlinear front-end component models - Different transmission BWs, number of CCs, FDD/TDD - Multiple UEs multiplexed in the same slot, possibly MU-MIMO - UL MIMO transmissions - Different fading channel models - Meeting existing emission requirements (IBE and OOBE) - gNB receiver framework - Combination with other features (e.g. FDSS) |
| MediaTek (2512) | - BS demod scope is included - The Study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact - I/Q imbalance and LO leakage in UE are not tighted in 5G-A. |
| China Unicom (2554) | - BS performance requirements to verify BS capability to compensate the non-linearity of transmission signals with modulation orders of 64QAM and 256QAM - The study is performed based on non-AI-based approach at BS receiver as baseline |
| Ericsson (2570) | - A system evaluation of the overall relevance of MPR reduction for high order modulations would be useful - It is important to base the evaluation on a realistic model for the UE RF front end - It is important to determine if there is a dependency of the potential gains on RB allocation - If the post-distortion compensation performance would differ between UEs then it would become difficult for the network to manage EVM relaxation - It has been proposed to consider non-AI based in 5G, and AI based is likely to be proposed for 6G study |
| Vodafone (2616) | - Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation order, i.e,. 64QAM, 256 QAM and enable/disable it under network control |
| CATT (2710) | - RAN to include this objective in the 5G-A package by initiating a study phase, with a checkpoint set for 26Q2 (RAN#112) to assess the outcomes and ongoing 6G studies, and determine whether follow-up normative work is necessary. |
| Huawei (2738/2744) | - Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM , 256QAM - BS demod scope is included. - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact - Note 1: This work will be started in Q1’26. If the similar scope is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, the study for 5G-A and 6G will be carried out in parallel. A check point is established upon completion to determine whether a dedicated WI for 5G-A is necessary. - Note 2: No RAN1/2 scope is expected - Note 3: the proper TU can follow the agreement in RP-251879 |
| Intel (2749) | - Evaluation of feasible Tx EVM relaxation for different UEs and assessment of associated MPR improvement. - Evaluation of achievable performance improvement incl. link-level and system-level performance and various scenarios incl. MU-MIMO. - Evaluation of BS complexity and implementation feasibility |

Moderator’s Observations

Following the guideline from RAN #109, offline discussion for EVM relaxation topics is to provide additional boundary of this study. Based on the companies’ proposal, moderator suggest to discuss the following study objectives as further guideline from RAN #109 if approved. Whether to capture such study objectives in 6G WG SI or a separate WF will follow further guideline from RAN.

- Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for higher modulation orders, i.e., 64QAM , 256QAM

  - Evaluation of achievable performance improvement from system perspective.

  - BS demod scope is included including post-distortion compensation performance.

  - The study is performed based on non-AI-based approach at BS receiver as baseline

  - Study the different UE PA and nonlinear front-end component model including the realistic PA modelling, memory effect, IQ image, DC leakage, CFR, DPD etc;

  - Study the feasibility of keeping other existing 5G-A UE Tx requirements unchanged including I/Q imbalance, LO leakage ACLR, SEM and SE

Offline discussions on EVM relaxation topics

Moderator suggest to add the following notes in 5G-A R20 UE RF WID Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. Agreement Add the following notes in 5G-A R20 UE RF WID Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

Conclusions