3GPP TSG RAN meeting #109			RP-252904

Beijing, China, September 15-18, 2025

Title:	Moderator's summary for RAN4 REL-20 5G-Adv: BS RF topics

Source:	Huawei

Agenda:	9.1.4.2

Document for:	Report

Introduction

This document summarizes the companies’ proposals of BS RF topics for RAN4 Rel-20 5G-Adavanced and recommend the objectives for the WID based on companies’ proposals and comments.

BS RF topics

Endorsed way forward in RAN#108

The following is the endorsed way forward for BS RF topics in RAN#108 meeting [1].

The high-level scope for WI on BS RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - Enhancement of co-location requirements - It is FFS on the applicable frequency ranges, e.g. >3GHz requirements, for OTA - R19 outcome should be considered - Coexistence requirement enhancement can be further discussed in RAN#109 - Down select from the following four options in RAN#109 - Additional Hybrid requirement sets for BS type 1-H/large antenna array - SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios - It is limited to RAN4 only solution - Neither - Both - No RAN1/2 impact is expected - Target 0.5RF TU in Q4’25 and 1 TU allocation from Q1’26.

General discussions of candidate objectives for BS RF topics for 5G-A Rel-20

The proposals from companies are summarized in Section 4, which is briefly highlighted below:

- Enhancement of co-location requirements

  - Support: ZTE (2034), Samsung (2067), CMCC (2610), Ericsson (2621), Nokia (2659), CATT (2711), Huawei (2739)

- SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios

  - Support: ZTE (2034), Samsung (2067), CMCC (2610), China Unicom et al (2691), CATT (2711), Huawei (2739)

  - Accept but need some limitation or modification of scope: Ericsson (2621, declared by BS vendor and TDD operation not impacted), Nokia (2659, e.g. limited to band n104 and Wide Area BS class only), ISSDU (2730)

- Additional Hybrid requirement sets for BS type 1-H/large antenna array

  - Support: Ericsson (2621)

  - De-prioritize/Move to 6G: ZTE (2034), CMCC (2610), CATT (2711), CMCC (2610), Huawei (2739)

Based on the summarized proposals above and the endorsed way forward [1] for TU status as shown below, the moderator would like to propose to take enhancement of co-location requirements and SBFD BS adjacent channel coexistence as objectives.

The overall TU status should be taking into consideration in RAN4-led R20 package discussion. - 4.5~7.5TU are available through the R20 time frame for RAN4-led non-spectrum package. - Based on the broad scope of the proposals from all companies, it is unrealistic to assume more than 1TU for any of individual RAN4-led WI (except NTN). - Majority of RAN4-led WI are expected to be allocated with no more than 0.5TU. - For the WI with 1 TU, no more than 3 objectives are expected, depending on the exact scope. - For the WI with 0.5 or less TU, no more than 2 objectives are expected, depending on the exact scope. …. The high-level scope for WI on BS RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - … - Target 0.5RF TU in Q4’25 and 1 TU allocation from Q1’26.

Moderator recommendation: it is recommended to take the following two objectives for BS RF topics in 5G-A Rel-20

- Co-location requirements enhancement:

- SBFD BS to SBFD BS adjacent channel coexistence

Companies’ comments:

| Companies | Comments |
| --- | --- |

Enhancement of co-location requirements

Based on companies’ contributions, the moderator suggests the following justification and objectives for discussions.

Justifications

According to moderator understanding, a number of motivations were proposed in the companies’ contributions. The main motivation is to improve the co-location OTA requirements for upper bands in NR FR1 due to the identified issues.

There are a number of reasons to improve the co-location requirements. Firstly, for the upper FR1 bands, the existing BS type 1-O co-location tests equipment/environment may not be easily available, and the tests may be not easily aligned among companies. Secondly, some companies questioned the existing CLTA for BS type 1-O, and accordingly proposed the enhanced test concept, e.g., based on OTA traditional parameters (TRP, EIPR and EIS). Moreover, some companies mentioned the assumption of 30dBc would not be valid for upper FR1 bands.

In moderator view, it would be reasonable to keep the general description of the justification based on the common understanding, and may not need to list the exact potential issues like 30dBc or CLTA to avoid the debate among companies due to potential different understanding.

The following text is proposed for discussions:

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

The upper bands in NR FR1 are becoming more important. A number of issues for BS type 1-O requirements were discussed in the previous releases. To make BS type 1-O more relevant for AAS BS operating in the upper bands of FR1, RAN4 need to evolve and improve the requirement concept for which the co-location requirements are based. The requirements should be defined in such a way that multi-vendor BS co-location and tower sharing at a common site can be guaranteed, and conformance tests can be reproduced.

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

Objectives

According to moderator understanding, there would mainly be a number of issues which need be discussed for the objectives. Firstly, companies proposed the different frequency range (>/=3.3GHz, or >/=4.9GHz), and one operator clearly indicate the request of no relaxation of requirements for existing bands. Secondly, regarding whether to use regular OTA parameters instead of CLTA, or other solutions, it may be better to further discuss in WI phase.

The following text is proposed for discussions:

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Core part objective:

- Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz

  - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements.

    - Consider impact on the following OTA co-location requirements:

      - Transmitter OFF power requirement

      - Transmitter co-location spurious emission requirement

      - Transmitter intermodulation requirement

      - Co-location OOBB requirement

  - Enhance OTA co-location requirements

    - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered

      - Option 1: Investigate and improve CLRA concept

      - Option 2: Investigate possibility to define requirements using existing OTA parameters, e.g., TRP, depending on the conclusion of MCL (maximum coupling loss)

      - -

Performance part objective:

- To define the conformance testing requirement of co-location requirements according to the core requirements

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

The following objectives are recommended

- Suggest to further study the co-location OTA requirements for BS type 1-H in 6G.

- The following objectives for co-location requirements are suggested

Core part objective: - Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Consider impact on the following OTA co-location requirements: - Transmitter OFF power requirement - Transmitter co-location spurious emission requirement - Transmitter intermodulation requirement - Co-location OOBB requirement - Enhance OTA co-location requirements - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered - Option 1: Investigate and improve CLRA concept - Option 2: Investigate possibility to define requirements using TRP, depending on the conclusion of MCL (maximum coupling loss) Performance part objective: - To define the conformance testing requirement of co-location requirements according to the core requirements.

Online agreements:

Core part objective: - Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Consider impact on the following OTA co-location requirements: - Transmitter OFF power requirement - Transmitter co-location spurious emission requirement - Transmitter intermodulation requirement - Co-location OOBB requirement - Enhance OTA co-location requirements - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered - Option 1: Investigate and improve CLRA concept - Option 2: Investigate possibility to define requirements using existing OTA metrics (such TRP, EIRP, EIS), if any Performance part objective: - To define the conformance testing requirement of co-location requirements according to the core requirements.

SBFD BS to SBFD BS adjacent channel co-existence

Based on companies’ contributions, the moderator suggests the following justification and objectives for discussions.

Justifications

The following text is proposed for discussions:

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

SBFD (Subband Full Duplex) is a critical BS evolution technique in the 5G-A. In Rel-19 TDD BS to SBFD BS co-existence study was conducted. And it was agreed that RAN4 do not introduce the specific requirements for SBFD BS to SBFD BS co-existence for FR1 WA or MR BS and SBFD BS to SBFD BS co-existence case can be considered for Rel-20.

Compared to TDD BS to SBFD BS co-existence case, ACIR in SBFD BS to SBFD BS co-existence case can be improved and better co-existence performance can be achieved, because of the nature of SBFD design where the SBFD downlink of operator B can be adopted as guard band for filter implementation to protect the SBFD uplink of adjacent operator A.

Therefore, in Rel-20 it is proposed to specify BS TX and RX RF requirement for SBFD-capable BS for the scenario of SBFD BS to SBFD BS adjacent channel coexistence.

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

Objectives

According to moderator understanding, there would mainly be a number of issues which need be discussed for the objectives. Firstly, one company proposed to only consider WA BS. Secondly, companies proposed the different example bands. Thirdly, one company proposed that whether the requirement to be complied depends on declaration, and that TDD operation should not be impacted. Besides, one company proposed the details for evaluations, which may be better to discuss in WI phase.

The following text is proposed for discussions:

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Core part objective:

- SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios

  - Focus on FR1 WA

  - Identify the realistic scenarios and if found realistic

  - Consider both scenarios including SBFD capable BS being co-located and non-collocated

  - Focus on bands: n104, n79, [n41].

  - Specify necessary BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include

    - TX unwanted emission

    - RX in-band blocking

  - Note 1: whether to comply with requirements is declared by BS.

  - Note 2: The existing TDD requirements apply for non-SBFD BSs

Performance part objective:

- Specify the conformance testing of core RF requirements for SBFD BS to SBFD BS coexistence.

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

The following objectives are recommended

- The following objectives for co-location requirements are suggested

Core part objective: - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated - Focus on bands: n104, n79, n41. - Specify necessary BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs Performance part objective: - Specify the conformance testing of core RF requirements for SBFD BS to SBFD BS coexistence.

Online agreements:

Core part objective: - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs Performance part objective: - Specify the conformance testing of core RF requirements for SBFD BS to SBFD BS coexistence.

Additional hybrid requirement sets for BS type 1-H/large antenna array

This section is reserved for the discussions of additional hybrid requirement sets for BS type 1-H/large antenna array, if the consensus can be reached.

Conclusions

This document summarized the contributions and discussions for 5G-A Rel-20 BS RF topics, and based on them the recommendations for 5G-A Rel-20 BS RF WID are provided.

Enhancement of co-location requirements

Online agreements:

Core part objective: - Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Consider impact on the following OTA co-location requirements: - Transmitter OFF power requirement - Transmitter co-location spurious emission requirement - Transmitter intermodulation requirement - Co-location OOBB requirement - Enhance OTA co-location requirements - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered - Option 1: Investigate and improve CLRA concept - Option 2: Investigate possibility to define requirements using existing OTA metrics (such TRP, EIRP, EIS), if any Performance part objective: - To define the conformance testing requirement of co-location requirements according to the core requirements.

SBFD BS to SBFD BS adjacent channel co-existence

Online agreements:

Core part objective: - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs Performance part objective: - Specify the conformance testing of core RF requirements for SBFD BS to SBFD BS coexistence.

Annex reference contributions

The Tdocs of BS RF topics submitted for RAN4 Rel-20 5G-Advanced in RAN#109 are listed in the table below.

| TDoc | Title | Source | Type | Agenda ittem |
| --- | --- | --- | --- | --- |
| RP-251922 | Moderator's summary for RAN4 REL-20 5G-Adv: BS RF topics | Huawei | report | 9.1.4.2 |
| RP-252034 | Views on Rel-20 BS RF topics for 5G-A | ZTE Corporation, Sanechips | discussion | 9.1.4.2 |
| RP-252067 | Views on 5G-A Rel-20 BS RF topics | Samsung | discussion | 9.1.4.2 |
| RP-252610 | Views on Rel-20 BS RF Topics | CMCC | discussion | 9.1.4.2 |
| RP-252621 | RAN4 BS RF Rel-20 work packages | Ericsson Hungary Ltd | discussion | 9.1.4.2 |
| RP-252659 | Considerations on RAN4-led Rel-20 5G-Adv BS RF topics | Nokia | discussion | 9.1.4.2 |
| RP-252691 | Proposal for SBFD-SBFD adjacent channel co-existence scenario | China Unicom, Huawei, HiSilicon, Samsung, ZTE, Verizon, CMCC, China Telecom, Vodafone, Qualcomm, ISSDU, CATT | discussion | 9.1.4.2 |
| RP-252711 | Views on BS RF works for Rel-20 5G-A | CATT | discussion | 9.1.4.2 |
| RP-252730 | Discussion on SBFD–SBFD Coexistence Configurations and BS RF Requirements | ISSDU | discussion | 9.1.4.2 |
| RP-252739 | Proposals for RAN4-led Rel-20 BS RF topics | Huawei, HiSilicon | discussion | 9.1.4.2 |

|  | Tdoc | Proposals |
| --- | --- | --- |
| 1 | ZTE (2034) | ZTE’s views on Additional Hybrid requirement sets for BS type 1-H/large antenna array, we suggest to discuss this issue in 6G SI phase since the physical layer needs also to be optimized for the analog beamforming in FR1. Proposal 1: Postpone the FR1 hybrid beamforming into 6G SI phase. Proposal 2: Consider the justifications and objectives in sections 2.1.2 and 2.1.3 for the WID of BS RF. Core part objective: - Enhancement of co-location requirements - The applicable FR1 frequency ranges, e.g. >3.5GHz requirements; - Focus on the following RF requirements: - Transmitter OFF power requirement - Transmitter spurious emission requirement for Co-location with other base station/Protection of BS receiver of own or different BS - Transmitter intermodulation requirement - Co-location OOBB requirement - SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios - Focus on FR1 WA and MR BS. - Example band: n104. - To specify the additional unwanted emissions and in-band blocking for SBFD coexisting with SBFD case Performance part objective: - To define the conformance testing requirement of co-location requirements - To define the conformance testing requirement of additional SBFD-SBFD coexistence requirements; |
| 2 | Samsung (2067) | Proposal 1: Prioritize the objective of SBFD BS to SBFD BS adjacent channel coexistence over the objective of additional hybrid requirement sets. Core part objective: - Enhancement of co-location requirements - The applicable frequency ranges shall be targeted above 3.5GHz - R19 outcome should be considered - SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios - The coexistence case shall at least include FR1 WA and MR BS. - If identified as feasible, specify BS TX and RX RF requirement for SBFD-capable BS for the scenario of SBFD BS to SBFD BS adjacent channel coexistence. - Example band: n104. - Limited to RAN4 only solution Performance part objective: - Specify conformance test cases for the specified enhanced co-location requirements - Specify conformance test cases for the specified BS RF requirements for the scenario of SBFD BS to SBFD BS adjacent channel coexistence. |
| 3 | CMCC (2610) | Enhancement of co-location requirement Objective for Rel-20 - Enhancement of co-location requirements - applicable frequency range: larger than 5GHz - no loose of current requirements for n41, n77, n78 and n79 SBFD BS to SBFD BS adjacent channel coexistence Objective for Rel-20 - Study and specify RF requirements for SBFD BS to SBFD BS adjacent channel coexistence. - All the sub-band configurations specified in Rel-19 should be considered. - Example bands: n79 - All classes of BS should be considered, including macro, micro and pico BS. Additional hybrid requirements Consideration for Rel-20 - It’s suggested to move “hybird requirements sets for BS type 1-H/large antenna array” to 6G |
| 4 | Ericsson (2621) | Evolution of co-location requirement concept: Observation 2.1-1: The height difference between to co-located base stations operating at different bands (900 MHz and 3.5 GHz) is far larger than what is captured in the CTLA description in current specification. Observation 2.1-2: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, Clause 4.9 is not practical for conformance testing of product operating above 3.5 GHz since the current description of the co-location test antenna and corresponding CLTA don’t match real deployment situations. Hence, BS type 1-O is not a feasible option for AAS BS implementations above 3.5 GHz. Observation 2.1-3: It can be noticed that information provided to describe the CLTA in the specification is overlapping and contradicting. The description provides information on both vertical radiating dimension and vertical beamwidth for the out-of-band case. Both conditions cannot be met at the same time. Observation 2.1-4: The CLTA antenna gain is indirectly specified via the horizontal beam width and the vertical radiating dimension, while the antenna efficiency is not specified. This means that large isolation between the test object and CLTA can be created by having a CLTA with poor antenna efficiency. Observation 2.1-4: The CLTA antenna efficiency is not captured by the description in current specification. Observation 2.1-5: Different types of CLTA from different vendors with very different characteristics would risk making the co-location requirements use-less. To guarantee co-location, the CLTA used should be of similar type and have the same characteristics. Observation 2.1-6: It can be noticed that the coupling loss tables created in Rel-19 are not complete. RAN4 need to further evaluate BS-to-BS coupling loss for different coupling situations relevant for considered requirements. Observation 2.1-7: To progress the evolution of co-location requirements the focus should be on transmitter spurious emission and receiver out-of-band blocking. Observation 2.1-8: To establish relevant coupling assumptions for all requirements RAN4 need to establish a common simulation methodology and conduct a simulation campaign. It’s not possible to establish coupling values based on measurements results only, due to the large scope and different combination of bands and antenna configuration. Observation 2.1-9: Improving co-location requirements will impact both BS RF core specification and Conformance test specification, since the coupling loss assumption needs to be re-evaluated and test issues related to current CLTA needs to be resolved. Observation 2.1-10: Improving co-location requirement in the upper region of FR1 includes re-evaluation requirement levels for following bands: n46, n47, n48, n77, n78, n79, n96, n102 and n104. Proposal 2.1-1: For BS RF in Rel-20 define a work package with clear goal to resolve open issues identified for BS type 1-O co-location requirements. Use outcome from Rel-19 coupling loss investigation as starting point and develop new co-location requirements based on regular OTA parameters for following bands: n46, n47, n48, n77, n78, n79, n96, n102 and n104. Derivation of SBFD-to-SBFD co-existence requirements: Observation 2.2-1: All operators in the band must deploy SBFD filtering. Observation 2.2-2: The proposed scenario will only suppress the inter-operator interference, meanwhile, the challenges on other types of interference still exist. Observation 2.2-3: Operators must deploy SBFD at every site to avoid sites at which SBFD does not work due to interference. Observation 2.2-4: The flexibility to configure carrier and sub-band allocations within a band without hardware changes would be restricted due to filtering and may need coordination between operators. Observation 2.2-5: If new types of emissions and blocking requirements requiring operator collaboration are created then they should ensure that all kinds of deployment scenarios can work, including re-use of existing sites, close and co-located BS. Observation 2.2-6: Further details on band allocations, operator spectrum allocations and plans for sub-band sizes are needed to determine requirements and filtering feasibility. Observation 2.2-7: Further analysis is needed on the filtering suppression and requirement levels. Observation 2.2-8: Both transmitter and receiver analogue filters are required per polarization per subarray to suppress inter-operator interference. The two types of analogue filters have different frequency positions, different pass bands and different roll-offs. Observation 2.2-9: An operator’s transmitter analogue filter is customized based on other operators’ sub-band configurations. Observation 2.2-10: The insertion-loss of both the TX and RX analogue filters should be carefully considered, as it can further reduce the theoretical UL coverage gain. Observation 2.2-11: Filter feasibility in terms of roll-off, size, shape needs to be studied. Proposal 2.2-1: If SBFD-to-SBFD co-existence requirements are created, it should be clarified that meeting the requirement is declared by the BS vendor and that TDD operation is not impacted. Introduction of new AAS NR BS type: Observation 2.3-1: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to improvements of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed. Observation 2.3-2: The new BS type (BS type 1-H enh) would require a new requirement interface (TAB2 after the RDN) to be defined. Observation 2.3-3: OTA receiver requirements rely on declarations of the element/sub-array characteristics. The description of the RDN needs to be enhanced to support more complex beamforming schemes (e.g. hybrid beamforming) to provide guidance for required declarations. Proposal 2.3-1: For BS RF in Rel-20, include WI to define new BS type with relevant requirement applicability and corresponding requirement interface relevant for AAS BS with large array antenna. |
| 5 | Nokia (2659) | The following Proposals were made in this document: - Proposal 1: it is proposed that “Enhancement of co-location requirements” is the main objective in BS RF centric evolution work item in REL-20. The applicable frequency range should start at 3.3 GHz in order to encompass the whole of n77/n78. - Proposal 2: Additionally, study of SBFD BS to SBFD BS adjacent channel coexistence can be considered if the scope is carefully defined, e.g. limited to band n104 and Wide Area BS class only. - Proposal 3: Great care needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items. |
| 6 | China Unicom et al (2691) | Proposal: it is proposed to define RF requirements for SBFD BS to SBFD BS adjacent channel coexistence in the Rel-20 5G-Advanced WI on BS RF centric evolution. |
| 7 | CATT (2711) | Proposal 1: RAN to enhance co-location requirements by considering a new co-location coupling loss assumption for the FR1 high frequency range in Rel-20 5G-A WID objective: - Introduce a new coupling loss assumption for BS co-location requirements for FR1 high frequency range. - Determine applicable frequency range. - Define the relevant transmission and reception co-location requirements under the new coupling loss assumption. - Transmitter OFF power, co-location spurious emissions, transmitter intermodulation, - Co-location blocking etc. Proposal 2: RAN to treat the objective of introducing additional hybrid requirement sets for BS Type 1-H / large antenna arrays as a low priority for the Rel-20 5G-A package Proposal 3: RAN to consider the SBFD coexisting with SBFD in FR1 scenario in Rel-20 5G-A phase. WID objective: - Study and specify requirements for additional Tx and Rx requirements of SBFD BS to SBFD BS adjacent channel scenario: - BS classes: FR1 WA BS and FR1 MR BS - DL and UL spectrum configurations - Additional Tx unwanted emission requirement(s) and Rx in-band blocking requirement(s) |
| 8 | ISSDU (2730) | Observation 1: UL-exposed configurations (UD/DU, UD/DUD, DUD/DU) require stricter filtering to suppress interference from adjacent DL. Observation 2: The protected configuration (DUD/DUD) avoids outer-edge UL exposure, but narrower DL sub-bands require higher filter selectivity. Proposal 1: Limit SBFD deployments to configurations that consider the DL subband frequency locations, ensuring DL is aligned to protect UL reception. Proposal 2: Restrict SBFD deployments to DL–DL boundary-compliant configurations (UD/DU, UD/DUD, DUD/DU, DUD/DUD), or equivalent arrangements that improve adjacent-channel coexistence. Proposal 3: Define differentiated BS RF requirements per configuration, with stricter values for exposed UL cases and more relaxed requirements for protected cases. |
| 9 | Huawei (2739) | Proposal 1: [BS RF: SBFD] It is proposed to define RF requirements for SBFD BS to SBFD BS adjacent channel coexistence in the Rel-20 5G-Advanced WI on BS RF centric evolution. Proposal 2: [BS RF: SBFD] The following WI objective is proposed for SBFD BS to SBFD BS adjacent channel coexistence in the Rel-20 5G-Advanced WI on BS RF centric evolution: Core part objective: SBFD BS to SBFD BS adjacent channel coexistence - Define FR1 Macro/Micro RF requirements for SBFD-SBFD adjacent operator scenario, considering the expected spectrum allocation and sub-band sizes, and relevant co-existence and co-location deployment scenarios. Compliance with these requirements may be declared by manufacturer for SBFD capable BS. - TX unwanted emission - RX in-band blocking Performance part objective: SBFD BS to SBFD BS adjacent channel coexistence: - Specify the conformance testing of the introduced core requirements for SBFD-SBFD adjacent channel coexistence Proposal 3: [BS RF: co-location] For Rel-20 BS RF scope on enhancements of co-location requirements, clarify that the applicable frequency range for OTA co-location requirements enhancements is above 4.9 GHz, and could be extended down to above 3.5 GHz, if necessary. Proposal 4: [BS RF: co-location] For Rel-20 5G-Advanced WI on BS RF centric evolution, agree on objective to re-evaluate OTA co-location requirements in Rel-20, with Rel-19 BS-BS isolation data as baseline. Proposal 5: [BS RF: co-location] Consider the following OTA co-location objectives for the Rel-20 BS RF WID: Core part objective: Enhancements for OTA co-location requirements: - Evaluate impact of Rel-19 BS-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Investigate necessity of OTA co-location requirements above 4.9 GHz. - Consider impact on the following OTA co-location requirements: - OTA transmitter OFF power requirement, - OTA co-location spurious emission requirement, - OTA transmitter intermodulation requirement, - OTA co-located OOBB requirement. Performance part objective: Enhancements for OTA co-location requirements: - Reflect modifications introduced for OTA co-location Core requirements, in related OTA co-location test requirements. Proposal 6: [BS RF: co-existence] Co-existence requirements objective shall be separated from the co-location requirements, as those two (i.e., co-location, and co-existence requirements) are separate and unrelated topics. Proposal 7: [BS RF: co-existence] Decision on potential need for Rel-20 follow-up of the ongoing Rel-19 work on co-existence requirements to be postponed till Rel-20 RAN4 check-point in June 2026. Proposal 8: [BS RF: hybrid] De-prioritize Rel-20 objective on additional Hybrid requirement sets for BS type 1-H/large antenna array. |

Reference

[1] RP-251879, Way forward on RAN4 R20 package, RAN4 Chair (Apple), RAN TSG meeting #108.