3GPP TSG RAN Meeting #110			RP-253077

Baltimore, US, December 8-11, 2025

Agenda item:	8.2.1.4

Title:	Views on 6G requirements for supporting AI services

Source:	SK Telecom

Document for:	Discussion

1. Introduction

At RAN#108, the revised SID on 6G requirements has been approved with updating the objectives as follows [1]:

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

In this contribution, we aim at providing our view on 6G requirements which should be taken into account.

2. Discussion

AI/ML is expected to become a foundational enabler of 6G networks, transforming both how the RAN operates and how the RAN supports AI-based applications. While 3GPP have explored AI/ML-assisted functionalities and management aspects during 5G timeline, a comprehensive framework towards AI-native RAN remains to be defined, which will be one of the important design targets for 6G. In 5G, 3GPP have more focused on how to utilize AI for improving network, e.g., for mobility management, CSI prediction, beam management, etc. For 6G, this aspect still needs to be pursued with more efficient and effective RAN operation. Building on the 5G use cases such as performance enhancement, network optimization, and energy saving, 6G should further pursue the evolution toward an autonomous RAN capable of self-configuration, self-optimization, and self-healing. Such capabilities will enable the network to operate with minimal human intervention while maintaining high efficiency, agility, and resilience.

On the other hand, 6G would aim at integrating AI as a native capability rather than an add-on feature like what has specified in 5G. Especially, to make 6G being a powerful platform to enable AI, the 6GR and 6G RAN architecture should be capable of orchestrating computing resources so that both RAN and AI workload can be properly/effectively supported by network infrastructure without delay or performance loss. By enabling both communication services and AI services to run concurrently on the same hardware, 6G networks can maximize infrastructure utilization and open up new monetization opportunities. Such convergence of computing and communication will allow operators to deliver differentiated AI-driven services while generating additional revenue streams from the existing network assets.

In summary, there would be two-fold perspectives for supporting AI services with 6G RAN as follows:

- AI for RAN: Leveraging AI to enhance network performance, stability and efficiency

- RAN for AI: Evolving the RAN architecture/interfaces/functions to support AI services (AI workload) for higher value of infrastructure with monetization opportunities

- Adopt the text proposal in section 3.

3. Text proposal

We propose the following text to TR 38.914.

### 5.4.5 AI The 6GR and 6G RAN architecture shall support use of AI for efficient/effective RAN operation including performance enhancement, network optimization, and energy saving, with evolution towards autonomous RAN. The 6GR and 6G RAN architecture shall support enabling AI applications especially considering the orchestration of computing resource (e.g., xPU) shared by communication and AI workloads, enabling concurrent operation of communication and AI services. The 6GR and 6G RAN architecture shall define a unified LCM framework and a data collection framework for AI/ML.

4. Conclusion

In this contribution, the following proposals are provided.

- Adopt the text proposal in section 3.

References

- RP-251395, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, RAN#108.