3GPP TSG RAN Meeting #110   											   RP-253115

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.0

Source:	CMCC

Title:	Text proposal for TR 38.914 for deployment scenarios

Document for:	Approval

Introduction

In RAN#107, the Revised SID: “Study on 6G Scenarios and requirements” has been approved [1]. In this contribution, we provide our views on 6G Scenarios and Requirements based on current version of TR 38.914 [2], RAN agreement [3] and RAN1 agreement [4].

Deployment scenarios for 6G

In this section, we discuss the deployment scenarios for 6G, including general consideration on UE antenna elements, high speed, urban coverage for massive connection, air-to-ground scenario, non-terrestrial network and unmanned aerial vehicle. The specific attributes for each essential scenario are listed in each subsection.

General consideration on UE antenna elements

During RAN#109 [3] and RAN1#123 meeting [4], parameters for UE antenna element are discussed but no consensus was reached. The proposal for specific parameters in RAN1#123 is as follows:

Proposal in RAN1#123: For 6GR evaluation, RAN1 to model the UE antenna as follows, - Note: Each of other topics could further decide to use which combination(s) for the evaluations. Other combinations are not precluded for evaluations, e.g., 2T6R, 3T6R, 6T6R, 6T8R. - Note: The antenna locations in Alt1 and Alt 2 in the following table are considered as examples and used for performance calibration. Any antenna array structures and/or antenna locations in section 7.3 in TR38.901 is possible for evaluations and up to companies to report. - Note: The antenna locations in Alt 2 not included in section 7.3 in TR38.901 are up to companies to report. - Note: The antenna element/location of T is a subset of the element/locations for R. - FFS: Alt1 or Alt2 is used for each of the combination. - FFS on UE antenna modelling for 30GHz. - Note: The mapping between the combination and the device types might be separately discussed UE antenna modelling for RAN1 evaluations; Total number of antenna elements; Total number of TXRU; Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901; Applicable carrier frequency. Combination0 Note: This combination is not for handheld UE.; 1; 1T1R,; Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) Alt 2: - 1T - 1R; 700MHz, 2GHz. Combination1; 2; 1T2R,; Alt 1: - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901.; 700MHz, 2GHz, 4GHz. Combination2; 4; 1T4R, 2T4R, 4T4R; Alt 1: - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901; 700MHz 2GHz, 4GHz, 7GHz, 15GHz. Combination3 Note: This combination is not intended for handheld UE.; 8; 1T8R, 4T8R, 8T8R; Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901; 2GHz, 4GHz, 7GHz, 15GHz Note: If number of TXRU and frequency combination is applicable. Combination4 Note: This combination is not for intended for handheld UE.; 16; FFS: 4T16R, 8T16R, 16T16R; Alt 1: - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; x, y) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization; 7GHz, 15GHz.

From our understanding, since RAN1 has already start a comprehensive and detailed discussion, it is recommended that the specific parameters (e.g., TXRU number, panel pattern, applicable carrier frequency) can be discussed in RAN1, and RAN plenary only focus on the upper bound capabilities to avoid duplicate work.

Proposal 1: To avoid duplicate work between RAN and RAN1, support the following on UE antenna elements:

- RAN plenary focus on the upper bound capabilities on UE antenna elements.

- RAN1 focus on specific parameters (e.g., TXRU number, panel pattern, applicable carrier frequency) for UE antenna elements.

Furthermore, for specific value on the upper bound capabilities of UE antenna elements, we support to take the existing values in TR 38.913 as baseline, if applicable.

Proposal 2: For upper bound capability on UE antenna elements, support to take the existing values in TR 38.913 as baseline, if applicable. The specific values are as follows:

- Around 700 MHz: Up to 4 Tx and Rx antenna elements

- Around 2 GHz: Up to 8 Tx and Rx antenna elements

- Around 4 GHz: Up to 8 Tx and Rx antenna elements

- Around 7 GHz: Up to 8 Tx and Rx antenna elements

- Around 15 GHz: Up to 8 Tx and Rx antenna elements

- Around 30 GHz: Up to 32 Tx and Rx antenna elements

High speed

The high speed deployment scenario focuses on continuous coverage along track in high speed trains. The key characteristics of this scenario are consistent passenger user experience and critical train communication reliability with very high mobility. In this deployment scenario, dedicated linear deployment along railway line and the deployments including SFN scenarios captured in Section 6.2 of 3GPP TR 36.878 [5] are considered, and passenger UEs are located in train carriages.

Figure 1 illustrates a typical layout of high speed deployment scenario. In this deployment scenario, multiple RRHs connect to one BBU with fiber and share the same cell ID. For each RRH, usually there are two TRPs which orient opposite directions along the track, and 8 RRHs are connected to one BBU with fiber and share the same cell ID to form one large macro cell.

Figure 1. Layout illustration of high speed deployment scenario

Some of its attributes are listed in Table 2.1.

Table 2.1: Attributes of High Speed

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro NOTE2 only: Around 4 GHz or Around 7 GHz |
| Aggregated system bandwidth NOTE3 | Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Macro only: - Around 4 GHz or 7 GHz: Dedicated linear deployment along the railway line as in Figure 1. - RRH site to railway track distance: 100m |
| ISD | - Around 4 GHz or 7 GHz: ISD 500m or 1732m between RRH sites, two TRxPs per RRH site. See Figure 1. |
| BS antenna elements NOTE4 | Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements |
| User distribution and UE speed | 100% of users in train For non-full buffer, 300 UEs per macro cell (assuming 1000 passengers per high-speed train and at least 10% activity ratio) Maximum mobility speed: 500km/h |
| Service profile | Alt 1: Full buffer Alt 2: FTP model 1/2/3 with packet size 0.5 Mbytes, 0.1 Mbytes (other value is not precluded) Other traffic models are not precluded, e.g., for critical train communications. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range.

NOTE2:	For Macro, it is assumed RRH sharing the same cell ID or having different cell ID.

NOTE3:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

Urban coverage for massive connection

The urban coverage for massive connection scenario focuses on large cells and continuous coverage to provide mMTC. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas, with very high connection density of mMTC devices. This deployment scenario is for the evaluation of the KPI of connection density.

Some of its attributes are listed in Table 2.2.

Table 2.2: Attributes of urban coverage for massive connection

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 700 MHz, or around 2 GHz as an option |
| Network deployment including ISD | Macro only, ISD = 1732m, 500m |
| Device deployment | Indoor, and outdoor in-car devices |
| Maximum mobility speed | 20% of users are outdoor in cars (100km/h) or 20% of users are outdoors (3km/h) 80% of users are indoor (3km/h) Users dropped uniformly in entire cell |
| Service profile | Non-full buffer with small packets |
| BS antenna elements | 2 and 4 Rx ports (8 Rx ports as optional) |
| UE antenna elements | 1Tx |

Air-to-Ground scenario

The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds. The commercial airlines aircrafts are likely equipped with an aggregation point (e.g. Relay).

Some of its attributes are listed in Table 2.3.

Table 2.3: Attributes for Air-to-Ground Scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro + relay: for BS to relay: 4 GHz, 7GHz |
| System Bandwidth | Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Macro + relay nodes NOTE1 |
| BS antenna elements NOTE4 | Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Option 1 for relay: omni-direction antenna Option 2 for relay: antenna array For antenna array, Around 4 GHz: Up to 32 Tx and Rx antenna elements Around 7 GHz: Up to 64 Tx and Rx antenna elements |
| Cell range | Macro cell: 100 km range to be evaluated through system level simulations. Feasibility of Higher Range shall be evaluated through Link level evaluation. Relay: up to 80 m |
| User density and UE speed | End user density per Macro: NOTE2 UE speed: Up to 1200 km/h Altitude: Up to 15 km |
| Traffic model | End User experienced data rate: 384 kbps DL. NOTE3 |

NOTE1:	BS to relay link should be the priority for study compared to relay to UE link.

NOTE2:	Evaluate how many users can be served per cell site when the range edge users are serviced with the target user experience data rate.

NOTE3:	Target values for UL are lower than DL, 1/3 of DL is desirable.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

Non-Terrestrial Network

// The main texts are the outputs from RANP#109 offline discussion specifically on NTN deployments and captured in RP-252915. Updates are provided with change marks.

These deployment scenarios are defined to allow for the provision of services for those areas including vertical space (e.g., up to several kilometers altitude) where terrestrial network service is not available (on a permanent or temporary basis), and also for those services that can be more efficiently supported by Non-Terrestrial Networks.

Non-Terrestrial Networks can overcome coverage limitations of terrestrial networks especially in low-density populated areas and maritime areas.

As such, NTN will mainly contribute to:

- Achieve ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improve overall 6G network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables by supporting in priority critical communications and emergency communications (messaging, voice calls).

Non-Terrestrial Networks may support messaging, enhanced Mobile BroadBand (eMBB), voice, video, positioning, delay tolerant, machine type communications and other data services. It may also support enablers for Navigation (UE in connected mode) and Timing services.

The attributes of the NTN deployment scenarios are listed in Table 2.4.

Table 2.4: Attributes for Non-Terrestrial Network Deployment Scenarios

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 1.5 GHz Around 2 GHz Around 4 GHz Around 11/13 GHz Around 20/30 GHz Around 40/50 GHz |
| Aggregated system Bandwidth | Up to [60] MHz (DL+UL): Around 1.5 GHz and Around 2 GHz Up to [100] MHz (DL+UL): Around 4 GHz Up to [200] MHz (DL+UL): Around 11/13 GHz Up to [800] MHz (DL+UL): Around 20/30 GHz Up to [1600] MHz (DL+UL): Around 40/50 GHz |
| Orbit types (Note 1) | Any orbit from VLEO (300 to 600 km altitude) to GSO (~35 786 km altitude), including LEO (600 to 2000 km altitude) and MEO (2000 to ~35000 km altitude), |
| Duplex mode | In paired bands (Satellite service allocated bands) and all orbits: FDD (Frequency Division Duplexing) or Half Duplex Frequency Division Duplexing (HD-FDD) mode In unpaired bands (Satellite service allocated bands) with Max RTD < [TBD] ms (e.g. VLEO/LEO only): TDD (Time Division Duplexing) mode (Note 4) |
| Architecture (Note: 2) | Transparent payload: BS on ground (at feeder gateway) Regenerative payload: [architectural impact TBD] |
| Satellite access node’s antenna characteristics | Antenna array of MxN elements [details FFS] and/or other types of directional antenna |
| Radio Cell type (Note: 3) | Earth fixed (GSO), Quasi Earth fixed (NGSO) or Earth moving (NGSO) |
| User distribution | Below 7.125 GHz: - Case 1 conditions: 100% Outdoor - [Case 2 conditions: [TBD%] Outdoor and [TBD%] in adverse propagation conditions (e.g. light indoor, in vehicle) ] Above 10.7 GHz: Outdoor only: [100%] Outdoor |
| Note 1: The orbit types can be characterised with a set of parameters (e.g. Maximum Round Trip Delay (ms), Maximum Doppler shift (Hz), Max Differential delay (ms), Max Differential Doppler (Hz), Max Doppler rate (Hz/s), Max delay variation (s-2), Channel model type in TR 38.811). Note 2: A transparent payload forward transparently the 3GPP radio protocol between the feeder and the service link and vice e versa. Note 3: A radio cell is able to serve UEs on the earth surface as well as in the space above. It may be realised with one or several beams generated by satellites. Only part of the beams generated by a given satellite may be active. Note 4: TDD mode design should consider at least for band around 1.5 GHz. Additional requirements on the maximum round trip delay may be required for TDD mode. |  |

Several types of User Equipment may be served by Non-Terrestrial Networks: handheld, automotive and IoT devices as well as moving platforms/building mounted terminals or Small size antenna Aperture Terminal (e.g. antenna size smaller than 20 x 20 cm). Moving platforms can refer to such as trains, ships, airplanes, drones.

Editor’s note: Whether and how NTN capable UE types are going to be defined is FFS, see clause 5.3.2.

Note:

- NTN may provide coverage in areas where TN also provides services (overlapping coverage) assuming different carrier frequencies, or where TN does not provide services on a permanent or temporary basis (non-overlapping coverage).

- NTN may be based on a combination of orbit types (e.g. GSO and LEO).

Unmanned Aerial Vehicle

The unmanned aerial vehicle (UAV) deployment scenario focuses on continuous coverage for UAVs with low altitude. The key characteristics of this scenario are continuous wide area coverage in low altitude and critical UAV communication reliability with high mobility. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 2.5.

Table 2.5: Attributes for Unmanned Aerial Vehicle

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 GHz Around 4 GHz Around 7 GHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 60 MHz(DL+UL) NOTE3 Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Hex |
| ISD | 500m 1000m |
| BS antenna elements NOTE4 | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements |
| User distribution and UE speed | Uniformly distributed Maximum mobility speed: 150km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450MHz – 960MHz identified for WRC-15 are currently being considered and around 700MHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	Consider larger aggregated system bandwidth if 20MHz cannot meet requirement.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

Conclusion

In this contribution, we provide our views on text proposal for TR 38.914 for deployment scenarios, and the following proposals are made:

Proposal 1: To avoid duplicate work between RAN and RAN1, support the followings on UE antenna element:

- RAN plenary focus on the upper bound capability on UE antenna element for each carrier frequency.

- RAN1 focus on specific parameters (e.g., TXRU number, panel pattern, applicable carrier frequency) for UE antenna element.

Proposal 2: For upper bound capability on UE antenna elements, support to take the existing values in TR 38.913 as baseline, if applicable. The specific values are as follows:

- Around 700 MHz: Up to 4 Tx and Rx antenna elements

- Around 2 GHz: Up to 8 Tx and Rx antenna elements

- Around 4 GHz: Up to 8 Tx and Rx antenna elements

- Around 7 GHz: Up to 8 Tx and Rx antenna elements

- Around 15 GHz: Up to 8 Tx and Rx antenna elements

- Around 30 GHz: Up to 32 Tx and Rx antenna elements

We also propose to agree on related text for the above to be captured in the new TR 38.914 during this meeting.

References

- RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements

- RP-252915, pCR on Deployment scenarios for TR 38.914 (e.g. for RAN#110 discussion continuation)

- R1-2509390, FLS#6 on evaluation assumptions for 6GR air interface

- 3GPP TR 36.878, Study on performance enhancements for high speed scenario in LTE