3GPP TSG RAN Meeting #110	RP-253122

Baltimore, USA, Dec 8-11, 2025

Agenda Item:	8.2.1.2

Source:	CMCC

Title:	Views on architecture of 6G RAN for Sensing

Document for:	Discussion

# 1 Introduction

In RAN#108, the new study item on 6G Radio was approved [1], where the study on Sensing was agreed with the following agreed objectives. Meanwhile, the study on Radio Access Network architecture, interface protocols and procedures for various services and functionalities (e.g. AI/ML, sensing, etc) was also a crucial part of this study item. The agreed objectives are listed below.

(9) Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914]

a) PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4]

b) Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1]

c) Aspects of integration with communication services [RAN1]

d) higher layer procedures and protocol aspects [RAN2]

e) RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

(6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

a) Overall RAN architecture aspects [RAN3, RAN2]

b) RAN-CN functional split, interface, protocol stack and procedures [RAN3]

c) RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we would like to provide our consideration on the sensing requirements for network architecture and sensing data in 6G.

# 2 Discussion

As we known, Integrated Sensing and Communication (ISAC) has been identified as one of the six usage scenarios for IMT-2030, providing sensing services for low-altitude economy, traffic surveillance, intrusion detection and micro-deformation detection while also assisting communication services. In 5G-A, only TRP mono-static sensing for UAV use case is supported, the performance and use cases are both limited. 6G is expected to support all six sensing modes (i.e., TRP mono-static, TRP-TRP bi-static, TRP-UE bi-static, UE-TRP bi-static, UE mono-static and UE-UE bi-static) including both mono-static and bi-static with both TRP and UE involved to explore more use cases and to further improve sensing performance.

TR 22.837 [2] identifies a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137 [3]. For example, sensing could be used for trajectory tracking of UAVs, vehicles, AGVs for collision avoidance. In these scenarios, sensing service latency should be one of the crucial performance metrics. Furthermore, sensing could also be used for object and intruder detection (e.g., UAV intrusion detection) in restricted areas or predefined secure areas around critical infrastructure, e.g., airports, light rail, government facilities, factories and research institutes, etc. Considering the sensing measurements for intrusion detection might be sensitive, localized data storage and processing that enables data to remain within the private wireless networks could better meet the requirements for data privacy and reduce network latency. Based on the potential ISAC use cases, we observe that crossroads, airports, railway, factories and the predefined secure areas etc. could be the important deployment areas of sensing. Thus, the deployment flexibility based on the regional characteristics should be considered for 6G sensing.

Observation 1: The following requirements should be considered for network deployment of 6G sensing:

- Data privacy: localized data storage and processing that enables data to remain within the private wireless networks for the regulatory requirements of sensitive sensing information in secure areas

- Service latency: efficient transmission and processing for the low-latency services (e.g., collision avoidance)

- Deployment flexibility: agile deployment based on various regional characteristics without additional architecture impacts

According to the above deployment requirements, we think the solution of Sensing Function (SF) deployed in RAN (illustrated in Fig.1) could be considered for network architecture of sensing in 6G. The main services and functions provided by SF could include service authorization, service initiation/termination, sensing data interaction and handling, etc.

Fig.1 Proposed Radio Access Network architecture for sensing in 6G

This kind of localized deployment solution could meet the requirements mentioned above, including data privacy, service latency and deployment flexibility, and could also serve mono-static and bistatic sensing modes. Furthermore, since sensing related network authentication differs from communication authentication process in mobile communications, SF deployed in RAN could support the authentication process for sensing related nodes. And SF deployed in RAN could also support the functions of billing and authorized opening data to the third party.

Observation 2: The network architecture solution of SF deployed in RAN (localized deployment) has the advantages, for example:

- To meet the requirements of data privacy, service latency and deployment flexibility

- To support the authentication process, billing and authorized opening data to the third party

Proposal 1: The network architecture solution for sensing that SF is deployed in RAN (localized deployment) is supported in 6G.

Regarding to the solution of SF deployed in RAN, from our point of view, the new interface procedures between base stations and SF and Inter-Interface data interaction need to be studied.

Proposal 2: The new interface procedures between base stations and SF and Inter-Interface data interaction is studied.

The sensing data obtained from base stations could be the following four levels:

- Level-1: Sensing raw data

Sensing raw data (e.g., IQ data) consists of time-domain samples or subcarrier-domain samples of original received signals from each antenna port. It contains all the information obtained by an ISAC system without signal processing.

- Level-2: Delay/angular/Doppler spectrums

Spectrums are the first-stage processing products from sensing raw data. It could retain most of the information using lower data volume than raw data. Delay spectrum, angular spectrum and Doppler spectrum are fundamental for most of the sensing algorithms.

- Level-3: Cloud of points

Cloud of points are acquired by implementing detection algorithms on delay, angular and Doppler spectrums. Most of the information may be discarded at this stage, which leads to a relatively low data volume.

- Level-4: Sensing result

Sensing result is the final output of sensing signal processing. A sensing result is described by target’s attributes, like its location, velocity and type. It has the lowest data volume.

Different customers or sensing services will have the various requirements for sensing processing and sensing data level. Thus, the sensing data level reported from base stations to SF need to be discussed according to the requirements of sensing services.

Proposal 3: According to the processing requirements of various sensing services, the sensing data level (e.g., Sensing raw data, Delay/angular/Doppler spectrums, Cloud of points and Sensing result) reported from base stations to SF are discussed case by case.

# 3 Conclusion

In this paper, we provide our consideration on the sensing requirements for network architecture and sensing data in 6G.

Observation 1: The following requirements should be considered for network deployment of 6G sensing:

- Data privacy: localized data storage and processing that enables data to remain within the private wireless networks for the regulatory requirements of sensitive sensing information in secure areas

- Service latency: efficient transmission and processing for the low-latency services (e.g., collision avoidance)

- Deployment flexibility: agile deployment based on various regional characteristics without additional architecture impacts

Observation 2: The network architecture solution of SF deployed in RAN (localized deployment) has the advantages, for example:

- To meet the requirements of data privacy, service latency and deployment flexibility

- To support the authentication process, billing and authorized opening data to the third party

Proposal 1: The network architecture solution for sensing that SF is deployed in RAN (localized deployment) is supported in 6G.

Proposal 2: The new interface procedures between base stations and SF and Inter-Interface data interaction is studied.

Proposal 3: According to the processing requirements of various sensing services, the sensing data level (e.g., Sensing raw data, Delay/angular/Doppler spectrums, Cloud of points and Sensing result) reported from base stations to SF are discussed case by case.

# Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator).

- TR 22.837: "Study on Integrated Sensing and Communication".

- TS 22.137: "Integrated Sensing and Communication".

# Annex: Text proposal to TR 38.914

Based on the discussion above we propose the following text proposal to TR 38.914.

5.2	Requirements for architecture and migration

5.2.X	Architecture for sensing

The minimum requirements for network deployment of sensing are as follows:

–	Data privacy: localized data storage and processing that enables data to remain within the private wireless networks for the regulatory requirements of sensitive sensing information in secure areas

–	Service latency: efficient transmission and processing for the low-latency services (e.g., collision avoidance)

–	Deployment flexibility: agile deployment based on various regional characteristics without additional architecture impacts

The architecture that Sensing Function (SF) deployed in RAN will be considered. The new interface procedures between base stations and SF and the Inter-Interface data interaction will be studied.