3GPP TSG RAN Meeting #110	RP-253123

Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.2

Source:	CMCC

Title:	Views on coexistence of 6G and 4G

Document for:	Discussion

- Introduction

During RAN#108, the following text for requirements for architecture and migration was agreed and captured in TR38.914[1], and MRSS between 6GR and NR is to be supported when designing 6GR, as highlighted below.

5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

And in this contribution, we will discuss the need of spectrum sharing between 4G and 6G.

- Discussion on 6GR and 4G coexistence

In current network deployments and near future, a large number of IoT devices based on LTE network will remain active according to predictions. Figure.1 provides the Global Cellular IoT Module Shipments for different IoT technologies [2], it can be seen that 4G cellular IoT module shipments will still occupy more than 50% in 2030. Given that these IoT applications have a slow phase-out rate, potentially lasting 10 years or more, we will face the challenge of coexisting 4G and 6G deployment.

Figure 1. Global Cellular IoT Module Shipments by Network Technology Distribution (Unit: Ten Thousand Units) [1]

If these 4G frequency bands cannot be utilized for 6G due to the continued presence of 4G IoT applications, the valuable low-frequency spectrum during non-busy hours would remain underutilized, resulting in significant resource wastage. To address this challenge, two options can be considered:

- Option 1: Spectrum Sharing between 4G and 6G. This approach requires 6GR to be designed with awareness of 4G's always-on channels, such as PSS, SSS, PBCH, CRS, and PDCCH, with rate matching or puncturing mechanisms to avoid interference. This means higher specification design complexity, but it offers more flexibility, enabling 6G to fully exploit 4G's spectrum resources during periods of low 4G traffic.

- Option 2: Compressing 4G spectrum allocation to free up dedicated bands for 6G deployment. This method reduces 4G's bandwidth, reallocating the remaining spectrum to 6G. For instance, compressing 4G's original 20 MHz allocation to 5 MHz, 3 MHz, or 1.4 MHz, thereby freeing up 15+ MHz for 6G. Although spec design is simple, this approach lacks flexible bandwidth adjustment capabilities and suffers from reduced data rates. This poses challenges for certain industrial manufacturing scenarios, such as in some industrial manufacturing scenarios, machine vision quality inspection that demands high-definition video and image uploads, where uplink speed requirements typically range from 5 Mbps to 50 Mbps.

Table 1 provides peak rates for different LTE bandwidths, taking into account PSS/SSS, PBCH, 2-port CRS, PDCCH with 2 OFDM symbols, 90% Layer 2 efficiency, Cat 1/1bis with 1 DL layer and Cat 4 with 2 DL layers. Uplink rates are provided for both 16QAM and 64QAM modulation schemes.

Table.1 Peak data rates for different bandwidths

| Bandwidth | # of RBs | DL Cat1/1bis (1 layer) | DL Cat4 (2 layers) | UL 64QAM | UL 16QAM |
| --- | --- | --- | --- | --- | --- |
| 1.4MHz | 6 | 3.69 Mbps | 7.38 Mbps | 4.33 Mbps | 2.57 Mbps |
| 3MHz | 15 | 9.22 Mbps | 18.44 Mbps | 10.85 Mbps | 6.43 Mbps |
| 5MHz | 25 | 15.37 Mbps | 30.74 Mbps | 18.08 Mbps | 10.73 Mbps |

Beyond the industrial manufacturing scenarios mentioned above, other 4G applications with high-speed requirements include

- Connected vehicle terminals application, traffic such as map updates, real-time traffic navigation, and online music/video streaming demand downlink rates of 10~50 Mbps and uplink rates of 5~20 Mbps, exceeding the data rate provided by 5 MHz bandwidth.

- Intelligent remote control of UAVs, it requires transmission of high-definition video or aerial imagery, with uplink requirements of 2~20 Mbps, 5 MHz bandwidth may also prove insufficient.

- Smart transportation applications, where video stream uploads can require uplink rates up to 10 Mbps, require a reserve of at least 5 MHz bandwidth allocation.

Therefore, Option 2 has the risk of failing to meet high-rate application demands if the bandwidth allocated to LTE falls below 5 MHz, while reserving larger bandwidth results in low spectrum utilization. In contrast, Option 1 enables flexible spectrum sharing based on real-time 4G traffic demands, achieving maximum efficiency without compromising the 4G user experience. Based on this analysis, it is recommended that 6GR be designed to support flexible spectrum sharing with 4G, semi-static rate matching pattern for the always on signals of 4G can be configured for 6G system.

Proposal 1: The 6G design shall support flexible Spectrum Sharing between 6GR and 4G network for IoT purpose, e.g., LTE cat. 1bis/1/4.

To reduce the specification design complexity, some semi-static rate matching patterns for 4G always on channel/signals such as PSS/SSS, PBCH, CRS can be configured for 6G system. Therefore, 6G should support coexistence with LTE cat.1, cat.1bis, and LTE cat.4 via semi-static configuration.

- Text proposal

Based on the discussion in Section 2, we propose the following text proposal for TR 38.914 for the general requirements for architecture of 6G RAN.

Proposal 2: It’s recommended to capture the following TP in TR 38.914.

< Unchanged text omitted >

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

-	6G should support coexistence with NB-IoT (all deployment modes) and eMTC, LTE cat.1, LTE cat.1bis, and LTE cat.4 via semi-static configuration.

< Unchanged text omitted >

- Conclusion

In this contribution we make the following observation, proposal and propose adding the text proposal in section 3 to TR 38.914 clause 5.2.

Proposal 1: The 6G design shall support flexible Spectrum Sharing between 6GR and 4G network for IoT purpose, e.g., LTE cat. 1bis/1/4.

Proposal 2: It’s recommended to capture the following TP in TR 38.914.

< Unchanged text omitted >

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

-	6G should support coexistence with NB-IoT (all deployment modes) and eMTC, LTE cat.1, LTE cat.1bis, and LTE cat.4 via semi-static configuration.

< Unchanged text omitted >

References

- 3GPP TR 38.914 V0.2.0, Study on 6G Scenarios and Requirements; (Release 20).

- 2025 Wide-Area IoT: China Cellular and Satellite IoT Industry Research White Paper, https://www.iotku.com/News/1057017807033597952.html.