3GPP TSG RAN Meeting #110	RP-253167

Baltimore, USA, 8th – 11th December 2025

Agenda item	: 8.2.1.1

Source	: Jio Platforms Limited

Title	: Mobility Interruption Time for Voice Call Services

Document for	: Approval

# Introduction

Mobility interruption time is a key performance indicator that influences the continuity and perceived quality of real-time services. As 6G evolves, certain services, particularly conversational voice, are highly sensitive to even short interruptions in packet delivery. While TR 38.914 already defines the baseline mobility interruption time requirements, additional clarification is needed to capture the expectations for user-perceived service continuity during ongoing voice calls. This contribution introduces text to further describe the mobility interruption expectations for conversational voice services within Section 5.1.12.

# Discussion

Mobility interruption time is a critical performance metric that directly impacts the continuity of real-time services. Among these services, conversational voice is uniquely sensitive, i.e., even short interruptions in packet delivery can lead to perceptible audio gaps that degrade the user’s experience, as reflected in the quality expectations for conversational real-time communication defined in [1]. Users generally expect voice communication to remain stable and uninterrupted, and therefore mobility interruption time becomes an essential consideration in supporting high-quality voice services in 6G.

Conversational voice is highly sensitive to short interruptions in packet delivery, and even brief gaps can be perceptible to users, consistent with the characteristics of conversational real-time communication.

Experience from previous generations further illustrates this need for clarity. In 5G networks, mobility interruption time can vary depending on the mobility mechanism, network load, and the nature of inter- or intra-cell transitions. While 5G achieves strong performance in many respects, interruptions do occur during mobility. Although these brief gaps may be acceptable for some data services, they can still be perceptible during conversational voice and may disrupt the natural flow of communication.

Experience from previous generations further illustrates this need for clarity. In 5G networks, mobility interruption time can vary depending on the mobility mechanism, network load, and the nature of inter- or intra-cell transitions. While 5G achieves strong performance in many respects, interruptions do occur during mobility. Although such interruptions may be tolerable for many data services, the mobility interruption time in 5G is not sufficiently short to avoid perceptible disruptions during conversational voice, which can negatively affect the natural flow of communication.

In 5G networks, mobility interruption time is not sufficiently short to prevent perceptible disruptions during conversational voice, even though such interruptions may be tolerable for many data services.

Additionally, observations from dense urban deployments show that frequent handovers can occur even when the UE is moving slowly or is nearly stationary. In certain environments, UEs may experience 20 to 30 handovers within just a few minutes due to dense site grids, sharp signal boundaries, or fluctuating radio conditions. When such frequent transitions occur during a voice call, the cumulative effect of repeated mobility interruptions becomes particularly problematic. Even if each individual interruption is short, the repeated occurrence may lead to intermittent audio gaps or distortions, significantly degrading the overall conversational quality and resulting in a poor user experience. These challenges highlight the importance of explicitly capturing voice-related expectations within the mobility interruption time requirements.

In dense urban deployments, frequent handovers can occur even for slowly moving or nearly stationary UEs, and the cumulative effect of repeated mobility interruptions during a voice call can lead to intermittent audio gaps and degraded conversational quality.

As 6G aims to deliver seamless, high-quality communication across diverse environments, the baseline text in Section 5.1.12 provides an initial foundation by requiring that mobility interruption time be minimized and by specifying a minimum of 0 ms for mobility within a single base station. However, the current wording does not explicitly acknowledge the heightened sensitivity of conversational voice to interruption, nor does it reflect the challenges observed in dense urban mobility scenarios. To ensure alignment across RAN working groups and provide clear guidance for future design and evaluation efforts, it is beneficial to extend Section 5.1.12 with expectations tailored to conversational voice services.

It is proposed to extend Section 5.1.12 with text that captures the specific expectation that mobility interruption time for conversational voice should be short enough to avoid perceptible disruption during an ongoing voice call.

To address the observations outlined above, it is important that TR 38.914 explicitly reflects the expectation that 6G mobility performance should enable conversational voice services to remain seamless and free from perceptible disruption. Introducing this clarification in Section 5.1.12 will help ensure that future 6G RAN designs and evaluations appropriately consider the sensitivity of voice services to interruption and the challenges encountered in practical deployments. The accompanying text proposal in Section 4 provides the necessary enhancement to the existing mobility interruption time requirements to better support high-quality voice call experiences in 6G.

Capture the text proposal for Mobility Interruption Time requirements in Section 4 to TR 38.914 to ensure seamless voice call services during mobility.

# Conclusion

This contribution highlights the need to strengthen the mobility interruption time requirements in TR 38.914 to better support seamless conversational voice services in 6G. By clarifying the expectation that mobility interruptions should not lead to perceptible degradation during an ongoing voice call, the proposed enhancement ensures that voice remains a high-quality, reliable service across diverse deployment scenarios. RAN is invited to consider and adopt the text proposal provided in Section 4.

To support the above conclusion, the key observations and proposals from the discussion section are summarized below for consideration.

Observation 1:	 Conversational voice is highly sensitive to short interruptions in packet delivery, and even brief gaps can be perceptible to users, consistent with the characteristics of conversational real-time communication.

Observation 2:	 In 5G networks, mobility interruption time is not sufficiently short to prevent perceptible disruptions during conversational voice, even though such interruptions may be tolerable for many data services.

Observation 3:	 In dense urban deployments, frequent handovers can occur even for slowly moving or nearly stationary UEs, and the cumulative effect of repeated mobility interruptions during a voice call can lead to intermittent audio gaps and degraded conversational quality.

Proposal 1:	 It is proposed to extend Section 5.1.12 with text that captures the specific expectation that mobility interruption time for conversational voice should be short enough to avoid perceptible disruption during an ongoing voice call.

Proposal 2:	 Capture the text proposal for Mobility Interruption Time requirements in Section 4 to TR 38.914 to ensure seamless voice call services during mobility.

# References

3GPP TR 22.105 V19.0.0: "Services and Service Capabilities".

# Annex

### 5.1.12	Mobility interruption time

Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The proponent can report the Mobility interruption time in other scenarios, where applicable.

In addition, for conversational voice services, the mobility interruption time should be kept short enough to avoid perceptible degradation of an ongoing voice call. As user-perceived audio gaps are highly sensitive to even brief interruptions, 6GR should aim to support mobility procedures that minimize or eliminate audible disruptions during a call. The intention is that the mobility interruption performance for voice enables a seamless user experience, consistent with the expectations for carrier-grade conversational services.