3GPP TSG RAN Meeting #110						           RP-253189

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source:	Apple

Title:	on requirements of new and existing services for 6G

Document for:	Discussion/Decision

# Introduction

The latest RAN 6G SID is approved in RP-251395 [1], was approved. The SID covers mainly two aspects of the 6G study, i.e., (1) study on ITU IMT-2030 TPR (Technical Performance Requirement), (2) study of 6G in general. Regarding the study of 6G in general, the followings are the objectives quoted from RP-251395

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework

In this contribution, we provide our views on requirements of new and existing services for 6G

# Existing and new services for 6G Day 1

Service support is an important factor to be considered during the 6G study. Over the course of 3GPP technology evolution, 3GPP has provided specification support for services that have achieved great commercial success, such as MBB (Mobile Broadband). On the other hand, 3GPP also has specified solutions for other services that are not that commercially successful. Most importantly, new services are needed not only to provide more revenues for operators, but also to further improve the user experience. Therefore, it is important that 6G study covers a set of carefully selected, essential, existing services, and new/enhanced services.

For the existing services supported by 3GPP, there are several services that 3GPP has spent considerable amount of time on the design and evolution but achieved no, or very limited, commercial success.  Given the limited time for the 6G first release, it is very crucial for 3GPP to consider only the essential services that have a high chance to be deployed. The services that we believe should not be considered for the 6G first release include sidelink, unlicensed spectrum access, i.e., similar technology as LTE LAA, and NR-U, Broadcast/Multicast, and 6G RAT dependent positioning, etc. Regarding positioning, it is part of ITU IMT-2030 capabilities and is likely to have the corresponding ITU TPR. However, this does not necessarily mean that there is a need to specify 6G RAT dependent positioning in 6G again.

For the new/enhancement services, we think that there are at least three services that 3GPP should consider. The first and foremost is AI/ML, for which 3GPP should investigate the potential performance gain that can be achieved by AI/ML in all layers, in all the WGs, i.e., RAN1/2/3/4. Secondly, ISAC (Integrated Sensing and Communication) is another area that 3GPP can exploit. With the new ISAC channel model ready in 38.901 as the outcome of Rel-19, it is important for 3GPP to investigate the solutions that can allow integrated sensing and communication, and investigate how sensing can be used to improve the user experience, improve the system operation efficiency, etc. Last but not the least, immersive communication, with XR/VR/AR as a good example, is another important topic that 3GPP should cover. Immersive communication has a great potential to significantly improve the user experience, and offer new ways that user can interact with the devices and environments. However, immersive communication is a challenging design as it includes multiple traffic flows with different QoS requirement.

With the above in mind have the following proposal.

Proposal 1: For Rel-20 RAN WG study on 6G, regarding service support

- Do not include the support of following services (not exhaustive list)

  - Sidelink and vehicles if vehicles support is related to sidelink

  - Unlicensed spectrum access, i.e., similar technology as LTE LAA, and NR-U

  - Broadcast, Multicast

  - 6G RAT dependent positioning further discuss how to meet IMT-2030 TPR on positioning

    - E-CID based positioning shall be supported in 6G

  - Mission Critical Communication, or at least the scope in RAN needs to be clarified for further check

  - HRLLC

- Include, at least, support of the following new/enhanced services

  - AI/ML

  - ISAC (Integrated Sensing and Communication)

  - Immersive communication

  - NTN

  - FWA

# Requirements of existing and new services for 6G Day 1

## eMBB

eMBB (Enhanced Mobile Broadband) is the most important and successful service that is supported by the 3GPP specification. We believe eMBB will continue to play the same critical role for the success of 6G. On the other side, over the course of 4G and 5G, and especially during the later releases of 5G, many advanced features were introduced without real benefit to the user experience, for example, features that are introduced for URLLC. This creates many problems, including complicated specification, excessive UE capability, etc. For 6G, we have the following proposal

Proposal 2.1: eMBB service remains to be one of the most important services for  6G Day-1

- eMBB study should consider balanced design considering performance of both the NW and UE, e.g., regarding energy saving, complexity reduction, etc.

- eMBB study should focus on improving the real user experience

  - Deprioritize features that aim at enhancement without, or with very limited, benefit in practical deployment, e.g.

    - Sub-ms latency enhancement and/or reliability enhancement beyond 99.999%

    - Extremely high peak data rate/spectral efficiency

## NTN

NTN is crucial to truly ensure ubiquitous coverage. In 5G, NTN was introduced at later releases in a “NBC” fashion. As results, legacy UEs not able to connect, requiring extra development efforts. In 6G, we believe NTN should be natively supported in the first release, with a single scalable/harmonized design. On the other side, it is also important to design 6G NTN based on the lessons learn from and the experience gained from NTN design in 5G. Therefore, we have the following proposal

Proposal 2.2: For Day-1 6GR air interface, support both TN and NTN with a single scalable/harmonized design

- Consider all orbits including LEO, MEO, and GEO.

- Consider at least the following areas for unified design with TN, considering

  - Radio propagation properties, including long propagation delay, and high doppler

  - Knowledge of the satellite orbit to improve performance like mobility, RRM, etc.

- Consider at least the following areas for enhancement

  - GNSS resilient/free NTN operations

  - Coexistence of UEs with GNSS and UEs without GNSS

  - Support TN-NTN in the same spectrum (e.g. TN bands) including coexistence mechanism for interference mitigation

## ISAC

For ISAC, it is also important to note that 3GPP NR mainly covers the channel model in Rel-19 and NW side UAV detection in Rel-20 without UE impact. More important technical discussions including both the performance evaluation and technology design are expected in Rel-20 6G study. To carry out the design and evaluation in the RAN WGs, it is important to agree on a set of sensing use cases. In our view use cases that involve the UE as a sensing transmitter and/or receiver and allow for all network nodes as sensing output consumers and sense objects in their environment are essential. This is especially important as proposals in the  R20 5G-A sensing track are very TRP focused with no UE involvement.

Furthermore, it is important to discuss the ISAC key principles of operation and RAN architecture, as those would be quite new and for which there is no 5G baseline. We believe it is important define a clean and usable ISAC architecture, unburdened from the 5G positioning architecture (which saw almost no deployments). To this end, the following ISAC modes of operations should be considered:

- Network-based,

- UE-based, and

- UE-centric.

In our view, the potential use cases for the R20 6G track should include the following:

- Sensing Aided Communications Use cases for both the network and the UE e.g. R19 SA1 ISAC Use Case 5.21

- Privacy focused use cases for sensing data/entity privacy e.g. R19 SA1 ISAC Use Case 5.16

- Public Safety Use cases for both the target and subscriber e.g. R19 SA1 ISAC Use Case 5.27

- Intruder Detection use cases for home e.g. R19 SA1 Use Case 5.1 and 5.6

Proposal 2.3: For ISAC in the 6G first release, use cases that involve the UE as a sensing transmitter and/or receiver, support network-based, UE-based and UE-centric modes of operation and allow for all network nodes as sensing output consumers are essential. Consider at least the following use cases in TR22.837

- Sensing Aided Communications Use cases for both the network and the UE e.g. R19 SA1 ISAC Use Case 5.21

- Privacy focused use cases for sensing data/entity privacy e.g. R19 SA1 ISAC Use Case 5.16

- Public Safety Use cases for both the target and subscriber e.g. R19 SA1 ISAC Use Case 5.27

- Intruder Detection use cases for home e.g. R19 SA1 Use Case 5.1 and 5.6

## AI/ML

There are at least two aspects regarding how AI/ML cam impact the air interface. On direction is that AI/ML for NW, in other words, how to use AI/ML to make the air interface more efficient, which including different layers of the air interface protocols. The other direction is NW for AI/ML, in other words, how to enhance the air interface to make the support of AI/ML traffic more important. In our view, both AI/ML for NW and NW for AI/ML are equally important for the success of 6G Day 1.

Additionally, as device vendor, we do believe that it is very crucial to design 6G AI/ML with many UE aspects in mind. One aspect is the UE privacy. AI/ML heavily relies on the data and the associated training, and may also require model transfer, or some assistance information for interference and/or LCM, all of which may impose risk to UE privacy. As results, it is very important to limit UE information leakage (privacy): device location, activity, context, usage. Moreover, based on NR AI/ML study, some AI/ML model may require high UE computational complexity even just for interference, and different AI/ML processing engine may even be needed for different use cases, all of which should be taken into consideration during the study. Last but not the least, we also prefer that UE should have flexibility in terms of the choice of AI/ML model, or even whether to use AI/ML, to meet certain performance requirement.

With the above in mind, we have the following proposal

Proposal 2.4: Native support of both AI/ML for NW and NW for AI/ML in 6G Day 1

- For NW for AI/ML

  - Study the characteristic of the AI/ML traffic and focus on the potential enhancement that can improve the system operation capacity/efficiency

- For AI/ML for NW

  - Consider the principle of designing AI/ML as a toolbox for air interface with UE privacy safeguards in place. From UE perspective

    - Limit UE ML complexity required by 3GPP

    - Limit UE information leakage (privacy): device location, activity, context, usage

    - For performance requirement, specify cellular KPIs without regard to whether AI/ML is used to achieve improved performance

## Immersive Communication

New services, especially the one that can provide enhanced user experience, are very crucial for 6G. Immersive communication is a good example that has great potential to provide user with better experience. One good example of immersive communication is XR, including both VR and AR, etc. XR has been studied in 5G, and based on the study, it is evident that system capability for XR service is quite limited. One special challenges for supporting XR is that XR traffic typically consists of multiple traffic flows, and different traffic flow may have different characteristics and different associated QoS requirement. The performance of the XR service is determined jointly by all the component traffic flows.

For immersive communication, we have the following proposal

Proposal 2.5: Immersive communication should be natively support in 6G Day 1, and its study should focus on improving both the single user performance and the system performance, considering the traffic characteristics and QoS requirement of immersive communication

- Study should aim at identifying the pain point of immersive communication using 5G study as starting point

# Conclusion

In this contribution, we provide our views on 6G first release in general, with the following proposals regarding the 6G Day 1 support of the existing and new services

Proposal 1: For Rel-20 RAN WG study on 6G, regarding service support

- Do not include the support of following services (not exhaustive list)

  - Sidelink and vehicles if vehicles support is related to sidelink

  - Unlicensed spectrum access, i.e., similar technology as LTE LAA, and NR-U

  - Broadcast, Multicast

  - 6G RAT dependent positioning further discuss how to meet IMT-2030 TPR on positioning

    - E-CID based positioning shall be supported in 6G

  - Mission Critical Communication, or at least the scope in RAN needs to be clarified for further check

  - HRLLC

- Include, at least, support of the following new/enhanced services

  - AI/ML

  - ISAC (Integrated Sensing and Communication)

  - Immersive communication

  - NTN

  - FWA

Proposal 2.1: eMBB service remains to be one of the most important services for  6G Day-1

- eMBB study should consider balanced design considering performance of both the NW and UE, e.g., regarding energy saving, complexity reduction, etc.

- eMBB study should focus on improving the real user experience

  - Deprioritize features that aim at enhancement without, or with very limited, benefit in practical deployment, e.g.

    - Sub-ms latency enhancement and/or reliability enhancement beyond 99.999%

    - Extremely high peak data rate/spectral efficiency

Proposal 2.2: For Day-1 6GR air interface, support both TN and NTN with a single scalable/harmonized design

- Consider all orbits including LEO, MEO, and GEO.

- Consider at least the following areas for unified design with TN, considering

  - Radio propagation properties, including long propagation delay, and high doppler

  - Knowledge of the satellite orbit to improve performance like mobility, RRM, etc.

- Consider at least the following areas for enhancement

  - GNSS resilient/free NTN operations

  - Coexistence of UEs with GNSS and UEs without GNSS

  - Support TN-NTN in the same spectrum (e.g. TN bands) including coexistence mechanism for interference mitigation

Proposal 2.3: For ISAC in the 6G first release, use cases that involve the UE as a sensing transmitter and/or receiver, support network-based, UE-based and UE-centric modes of operation and allow for all network nodes as sensing output consumers are essential. Consider at least the following use cases in TR22.837

- Sensing Aided Communications Use cases for both the network and the UE e.g. R19 SA1 ISAC Use Case 5.21

- Privacy focused use cases for sensing data/entity privacy e.g. R19 SA1 ISAC Use Case 5.16

- Public Safety Use cases for both the target and subscriber e.g. R19 SA1 ISAC Use Case 5.27

- Intruder Detection use cases for home e.g. R19 SA1 Use Case 5.1 and 5.6

Proposal 2.4: Native support of both AI/ML for NW and NW for AI/ML in 6G Day 1

- For NW for AI/ML

  - Study the characteristic of the AI/ML traffic and focus on the potential enhancement that can improve the system operation capacity/efficiency

- For AI/ML for NW

  - Consider the principle of designing AI/ML as a toolbox for air interface with UE privacy safeguards in place. From UE perspective

    - Limit UE ML complexity required by 3GPP

    - Limit UE information leakage (privacy): device location, activity, context, usage

    - For performance requirement, specify cellular KPIs without regard to whether AI/ML is used to achieve improved performance

Proposal 2.5: Immersive communication should be natively support in 6G Day 1, and its study should focus on improving both the single user performance and the system performance, considering the traffic characteristics and QoS requirement of immersive communication

- Study should aim at identifying the pain point of immersive communication using 5G study as starting point

# Reference

[1] RP-251395, Revised SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2] TR 22.837, Feasibility Study on Integrated Sensing and Communication, 3GPP Technical Specification Group TSG SA