3GPP TSG RAN Meeting #110	   RP-253193

Baltimore, USA, December 8-11, 2025

Source:          ZTE Corporation, Sanechips

Title:             Views on RAN KPIs

Agenda Item:     8.2.1.1

Document for:    Discussion and Decision

- Introduction

In RAN#103 meeting, the following RAN plenary work plan was agreed for IMT-2030 aspects [1] where the RAN study includes two phases corresponding to ITU IMT-2030 TPR and 6G general aspects respectively.

RAN plenary work is split into an ITU focused SI (study item) and General RAN SI - IMT-2030 discussion is expected in RAN from 09/24 to 12/24 - RP SI Rel-20 focusing on ITU– IMT-2030: approval 12/24 until 06/25 - RP SI Rel-20 focusing on 6G General: approval 03/25 (after WS), until 06/26

In RAN#106 meeting, the new SID was endorsed for Study on 6G scenarios/use cases and requirements where the first phase on ITU TPR part is included.

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

In RAN#107, the moderator submitted a document comprising of all the proposals received from the meeting in RP-250819 [6].

In RAN#108 meeting, some preliminary agreements have been made regarding the TPRs and their values, as captured in the reply LS to ITU-R WP 5D RP-251825 [9].

In RAN#109, another LS provided 3GPP RAN views on IMT-2030 TPRs [12], and the moderator provided a summary of RAN KPIs in [11].

In this contribution, we provide our views on some KPI values that are still pending.

- KPI candidates and values

From the moderator summary [11], there some KPIs pending further discussions. Our views are provided below.

Coverage

Coverage refers to the ability to provide access to communication services for users in a desired service area. In TR 38.913, coverage and extreme coverage are already defined as a KPI in section 7.10 and 7.10.1 which are evaluated by calculating MaxCL.

However, for 6GR, the following two coverage related aspects have been listed in the SID[2][14].

- Enhanced overall coverage, focus on cell-edge performance and UL coverage - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

In RAN1#123, the following agreement is made to define the “interpretation and target” for the comparison cross different frequencies. In which, the template candidates 2 is aligned with MaxCL definition in TR 38.913.

Agreement - For the RAN1 study of “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, - The link budget template candidates 1 and 2 are used to calculate the metric(s) as starting point to compare existing 5G mid-band and 6G deployments in at least around 7 GHz, with potential future update. - During initial access/random access - Coverage target is referring the bottleneck channel (i.e. Rel-15 NR Msg3) during initial access/random access for existing 5G mid-band - FFS target value(s) of data rate for data channels relative to 5G mid-band - Following deployment scenarios are considered - Urban macro (both O2I and outdoor) - Sub-urban macro (both O2I and outdoor) - Following carrier frequencies are considered to calculate the metric(s) - [4 GHz] as the existing 5G mid-band - 7 GHz as 6G deployment Template in R1-2509615 is to be used for collecting inputs on the values from companies.

For KPI definition for coverage,

- Regarding the 1st objective listed above:

Regarding the metric defined for general coverage, similar to 5G, it’s recommended to reuse MaxCL defined in TR 38.913 (i.e., Template candidate-2 agreed in RAN1#122-bis). The reason behind is that this KPI will be used as the guidance for different services and deployment (e.g., IoT vs. eMBB, and TN vs. NTN). Given the diversity and different assumption, it’s not possible to align the detailed deployment assumption, hardware implementation and impairments at both UE and BS side in current stage. Additionally, comparison between 5G and 6G is also expected, and it’s reasonable to keep the same metric.

For the detailed value, since 6GR is aimed to support coverage enhancements as the native design, it’s reasonable to define the same target for coverage between eMBB and IoT service. However, it should be noticed that:

  - With more allocated resource to ensure the coverage, the associated data rate for eMBB will be potentially degraded for the case that extreme coverage is satisfied.

  - As state in the SID, the whole 6GR design should be conducted with prioritization on eMBB service. Although it is still under discussion, NB-IoT like service should not be the target for 6G IoT. Then, the value corresponding to NB-IoT, e.g., 164 dB should not be pursued.

Then, as the baseline, it’s preferred to take 154 dB as the initial value, at least for DL. For uplink, it’s better to align with the assumption on the detailed parameter firstly, e.g., number of Tx Ant, Power class and noise figure (7dB vs. 9 dB as agreed in RAN1#123), especially for newly introduced frequency bands, e.g., around 7 GHz.

- Regarding the 2nd objective listed above:

As discussed in the SID, this objective itself is already kind of metric/requirement for 6GR design. Additionally, the assessment of “Re-use of existing 5G mid-band (~3.5GHz) site ” and “comparable coverage” highly depends on the set-up assumed for existing and future network, e.g., Tx/Rx AntConfig, network layout. Then, in RAN-level, there is no need to further introduce any quantitative KPI for this aspect. The study can be conducted in RAN1 with aligned assumption.

Additionally, regarding the pending assumption for frequency in 5G-Mid band, to align with the scenario deployment highlighted in Section 4 in TR38.913, 4GHz can be considered as the representative value for 6GR design. Meanwhile, as mentioned above, without defining any quantitative KPI, other interested frequencies (e.g., 2.6 GHz, 3.5 GHz) along with proper configurations can also be investigated to identify the potential differences, which may also be taken into account for 6GR design.

Proposal 1: Regarding the KPI for overall coverage, MaxCL defined in section 7.10 in TR 38.913 is reused as the metric.

- For DL, 154 dB can be considered as the starting point for both eMBB and IoT service.

- For UL, further alignment on UE configuration is expected.

Proposal 2: For “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, no quantitative KPI is defined in RAN-level.

- For the assumption on carrier frequency, 4 GHz is considered as the reference. Other values, e.g., 3.5 GHz and 2.6 GHz can be considered additionally.

Energy efficiency

Agreement in RAN1#122b Apply the following evaluation methodology framework for Quantitative analysis, - For NW unloaded/empty load case or UE idle/inactive mode: - For energy saving: analytical calculation - For performance impact: analytical calculation, LLS - For loaded cases and connected-mode UEs - For energy saving: SLS - For performance impact: LLS, SLS Agreement in RAN1#122b At least the following NR metrics, - Network energy saving gain relative to baseline for BS - UE energy saving gain relative to baseline for UE - Impact to UPT (User-Perceived Throughput), if applicable, as well as the metrics - Impact to latency, if applicable - Impact to QoS/delay budget satisfaction rate, if applicable are used for 6G energy efficiency evaluation.

For energy efficiency, there have been some discussion in RAN1#122bis and RAN1#123 meetings, where relative BS power consumption model and relative UE power consumption model are to be used for energy saving evaluation as shown in the above box. Additionally, the power models for BS and devices are under discussion in RAN1. In draft TR 38.914, a definition for a KPI for network EE is given as follows. It is also in line with RAN1 discussions and agreements.

### 5.1.15 Energy efficiency Editor note: The text captured in RAN#109 is based on ITU-R WP5D and is for ITU-R evaluation; 3GPP internal evaluation metrics will be captured in future meetings.Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving [in relation to the traffic characteristics]. The requirement is defined as the relative energy [savings/consumption] (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case. Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies. The minimum requirements for Energy efficiency are summarized in Tables below. Table 5.1.15-1: The minimum requirements for Network Energy efficiency [Test environment(s)]**; Selected load case(s)*; The reference case; Relative energy [saving/consumption] (%). N. A; empty load; Fully loaded case*; [X1%]. [Dense urban-IC]**; [A%]; [X2%]. [Rural-IC]**; [B%]; [X3%]. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Table 5.1.15-2: The minimum requirements for Device Energy efficiency [Test environment(s)]**; Selected load case(s)*; The reference case; Relative energy [saving/consumption] (%). N. A; empty load; Fully loaded case; [Y1%]. [Dense urban-IC]**; [C%]; [Y2%]. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

Above all, we propose to reuse quantitative KPIs defined for ITU TPR for EE in RAN.

Proposal 3: Define energy efficiency as a quantitative KPI in 3GPP for both network and device.

- Use the same approach established in ITU-R.

Since ITU-R has updated the description for energy efficiency as below, we propose to update TR 38.914 accordingly.

Energy efficiency is an important metric of sustainability. Energy efficiency of the candidate RIT/SRIT can be characterized by evaluating network and device energy efficiency. Energy efficiency can be estimated as the relative energy consumption (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case. The RIT/SRIT shall have the capability to support low relative energy consumption in the selected load cases, considering at least the unloaded (L= 0%) and loaded scenario of [0% < L ≤ 15%] / [15% < L ≤ 30%] / [30% < L ≤ 50%] / [0% < L ≤ 30%], where L is load level, i.e., network resource utilization ratio. Proponents should report the evaluated energy efficiency values and the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies. Proponents are encouraged to describe mechanisms of the RIT/SRIT that improve the support of low energy consumption operation for both network and device. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

Composite requirement

For composite requirement, 3GPP RAN agreed to define the following.

| Test environment | Data rate (Mbits/s) | Latency (ms) | Packet success probability | Number of users (TRxP) |
| --- | --- | --- | --- | --- |
| Dense Urban - Immersive Communication | DL = 30 Mbps/UL=10 Mbps | DL=10 ms / UL=30 ms (or DL+UL total 40 ms) | 99% | 6 |

An emerging use case for IC is XR services. Given that the composite requirement is also meant to characterize XR-like services, we suggest to distinguish between XR and traditional video services by demonstrating the multi-modal nature of XR. The current definition only specifies the DL or UL data rate for XR service, without specifying the detailed traffic model. To better characterize XR and make it clear that 3GPP technologies can support XR-like multi-modal data, we suggest to define a mixed traffic model that follows the DL and UL data rate.

Observation 1: Multi-modality (mixed traffic model of video, haptic, etc) is a nature of XR services that distinguishes them from traditional video services.

For immersive communication, QoS requirements are different among data streams. According to TR 22.856, the haptic stream has the QoS requirement  higher than 99% on reliability and less than 5ms on latency, while video stream has the QoS requirement 99% on reliability and around 10ms on latency. For 6G immersive communication, good immersive experience relies on all streams meeting their own QoS requirements. Based on above observations and analysis, we propose to define the traffic model as below:

| Traffic model | DL Video: 30Mbps@60FPS, reliability = 99%, PDB = 10ms Haptic: fixed 100 Byte/s, reliability = 99.9%, PDB=5ms, pareto distribution arrival |
| --- | --- |

Proposal 4: Define a multi-modal traffic model for composite requirement as in the table above, which comprises of video and haptic data.

For latency requirements within the composite requirement, we believe that when doing simulations, there needs to be a clear cap on the maximum latency on DL and UL, so it is better to clearly define this cap by specifying DL and UL latency requirement respectively. If we additionally define 40 ms in total, it is likely that different companies will use different DL and UL latency combination, making calibration and comparison of simulation results more challenging. In short, we prefer to only define the latency requirement as 10ms for DL and 30ms for UL as in the composite requirement.

Proposal 5: Only define latency requirement within composite requirement as 10ms for DL and 30ms for UL, i.e., removing the part of “(or DL+UL total 40 ms)”.

ISAC

For ISAC, we also need to define new KPIs to characterize the system performance. We suggest to define detection probability and false alarm rate, localization accuracy and velocity estimation accuracy as three KPIs, with their proposed definition listed below. Although 3GPP did not reach consensus on ISAC KPIs during RAN 109, ITU-R WP 5D already made some progress and our proposal is also aligned with the latest ITU progress.

Sensing-related capabilities are measured in terms of the following KPIs: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Estimation Accuracy: Velocity estimation accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

Furthermore, the following KPIs have been agreed in RAN1#123 meeting for UAV detection and tracking for 5G-A SI.

| Metric | Value |
| --- | --- |
| Missed detection Probability | 5% |
| False Alarm Probability Type 1 | 5% |
| False Alarm Probability Type 2 | 5% |
| Horizontal Positioning Accuracy | 10 m with confidence level 90% |
| Vertical Positioning Accuracy | 10 m with confidence level 90% |
| Velocity Accuracy | 5 m/s with confidence level 90% |

Considering factors including larger channel bandwidth, larger antenna array size, and more comprehensive sensing modes and methods, it is natural that 6GR sensing KPI requirements should be higher compared to 5G-A. Considering SA1 requirements and achievable performance, we suggest the following sensing performance requirements: detection probability 95%, false alarm probability 5%, horizontal localization accuracy 2m, vertical localization accuracy 5m, velocity accuracy 1m/s.

Proposal 6: Define detection probability and false alarm probability, localization accuracy and velocity estimation accuracy as sensing KPIs.

- Detection probability: 95%

- False alarm probability: 5%

- Horizontal localization accuracy: 2m

- Vertical localization accuracy: 5m

- Velocity accuracy: 1m/s

Positioning

Positioning is supported by NR and is expected to be supported by 6GR as well. The KPI for positioning is naturally the positioning accuracy. We believe it is critical to consider both horizontal and vertical position accuracy at the same time. There are some use cases demanding vertical positioning accuracy, such as emergency (911) services and positioning for UAVs. We propose reusing the definition of ITU for positioning KPI as shown below.

Positioning is the ability to estimate the position of connected devices. The positioning accuracy is the 90% point of the CDF of the device positioning error for both horizontal and vertical directions. Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude). The minimum requirements for the device positioning accuracy are TBD.

Based on ITU-R M.2160 and achievable positioning accuracy captured in TR 38.855 and TR 38.857, the target of the positioning accuracy is proposed as below

Proposal 7: Define both horizontal and vertical positioning accuracy.

  - Horizontal positioning accuracy: 20cm-1m (indoor), 3m (outdoor) for 90% of UEs

  - Vertical positioning accuracy: 5m for 90% of UEs

Peak data rate

During RAN #109 meeting, there were a lot of online and offline discussions trying to find a common understanding for the peak data rate for 6GR. Our view is that the peak data rate shall be at least 100 Gbps in DL and 50 Gbps in UL, which is supported by 32 companies including 11 operators from different regions in RAN #109 [13].

From the perspective of demands on data rate from potential 6G use cases, 3GPP SA1 TR 22.870 “Study on 6G Use Cases and Service Requirements” [8] identified holographic communication which demands '100 Gbps range or beyond' data rate to support. There has also been lively discussions on different UE types, such as CPEs for FWA that may demand a high data rate.

Observation 2: TR 22.870 “Study on 6G Use Cases and Service Requirements” has identified 6G use cases that demand a high data rate on 100 Gbps range or beyond.

The KPI on peak data rate also has another implication on available spectrum for 6G in the IMT industry. The peak data rate can be calculated by the peak spectral efficiency and the bandwidth, and thus, the change of the available bandwidth can be easily reflected by comparing the peak data rate from 5G and 6G. In this sense, defining moderately advanced KPI values for 6G peak data rate can place the IMT industry in a better position to advocate for quality spectrum, which will in turn benefit the whole industry. On the other hand, if the peak data rate is defined as less than 40 Gbps in DL or 20 Gbps in UL (2 times of IMT-2020), given that the peak spectral efficiency is already agreed to be 2 times, there could be negative impression that the available bandwidth for IMT technologies shrinks from 5G to 6G. For example, we can consider that the expected available 5G bandwidth is IMT-2020_peak_data_rate/IMT-2020_peak_spectral_efficiency=(20Gbps)/(30bit/s/Hz)=666.66MHz. If the gain of 6G peak data rate is 2 times of IMT-2020 peak data rate or less, it implies that the expected available 6G bandwidth is no larger than 666.66MHz.

Observation 3: If the peak data rate is defined as less than 40 Gbps in DL or 20 Gbps in UL (2 times of IMT-2020), there could be confusion or negative impression that the available bandwidth for IMT technologies shrinks from 5G to 6G.

We also observe that 3GPP NR technologies can already achieve quite high data rate in both DL and UL. For instance, DL peak data rate can be as high as 171.2 Gbps (in FR2) [5] and 64.6 Gbps (in FR2) for UL peak data rate. Thus, even if we set the KPI of peak data rate to be 100 Gbps in DL and 50 Gbps in UL, NR can already satisfy this goal. On the other hand, if we set a lower goal, it might create confusions why 6GR, with all the potential enhancements, cannot do as well as 5G NR.

Observation 4: If sufficient spectrum is assumed, peak data rates as 100 Gbps in DL and 50 Gbps in UL are achievable goals.

There were views from several operators that a KPI titled “peak data rate” might leave a wrong impression to the non-technical people and cause problems. Although we believe the main goal of 3GPP is to conduct high-quality work based on technical foundations, we provide one possible solution to this concern to be considered. As demonstrated in TR 37.910 (Table 5.2.1.1-1 and Table 5.2.1.2-1, copied below),

Table 5.2.1.1-1 NR DL peak data rate

| Duplexing | SCS [kHz] |  | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Aggregated peak data rate over 16 CCs (Gbit/s) | Required DL bandwidth to meet the requirement (MHz)1 | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | FR1 | 15 | 50 | 2.31~2.41 | 37.0~38.6 | 414~433 | 20 |
|  |  | 30 | 100 | 4.67~4.89 | 74.7~78.2 | 409~428 |  |
|  |  | 60 | 100 | 4.62~4.82 | 73.9~77.1 | 415~433 |  |
| TDD (DDDSU) | FR1 | 15 | 50 | 1.81 | 29.0 | 552 |  |
|  |  | 30 | 100 | 3.68 | 58.9 | 543 |  |
|  |  | 60 | 100 | 3.62 | 57.9 | 552 |  |
|  | FR2 (Nlayer=6) | 60 | 200 | 5.33 | 85.3 | 750 |  |
|  |  | 120 | 400 | 10.7 | 171.2 | 748 |  |
| TDD (DSUUD, S slot= 11DL:2GP:2UL) | FR1 | 15 | 50 | 1.32 | 21.1 | 757 |  |
|  |  | 30 | 100 | 2.69 | 43.0 | 745 |  |
|  |  | 60 | 100 | 2.64 | 42.3 | 757 |  |
|  | FR2 (Nlayer=6) | 60 | 200 | 3.86 | 61.8 | 1036 |  |
|  |  | 120 | 400 | 7.81 | 125.0 | 1024 |  |
| TDD (DSUUD, S slot= 6DL:2GP:6UL) | FR1 | 15 | 50 | 1.13 | 18.1 | 885 |  |
|  |  | 30 | 100 | 2.30 | 36.8 | 870 |  |
|  |  | 60 | 100 | 2.26 | 36.2 | 885 |  |
|  | FR2 (Nlayer=8) | 60 | 200 | 4.38 | 70.1 | 913 |  |
|  |  | 120 | 400 | 8.76 | 140.2 | 913 |  |
| NOTE 1: The value only indicates the required bandwidth to meet the DL peak data rate. It is not necessarily supported as NR Transmission bandwidth. |  |  |  |  |  |  |  |

Table 5.2.1.2-1 NR UL peak data rate

| Duplexing | SCS [kHz] |  | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Aggregated peak data rate over 16 CCs (Gbit/s) | Required UL bandwidth to meet the requirement (MHz)1 | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | FR1 | 15 | 50 | 1.12~1.18 | 17.9~18.9 | 424~446 | 10 |
|  |  | 30 | 100 | 2.28~2.39 | 36.5~38.2 | 418~439 |  |
|  |  | 60 | 100 | 2.27~2.38 | 36.3~38.1 | 420~441 |  |
| TDD (DDDSU) + SUL | FR1 | 15 | 50 | 1.12~1.18 | 17.9~18.9 | 424~446 |  |
|  |  | 30 | 100 | 2.28~2.39 | 36.5~38.2 | 418~439 |  |
|  |  | 60 | 100 | 2.27~2.38 | 36.3~38.1 | 420~441 |  |
| TDD (DSUUD, S slot =11DL:2GP:2UL) | FR1 | 30 | 100 | 1.06 | 17.0 | 943 |  |
|  |  | 60 | 100 | 1.05 | 16.8 | 952 |  |
|  | FR2 | 60 | 200 | 1.91 | 30.6 | 1047 |  |
|  |  | 120 | 400 | 3.85 | 61.6 | 1039 |  |
| TDD (DSUUD, S slot =6DL:2GP:6UL) | FR1 | 30 | 100 | 1.05 | 16.8 | 952 |  |
|  |  | 60 | 100 | 1.04 | 16.6 | 962 |  |
|  | FR2 | 60 | 200 | 2.02 | 32.3 | 990 |  |
|  |  | 120 | 400 | 4.04 | 64.6 | 990 |  |
| NOTE 1: The value only indicates the required bandwidth to meet the DL peak data rate. It is not necessarily supported as NR Transmission bandwidth. |  |  |  |  |  |  |  |

It can be seen that in 3GPP self-evaluation for 5G, aggregated peak data rate over 16CCs are provided that can definitely satisfy the requirements. However, 16 CC aggregation does not reflect the real deployment in many cases. Alternatively, there is a metric on peak data rate per CC calculated. This metric has much lower risk of being misunderstood/misinterpreted by non-technical people, and thus, it is also possible to consider defining a “peak data rate per CC”. This metric can be reported to ITU-R WP5D if it is hard to reach consensus on the peak data rate aggregated for all CCs.

By simple calculation, the peak data rate per CC in FR1 should be up to 4 * 2 = 8 times larger than 5G value (4 times maximum bandwidth per CC, 2 times peak SE), i.e. 30 bit/s/Hz * 2 * 400 MHz = 24 Gbps for DL and 15 bit/s/Hz * 2 * 400 MHz = 12 Gbps for UL. Here, 30 bit/s/Hz is the IMT-2020 requirement and used as baseline, while 5G NR can already achieve higher peak spectral efficiency (see Table 5.1.1.1-1, Table 5.1.1.1-2, Table 5.1.1.2-1 and Table 5.1.1.2-2 in TR 37.910).

Proposal 8: Define peak data rates as 100 Gbps in DL and 50 Gbps in UL.

- Alternatively, define “peak data rate per CC” and adopt 24 Gbps for DL and 12 Gbps for UL in FR1 (8 times of NR peak data rate per CC by simple calculation). Follow the same logic for FR2 depending on future RAN1 agreement on maximum bandwidth per CC.

AI

Although RAN #109 did not reach consensus on the text or definition of AI-related capabilities, ITU-R WP 5D has made significant progress during its 50th meeting in Geneva. Based on the ITU progress, we suggest to define the AI-related capability as a qualitative one with the following definition:

AI-related capabilities AI-related capabilities refer to the ability to provide certain functionalities throughout the network to support AI enabled applications, including. - 6GR facilitating AI applications i.e. “Network for AI”. - AI to improve 6GR performance, i.e. “AI for network”. AI-related capabilities refer to the ability to support mechanisms and/or signaling/ related to AI functionalities. This should include support for at least the following: - Data collection - Distributed processing - Distributed learning - AI model training - AI computing - AI model inference

Proposal 9: Define AI-related capabilities as in the table above.

Latency for infrequent small packets

In TR 38.913, there is a clause on latency for infrequent small packets:

7.6 Latency for infrequent small packets For infrequent application layer small packet/message transfer, the time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point at the mobile device to the radio protocol layer 2/3 SDU egress point in the RAN, when the mobile device starts from its most "battery efficient" state. For the definition above, the latency shall be no worse than 10 seconds on the uplink for a 20 byte application packet (with uncompressed IP header corresponding to 105 bytes physical layer) measured at the maximum coupling loss (MaxCL) of 164dB. Analytical evaluation is the baseline evaluation methodology and system level evaluation can be considered if needed.

This corresponds to small data transmission (SDT) that is useful for transmitting small data packets in INACTIVE mode. We believe that this is also a highly relevant feature to be considered for 6GR, and thus, this KPI from TR 38.913 is proposed to be retained, with detailed values FFS.

Proposal 10: Retain KPI on “latency for infrequent small packets” in TR 38.913 and further study the values.

- Text proposals for TR 38.914

<start of change 1>

### 5.1.1	Peak data rate

Peak data rate is the theoretical maximum achievable data rate under ideal conditions, which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands, then the total peak data rate is

R=i=1Q  Wi × SEpi

where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is 100 Gbit/s.

–	Uplink peak data rate is 50 Gbit/s.

The minimum requirements for peak data rate per carrier for FR1 are as follows:

–	Downlink peak data rate is 24 Gbit/s.

–	Uplink peak data rate is 12 Gbit/s.

### 5.1.2	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is 60 bit/s/Hz.

–	Uplink peak spectral efficiency is 30 bit/s/Hz.

<end of change 1>

<start of change 2>

### 5.1.18	AI

AI-related capabilities refer to the ability to provide certain functionalities throughout the network to support AI enabled applications, including.

-	6GR facilitating AI applications i.e. “Network for AI”.

-	AI to improve 6GR performance, i.e. “AI for network”.

AI-related capabilities refer to the ability to support mechanisms and/or signalling related to AI functionalities. This should include support for at least the following:

-	Data collection

-	Distributed processing

-	Distributed learning

-	AI model training

-	AI computing

-	AI model inference

<end of change 2>

<start of change 3>

### 5.1.11	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

-	Stationary: 0 km/h

-	Pedestrian: 0 km/h to 10 km/h

-	Vehicular: 10 km/h to 120 km/h

-	High speed vehicular: 120 km/h to 500 km/h

-	Airplane: 500 km/h to 1000/1200 km/h

Table 5.1.11-1 defines the mobility classes that shall be supported in the respective [test environments].

Table 5.1.11-1: Mobility classes

|  | [Test environments] for IC |  |  |
| --- | --- | --- | --- |
|  | Indoor Hotspot – IC | Dense Urban – IC | Rural – IC |
| Mobility classes supported | Stationary, Pedestrian | Stationary, Pedestrian, Vehicular | Pedestrian, Vehicular, [High speed vehicular] |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 5.1.11-2. This assumes the user is moving at the maximum speed in that mobility class in each of the [test environments].

Table 5.1.11-2: Traffic channel link data rates normalized by bandwidth

| [Test environment] | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – IC | 2.25 | 10 |
| Dense Urban – IC | 1.68 | 30 |
| Rural – IC | 0.88~1.2 | 120 |
|  | 0.495~0.675 | 500 |

-

<end of change 3>

<start of change 4>

### 5.1.14	Positioning

Positioning is the ability to estimate the position of connected devices.

The positioning accuracy is the 90% point of the CDF of the device positioning error for both horizontal and vertical directions.

Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude).

The minimum requirements for the device positioning accuracy for various [test environments] are summarized in Table 5.1.14-1.

Table 5.1.14-1: Positioning accuracy

| [Test Environment] | Horizontal Accuracy | Vertical Accuracy |
| --- | --- | --- |
| Indoor Factory - ISAC | 20cm-1m | 5m |
| Urban Macro – ISAC | 3m | 5m |

<end of change 4>

<start of change 5>

### 5.1.16	Composite requirement

Editor note: The text captured in RAN#109 is based on ITU-R WP5D and is for ITU-R evaluation; 3GPP internal evaluation metrics will be captured in future meetings.

The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP.

This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface.

The minimum performance requirement of composite requirement is summarized in Table 5.1.16-1. The traffic model for the downlink and/or uplink can be a mixed traffic model comprising of more than one type of traffics (such as video and haptic) that add up to the total data rate.

Table 5.1.16-1: Composite requirement

| [Test environment] | Data rate (Mbit/s) | Latency (ms) | Packet success probability | Number of users (TRxP) |
| --- | --- | --- | --- | --- |
| Dense Urban - Immersive Communication | DL = 30 Mbps/UL=10 Mbps | DL=10 ms / UL=30 ms | 99% | 6 |

### 5.1.17	Sensing

Sensing-related capabilities are measured in terms of the following technical performance requirements:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error.

•	Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

The minimum requirements for sensing related capabilities are summarized in Table 5.1.17-1.

Table 5.1.17-1 Sensing related capabilities

| Requirement |  | Test Environment |  |
| --- | --- | --- | --- |
|  |  | Indoor factory-ISAC | Urban Macro-ISAC |
| Detection Probability |  | 95% | 95% |
| False Alarm Probability |  | 5% | 5% |
| Localization accuracy | Horizontal | 2m | 2m |
|  | Vertical | 5m | 5m |
| Velocity accuracy |  | 1m/s | 1m/s |

<end of change 5>

<start of change 6>

### 5.1.15	Energy efficiency

Editor note: The text captured in RAN#109 is based on ITU-R WP5D and is for ITU-R evaluation; 3GPP internal evaluation metrics will be captured in future meetings.Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving [in relation to the traffic characteristics].

The requirement is defined as the relative energy [savings/consumption] (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies.

The minimum requirements for Energy efficiency are summarized in Tables below.

Table 5.1.15-1: The minimum requirements for Network Energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy [saving/consumption] (%) |
| --- | --- | --- | --- |
| N. A | empty load | Fully loaded case* | [X1%] |
| [Dense urban-IC]** | [A%] |  | [X2%] |
| [Rural-IC]** | [B%] |  | [X3%] |
| * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation |  |  |  |

Table 5.1.15-2: The minimum requirements for Device Energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy [saving/consumption] (%) |
| --- | --- | --- | --- |
| N. A | empty load | Fully loaded case | [Y1%] |
| [Dense urban-IC]** | [C%] |  | [Y2%] |
| * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation |  |  |  |

Energy efficiency is an important metric of sustainability. Energy efficiency of the candidate RIT/SRIT can be characterized by evaluating network and device energy efficiency.

Energy efficiency can be estimated as the relative energy consumption (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

The RIT/SRIT shall have the capability to support low relative energy consumption in the selected load cases, considering at least the unloaded (L= 0%) and loaded scenario of [0% < L ≤ 15%] / [15% < L ≤ 30%] / [30% < L ≤ 50%] / [0% < L ≤ 30%], where L is load level, i.e., network resource utilization ratio.

Proponents should report the evaluated energy efficiency values and the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies.

Proponents are encouraged to describe mechanisms of the RIT/SRIT that improve the support of low energy consumption operation for both network and device.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

It is a quantitative KPI.

<end of change 6>

<start of change 7>

### 5.X	Latency for infrequent small packets

For infrequent application layer small packet/message transfer, the time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point at the mobile device to the radio protocol layer 2/3 SDU egress point in the RAN, when the mobile device starts from its most "battery efficient" state.

For the definition above, the latency shall be no worse than [10] seconds on the uplink for a [20] byte application packet (with uncompressed IP header corresponding to [105] bytes physical layer) measured at the maximum coupling loss (MaxCL) of [164]dB.

Analytical evaluation is the baseline evaluation methodology and system level evaluation can be considered if needed.

<end of change 7>

- Summary

Proposal 1: Regarding the KPI for overall coverage, MaxCL defined in section 7.10 in TR 38.913 is reused as the metric.

- For DL, 154 dB can be considered as the starting point for both eMBB and IoT service.

- For UL, further alignment on UE configuration is expected.

Proposal 2: For “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, no quantitative KPI is defined in RAN-level.

- For the assumption on carrier frequency, 4 GHz is considered as the reference. Other values, e.g., 3.5 GHz and 2.6 GHz can be considered additionally.

Proposal 3: Define energy efficiency as a quantitative KPI in 3GPP for both network and device.

- Use the same approach established in ITU-R.

Proposal 4: Define a multi-modal traffic model for composite requirement as in the table above, which comprises of video and haptic data.

Proposal 5: Only define latency requirement within composite requirement as 10ms for DL and 30ms for UL, i.e., removing the part of “(or DL+UL total 40 ms)”.

Proposal 6: Define detection probability and false alarm probability, localization accuracy and velocity estimation accuracy as sensing KPIs.

- Detection probability: 95%

- False alarm probability: 5%

- Horizontal localization accuracy: 2m

- Vertical localization accuracy: 5m

- Velocity accuracy: 1m/s

Proposal 7: Define both horizontal and vertical positioning accuracy.

  - Horizontal positioning accuracy: 20cm-1m (indoor), 3m (outdoor) for 90% of UEs

  - Vertical positioning accuracy: 5m for 90% of UEs

Proposal 8: Define peak data rates as 100 Gbps in DL and 50 Gbps in UL.

- Alternatively, define “peak data rate per CC” and adopt 24 Gbps for DL and 12 Gbps for UL in FR1 (8 times of NR peak data rate per CC by simple calculation). Follow the same logic for FR2 depending on future RAN1 agreement on maximum bandwidth per CC.

Proposal 9: Define AI-related capabilities as in the table above.

Proposal 10: Retain KPI on “latency for infrequent small packets” in TR 38.913 and further study the values.

- References

- RP-240823 Additional Considerations for 6G Timeline Source: TSG Chairs

- RP-250810 “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- RP-243202	Liaison Statement to External Organizations Minimum requirements related to technical performance for IMT-2030 radio interface(s), source ITU-R WP5D, 3GPP TSG#106, December 2024

- ITU-R WP5D 563 Report on the 48th meeting of Working Party 5D (Geneva, 4-13 February 2025) Chair, WP 5D

- 3GPP RAN, TR 37.910, “Study on self evaluation towards IMT-2020 submission”

- RP-250819 Moderator's summary for IMT-2030 TPRs, Moderator (CMCC)

- Report ITU-R M.2412-0 “Guidelines for evaluation of radio interface technologies for IMT-2020”

- 3GPP SA1, TR 22.870, “Study on 6G Use Cases and Service Requirements”

- RP-251825 Reply LS to ITU-R WP5D =RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN

- ITU-R Document 5D/TEMP/345-R1: Working Document Towards A Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]

- RP-252802 Moderator's summary for RAN led 6G SI: Key performance indicators, Moderator (CMCC)

- RP-252946 LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s)

- RP-252041 Views on IMT-2030 TPRs, ZTE Corporation, T-Mobile USA, SoftBank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Charter Communications, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH, Sanechips, InterDigital, VIAVI, Fujitsu

- RP-251881, New SID: Study on 6G Radio.