3GPP TSG RAN Meeting #110												RP-253281

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.0

Source:	Huawei, HiSilicon, CAICT, China Telcom, China Unicom, KT Corp., LG Uplus，SK Telecom, CATT, CICTCI

Title:	Discussion on 6G ISAC deployment scenarios

Document for:	Discussion and decision

# Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the scenario and relevant key assumptions in the scenario, the objective of this SID includes:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- …...

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure subsequent gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

…..

In this contribution, the considerations and views on some of the 6G ISAC scenarios and assumptions are discussed.

# 6G deployment scenario regarding the 6G ISAC

In recommendation ITU-R M.2160-0, integrated sensing and communication (ISAC) is defined as one of the key usage scenarios of IMT-2030. Its typical use cases include navigation, activity detection and movement tracking, environmental monitoring, and provision of sensing data/information of the surroundings for AI, XR, and digital twin applications[2]. The study[3] has identified multiple sensing-related use cases that demonstrate significant potential for enhancing network performance and enabling new service paradigms.

In 6G, the following services should be prioritized for study and meeting their requirements, considering the potential for huge markets.[4]

- Target detection

- Digital twin services enabled by Environment Object Reconstruction

In practical deployment, it is cost-effective that operators can utilize the communication network to provide ISAC services. Therefore, it is important to reflect needs on ISAC evaluation into related deployment scenarios.

In case of the indoor and outdoor target detection and the environment object reconstruction in practice, the indoor factory, urban grid and highway shall be considered. A generic mapping of the use cases to the scenario is provided in Table 1.

Table 1 Mapping of the ISAC use cases and deployment scenarios

| Use cases Scenarios | Target detection | Digital twin services enabled by Environment Object Reconstruction | Health monitoring services |
| --- | --- | --- | --- |
| Indoor hotspot | Human |  | Human |
| Indoor Factory | AGV |  |  |
| Highway | Vehicle |  |  |
| Urban Grid | Vehicle, human | buildings |  |

In RAN#109 meeting, there are multiple inputs on ISAC deployments and it is important that new scenarios can get treated at RAN#110. In this contribution, we provide the details of the deployment and scenario configuration for further discussion.

## Urban Grid scenario

TR 37.885 defines an urban grid scenario for V2X communication study. In R19 channel modelling, this urban grid deployment has been designed for integrated sensing and communication purpose as illustrated in Figure 2.1-1 and Figure 2.1-2[5]. In these two figures there are 9 buildings (the green blocks in the figure) with four lanes and two sidewalks in between, which can deploy the vehicle and human as the sensing targets or sensing users. The buildings are extended to a 3D size of 413m x 230m x 20m in order to model the environment object properly in practice. With the 3D building dropped, the base station is then dropped in the middle of the building edge with 25m height to simulate the practical site at urban scenario. This scenario will be noise-limited and/or interference-limited, using macro TRxPs. The macro BSs are deployed with ISD of 250m or 500 m.

With the 3D practical deployment, the environment object (EO) can provide the environment information to enable the RAN WG-level studies such as the environmental reconstruction, NLOS detection and tracking, sensing assisted communication and positioning.

The urban macro deployment scenario focuses on scenario of highly densely deployed vehicles placed in urban area. It could cover a scenario where freeways lead through an urban grid. The main KPI evaluated under this scenario are continuous and ubiquitous coverage, energy saving, positioning performance, sensing-related performance and AI-related performance in high network load and high UE density scenarios.

Figure 2.1-1

Sketch of hexagonal site layout with urban grid in 500m ISD

Figure 2.1-2

Sketch of hexagonal site layout with urban grid in 250m ISD

Therefore, this scenario deployment shall be considered for 6G and the values or assumptions are described as Table 2.

Table 2 Attributes of Urban Grid scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4GHz, Around 7GHz |
| System bandwidth | Around 4GHz: 300MHz, Around 7GHz: 400MHz NOTE1,2 |
| Layout | Single layer with hexagon cell |
| ISD | 500m see FIGURE 2.1-1 250m see FIGURE 2.1-2 |
| BS antenna elements | Around 4GHz: Tx: Up to 512Tx Rx: Up to 512 Rx Around 7GHz: Tx: Up to 2048Tx Rx: Up to 2048 Rx |
| UE antenna elements | Around 4GHz: Tx: Up to [8] Tx Rx: Up to [8] Rx Around 7GHz: Tx: Up to [16] Tx Rx: Up to [16] Rx |
| User distribution and UE speed | - Pedestrian UE dropping using equal space along the sidewalk with a fixed inter-pedestrian X m dropped - Total number of pedestrian UEs is 500 - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/500’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). For example, if the pedestrian UEs are dropped in ‘14’ road grids, the inter-pedestrian UE distance (m) is ‘36.344’. - Companies should explain how many road grids (i.e., ‘N”) are assumed in the evaluation. - Pedestrian UE speed is 3 km/h |
| Sensing target distribution and Sensing target speed | Vehicle is up to 120km/h. how many sensing targets from vehicle distribution per TR 37.885 depend on the evaluation methodology. |
| Environment Objects when sensing enabled | At least including 4 walls modelled per building of size 413m x 230m x 20m and/or other sizes, e.g. 40m x 20m x 30m. How many buildings and building types depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	The vehicle type is type 1/2 (passenger vehicle) and/or type 3 (truck/bus)

Proposal 1: Urban Grid shall be included as deployment scenario for 6G ISAC evaluation.

Proposal 2: The values or assumptions in Table 2 and notes are proposed to be adopted into the TR 38.914.

## Highway scenario

The highway is a valuable scenario for high speed vehicle communication and sensing. TR37.885 defines the highway deployment and scenario. The vehicle can be either connected or unconnected. Some of its attributes are listed in the following table.

Table 3 Attributes of Highway

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 7GHz, Around 4GHz |
| System bandwidth | Around 7GHz: Up to 400MHz Around 4GHz: Up to 300MHz |
| Layout | Single layer with hexagon cell |
| ISD | Around 4/7GHz: ISD=1732m, see Figure 2.2-1 |
| BS antenna elements | Around 4GHz: Tx: Up to 512Tx Rx: Up to 512 Rx Around 7GHz: Tx: Up to 2048 Tx Rx: Up to 2048Rx |
| UE antenna elements | Around 4GHz: Tx: Up to [8] Tx Rx: Up to [8] Rx Around 7GHz: Tx: Up to [16] Tx Rx: Up to [16] Rx |
| Vehicle distribution | Randomly distributed in the middle of each lane. The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is set to be the maximum between 2m and a value generated by a variable following exponential distribution with a mean of 2 * speed m. |
| Vehicle size | Size: 5m x 2m x 1.6m Antenna height 1.6m |
| User distribution and UE speed | 100% vehicle Speed is 140km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

Figure 2.2-1

Hex topology for around 7GHz and 4GHz with ISD=1732m

In realistic, the highway is a usual scenario for automotive vehicle detection and tracking for road safety. Furthermore, in R19 ISAC channel modelling SI, the highway is captured as an essential scenario for high-speed vehicle detection and tracking.

Proposal 3:  The highway shall be included as 6G deployment scenario for ISAC evaluation.

Proposal 4: The values or assumptions in Table 3 and notes are proposed to be adopted into TR 38.914.

## Indoor factory

The InF scenario focuses on factory halls of varying sizes and with varying levels of density of "clutter", e.g. machinery, assembly lines, storage shelves, etc. The key characteristics of this deployment scenario are clutter type, Clutter density, height of Tx and Rx and LOS/NLOS. The InF scenario is classified further into InF-SL, InF-DL, InF-SH, InF-DH and InF-HH based on a) density of the clutter (S-sparse and D-dense) and b) the height of Tx and Rx (L- low and H-high).

In addition, the indoor factory deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor supporting High-Reliability-Low-Latency communication, positioning and sensing.

Figure 2.3-1

Layout for Indoor Factory

Table 4 Attributes of Indoor Factory

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7 GHz Around 30 GHz Around 4GHz+Around 7GHz Note: The same layout is applied to the case of a single carrier frequency or the case of X+Y GHz. |
| Aggregated system bandwidth | Around 4 GHz: Up to 300MHz (DL+UL) Around 7 GHz: Up to 400MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) |
| Layout | Single layer - Indoor floor (Factory) |
| ISD | 50m (Equivalent to 18 TRxPs per 300m x 150m) |
| BS antenna elements | Around 4 GHz: Up to 128 Tx and Rx antenna elements Around 7 GHz: Up to 512 Tx and Rx antenna elements Around 30 GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements | Around 4 GHz: Up to [4-8] Tx and Rx antenna elements Around 7 GHz: Up to [8-16] Tx and Rx antenna elements Around 30GHz: Up to [32] Tx and Rx antenna elements Note: For around 30 GHz, with multi-panel UE, it is the number of simultaneously used antenna elements. |
| User distribution and UE speed | HRLLC: - Profile 1 100% Indoor: 3km/h 10 users per TRxP - Profile 2 (Optional, stationary machine) 100% Indoor: 0 km/h Positioning: 100% Indoor: 3km/h Sensing Targets: Human, AGV - 100% Indoor Sensing target assumptions as in Table 7.9.1-3 or Table 7.9.1-4 (TR. 38.901) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

Proposal 5: The indoor factory shall be included as 6G deployment scenario for ISAC evaluation.

Proposal 6: The values or assumptions in Table 4 and notes are proposed to be adopted into TR 38.914.

As discussed above, the text proposal is provided in section 5 for the urban grid, highway and indoor factory scenario.

Proposal 7: Adopt the text proposal in section 5 for urban grid, highway and indoor factory for deployment scenario in TR 38.914.

# Conclusion

In conclusion, the considerations on the 6G ISAC deployment and scenarios are discussed and the proposals are given as follows:

Proposal 1: Urban Grid shall be included as deployment scenario for 6G ISAC evaluation.

Proposal 2: The values or assumptions in Table 2 and notes are proposed to be adopted into the TR 38.914.

Proposal 3:  The highway shall be included as 6G deployment scenario for ISAC evaluation.

Proposal 4: The values or assumptions in Table 3 and notes are proposed to be adopted into TR 38.914.

Proposal 5: The indoor factory shall be included as 6G deployment scenario for ISAC evaluation.

Proposal 6: The values or assumptions in Table 4 and notes are proposed to be adopted into TR 38.914.

Proposal 7: Adopt the text proposal in section 5 for urban grid, highway and indoor factory for deployment scenario in TR 38.914.

# Reference

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

- 3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

- RP-252133, Consideration on supporting ISAC services for 6G, RAN#109 meeting.

- R1-2505006, CR to introduce channel model for ISAC.

# TP for 38.914

In section 4 Deployment scenarios in TR38.914, the text proposal for section 4.11, section 4.12 and section 4.13 are provided as follow:

-------------------------------------------------Start of the text proposal ------------------------------------------------

4.11	 Urban grid

The urban macro deployment scenario focuses on scenario of highly densely deployed vehicles placed in urban area. It could cover a scenario where freeways lead through an urban grid. The main KPI evaluated under this scenario are continuous and ubiquitous coverage, energy saving, positioning performance and sensing-related performance in high network load and high UE density scenarios.

Some of its attributes are listed in Table 4.11.

Table 4.11 Attributes for Urban Grid scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4GHz, Around 7GHz |
| System bandwidth | Around 4GHz: 300MHz, Around 7GHz: 400MHz NOTE1,2 |
| Layout | Single layer with hexagon cell |
| ISD | 500m 250m |
| BS antenna elements | Around 4GHz: Tx: Up to 512 Tx Rx: Up to 512Rx Around 7GHz: Tx: Up to 2048Tx Rx: Up to 2048 Rx |
| UE antenna elements | Around 4GHz: Tx: Up to [8] Tx Rx: Up to [8] Rx Around 7GHz: Tx: Up to [16] Tx Rx: Up to [16] Rx |
| User distribution and UE speed | - Pedestrian UE dropping using equal space along the sidewalk with a fixed inter-pedestrian X m dropped - Total number of pedestrian UEs is 500 - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/500’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). For example, if the pedestrian UEs are dropped in ‘14’ road grids, the inter-pedestrian UE distance (m) is ‘36.344’. - Companies should explain how many road grids (i.e., ‘N”) are assumed in the evaluation. - Pedestrian UE speed is 3 km/h |
| Sensing target distribution and Sensing target speed | Vehicle is up to 120km/h. how many sensing targets from vehicle distribution per TR 37.885 depend on the evaluation methodology. |
| Environment Objects when sensing enabled | At least including 4 walls modelled per building of size 413m x 230m x 20m and/or other sizes, e.g. 40m x 20m x 30m. How many buildings and building types depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	The vehicle type is type 1/2 (passenger vehicle) and/or type 3 (truck/bus)

4.12 Highway

The highway deployment scenario focuses on scenario of vehicles placed in highways with high speeds. The main KPIs evaluated under this scenario would be reliability/availability under high speeds/mobility (and thus frequent handover operations) and sensing-related performance.

Some of its attributes are listed in Table 4.12.

Table 4.12 Attributes for highway scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 7GHz, Around 4GHz |
| System bandwidth | Around 7GHz: Up to 400MHz Around 4GHz: Up to 300MHz |
| Layout | Single layer with hexagon cell |
| ISD | Around 4/7GHz: ISD=1732m |
| BS antenna elements | Around 4GHz: Tx: Up to 512 Tx Rx: Up to 512 Rx Around 7GHz: Tx: Up to 2048 Tx Rx: Up to 2048 Rx |
| UE antenna elements | Around 4GHz: Tx: Up to [8] Tx Rx: Up to [8] Rx Around 7GHz: Tx: Up to [16] Tx Rx: Up to [16] Rx |
| Vehicle distribution | Randomly distributed in the middle of each lane. The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is set to be the maximum between 2m and a value generated by a variable following exponential distribution with a mean of 2 * speed m. |
| Vehicle size | Size: 5m x 2m x 1.6m Antenna height 1.6m |
| User distribution and UE speed | 100% vehicle Speed is 140km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

4.14 Indoor Factory

The InF scenario focuses on factory halls of varying sizes and with varying levels of density of "clutter", e.g. machinery, assembly lines, storage shelves, etc. The key characteristics of this deployment scenario are clutter type, Clutter density, height of Tx and Rx and LOS/NLOS. The InF scenario is classified further into InF-SL, InF-DL, InF-SH, InF-DH and InF-HH based on a) density of the clutter (S-sparse and D-dense) and b) the height of Tx and Rx (L- low and H-high).

In addition, the indoor factory deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor supporting High-Reliability-Low-Latency communication, positioning and sensing.

Some of its attributes are listed in Table 4.14-1 and 4.14-2.

Table 4.14-1: Attributes for Indoor Factory (InF)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 4 GHz Around 7 GHz Around 30 GHz Around 4GHz+Around 7GHz Note: The same layout is applied to the case of a single carrier frequency or the case of X+Y GHz. |
| Aggregated system bandwidth NOTE2 | Around 4 GHz: Up to 300MHz (DL+UL) Around 7 GHz: Up to 400MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) NOTE3 |
| Layout | Single layer - Indoor floor (Factory) |
| ISD | 50m (Equivalent to 18 TRxPs per 300m x 150m) |
| BS antenna elements | Around 4 GHz: Up to 128 Tx and Rx antenna elements Around 7 GHz: Up to 512 Tx and Rx antenna elements Around 30 GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements | Around 4 GHz: Up to [4-8] Tx and Rx antenna elements Around 7 GHz: Up to [8-16] Tx and Rx antenna elements Around 30GHz: Up to [32] Tx and Rx antenna elements Note: For around 30 GHz, with multi-panel UE, it is the number of simultaneously used antenna elements. |
| User distribution and UE speed | HRLLC: - Profile 1 100% Indoor: 3km/h 10 users per TRxP - Profile 2 (Optional, stationary machine) 100% Indoor: 0 km/h Positioning: 100% Indoor: 3km/h Sensing Targets: Human, AGV - 100% Indoor Sensing target assumptions as in Table 7.9.1-3 or Table 7.9.1-4 (TR. 38.901) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range. A range of bands from 24.25 GHz – 52.6 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the aggregated system bandwidth used for either DL or UL via switching in time-domain.

Table 4.14-2: Attributes for Indoor Factory (InF)

| Parameters |  | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (high Tx, high Rx) |
| --- | --- | --- | --- | --- | --- | --- |
| Layout | Room size | Rectangular: 45000 m2 i.e. 300 x 150 m |  |  |  |  |
|  | Ceiling height | 5-25 m | 5-15 m | 5-25 m | 5-15 m | 5-25 m |
|  | Effective clutter height hc | < Ceiling height, 0-10 m |  |  |  |  |
|  | External wall and ceiling type | Concrete or metal walls and ceiling with metal-coated windows |  |  |  |  |
| Clutter type |  | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Any |
| Typical clutter size, dclutter |  | 10 m | 2 m | 10 m | 2 m | Any |
| Clutter density r (percentage of surface area occupied by clutter) |  | Low clutter density (<40%) | High clutter density (≥40%) | Low clutter density (<40%) | High clutter density (≥40%) | Any |
| BS antenna height |  | Clutter-embedded, i.e. the BS antenna height is below the average clutter height |  | Above clutter |  | Above clutter |
| UT location | LOS/NLOS | LOS and NLOS |  |  |  | 100% LOS |
|  | Height | Clutter-embedded |  |  |  | Above clutter |

-------------------------------------------------End of the text proposal ------------------------------------------------