3GPP TSG RAN Meeting #110                                                                                   RP-253283

Baltimore, USA, December 8-11, 2025

Agenda Item:	 8.2.1.3.1

Source:	Huawei, HiSilicon, China Unicom, VODAFONE

Title:	Consideration on spectrum requirements

Document for:	Discussion and Decision

# Introduction

In the RAN#108 meeting, a New SID: Study on 6G Radio was approved [1]. Its objectives include the following:

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

This contribution discusses spectrum requirements for 6G initial access, with a particular focus on how to support diverse spectrum allocations.

# Main Considerations

## Baseline bandwidth for initial access signals/channels

From a spectrum allocation perspective, the 6GR initial access design should primarily target typical spectrum allocations, while still maintaining basic functionality for non-typical narrow allocations (e.g., 3 MHz in FDD bands).

Initial access mainly involves signals/channels such as SSB, common PDCCH and PDSCH (SIB1), PUSCH (Msg3), etc. The bandwidth of these signals/channels has a fundamental impact on the coverage performance and network efficiency, and such an impact will take effect even in future releases of the whole generation, due to the existence of legacy UEs from 6G Day One.

Taking PBCH as an example, the NR PBCH occupies 20 RBs, which was not designed for very narrow channel bandwidths (e.g., <5 MHz @ 30 kHz SCS or <3 MHz @ 15 kHz SCS), but rather to ensure the overall system performance. For a carrier with a narrow channel bandwidth of 3 MHz, NR introduced puncturing-based solutions to avoid fundamental impact on initial access for other scenarios. Our initial evaluation indicates that limiting the NR PBCH to 3 MHz (e.g., 12 RB with 15 kHz SCS) results in over 3 dB coverage loss compared with 20 RBs, and the ~7 GHz and ~3.5 GHz co-site deployment requirement introduces approximately 10 dB additional gap. Potential extension of synchronization signal periodicity for energy saving in 6GR may further widen the coverage gap. To compensate such a coverage gap in narrow bandwidths, more time-domain resources are needed for PBCH (and SIB1) transmission (e.g., A coverage gap of about ~13 dB requires about 20 repetitions to be compensated).

Therefore, if the minimum spectrum allocation includes bandwidths smaller than 5MHz, it would significantly constrain the SSB design and degrade coverage performance, especially for ~7 GHz bands, which already experience higher propagation loss. In this case, more time-domain resources, such as repetitions or larger burst set, would be needed to compensate for the coverage gap, leading to an increase in always-on overhead and negatively impacts UE access latency and energy efficiency.

Based on the above considerations, we propose to support 6GR SSB design assuming at least 5 MHz @ 15 kHz SCS (FDD) and 10 MHz @ 30 kHz SCS (TDD) as baseline configurations, ensuring adequate coverage and performance for common spectrum allocations (e.g., especially at around 7 GHz).  Larger bandwidths (e.g., 20 MHz) can be further considered for bands with larger spectrum resources.

Proposal 1: The minimum bandwidth for initial access common signals/channels is 5 MHz for FDD with 15 kHz SCS, and 10 MHz for TDD with 30 kHz SCS.

## Support for minimum spectrum allocations

While typical spectrum allocations should drive the baseline design of common signals/channels bandwidth for initial access, it is also important to consider operators’ minimum spectrum allocations and certain irregular cases. Examples include at least:

- Very narrow channel bandwidths, e.g., 3 MHz in some FDD bands

The following agreement was made in RAN1 #123 meeting [3]:

Agreement

If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR,

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

- Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

As illustrated in Section 2.1, using Opt2 to cover very narrow minimum allocations (e.g., 3 MHz) would significantly limit the SSB design and hurt coverage and efficiency. Instead, Opt1 is more desirable to avoid such constraints and negative impact on performance like coverage, system overhead and energy efficiency. At the same time minimum spectrum allocations (e.g., 3 MHz) should also be supported via case-by-case designs. Solutions like puncturing-based or other tailored scalable designs can be further studied in RAN1.

Proposal 2:  Spectrum allocation(s) smaller than 5MHz FDD is/are supported based on the initial access common signals/channels for 5MHz FDD, but not necessarily optimized.

# Conclusions

In this contribution, we discussed spectrum requirements for 6G initial access, with particular emphasis on how to support operators’ minimum spectrum allocations while maintaining good coverage and efficiency for typical spectrum scenarios. Based on the considerations above, we propose the following:

Proposal 1: 
The minimum bandwidth for initial access common signals/channels is 5 MHz for FDD with 15 kHz SCS, and 10 MHz for TDD with 30 kHz SCS.

Proposal 2: 
Spectrum allocation(s) smaller than 5MHz FDD is/are supported based on the initial access common signals/channels for 5MHz FDD, but not necessarily optimized.

# Text Proposal for 38.914

-------------------------------------------Text Proposal----------------------------------------------------

### 5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

The minimum bandwidth for initial access common signals/channels is 5 MHz for FDD with 15 kHz SCS, and 10 MHz for TDD with 30 kHz SCS.

Spectrum allocation(s) smaller than 5MHz FDD is/are supported based on the initial access common signals/channels for 5MHz FDD, but not necessarily optimized.

# References

RP-251881, “New SID: Study on 6G Radio”, 3GPP TSG RAN meeting #108, Prague, Czech Republic, June 9-13, 2025.

RP-252882 TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements.

R1-2509291, FL summary#4 on overview of 6GR air interface, Moderator (NTT DOCOMO), Dallas, USA, Nov 17th-21st, 2025

R1-2508733, Overview of 6GR air interface, Huawei, HiSilicon, Dallas, USA, November 17 – 21, 2025